| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0423-v2 |
| --- | --- |

| Title: | Hypothetical Reference Decoder for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Specification text is proposed for Hypothetical Reference Decoder (HRD) for VVC as a starting point.

# Introduction

VVC WD [1] currently does not include Hypothetical Reference Decoder (HRD) buffer model.

VVC WD [1] already includes reference picture management including reference picture marking process. Additionally one syntax element related to SPS maximum decoder picture buffering is included. It is asserted that with the reference picture management included in VVC WD, the current state of VVC WD is amenable to add support for starting point HRD specification text.

In this document specification text is proposed as a starting point for HRD for VVC.

The proposed starting point text includes an Access Unit (AU) based HRD. It is asserted that additionally a Decoding Unit (DU)/ sub-picture based HRD may be added to VVC in future when the corresponding technologies mature. However, it is asserted that for many typical applications a simple AU based HRD model is useful and would be necessary.

Compared to HEVC HRD specification text, at high-level following modifications were made to the proposed starting point text for VVC.

- Specification text corresponding to VPS is not included as VVC WD does not currently include VPS.

- Specification text corresponding to BLA pictures is not included as VVC WD does not currently include BLA pictures.

- Operation point definition is adapted to VVC WD and thus not using a target layer identifier list.

- Specification text is updated to not include layer set, layer id list aspect.

- Specification text is updated to not include filler data NAL unit aspect as that NAL unit is not currently included in VVC WD.

- Specification text corresponding to decoding unit based/ sub-picture based operation is not included for now. It is expected that since decoding unit based operation can achieve low latency, this should be added in future.

- Specification text for RPS related aspects is modified to refer to method used in VVC WD.

- The conditions which use NAL unit type for conditional signalling are updated.

- Specification text for cropping related aspect is not currently included. It is asserted that it could be straightforward to include this aspect later.

# Proposal

The proposed text is provided in the accompanying document as a mark-up compared to VVC WD [1].

In v2 revision the proposed text also additionally shows changes compared to corresponding HEVC text.

# Conclusion

This document proposes starting point text for HRD for VVC. It is proposed to adopt the proposed text in the next version of VVC WD.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-M1001-v6, Versatile Video Coding (Draft 4), Marrakech, March 2019