| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0425-v2 |
| --- | --- |

| Title: | Simplification of CABAC Initialization Process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Dong, Amir Said, Vadim Seregin, Marta Karczewicz, | Tel: Email: | +1-408-533-9515 jiedong@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a simplified CABAC initialization process, where the initial probability states represent the probability in the linear domain and can be used for the arithmetic coding engine directly. The proposed simplification saves a logarithmic to linear domain mapping process using a 256-byte look-up table, while achieves BD-rate reduction -0.01% AI, -0.02% RA, -0.01% LDB, and 0.01% LDP.

# Introduction

In the current VVC, CABAC has a QP dependent initialization process invoked at the beginning of each slice. Given the initial value of luma QP for the slice, the initial probability state of a context model, denoted as preCtxState, is derived by (1) to (3),

m = slopeIdx × 5 – 45                                                               (1)

n = (offsetIdx << 3) – 16                                                            (2)

preCtxState = Clip3(0, 127, ((m × QP) >> 4) + n)                                     (3)

where slopeIdx and offsetIdx (both 4-bit integers from 0 to 15 inclusive) are packed into a single 8-bit initialization value, initVal (i.e., initVal=slopeIdx×16+offset). The variable initVal is predefined and stored for each context model. Eq. (3) means that preCtxState is modeled by a linear function of QP, with the slope approximately (m>>4) and the intersection n at QP = 0.

This is a legacy initialization process used in HEVC, where the output probability state preCtxState represents the probability in the logarithmic domain. However, as adopted in the Marrakesh meeting, the arithmetic coding engine in the current VVC uses linear probability representation. To fix this mismatch, a 128-entry look-up table (256 bytes the size), denoted as initStateIdxToState, is used in the current VVC for logarithmic to linear domain mapping, shown as below.

pStateIdx0 = initStateIdxToState[preCtxState] >> 4                                         (4)

pStateIdx1 = initStateIdxToState[preCtxState]                                             (5)

# Proposed method

The contribution proposes to make the probability state preCtxState represent the probability in the linear domain directly. By doing this, preCtxState only needs proper shifting operations before input to arithmetic coding engine, and the logarithmic to linear domain mapping as well as the 256-byte table is saved. The detailed operations are showed as below.

m = slopeIdx × 5 – 45                                                                 (6)

n = (offsetIdx << 3) + 7                                                                (7)

preCtxState = Clip3(1, 127, ((m × (QP – 32)) >> 4) + n)                                    (8)

pStateIdx0 = preCtxState << 3                                                        (9)

pStateIdx1 = preCtxState << 7                                                      (10)

# Experimental results

The proposed simplification method is integrated into VTM4.0 and tested under CTC. The initialization parameters are retrained to fit the new physical meanings of slope and offset. The average BD-rate reductions are shown in Tables 1 to 4, and more detailed results can be found in the attached spreadsheet.

Table 1 Simplified initialization process in AI

|  |  | All IntraMain10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM4.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.02% | 103% | 104% |
| Class A2 | -0.02% | 0.07% | -0.01% | 99% | 101% |
| Class B | -0.01% | -0.06% | 0.07% | 100% | 95% |
| Class C | 0.02% | -0.06% | -0.03% | 99% | 95% |
| Class E | -0.02% | 0.00% | -0.11% | 99% | 101% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 98% |
| Class D | 0.00% | 0.03% | -0.06% | 102% | 102% |
| Class F | 0.05% | -0.13% | -0.15% | 101% | 98% |

Table 2 Simplified initialization process in RA

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | -0.04% | 102% | 99% |  |
| Class A2 | -0.02% | -0.09% | -0.02% | 97% | 104% |  |
| Class B | -0.02% | -0.09% | -0.22% | 100% | 100% |
| Class C | -0.05% | -0.22% | -0.21% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.06% | 0.06% | -0.09% | 100% | 101% |
| Class F | -0.07% | -0.22% | -0.17% | 102% | 104% |

Table 3 Simplified initialization process in LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.08% | -1.08% | 99% | 105% |
| Class C | -0.04% | 0.48% | -0.32% | 99% | 105% |
| Class E | 0.04% | 0.45% | -0.39% | 101% | 110% |
| Overall | -0.01% | 0.24% | -0.65% | 100% | 106% |
| Class D | -0.03% | 0.15% | -1.45% | 101% | 107% |
| Class F | -0.29% | -0.33% | -0.19% | 103% | 105% |

Table 4 Simplified initialization process in LDP

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.00% | -0.13% | -0.72% | 101% | 95% |
| Class C | -0.03% | 0.07% | -0.91% | 100% | 93% |
| Class E | 0.10% | 0.04% | -0.23% | 101% | 102% |
| Overall | 0.01% | -0.02% | -0.66% | 101% | 96% |
| Class D | 0.00% | 0.60% | -0.13% | 101% | 95% |
| Class F | -0.08% | -0.02% | -0.11% | 103% | 95% |

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).