| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0427 |
| --- | --- |

| Title: | CE3-related: Harmonization between ISP and WAIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In VTM4.0, In this contribution, a harmonized scheme between Intra Sub-Partition (ISP) and Wide Angular Intra Prediction (WAIP) is proposed. Firstly, 1x64 and 64x1 partitions are disabled for ISP mode. Secondly, it is proposed that WAIP mode mapping is always performed at the TU level. It is reportedly shown that, the proposed method achieved 0.00% and XXX% luma BD-rates with around 100% and XXX% encoding time, and 100% and XXX% decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

Introduction

As shown in Figure 1, there are Planar, DC and 93 angular intra prediction modes used in VTM4.0 [1], wherein modes -1 ~ -14 and modes 67 ~ 80 are called Wide Angular Intra Prediction (WAIP) modes. WAIP modes was designed for non-square blocks. However, after the adoption of ISP, WAIP directions are sometimes applied at CU level (when ISP on and CU is non-square) while sometimes at TU level (when ISP off and TU is non-square). As a result, WAIP modes sometimes are applied to non-square blocks while sometimes applied to square blocks, which is not a consistent design. For example, as shown in Figure 2, current CU is non-square and it is split into 4 square sub-partitions. Since current CU is non-square, WAIP modes are applied to four sub-partitions of current CU even though they are all square blocks.

Figure 1: Intra prediction modes in VTM4.0

Figure 2: One non-square CU is split into 4 square sub-partitions

Proposed methods

Firstly, it is proposed to disable 1x64 or 64x1 sub-partitions when ISP mode is on. (Namely Method #1)

Secondly, it is proposed to always perform WAIP mode mapping at the TU level. When current TU is non-square block, WAIP mode mapping is performed. Otherwise, WAIP mode mapping is not performed. (Namely Method #2)

Experimental results

The proposed method is implemented on top of VTM4.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-M1010 [2]. Two tests are conducted to evaluate the performance of the proposed methods.

- Test 1: Method #1

- Test 2: Method #1 + Method #2

- Table 1: Coding performance of Test 1 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |  |  |  |  |  |
| Class E | 0.00% | 0.00% | 0.00% | 98% | 99% |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 97% |  |  |  |  |  |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 103% |  |  |  |  |  |

Table 2: Coding performance of Test 2 over VTM-4.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.02% | 98% | 99% |  |  |  |  |  |
| Class A2 | -0.01% | 0.06% | 0.01% | 101% | 102% |  |  |  |  |  |
| Class B | 0.01% | 0.03% | -0.03% | 101% | 103% |  |  |  |  |  |
| Class C | 0.00% | -0.04% | -0.01% | 101% | 99% |  |  |  |  |  |
| Class E | 0.02% | -0.01% | 0.07% | 98% | 99% |  |  |  |  |  |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class D | 0.01% | -0.05% | -0.12% | 102% | 99% |  |  |  |  |  |
| Class F | 0.02% | -0.04% | -0.06% | 100% | 101% |  |  |  |  |  |

Conclusion

In this contribution, a harmonized scheme between Intra Sub-Partition (ISP) and Wide Angular Intra Prediction (WAIP) is proposed. Firstly, 1x64 and 64x1 partitions are disabled for ISP mode. Secondly, it is proposed that WAIP mode mapping is always performed at the TU level. The proposed methods have no encoding and decoding time impact and achieves average XXX% and XXX% luma BD-rates on top of VTM4.0 under AI and RA configurations. It is proposed to adopt this proposal.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-M1010, Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).