| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0434 |
| --- | --- |

| Title: | Non-CE4: Unified context model of AMVR and Affine AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ciao Yang Po-Han Lin | Tel: Email: | yuciaoyang@fginnov.com PohanLin@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

This contribution proposes a unified context model method for adaptive motion vector resolution (AMVR) [1] and Affine adaptive motion vector resolution (Affine AMVR) [2]; In the current VTM-4.0, up to 4 context models in amvr_flag and 2 context models in amvr_precision_flag are supported and are selected according to inter_affine_flag; To reduce the number of context models, a unified context model in amvr_flag is proposed for both AMVR and Affine AMVR, instead of separate context set for AMVR and Affine AMVR. Compared to VTM4.0, 0.00%/-0.08%/-0.06% YUV-BD rate with 100/100% coding time change are observed in RA condition, and -0.01%/0.15%/-0.12% YUV-BD rate with 100/100% coding time change are observed in LB condition.

Introduction

In current VTM-4.0, the syntax element amvr_flag has a different context sets under AMVR and Affine AMVR. The context set and binarization of amvr_flag and amvr_precision_flag are described in Table 1 and Table 2.

Table 1. Context set and binarization for AMVR

| AMVR | Binarization |  |
| --- | --- | --- |
| Precision | amvr_flag (context index: 0, 1, 2) | amvr_precision_flag (context index: 3) |
| 1/4 | 0 |  |
| 1 | 1 | 0 |
| 4 | 1 | 1 |

Table 2. Context set and binarization for Affine AMVR

| Affine AMVR | Binarization |  |
| --- | --- | --- |
| Precision | amvr_flag (context index: 4) | amvr_precision_flag (context index: 5) |
| 1/4 | 0 |  |
| 1/16 | 1 | 0 |
| 1 | 1 | 1 |

In Affine AMVR, one context model is used for the amvr_flag. In AMVR, the context model for current amvr_flag is selected by context index increment (ctxInc), and it is derived by the amvr_flag of left block and the amvr_flag of above block, the equation is described as follows:

ctxInc = availability(Left) && amvr_flag(Left) + availability(Above) && amvr_flag(Above)

Proposed Unified Context Model Method

In current VTM-4.0, the context sets of amvr_flag are shown in Table 3. The syntax element inter_affine_flag indicates whether the coding unit is predicted in affine mode.

Table 3. Context model set of amvr_flag in VTM-4.0

| Syntax element | amvr_flag |
| --- | --- |
| Context index | inter_affine_flag ? 3 : (0, 1, 2) |

For the amvr_flag, two different context sets are used under different conditions, and the number of context models in different context sets are not equal. To simplify it, a unified context model of the syntax amvr_flag is proposed and shown in Table 4, one context model of amvr_flag instead of 4 context models is used. Therefore, the number of context model of amvr_flag can reduce from 4 to 1 and amvr_flag can share the same context model under AMVR and Affine AMVR.

Table 4. Proposed context model set of amvr_flag

| Syntax element | amvr_flag |
| --- | --- |
| Context index | 0 |

Experiment result

The proposed unified context model method is implemented on top of VTM-4.0 and the software is performed under common test conditions [3]. Table 5 shows the experimental results.

Table 5. Summary of experimental results (The proposed unified context model method over VTM-4.0 anchor)

|  | Random access Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over VTM-4.0 |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 | 0.00% |  | -0.07% |  | -0.04% | 100% | 100% |
| Class A2 | -0.02% |  | -0.13% |  | 0.02% | 100% | 100% |
| Class B | 0.02% |  | -0.07% |  | -0.07% | 100% | 100% |
| Class C | 0.01% |  | -0.07% |  | -0.12% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% |  | -0.08% |  | -0.06% | 100% | 100% |
| Class D | -0.01% |  | -0.07% |  | -0.08% | 100% | 99% |
| Class F | 0.00% |  | 0.01% |  | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  |  |  |  | Over VTM-4.0 |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.02% |  | -0.08% | 100% | 100% |
| Class C | -0.04% |  | 0.19% |  | -0.04% | 100% | 100% |
| Class E | 0.01% |  | 0.33% |  | -0.31% | 100% | 100% |
| Overall | -0.01% |  | 0.15% |  | -0.12% | 100% | 100% |
| Class D | -0.02% |  | 0.07% |  | -1.05% | 100% | 100% |
| Class F | -0.13% |  | 0.63% |  | -0.15% | 100% | 100% |

Conclusion

This contribution proposes a unified context model method for amvr_flag of AMVR and Affine AMVR. Experimental results show that 0.00%/-0.08%/-0.06% YUV-BD rate change in RA and -0.01%/0.15%/-0.12% YUV-BD rate change in LB over VTM-4.0 without any encoding/decoding time impact. It is observed that the proposed method shows that no significant impact on RA and LB conditions. It is suggested to adopt proposed unification method into the next VVC working draft.

Reference

- Y. Zhang, Y. Han, C.-C. Chen, C.-H. Hung, W.-J. Chien, M. Karczewicz, “CE4.3.3: Locally adaptive motion vector resolution and MVD coding,” JVET-K0357, Ljubljana, SI, July 2018.

- H. Liu, K. Zhang, L. Zhang, J. Xu, Y. Wang, P. Zhao, D. Hong, “CE2: Adaptive Motion Vector Resolution for Affine Inter Mode (Test 2.1.2),” JVET-M0246, Marrakech, MA, Jan. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, Marrakech, MA, Jan. 2018.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).