| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0453 |
| --- | --- |

| Title: | AHG16/Non-CE3: on chroma 2xN and Nx2 intra prediction in single tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao Karabutov Alexander Haitao Yang Jianle Chen | Tel: Email: | +86-571-28163645 yin.zhao@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution extends the “no chroma split for small CU” scheme in JVET-L0372 to avoid 2xN or Nx2 (N<=4) small-sized chroma intra prediction in single tree. In Test 1, when a split mode of node A leads to small-sized chroma blocks in child nodes, the luma block of the node A splits according to the split mode, while the chroma blocks do not further split. In Test 2, in P/B picture, the CUs covered by node A are further constrained to only use Inter mode or to only use Intra/IBC mode. If the region of node A are constrained with only Intra/IBC mode (i.e., an Intra region), the luma block of the node splits using the split mode while the chroma blocks do not further split; otherwise (i.e., an Inter region), the luma and chroma blocks of node A are still jointly split in a single tree. For dual-tree I picture, the CE3-2.1.1 is used to avoid small-sized chroma block. In single tree AI configuration, Y/U/V BD-rates of Test 1 (same as Test 2) are -0.02% / -0.22% / -0.46%. In CTC RA configuration, Y/U/V BD-rates of Test 1 are x.x% / x.x% / x.x%, and those of Test 2 are x.x% / x.x% / x.x%. In CTC LDB configuration, Y/U/V BD-rates of Test 1 are x.x% / x.x% / x.x%, and those of Test 2 are x.x% / x.x% / x.x%.

Introduction

In VTM4, chroma block size may be 2xN or Nx2. This may reduce the throughput of intra coded CUs. In JVET-L0372, a no chroma split for small CU scheme was proposed to avoid 2xN and Nx2 chroma blocks in single-tree I picture. More specifically, when a split mode of node A leads to small-sized chroma blocks in child nodes, the luma block of the node A splits according to the split mode, while the chroma blocks do not further split. By this method, in the single-tree AI configuration, negligible coding loss (0.08%/-0.35%/-0.18% in Y/U/V) was observed while the small-sized intra chroma prediction was removed in I picture.

However, JVET-L0372 only implemented and tested the method for I picture. This contribution continues to handle small chroma intra block in P/B pictures, and two tests are conducted to investigate the loss from completely removing chroma intra prediction blocks of fewer than 16 chroma samples.

For dual-tree I picture, the CE3-2.1.1 is used to avoid small-sized chroma block, and thus CTC AI simulation results are identical to CE3-2.1.1.

Proposed solution

Test 1: No chroma split for small CU

When a split mode of node A leads to small-sized chroma blocks in child nodes, the luma block of the node A splits according to the split mode, creating a set of luma CUs, while the chroma blocks do not further split, creating a chroma CU.

The prediction mode of the chroma CU is determined as follows:

- if the central position of the chroma CU corresponds to a luma CU coded in MODE_INTRA, the chroma CU is in MODE_INTRA, and chroma intra prediction mode are parsed to determine the intra prediction mode. This is similar to chroma CU in dual-tree I picture.

- if the central position of the chroma CU corresponds to a luma CU coded in MODE_IBC, a pred_mode_ibc_flag is parsed to determine whether MODE_IBC or MODE_INTRA is used. If it is in MODE_IBC, follow the 2x2-based IBC processing for chroma CU in dual-tree I picture (all 2x2 blocks have displacement vectors, guaranteed by encoder conformance check), otherwise, parse chroma intra prediction mode.

- if the central position of the chroma CU corresponds to a luma CU coded in MODE_INTER, the chroma CU is in MODE_INTER, and the motion vector(s) of the central position is applied to the chroma CU. The chroma CU is predicted by block-based uni-prediction or bi-prediction. The chroma CU motion vector does not override the motion vector field or update HMVP list.

Further harmonization with VTM tools are as follows:

- DMVR, BDOF, SBT, MTS and chroma residual scaling are not allowed in chroma CU

- GBI and TPM are not allowed in chroma CU, averaging weight always 0.5:0.5; TPM is disabled in luma CU as well (which only affects the center CU from a TT split on a 128-pixel node), because if TPM is chosen for the TT central node, the chroma CU will inherit TPM motion vectors but simple averaging is used for chroma CU bi-prediction

In the proposed scheme, the luma CUs of node A are parsed/decompressed/deblocked before the chroma CU of node A.

Test 2: No chroma split for small CU with mode restriction

In P/B pictures, when a split mode of node A leads to small-sized chroma blocks in child nodes, a flag (namely mode_cons_flag) is parsed after the split mode syntax. If mode_cons_flag is 1, the region of node A is an Intra/IBC region, in which all CUs are allowed to use only Intra/IBC mode, and the luma blocks split while and chroma blocks does not split; otherwise, the region is an Inter region, in which all CUs are allowed to use only Inter mode, and the luma and chroma are still in single tree.

In RDO, a region is coded in two passes. The region is first coded as Inter region, and then coded as Intra region. If all CUs in the Inter region do not have residual, the Intra region pass is skipped.

Comparison of the tests

The impact of the tests to small-sized prediction/transform are listed in Table 1.

Table 1 Comparison of tests

|  | intra prediction < 16 pixels | inter prediction < 16 pixels | transform block < 16 pixels | BD-rate YUV in RA | BD-rate YUV in LDB |
| --- | --- | --- | --- | --- | --- |
| Test 1 | remove | remove | Remove | x.x% / x.x% / x.x% | x.x% / x.x% / x.x% |
| Test 2 | remove | same as VTM4 | same as VTM4 | x.x% / x.x% / x.x% | x.x% / x.x% / x.x% |

Simulation results

Single-tree AI configuration of Test 1/ Test 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | -0.04% | 102% | 101% |
| Class A2 | 0.09% | -0.47% | -0.13% | 101% | 101% |
| Class B | 0.00% | -0.58% | -1.18% | 102% | 101% |
| Class C | -0.05% | 0.35% | 0.03% | 104% | 103% |
| Class E | -0.14% | -0.43% | -0.65% | 102% | 102% |
| Overall | -0.02% | -0.22% | -0.46% | 102% | 102% |
| Class D | 0.10% | 0.10% | 0.53% | 104% | 101% |
| Class F (optional) | -0.15% | 1.16% | 0.16% | 102% | 102% |

CTC configuration of CE3-2.1.1 + Test 1

Table 6 - RA simulation results of CE3-2.1.1 + Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.37% | 2.20% | 2.39% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.57% | 4.44% | 5.16% | 101% | 100% |
| Class F (optional) |  |  |  |  |  |

CTC configuration of CE3-2.1.1 + Test 2

Table 7 - RA simulation results of CE3-2.1.1 + Test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | 0.60% | 0.59% | 102% | 100% |
| Class C | 0.32% | 1.48% | 1.68% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.64% | 2.58% | 3.27% | 106% | 99% |
| Class F (optional) |  |  |  |  |  |

Conclusion

This contribution studies methods to completely remove intra prediction blocks in all picture. Some chroma loss and luma loss are observed from small resolution sequences. The coding efficiency penalty for HD are marginal. For single-tree I slice, there is no loss from this method.

Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).