| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0458-v2 |
| --- | --- |

| Title: | CE8-1.2: Block vector coding for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, jaehyun.lim, Seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the CE test results for CE8-1.2b, CE8-1.2c, and CE8-1.2d. In CE8-1.2b, binarization process for abs_mvd_minus2 syntax, which is coded by Exp-Golomb order K equal to 1 (EG1), is changed to use EG3. In CE8-1.2c, four contexts are added for signaling the abs_mvd_greater0_flag and abs_mvd_greater1_flag. CE8-1.2d is combined test of CE8-1.2b and CE8-1.2c. From experiment results, following BD rate changes are observed over VTM-4.0 with IBC:

CE8-1.2b, (EG3 coding for abs_mvd_minus2)

- AI: -0.01% (CTC), -0.21% (Class F), -0.58% (TGM)

- RA: 0.00% (CTC), -0.21% (Class F), -0.25% (TGM)

CE8-1.2c, (+2 context models for abs_mvd_greater0_flag and abs_mvd_greater1_flag, respectively)

- AI: 0.00% (CTC), -0.10% (Class F), -0.24% (TGM)

- RA: 0.00% (CTC), -0.15% (Class F), -0.12% (TGM)

CE8-1.2d, (combined test of CE8-1.2b and CE8-1.2c)

- AI: -0.01% (CTC), -0.32% (Class F), -0.85% (TGM)

- RA: 0.00% (CTC), -0.26% (Class F), -0.35% (TGM)

Introduction

In VVC Draft 4, block vector difference (BVD) coding for IBC is signaled and decoded by reusing normal motion vector difference (MVD) coding which consists of four parts; abs_mvd_greater0_flag, abs_mvd_greater1_flag, abs_mvd_minus2, mvd_sign_flag. Binarization process for abs_mvd_minus2 syntax is coded by Exp-Golomb order K equal to 1. One context model for abs_mvd_greater0_flag and abs_mvd_greater1_flag is used, respectively.

Proposed method

This contribution proposes a coding method of BVD when current coding block is coded by IBC mode. There are two modifications.

CE8-1.2b: Binarization process for abs_mvd_minus2 syntax

When prediction mode of current block is IBC mode, binarization process for abs_mvd_minus2 syntax is signaled by Exp-Golomb order K equal to 3. However, when prediction mode of current block is inter mode, binarization process for abs_mvd_minus2 syntax is signaled by Exp-Golomb order K equal to 1 as same as anchor.

CE8-1.2c: Context modeling for abs_mvd_greater0_flag and abs_mvd_greater1_flag

For IBC blocks, four contexts are additionally introduced for signaling the abs_mvd_greater0_flag and abs_mvd_greater1_flag. The contexts are separated from contexts of MVD coding. Two context models for abs_mvd_greater0_flag and abs_mvd_greater1_flag are used, respectively.

CE8-1.2d: Combination of CE8-1.2b and CE8-1.2c

CE8-1.2d is tested Combination of CE8-1.2b and CE8-1.2c tests.

Experimental results

The proposed methods have been implemented on top of VTM-4.0 and tested under CE8 test conditions [2].

Table 1. Performance of test 8-1.2b with IBC in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.02% | -0.08% | 100% | 99% |
| Class A2 | 0.00% | -0.05% | -0.01% | 100% | 100% |
| Class B | -0.01% | -0.02% | 0.03% | 100% | 99% |
| Class C | -0.03% | 0.08% | -0.03% | 100% | 98% |
| Class E | -0.04% | -0.04% | 0.06% | 99% | 97% |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 99% |
| Class D | -0.01% | 0.02% | -0.04% | 100% | 97% |
| Class F | -0.21% | -0.17% | -0.15% | 100% | 100% |
| Class SCC | -0.58% | -0.55% | -0.54% | 100% | 100% |

Table 2. Performance of test 8-1.2b with IBC in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.11% | -0.05% | 100% | 100% |
| Class A2 | -0.01% | 0.02% | 0.13% | 100% | 100% |
| Class B | 0.01% | -0.08% | -0.10% | 100% | 100% |
| Class C | -0.01% | 0.24% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 100% | 100% |
| Class D | 0.02% | -0.07% | 0.04% | 100% | 100% |
| Class F | -0.21% | -0.28% | -0.26% | 100% | 100% |
| Class SCC | -0.25% | -0.21% | -0.25% | 100% | 100% |

Table 3. Performance of test 8-1.2b with IBC in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.09% | 99% | 98% |
| Class C | -0.04% | 0.08% | 0.02% | 100% | 99% |
| Class E | -0.05% | -0.07% | 1.05% | 100% | 101% |
| Overall | -0.02% | 0.03% | 0.31% | 99% | 99% |
| Class D | -0.11% | 0.67% | 0.72% | 100% | 99% |
| Class F | -0.29% | 0.31% | 0.31% | 100% | 100% |
| Class SCC | -0.11% | -0.13% | -0.06% | 100% | 99% |

Table 4. Performance of test 8-1.2c with IBC in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | -0.05% | 100% | 100% |
| Class B | -0.01% | 0.00% | -0.05% | 100% | 99% |
| Class C | 0.00% | 0.03% | -0.03% | 100% | 98% |
| Class E | -0.02% | -0.06% | 0.06% | 100% | 97% |
| Overall | 0.00% | -0.01% | -0.02% | 100% | 99% |
| Class D | 0.00% | 0.01% | 0.10% | 100% | 98% |
| Class F | -0.10% | -0.05% | -0.18% | 101% | 100% |
| Class SCC | -0.24% | -0.27% | -0.25% | 100% | 100% |

Table 5. Performance of test 8-1.2c with IBC in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01% | -0.02% | -0.06% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.00% | 99% | 100% |
| Class B | 0.01% | -0.07% | -0.31% | 100% | 100% |
| Class C | -0.03% | 0.25% | 0.19% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | -0.07% | 100% | 100% |
| Class D | 0.03% | -0.09% | 0.01% | 100% | 101% |
| Class F | -0.15% | -0.22% | -0.19% | 100% | 101% |
| Class SCC | -0.12% | -0.11% | -0.08% | 100% | 100% |

Table 6. Performance of test 8-1.2c with IBC in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.15% | 0.25% | 99% | 97% |
| Class C | -0.03% | -0.22% | -0.11% | 100% | 98% |
| Class E | -0.01% | -0.30% | 0.95% | 100% | 101% |
| Overall | -0.02% | -0.09% | 0.30% | 99% | 98% |
| Class D | -0.09% | 0.57% | -0.38% | 100% | 99% |
| Class F | -0.09% | 0.13% | -0.07% | 99% | 99% |
| Class SCC | 0.00% | 0.05% | 0.05% | 99% | 100% |

Table 7. Performance of test 8-1.2d with IBC on in AI

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.00% | -0.03% | -0.06% | 100% | 99% |
| Class A2 | 0.00% | -0.04% | -0.05% | 100% | 101% |
| Class B | -0.01% | -0.03% | 0.01% | 100% | 99% |
| Class C | -0.02% | 0.04% | -0.01% | 100% | 98% |
| Class E | -0.05% | -0.05% | -0.06% | 100% | 97% |
| Overall | -0.01% | -0.02% | -0.03% | 100% | 99% |
| Class D | -0.01% | -0.06% | 0.05% | 100% | 97% |
| Class F | -0.32% | -0.36% | -0.38% | 100% | 101% |
| Class SCC | -0.85% | -0.80% | -0.81% | 100% | 100% |

Table 8. Performance of test 8-1.2d with IBC in RA

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0.01% | -0.15% | 0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.02% | 0.09% | 99% | 100% |
| Class B | 0.01% | -0.13% | -0.28% | 99% | 100% |
| Class C | -0.03% | 0.15% | 0.13% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.04% | 99% | 99% |
| Class D | 0.00% | -0.10% | 0.24% | 100% | 99% |
| Class F | -0.26% | -0.44% | -0.33% | 100% | 100% |
| Class SCC | -0.35% | -0.20% | -0.32% | 100% | 101% |

Table 9. Performance of test 8-1.2d with IBC in LD

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.38% | -0.13% | 99% | 96% |
| Class C | 0.00% | 0.04% | 0.10% | 100% | 98% |
| Class E | 0.00% | 0.84% | 1.45% | 99% | 99% |
| Overall | -0.01% | 0.38% | 0.34% | 99% | 97% |
| Class D | -0.03% | 0.99% | 0.29% | 100% | 98% |
| Class F | -0.21% | -0.16% | 0.08% | 100% | 99% |
| Class SCC | -0.13% | -0.03% | -0.04% | 99% | 99% |

Conclusion

In this contribution, two modifications of block vector coding for IBC mode are tested in CE8-1.2b. It is recommended to adopt the proposed BVD coding for IBC mode.

Reference

- J. Nam, et al., “Non-CE8: Coding on block vector difference,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M0333, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- X. Xu, et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” , Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1028, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).