| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0459-v2 |
| --- | --- |

| Title: | CE8-related: Modified block vector coding for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hyeongmun Jang Jungah Choi Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | {junghak.nam, hm.jang, jungah.choi, jin78.heo, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

- Abstract

This contribution proposes some improvements on block vector coding for IBC mode. Syntaxes for block vector coding are used by separating those of motion vector coding. Especially, context modeling and binarization method are designed for block vector coding. In addition, block vector predictors are clipped based on position of current coding block. From experiment results, following BD rate changes are observed over VTM-4.0 with CPR:

- AI: -0.03% (CTC), -0.44% (Class F), -1.09% (TGM)

- RA: -0.01% (CTC), -0.34% (Class F), -0.50% (TGM)

Introduction

In VVC Draft 4, block vector difference (BVD) coding for IBC is signaled and decoded by reusing normal motion vector difference (MVD) coding which consists of four parts; abs_mvd_greater0_flag, abs_mvd_greater1_flag, abs_mvd_minus2, mvd_sign_flag. Binarization process for syntax elements of MVD coding is shown in Table 1.

Table 1: Binarization process for syntax elements of MVD coding

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |

Assignment of ctxInc to syntax elements of MVD coding is shown in Table 2.

Table 2: Assignment of ctxInc to syntax elements for MVD coding

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_mvd_greater0_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_greater1_flag[ ] | 0 | na | na | na | na | na |
| abs_mvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| mvd_sign_flag[ ] | bypass | na | na | na | na | na |

Proposed method

In this contribution, syntaxes for block vector coding are separated from those of motion vector coding. For the BVD coding, four syntaxes are introduced, i.e. abs_bvd_greater0_flag, abs_bvd_greater1_flag, abs_bvd_minus2, bvd_sign_flag.

Firstly, binarization process is modified for signaling abs_mvd_minus2. An x-component of abs_bvd_minus2 syntax is binarized by truncated unary plus fixed length coding with four (TU_FLC4). A y-component of abs_bvd_minus2 syntax is binarized by truncated unary plus fixed length coding with three (TU_FLC3). In general, smaller FLC is used for the y-component because search range of y-axis is smaller than search range of x-axis. Binarization process for syntax elements of BVD coding is shown in Table 3.

Table 3: Binarization process for syntax elements of BVD coding

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| mvd_coding( ) | abs_bvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_bvd_minus2[ 0 ] | TU + FL4 | cRiceParam = 0, cMax = 16 / cMax = 15 |
|  | abs_bvd_minus2[ 1 ] | TU + FL3 | cRiceParam = 0, cMax = 16 / cMax = 7 |
|  | bvd_sign_flag[ ] | FL | cMax = 1 |

Secondly, four context models are added for signaling the abs_mvd_greater0_flag and abs_mvd_greater1_flag. It is identical to CE8-1.2c. Assignment of ctxInc to syntax elements of BVD coding is shown in Table 4.

Table 4: Assignment of ctxInc to syntax elements for bvd coding

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| abs_bvd_greater0_flag[ 0 ] | 0 | na | na | na | na | na |
| abs_bvd_greater0_flag[ 1 ] | 0 | na | na | na | na | na |
| abs_bvd_greater1_flag[ 0 ] | 0 | na | na | na | na | na |
| abs_bvd_greater0_flag[ 1 ] | 0 | na | na | na | na | na |
| abs_bvd_minus2[ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| bvd_sign_flag[ ] | bypass | na | na | na | na | na |

In addition, in worst case, candidates of block vector prediction could be out of range for current coding block. It may occur quite large BVD values. In this contribution, block vector predictors (BVP) are clipped based on position of current coding block as follows;

BVP_x = Clip3 (– ((PosX – CTUPosX) + CTUSize), CTUSize – ((PosX – CTUPosX) + Width), BVP_x)

BVP_y = Clip3 (– (PosY – CTUPosY), CTUSize – ((PosY – CTUPosY) + Height), BVP_y)

where (PosX, PosY) specifies the top-left sample of the current coding block relative to the top-left sample of the current picture and (CTUPosX, CTUPosY) specifies the top-left sample of the current CTU block relative to the top-left sample of the current picture. Width and Height specifies the width and height of the current coding block, respectively. BVP_x and BVP_y specifies x- and y-component of BVP.

Experimental results

The proposed methods have been implemented on top of VTM-4.0 and tested under CE8 test conditions [1].

Table 5. Performance of proposed method in AI

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | -0.09% | 102% | 103% |
| Class A2 | -0.01% | -0.05% | -0.04% | 102% | 103% |
| Class B | -0.02% | -0.07% | -0.04% | 101% | 103% |
| Class C | -0.03% | 0.02% | 0.06% | 101% | 99% |
| Class E | -0.10% | -0.14% | -0.04% | 100% | 100% |
| Overall | -0.03% | -0.06% | -0.03% | 101% | 102% |
| Class D | -0.02% | -0.08% | 0.06% | 100% | 98% |
| Class F | -0.44% | -0.41% | -0.49% | 100% | 101% |
| Class SCC | -1.09% | -1.03% | -0.99% | 100% | 101% |

Table 6. Performance of proposed method in RA

|  |  |  | RA Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.06% | -0.05% | 103% | 104% |
| Class A2 | -0.02% | -0.03% | 0.06% | 102% | 102% |
| Class B | 0.00% | 0.03% | -0.31% | 102% | 101% |
| Class C | -0.01% | 0.29% | 0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.07% | -0.08% | 102% | 101% |
| Class D | 0.01% | -0.04% | 0.13% | 101% | 100% |
| Class F | -0.34% | -0.40% | -0.44% | 101% | 100% |
| Class SCC | -0.50% | -0.32% | -0.34% | 101% | 100% |

Table 7. Performance of proposed method in LD

|  |  |  | LD Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.14% | 0.26% | 103% | 103% |
| Class C | -0.04% | -0.23% | -0.13% | 102% | 103% |
| Class E | -0.03% | -0.35% | 0.87% | 101% | 101% |
| Overall | -0.02% | -0.11% | 0.28% | 102% | 102% |
| Class D | -0.05% | 1.03% | -0.01% | 102% | 101% |
| Class F | -0.32% | -0.84% | -0.33% | 100% | 101% |
| Class SCC | -0.15% | 0.00% | -0.09% | 100% | 100% |

Conclusion

In this contribution, modifications of block vector coding are proposed. It is recommended to adopt the proposed method for IBC mode.

Reference

- X. Xu, et al., “Description of Core Experiment 8 (CE8): Screen Content Coding Tools,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1028, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).