| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0466 |
| --- | --- |

| Title: | CE8-related : The corner case handling regarding mv derivation for Chroma IBC in dual tree structure. |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution suggests corner case handling methods about motion vector derivation for the chroma block which is coded in IBC mode. Current VVC WD4 allows IBC mode as a separate prediction mode. When a chroma coding block is coded by IBC mode in the dual tree structure, the motion vector of IBC chroma block is derived from motion vector of corresponding luma block based on 2x2 sub-block. But the motion vector is not available when the corresponding luma block is coded by intra prediction. This case can cause irregular decoder behavior. In the proposed method, in encoder side, if the center position of corresponding luma block is IBC coded then the chroma block allows Rate-Optimization process for IBC mode with a default IBC motion vector for the abovementioned corner case. The default IBC motion vector for chroma block is set equal to the IBC motion vector of center position in the corresponding luma block.

Problem statement

In the current VVC reference software VTM-4.0, IBC chroma Rate-Optimization is applied when the entire corresponding luma block of current chroma block is IBC coded. When the luma block is partially IBC coded or none IBC coded, the Chroma IBC Rate-Optimization is not applied and ibc_pred_mode is set as 0. However current VVC WD allows Chroma IBC mode despite of that the corresponding luma block is partially IBC coded. It causes a corner case which is corresponding luma block with being intra coded. In this case, current chroma subblock derives motion vector as (0, 0), predFlagL0 and predFlagL1 as 0 also. Using this information, current chroma IBC sub-block cannot derive prediction sample. Table 1-1 shows current motion vector derivation process for the IBC chroma block..

8.6 Decoding process for coding units coded in IBC prediction mode ### 8.6.1 General decoding process for coding units coded in IBC prediction mode Inputs to this process are: - a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples, - a variable treeType specifying whether a single or a dual tree is used and if a dual tree is used, it specifies whether the current tree corresponds to the luma or chroma components. Output of this process is a modified reconstructed picture before in-loop filtering. The derivation process for quantization parameters as specified in clause 8.7.1 is invoked with the luma location ( xCb, yCb ), the width of the current coding block in luma samples cbWidth and the height of the current coding block in luma samples cbHeight, and the variable treeType as inputs. The decoding process for coding units coded in ibc prediction mode consists of the following ordered steps: - The motion vector components of the current coding unit are derived as follows: - If treeType is equal to SINGLE_TREE or DUAL_TREE_LUMA, the following applies: - The derivation process for motion vector components as specified in clause 8.6.2.1 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth and the luma coding block height cbHeight as inputs, and the luma motion vector mvL[ 0 ][ 0 ] as output. - When treeType is equal to SINGLE_TREE, the derivation process for chroma motion vectors in clause 8.6.2.9 is invoked with luma motion vector mvL[ 0 ][ 0 ] as input, and chroma motion vector mvC[ 0 ][ 0 ] as output. - The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are both set equal to 1. - Otherwise, if treeType is equal to DUAL_TREE_CHROMA, the following applies: - The number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY are derived as follows: numSbX = ( cbWidth >> 2 ) (8-955) numSbY = ( cbHeight >> 2 ) (8-956) - The chroma motion vectors mvC[ xSbIdx ][ ySbIdx ] are derived as follows for xSbIdx = 0..numSbX − 1, ySbIdx = 0..numSbY − 1: - The luma motion vector mvL[ xSbIdx ][ ySbIdx ] is derived as follows: - The location ( xCuY, yCuY ) of the collocated luma coding unit is dervied as follows: xCuY = xCb + xSbIdx*4 (8-957) yCuY = yCb + ySbIdx*4 (8-958) - If CuPredMode[ xCuY ][ yCuY ] is equal to MODE_INTRA, the following applies. mvL[ xSbIdx ][ ySbIdx ][ 0 ] = 0 (8-959) mvL[ xSbIdx ][ ySbIdx ][ 1 ] = 0 (8-960) predFlagL0[ xSbIdx ][ ySbIdx ] = 0 (8-961) predFlagL1[ xSbIdx ][ ySbIdx ] = 0 (8-962) - Otherwise ( CuPredMode[ xCuY ][ yCuY ] is equal to MODE_IBC ), the following applies: mvL[ xSbIdx ][ ySbIdx ][ 0 ]=MvL0[ xCuY ][ yCuY ][ 0 ] (8-963) mvL[ xSbIdx ][ ySbIdx ][ 1 ]=MvL0[ xCuY ][ yCuY ][ 1 ] (8-964) predFlagL0[ xSbIdx ][ ySbIdx ] = 1 (8-965) predFlagL1[ xSbIdx ][ ySbIdx ] = 0 (8-966) - The derivation process for chroma motion vectors in clause 8.6.2.9 is invoked with mvL[ xSbIdx ][ ySbIdx ] as inputs, and mvC[ xSbIdx ][ ySbIdx ] as output. - It is a requirement of bitstream conformance that the chroma motion vector mvC[ xSbIdx ][ ySbIdx ] shall obey the following constraints: - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ), yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) ) as inputs, and the output shall be equal to TRUE. - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current chroma location ( xCurr, yCurr ) set equal to ( xCb / SubWidthC, yCb / SubHeightC ) and the neighbouring chroma location ( xCb / SubWidthC + ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + cbWidth / SubWidthC − 1, yCb / SubHeightC + ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + cbHeight / SubHeightC − 1 ) as inputs, and the output shall be equal to TRUE. - One or both of the following conditions shall be true: - ( mvC[ xSbIdx ][ ySbIdx ][ 0 ] >> 5 ) + xSbIdx * 2 + 2 is less than or equal to 0. - ( mvC[ xSbIdx ][ ySbIdx ][ 1 ] >> 5 ) + ySbIdx * 2 + 2 is less than or equal to 0.

Proposed method

In order to handle the corner case, the chroma block with corresponding luma block being coded with intra uses a default IBC motion vector. The default IBC motion vector is set to the IBC motion vector of center position in the corresponding luma block. In encoder side, only if the center position of corresponding luma block is IBC coded, the chroma block allows Rate-Optimization process for IBC mode.

Experimental results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class F (optional) | -0.02% | -0.07% | -0.12% | 101% | 100% |
| Class SCC 1080p | -0.14% | -0.21% | -0.25% | 102% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | -0.03% | -0.05% | -0.17% | 101% | 100% |
| Class SCC 1080p | -0.06% | -0.07% | -0.12% | 102% | 99% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).