| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0468 |
| --- | --- |

| Title: | AhG2 : Editorial Suggestion on the text specification for Inter and IBC mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposed draft |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seunghwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In VTM4.0, IBC mode is signalled as a separate CU prediction mode, and IBC mode has its own candidate list including IBC coded spatial neighbor blocks. This contribution introduces editorial suggestions to VVC WD specification for clarifying regarding IBC mode addition.

Proposed method

#1. Derivation process for spatial merging candidates from the neighboring spatial blocks which is coded MODE_INTER.

- 8.5.2.3. Derivation process for spatial merging candidates Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. Outputs of this process are as follows, with X being 0 or 1: - the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units, - the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0, refIdxLXB1 and refIdxLXB2 of the neighbouring coding units, - the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0, predFlagLXB1 and predFlagLXB2 of the neighbouring coding units, - the motion vectors in 1/16 fractional-sample accuracy mvLXA0, mvLXA1, mvLXB0, mvLXB1 and mvLXB2 of the neighbouring coding units, - the bi-prediction weight indices gbiIdxA0, gbiIdxA1, gbiIdxB0, gbiIdxB1, and gbiIdxB2. For the derivation of availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 the following applies: - The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb + cbHeight − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1. - The variables availableFlagA1, refIdxLXA1, predFlagLXA1 and mvLXA1 are derived as follows: - If one or more of the following conditions are true, availableFlagA1 is set equal to 0, both components of mvLXA1 are set equal to 0, refIdxLXA1 is set equal to −1 and predFlagLXA1 is set equal to 0, with X being 0 or 1, and gbiIdxA1 is set equal to 0 - availableA1 is equal to FALSE. - availableA1 is equal to TRUE and CuPredMode[ xNbA1][ yNbA1] is not equal to MODE_INTER. - Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made: mvLXA1 = MvLX[ xNbA1 ][ yNbA1 ] (8-294) refIdxLXA1 = RefIdxLX[ xNbA1 ][ yNbA1 ] (8-295) predFlagLXA1 = PredFlagLX[ xNbA1 ][ yNbA1 ] (8-296) gbiIdxA1 = GbiIdx[ xNbA1 ][ yNbA1 ] (8-297) For the derivation of availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 the following applies: - The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1. - The variables availableFlagB1, refIdxLXB1, predFlagLXB1 and mvLXB1 are derived as follows: - If one or more of the following conditions are true, availableFlagB1 is set equal to 0, both components of mvLXB1 are set equal to 0, refIdxLXB1 is set equal to −1 and predFlagLXB1 is set equal to 0, with X being 0 or 1, and gbiIdxB1 is set equal to 0: - availableB1 is equal to FALSE. - availableB1 is equal to TRUE and CuPredMode[ xNbB1][ yNbB1] is not equal to MODE_INTER. - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors and the same reference indices. Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made: mvLXB1 = MvLX[ xNbB1 ][ yNbB1 ] (8-298) refIdxLXB1 = RefIdxLX[ xNbB1 ][ yNbB1 ] (8-299) predFlagLXB1 = PredFlagLX[ xNbB1 ][ yNbB1 ] (8-300) gbiIdxB1 = GbiIdx[ xNbB1 ][ yNbB1 ] (8-301) For the derivation of availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 the following applies: - The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0. - The variables availableFlagB0, refIdxLXB0, predFlagLXB0 and mvLXB0 are derived as follows: - If one or more of the following conditions are true, availableFlagB0 is set equal to 0, both components of mvLXB0 are set equal to 0, refIdxLXB0 is set equal to −1 and predFlagLXB0 is set equal to 0, with X being 0 or 1, and gbiIdxB0 is set equal to 0: - availableB0 is equal to FALSE. - availableB0 is equal to TRUE and CuPredMode[ xNbB0][ yNbB0] is not equal to MODE_INTER. - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices. - availableA1 is equal to TRUE, the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors and the same reference indices and merge_triangle_flag[ xCb ][ yCb ] is equal to 1. - Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made: mvLXB0 = MvLX[ xNbB0 ][ yNbB0 ] (8-302) refIdxLXB0 = RefIdxLX[ xNbB0 ][ yNbB0 ] (8-303) predFlagLXB0 = PredFlagLX[ xNbB0 ][ yNbB0 ] (8-304) gbiIdxB0 = GbiIdx[ xNbB0 ][ yNbB0 ] (8-305) For the derivation of availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 the following applies: - The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb + cbWidth ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0. - The variables availableFlagA0, refIdxLXA0, predFlagLXA0 and mvLXA0 are derived as follows: - If one or more of the following conditions are true, availableFlagA0 is set equal to 0, both components of mvLXA0 are set equal to 0, refIdxLXA0 is set equal to −1 and predFlagLXA0 is set equal to 0, with X being 0 or 1, and gbiIdxA0 is set equal to 0: - availableA0 is equal to FALSE. - availableA0 is equal to TRUE and CuPredMode[ xNbA0][ yNbA0] is not equal to MODE_INTER. - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices. - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices and merge_triangle_flag[ xCb ][ yCb ] is equal to 1. - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors and the same reference indices and merge_triangle_flag[ xCb ][ yCb ] is equal to 1. - Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made: mvLXA0 = MvLX[ xNbA0 ][ yNbA0 ] (8-306) refIdxLXA0 = RefIdxLX[ xNbA0 ][ yNbA0 ] (8-307) predFlagLXA0 = PredFlagLX[ xNbA0 ][ yNbA0 ] (8-308) gbiIdxA0 = GbiIdx[ xNbA0 ][ yNbA0 ] (8-309) For the derivation of availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 the following applies: - The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2. - The variables availableFlagB2, refIdxLXB2, predFlagLXB2 and mvLXB2 are derived as follows: - If one or more of the following conditions are true, availableFlagB2 is set equal to 0, both components of mvLXB2 are set equal to 0, refIdxLXB2 is set equal to −1 and predFlagLXB2 is set equal to 0, with X being 0 or 1, and gbiIdxB2 is set equal to 0: - availableB2 is equal to FALSE. - availableB2 is equal to TRUE and CuPredMode[ xNbB0][ yNbB0] is not equal to MODE_INTER. - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices. - availableB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices. - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices and merge_triangle_flag[ xCb ][ yCb ] is equal to 1. - availableA0 is equal to TRUE, the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors and the same reference indices and merge_triangle_flag[ xCb ][ yCb ] is equal to 1. - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4 and merge_triangle_flag[ xCb ][ yCb ] is equal to 0. - Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made: mvLXB2 = MvLX[ xNbB2 ][ yNbB2 ] (8-310) refIdxLXB2 = RefIdxLX[ xNbB2 ][ yNbB2 ] (8-311) predFlagLXB2 = PredFlagLX[ xNbB2 ][ yNbB2 ] (8-312) gbiIdxB2 = GbiIdx[ xNbB2 ][ yNbB2 ] (8-313)

#2. Derivation process for spatial motion vector predictor candidates from the neighboring spatial blocks which is coded MODE_INTER.

- 8.5.2.10 Derivation process for spatial motion vector predictor candidates Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples, - the reference index of the current coding unit partition refIdxLX, with X being 0 or 1. Outputs of this process are (with N being replaced by A or B): - the motion vectors mvLXN in 1/16 fractional-sample accuracy of the neighbouring coding units, - the availability flags availableFlagLXN of the neighbouring coding units. Figure 8-2 provides an overview of spatial motion vector neighbours. Figure 8-2 – Spatial motion vector neighbours (informative) The variable currCb specifies the current luma coding block at luma location ( xCb, yCb ) and the variable currPic specifies the current picture. The variable isScaledFlagLX, with X being 0 or 1, is set equal to 0. The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps: - The sample location ( xNbA0, yNbA0 ) is set equal to ( xCb − 1, yCb + cbHeight ) and the sample location ( xNbA1, yNbA1 ) is set equal to ( xNbA0, yNbA0 − 1 ). - The availability flag availableFlagLXA is set equal to 0 and both components of mvLXA are set equal to 0. - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0. - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1. - If one or more of the following conditions are TRUE, the variable isScaledFlagLX is set equal to 1: - availableA0 is euqal to TRUE and CuPredMode[ xNbA0][ yNbA0] is equal to MODE_INTER - availableA1 is euqal to TRUE and CuPredMode[ xNbA1][ yNbA1] is equal to MODE_INTER - The following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ): - When availableAk is equal to TRUE and CuPredMode[ xNbAk][ yNbAk] is equal to MODE_INTER and availableFlagLXA is equal to 0, the following applies: - If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 and DiffPicOrderCnt( RefPicList[ X ][ RefIdxLX[ xNbAk ][ yNbAk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies: mvLXA = MvLX[ xNbAk ][ yNbAk ] (8-385) - Otherwise, when PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicList[ Y ][ RefIdxLY[ xNbAk ][ yNbAk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXA is set equal to 1 and the following applies: mvLXA = MvLY[ xNbAk ][ yNbAk ] (8-386) - When availableFlagLXA is equal to 0, the following applies for ( xNbAk, yNbAk ) from ( xNbA0, yNbA0 ) to ( xNbA1, yNbA1 ) or until availableFlagLXA is equal to 1: - When availableAk is equal to TRUE and CuPredMode[ xNbAk][ yNbAk] is equal to MODE_INTER and availableFlagLXA is equal to 0, the following applies: - If PredFlagLX[ xNbAk ][ yNbAk ] is equal to 1 , availableFlagLXA is set equal to 1 and the following assignments are made: mvLXA = MvLX[ xNbAk ][ yNbAk ] (8-387) refIdxA = RefIdxLX[ xNbAk ][ yNbAk ] (8-388) refPicListA = RefPicList[ X ] (8-389) - Otherwise, if PredFlagLY[ xNbAk ][ yNbAk ] (with Y = !X) is equal to 1, availableFlagLXA is set equal to 1 and the following assignments are made: mvLXA = MvLY[ xNbAk ][ yNbAk ] (8-390) refIdxA = RefIdxLY[ xNbAk ][ yNbAk ] (8-391) refPicListA = RefPicList[ Y ] (8-392) - When availableFlagLXA is equal to 1, DiffPicOrderCnt( refPicListA[ refIdxA ], RefPicList[ X ][ refIdxLX ] ) is not equal to 0, mvLXA is derived as follows: tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-393) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-394) mvLXA = Clip3( −131072, 131071, Sign( distScaleFactor * mvLXA ) * ( ( Abs( distScaleFactor * mvLXA ) + 127 ) >> 8 ) ) (8-395) where td and tb are derived as follows: td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListA[ refIdxA ] ) ) (8-396) tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) ) (8-397) The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps: - The sample locations ( xNbB0, yNbB0 ), ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) are set equal to ( xCb + cbWidth, yCb − 1 ), ( xCb + cbWidth − 1, yCb − 1 ) and ( xCb − 1, yCb − 1 ), respectively. - The availability flag availableFlagLXB is set equal to 0 and the both components of mvLXB are set equal to 0. - The following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ): - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk. - When availableBk is equal to TRUE and CuPredMode[ xNbBk][ yNbBk] is equal to MODE_INTER and availableFlagLXB is equal to 0, the following applies: - If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, and DiffPicOrderCnt( RefPicList[ X ][ RefIdxLX[ xNbBk ][ yNbBk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made: mvLXB = MvLX[ xNbBk ][ yNbBk ] (8-398) refIdxB = RefIdxLX[ xNbBk ][ yNbBk ] (8-399) - Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1 and DiffPicOrderCnt( RefPicList[ Y ][ RefIdxLY[ xNbBk ][ yNbBk ] ], RefPicList[ X ][ refIdxLX ] ) is equal to 0, availableFlagLXB is set equal to 1 and the following assignments are made: mvLXB = MvLY[ xNbBk ][ yNbBk ] (8-400) refIdxB = RefIdxLY[ xNbBk ][ yNbBk ] (8-401) - When isScaledFlagLX is equal to 0 and availableFlagLXB is equal to 1, availableFlagLXA is set equal to 1 and the following applies: mvLXA = mvLXB (8-402) - When isScaledFlagLX is equal to 0, availableFlagLXB is set equal to 0 and the following applies for ( xNbBk, yNbBk ) from ( xNbB0, yNbB0 ) to ( xNbB2, yNbB2 ) or until availableFlagLXB is equal to 1: - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbBk, yNbBk ) as inputs, and the output is assigned to the block availability flag availableBk. - When availableBk is equal to TRUE and CuPredMode[ xNbBk][ yNbBk] is equal to MODE_INTER and availableFlagLXB is equal to 0, the following applies: - If PredFlagLX[ xNbBk ][ yNbBk ] is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made: mvLXB = MvLX[ xNbBk ][ yNbBk ] (8-403) refIdxB = RefIdxLX[ xNbBk ][ yNbBk ] (8-404) refPicListB = RefPicList[ X ] (8-405) - Otherwise, when PredFlagLY[ xNbBk ][ yNbBk ] (with Y = !X) is equal to 1, availableFlagLXB is set equal to 1 and the following assignments are made: mvLXB = MvLY[ xNbBk ][ yNbBk ] (8-406) refIdxB = RefIdxLY[ xNbBk ][ yNbBk ] (8-407) refPicListB = RefPicList[ Y ] (8-408) - When availableFlagLXB is equal to 1, DiffPicOrderCnt( refPicListB[ refIdxB ], RefPicList[ X ][ refIdxLX ] ) is not equal to 0, mvLXB is derived as follows: tx = ( 16384 + ( Abs( td ) >> 1 ) ) / td (8-409) distScaleFactor = Clip3( −4096, 4095, ( tb * tx + 32 ) >> 6 ) (8-410) mvLXB = Clip3( −131072, 131071, Sign( distScaleFactor * mvLXB ) * ( ( Abs( distScaleFactor * mvLXB ) + 127 ) >> 8 ) ) (8-411) where td and tb are derived as follows: td = Clip3( −128, 127, DiffPicOrderCnt( currPic, refPicListB[ refIdxB ] ) ) (8-412) tb = Clip3( −128, 127, DiffPicOrderCnt( currPic, RefPicList[ X ][ refIdxLX ] ) ) (8-413)

#3. Derivation process for IBC spatial merging candidates from the neighboring spatial blocks which is coded MODE_IBC.

- 8.6.2.3 Derivation process for spatial merging candidates - Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. Outputs of this process are as follows: - the availability flags availableFlagA0, availableFlagA1, availableFlagB0, availableFlagB1 and availableFlagB2 of the neighbouring coding units, - the motion vectors in 1/16 fractional-sample accuracy mvA0, mvA1, mvB0, mvB1 and mvB2 of the neighbouring coding units, For the derivation of availableFlagA1 and mvA1 the following applies: - The luma location ( xNbA1, yNbA1 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb + cbHeight − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA1, yNbA1 ) as inputs, and the output is assigned to the block availability flag availableA1. - The variables availableFlagA1 and mvLA1 are derived as follows: - If one or more of the following conditions are true, availableFlagA1 is set equal to 0 and both components of mvA1 are set equal to 0 - availableA1 is equal to FALSE. - availableA1 is equal to TRUE and CuPredMode[ xNbA1 ][ yNbA1 ] is not equal to MODE_IBC. - Otherwise, availableFlagA1 is set equal to 1 and the following assignments are made: mvA1 = MvL0[ xNbA1 ][ yNbA1 ] (8-979) For the derivation of availableFlagB1 and mvB1 the following applies: - The luma location ( xNbB1, yNbB1 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth − 1, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB1, yNbB1 ) as inputs, and the output is assigned to the block availability flag availableB1. - The variables availableFlagB1 and mvB1 are derived as follows: - If one or more of the following conditions are true, availableFlagB1 is set equal to 0 and both components of mvB1 are set equal to 0: - availableB1 is equal to FALSE. - availableB1 is equal to TRUE and CuPredMode[ xNbB1 ][ yNbB1 ] is not equal to MODE_IBC. - availableFlagA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB1, yNbB1 ) have the same motion vectors. Otherwise, availableFlagB1 is set equal to 1 and the following assignments are made: mvB1 = MvL0[ xNbB1 ][ yNbB1 ] (8-980) For the derivation of availableFlagB0 and mvB0 the following applies: - The luma location ( xNbB0, yNbB0 ) inside the neighbouring luma coding block is set equal to ( xCb + cbWidth, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB0, yNbB0 ) as inputs, and the output is assigned to the block availability flag availableB0. - The variables availableFlagB0 and mvB0 are derived as follows: - If one or more of the following conditions are true, availableFlagB0 is set equal to 0 and both components of mvB0 are set equal to 0: - availableB0 is equal to FALSE. - availableB0 is equal to TRUE and CuPredMode[ xNbB0 ][ yNbB0 ] is not equal to MODE_IBC. - availableFlagB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB0, yNbB0 ) have the same motion vectors. - availableFlagA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB0, yNbB0 ) have the same motion vectors. - Otherwise, availableFlagB0 is set equal to 1 and the following assignments are made: mvB0 = MvL0[ xNbB0 ][ yNbB0 ] (8-981) For the derivation of availableFlagA0 and mvA0 the following applies: - The luma location ( xNbA0, yNbA0 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb + cbWidth ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbA0, yNbA0 ) as inputs, and the output is assigned to the block availability flag availableA0. - The variables availableFlagA0 and mvA0 are derived as follows: - If one or more of the following conditions are true, availableFlagA0 is set equal to 0 and both components of mvA0 are set equal to 0: - availableA0 is equal to FALSE. - availableA0 is equal to TRUE and CuPredMode[ xNbA0 ][ yNbA0 ] is not equal to MODE_IBC. - availableA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbA0, yNbA0 ) have the same motion vectors. - availableB1 is equal to TRUE, the luma locations ( xNbB1, yNbB1 ) and ( xNbA0, yNbA0 ) have the same motion vectors. - availableB0 is equal to TRUE, the luma locations ( xNbB0, yNbB0 ) and ( xNbA0, yNbA0 ) have the same motion vectors. - Otherwise, availableFlagA0 is set equal to 1 and the following assignments are made: mvA0 = MvL0[ xNbA0 ][ yNbA0 ] (8-982) For the derivation of availableFlagB2 and mvB2 the following applies: - The luma location ( xNbB2, yNbB2 ) inside the neighbouring luma coding block is set equal to ( xCb − 1, yCb − 1 ). - The availability derivation process for a block as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xNbB2, yNbB2 ) as inputs, and the output is assigned to the block availability flag availableB2. - The variables availableFlagB2 and mvB2 are derived as follows: - If one or more of the following conditions are true, availableFlagB2 is set equal to 0 and both components of mvB2 are set equal to 0: - availableB2 is equal to FALSE. - availableB2 is equal to TRUE and CuPredMode[ xNbB2 ][ yNbB2 ] is not equal to MODE_IBC. - availableFlagA1 is equal to TRUE and the luma locations ( xNbA1, yNbA1 ) and ( xNbB2, yNbB2 ) have the same motion vectors. - availableFlagB1 is equal to TRUE and the luma locations ( xNbB1, yNbB1 ) and ( xNbB2, yNbB2 ) have the same motion vectors. - availableFlagB0 is equal to TRUE and the luma locations ( xNbB0, yNbB0 ) and ( xNbB2, yNbB2 ) have the same motion vectors. - availableFlagA0 is equal to TRUE and the luma locations ( xNbA0, yNbA0 ) and ( xNbB2, yNbB2 ) have the same motion vectors. - availableFlagA0 + availableFlagA1 + availableFlagB0 + availableFlagB1 is equal to 4. - Otherwise, availableFlagB2 is set equal to 1 and the following assignments are made: mvB2 = MvL0[ xNbB2 ][ yNbB2 ] (8-983)

#4. Unreferenced text specification clean-up

The variable CurrPicIsOnlyRef, specifying that the current decoded picture is the only reference picture for the current tile group, is derived as follows: CurrPicIsOnlyRef = sps_cpr_enabled_flag && ( tile_group_type = = P ) && (7-39) ( num_ref_idx_active_minus1[ 0 ] = = 0 )

- Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).