| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0485-v2 |
| --- | --- |

| Title: | Non-CE6: Efficient separable Multiple-Transform-Selection (MTS) without zero-out |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The “zeroing-out” of transform coefficients is a straightforward technique to reduce the computational complexity of large-dimension transforms. However, its apparent simplicity can hide the fact that, when it is used in different transforms and dimensions, it creates two logical types of transform blocks, resulting in very significant additional complications in the codec’s design and implementation code. To avoid those problems, this proposal presents a new implementation of the 64-point DCT-2, together with Transform Adjustments (JVET-M0538) for DCT-2-based computation of DCT-8 DST-7. Its complexity analysis and simulation results show that the propose method can reduce the complexity of the large separable transforms used by Multiple-Transform-Selection (MTS), to values very similar to the “zero-out,” without incurring in its coding loss.

Simulations results using VTM-4.0 and CE6 test conditions showed that the proposed method gives B-D rate average coding gain of 0.24% for AI tests. As expected, larger coding gains were obtained on class A sequences, where larger blocks are used more frequently. For AI configuration the gains are 0.59% and 0.27% for classes A1 and A2, respectively.

# Introduction

For a transform of dimension N, “zeroing-out” consists in computing only a certain fraction N / K of transform coefficients (those corresponding to most important basis vectors, or lowest frequencies), and setting the remaining coefficients to zero. While this is theoretically sub-optimal, it is an easy way to reduce the computational complexity of large transforms by a factor of K, and it has been shown to produce relatively small losses for large N on low-resolution sequences where larger blocks (e.g., of size 32 and 64) are less frequently used.

However, to really evaluate its effects, it is necessary to also consider factors such as:

- It can cause significant losses on high resolution video sequences where large blocks are used more often.

- It is essentially a block-shape dependent dead-zone quantization applied before the actual quantization step, which may adversely affect the RD-optimized quantization.

- In a practical encoder, many decisions are made based on the remaining energy after prediction so that a potential zeroing-out of coefficients can make a practical encoder deviate from a good coding decision.

- It requires a codec to support separate implementations for two logical types of transform blocks (with and without zero-out), where additional processing units (such as multiple types of scans and coefficient coding methods) are needed to handle multiple types of transform coefficient block with zeros at different locations.

- It adds another level of logical transform block partitioning (below the main block partitioning) depending on which transform combinations have zero-out, and how they are located.

The first factor can be identified by simply analyzing the coding gains on videos of different resolutions, while the others are equally important but subtler, because they involve codec design and code structure.

The existence of two types of logical transform blocks affect functions related to (a) transforms; (b) quantization; and (c) coefficient entropy coding. For that reason, the zero-out rules result in dozens of code “patches” on all those functions, in order to deal with all the cases where the two types of blocks have different dimensions and shapes.

Even in a codec that has been designed with an architecture to support such duality of transform blocks, those complications make the codec more expensive to implement, because it may be difficult to deal with the combinatorial number of possible cases and exceptions. Of course, those problems are much worse if the codec has not been originally designed to support different transform block types.

This contribution proposes alternatives to the zero-out technique that are (i) conceptually, self-contained and transform-only solutions, not requiring any other non-transform related changes in a codec design and (ii) are slightly more complex as compared to the zero-out technique.

In a nutshell, the proposal combines two techniques:

- For the 64-point DCT-2 only, a DCT-4 factorization using the same DCT-2 “partial-butterfly” that is part of the VTM code is proposed, and an additional low-complexity computation reducing (besides the DCT-2 one level partial butterfly) the matrix dimensions and number of multiplications by half. This is a well-known alternative computation of the DCT-2 and is not an approximation.

- For supporting the DCT-8 and DST-7, we use the Transform Adjustment method of JVET-M0538, which approximates those transforms using the DCT-2 as main computational engine. Since those transforms use the DCT-2, they are independent of how it is computed, so it can be readily used together with the DCT-4 factorization.

In the next sections it is explained why the proposed approach is suitable for both software and hardware implementations, and it is shown that its computational complexity analysis is comparable or below that of zero-out. The simulation results show that with this approach the coding gains of large transforms are recovered, especially for Classes A1 and A2.

# Summary of features

The main features of this proposal are:

- The minimum memory to required to compute all the DCT-2 transformations, plus the DCT-8 and DST-7 of dimensions 32 and 64, is only 437 bytes, compared to 1365 bytes currently required by the VTM-4.0, using zero-out, for the same transforms.

- Coding gains of large transforms are restored without increase in encoder or decoder speeds.

- New DCT-4 factorization into DCT-4s allows using the same “partial-butterfly” structure as the DCT-2.

Since this proposal uses the same Transform Adjustment method as in JVET-M0538, we repeat here some of the features listed in that document.

- With the adoption of 8-bit coefficients for primary transforms, the use of the DCT-2 in this proposal now corresponds to current HEVC implementations, and previously optimized transform SW and HW can be readily re-used.

- No additional HW is needed for supporting the forward DCT at decoder-only implementations.

- Arithmetic precision is maintained:

  - The coefficients of the 8×8 adjustment matrix are represented with 8 bits.

  - After the adjustment stages the computed values are re-normalized and “clamped” to guarantee that inputs to the inverse DCT-2 are limited to 16-bit integers.

- The band-matrix previously used for adjustment computations is replaced by a single small matrix, which was indicated as the preferred form of computation.

- All computations of the DCT-2 and the adjustments have very regular patterns, greatly simplifying efficient parallel implementation using SIMD instructions or custom HW (a SIMD-based code is shared as example).

- The additional memory (beyond that of used for the DCT-2) needed for computing the MTS transforms of dimensions 16, 32, and 64, is only 64 bytes. This represents a very significant reduction compared to the memory required to store the DST-7/DCT-8 matrices (cf. Section 4).

- Due to the “zero-out” technique used in DCT-2 of dimension 64, the worst-case transform complexity in VTM-3, which defines the custom HW costs, boils down to the matrix-based DST-7 and DCT-8 on blocks of size 32×32. In Section 6 it is shown how the worst-case number of multiplications per transform coefficient is reduced from 64 to 36 (i.e., approximately a 40% reduction).

- A comparison of simulation results in CE6-1.4c and CE6-1.4d show that MTS transforms of dimension 64 now have a reasonable computational complexity, which is practically equal to the DCT-2 transform of dimension 64 (when using “zero-out”). So, additional gains from extending MTS to larger blocks do not increase encoding or decoding times.

- Transform coefficient throughput measurements, based on the throughput profiler proposed in JVET-M0540, show significant improvements, especially at decoder inverse transformations (leading from 2 to 5 times decoder speedups).

- A separate proposal [JVET-M0539] (exploiting special properties of the DCT-2 and the corresponding adjustments for DST-7 and DCT-8,) describes how it is possible to significantly increase the throughput at the encoder side, because all four MTS combinations (DST-7 or DCT-8 for horizontal or vertical passes) can be computed with complexity similar to that of computing a single separable transformation.

# Proposed DCT-2 and DCT-4 factorization

Using TN(C2) and TN(C4) to represent the matrices of the DCT-2 and DCT-4 transforms of dimension N, the “partial-butterfly” implementation of the DCT-2 in VTM is based on the factorization

TN(C2)=PTN/2(C2)00TN/4(C4) H,                                                           (1)  ,

where P is an even-odd permutation matrix and the matrix H, with Hadamard parallel rotations, is defined by

Hi,j=1,i=j<N/2,1,i=N-1-j<N/2,1,i=j+N/2,-1,i=3N/2-1-j,0,otherwise.                                                      (2)

The DCT-2 can be recursively factorized using equation (1), for a complete or incomplete “partial-butterfly” factorization, with the advantage that, because of the factors in H, all versions using integer arithmetic produce the same results.

The DCT-4 can also be recursively factorized, for example using

TN(C4)=GTN/2(C2)00TN/4(C2) H  ,

where G is compose of parallel Givens rotations, and H has parallel Hadamard rotations, but not as regular as in H.

In this proposal the following alternative (and less known) factorization of the DCT-4

TN(C4)=GTN/2(C4)00TN/4(C4) H  ,

is used only for the 64-point DCT-2, to enable reducing its complexity. The advantage of this factorization is that it uses the simpler matrix H that is also used by the DCT-2, enabling its re-use. Furthermore, it uses the same scaling as the DCT-2, so it does not need memory to store coefficients beyond those needed for N = 32.

# Transform adjustments

The type of Transform Adjustments used in this proposal were proposed in JVET-M0538 and as a method for replacing computationally the costly trigonometric transforms DST-7, and DCT-8, with approximations that could

- Provide the codec with the same type of transform diversity as that provided by the trigonometric transforms, and thus yield practically the same coding gains;

- Be computed using only an “adjustment” of very low complexity, plus the DCT-2 as the “main computational engine,” since it is the transform already used in HEVC, has been widely studied and optimized, and enables re-using previously developed hardware and software implementations.

A diagram with the new method is shown in Fig. 1, where the adjustments are computed after the forward DCT-2, and their inverse before the inverse DCT-2. In this new version those adjustments correspond to simply multiplying the vector with the 8 lowest-frequency transform coefficients by an 8×8 matrix. The same matrix is used for transforms of dimension 16, 32, and 64, and for the DST-7 and DCT-8 adjustments.

Fig. 1 – Implementation of transform adjustment that needs only the inverse DCT-2 at the decoder. For transforms of dimension 32 and 64, the adjustment blocks correspond to multiplying the 8 lowest frequency transform coefficients by the same 8×8 matrix.

# Implementation

For a transform of dimension N = 16, 32, or 64, if c(DCT-2) is the N-dimensional vector with coefficients of the DCT-2 transform (in increasing frequency), we can split this vector as

c(DCT-2) = [  x | y ],

where x has dimension 8 (i.e., the coefficients of the 8 lowest frequencies) and y has dimension N – 8. The transform adjustments corresponding to the DCT-8 and DST-7 MTS options are defined as

ĉ(DCT-8) = [  A(DCT-8) x | y ],

ĉ (DST-7) = [  A(DST-7) x | y ].

where A is an 8×8 matrix (symbol ĉ is used instead of c to indicate that those are not the true DCT-8 and DST-7 coefficients).

Matrix A should be orthogonal and designed to maximize coding gain. In this proposal the matrix used to compute the orthogonal transform associated with the DST-7 is scaled by 128, and defined by the C++ code

const TMatrixCoeff g_TAF_DST7[8][8] =

{

{ 118,-48,-12, -2, -2, -1, -1,  0 },

{  38,108,-54,-15, -8, -6, -3, -2 },

{  24, 36,107,-51,-16, -9, -4, -4 },

{  14, 23, 34,111,-44,-13, -8, -5 },

{  10, 16, 19, 29,114,-38,-14, -8 },

{   8, 13, 13, 14, 26,118,-33,-12 },

{   6,  9,  9, 11, 15, 24,120,-28 },

{   5,  8,  8,  8, 11, 14, 23,124 }

};

while the matrix corresponding to the DCT-8 computation is

const TMatrixCoeff g_TAF_DCT8[8][8] =

{

{ 118, 48,-12,  2, -2,  1, -1,  0 },

{ -38,108, 54,-15,  8, -6,  3, -2 },

{  24,-36,107, 51,-16,  9, -4,  4 },

{ -14, 23,-34,111, 44,-13,  8, -5 },

{  10,-16, 19,-29,114, 38,-14,  8 },

{  -8, 13,-13, 14,-26,118, 33,-12 },

{   6, -9,  9,-11, 15,-24,120, 28 },

{  -5,  8, -8,  8,-11, 14,-23,124 }

};

It can be observed that the elements of the two matrices have same magnitudes, but different signs, according to a chessboard pattern. This is not a coincidence and is a consequence of properties of the DCT-2, and DCT-8 and DST-7. Thus, only one of these matrices need to be stored, and used in the matrix-vector computations (as shown in the code for this CE). For various optimized HW or SW implementations it may be more useful to store both versions, yet it can be represented using a single 8x8 matrix requiring only 64 additional bytes of memory. Since the matrices above are scaled versions of orthogonal matrices, their inverses are defined by simply transposing (or left-side multiplication).

Note that in this proposal the same matrix coefficients are used for transforms of dimensions 16, 32, and 64, and thus the total memory requirement, beyond the DCT-2, is only 64 bytes.

# Simulation results

The following simulations were done in a heterogenous cluster, so time measurements are not accurate. Results with more precise encoder and decoder times will be available in future versions of this document.

## CTC (inter MTS disabled)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.59% | -0.12% | -0.21% | 81% | 93% |
| Class A2 | -0.27% | 0.29% | 0.17% | 79% | 93% |
| Class B | -0.14% | 0.12% | 0.13% | 78% | 95% |
| Class C | -0.06% | -0.08% | -0.10% | 88% | 94% |
| Class E | -0.28% | 0.03% | -0.10% | 87% | 98% |
| Overall | -0.24% | 0.05% | -0.01% | 82% | 95% |
| Class D | -0.07% | 0.01% | -0.09% | 86% | 94% |
| Class F | 0.03% | 0.05% | -0.12% | 104% | 96% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.17% | 0.00% | -0.33% | 87% | 96% |
| Class C | -0.05% | -0.13% | -0.29% | 90% | 97% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.08% | -0.16% | 0.04% | 92% | 95% |
| Class F | -0.05% | -0.05% | -0.10% | 100% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.20% | 0.32% | -0.29% | 82% | 96% |
| Class E | 0.05% | 0.39% | 0.10% | 82% | 100% |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.10% | -0.31% | 86% | 89% |
| Class F |  |  |  |  |  |

## CTC + inter MTS enabled

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | 0.17% | -0.06% | 102% | 101% |
| Class C | -0.05% | -0.03% | -0.13% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.15% | 0.04% | -0.04% | 104% | 98% |
| Class F | -0.03% | 0.00% | -0.10% | 97% | 101% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.19% | 0.16% | -0.03% | 115% | 105% |
| Class E | 0.01% | 0.43% | 1.44% | 102% | 102% |
| Overall |  |  |  |  |  |
| Class D | -0.09% | -0.11% | 0.43% | 100% | 96% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

## Low QP test

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 - low QP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.12% | -0.18% | 100% | 96% |
| Class A2 | -0.09% | 0.13% | 0.10% | 101% | 101% |
| Class B | -0.01% | 0.00% | 0.02% | 101% | 98% |
| Class C | 0.02% | 0.06% | 0.10% | 97% | 97% |
| Class E | -0.09% | 0.14% | -0.06% | 99% | 106% |
| Overall | -0.05% | 0.04% | 0.01% | 100% | 99% |
| Class D | -0.02% | 0.11% | 0.09% | 101% | 100% |
| Class F | 0.02% | -0.03% | 0.06% | 100% | 99% |

# Conclusions

This contribution proposes to use the Transform Adjustment (JVET-M0538) method to approximate DST-7 and DCT-8 of larger dimensions instead of zeroing-out. It has been experimentally demonstrated that the proposed method can considerably outperform the zero-out method in terms of coding gains achieved on higher-resolution sequences (class A1 and A2) at a reasonable complexity.

It is recommended to adopt the proposed Transform Adjustment method for efficient DST-7 and DCT-8 implementations and further study the proposed DCT-2 factorization in a CE.

# References

- A. Said et al., “CE6-1.4:  Efficient Implementations of MTS with Transform Adjustments,” JVET-M0538, 13th meeting, Marrakech, MA, Jan. 2019.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

For consistency with signal processing notation, the matrix indexes are in the range 0, 1, …, N – 1.