| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0491-r2 |
| --- | --- |

| Title: | CE6-related: Transform Candidate Ordering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christopher Hollmann, Davood Saffar, Jacob Ström, Per Wennersten Torshamnsgatan 23 16480 Stockholm, Sweden | Tel: Email: | +46107190000 {christopher.hollmann, davood.saffar, jacob.strom, per.wennersten} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This proposal describes a modification of the transform selection in the MTS tool. It consists of three changes: removing the combination (DCT-8, DCT-8), replacing the size-32 DCT-8 with the existing DCT-2, and adding an algorithm to sort the combinations (DST-7, DCT-8) and (DCT-8, DST-7) based on block characteristics.

Two different configurations are tested: The default CTC configuration where at most 3 candidates besides (DCT-2, DCT-2) are tested for intra-predicted blocks and at most 4 candidates for inter-predicted blocks, and a configuration where at most 2 candidates are tested for both intra and inter. When comparing the CTC configuration with the anchor [T1], a gain of -0.04% / -0.01% / -0.04% AI/RA/LD is reported with unchanged encoder and decoder run times. When comparing the variant of testing 2 candidates to an anchor testing 2 candidates [T3], the reported gain is -0.07% / -0.02% / -0.03% AI/RA/LD without significant run time changes. A third test comparing the proposed software with 2 candidates to the CTC anchor [T2] reportedly shows a loss of 0.06% / 0.03% / 0.02% AI/RA/LD with a reported encoder run time reduction to 87% / 98% / 97% AI/RA/LD.

In the case when inter MTS is enabled, the reported results are 0.01% / 0.05% RA/LD for the test with default values [T1], -0.02% / -0.03% RA/LD for the case with 2 candidates in both test and anchor [T3], and 0.14% / 0.15% RA/LD for the third case with 2/2 candidates in the proposal and 3/4 candidates in the anchor [T2]. Encoder run times are 94%/95% [T1], 99%/98% [T3], and 92%/93% [T2] RA/LD for the three test cases.

In version 2 of this document, results for inter MTS were provided.

# Introduction

In the current VTM-4.0, there are a total of six different transform modes:

- (DCT-2, DCT-2), the default transform,

- TransformSkip, a method that does not use any transform and is mostly used for screen content, and

- Four combinations of DST-7 and DCT-8, commonly referred to as MTS candidates.

The encoder determines a cost estimate for each mode by performing a forward transform and summing up the absolute coefficient magnitudes. A threshold is calculated by multiplying the estimated cost of the default mode (DCT-2, DCT-2) with a factor based on the size of the current transform unit. The estimated costs of the remaining modes are compared to the threshold, and if a cost estimation is over the threshold, the respective mode will not be evaluated. Furthermore, the maximum number of modes that can be evaluated, can be limited by setting the encoder configuration parameter MTSIntraMaxCand to a value between one and five. Correspondingly, the parameter MTSInterMaxCand can be used to limit the number of evaluated candidates in case MTS is used for inter-predicted blocks.

The default value in the CTC for intra is three, which means that at most three of the five possible candidates (TransformSkip and the four MTS candidates) are evaluated.

# Technology Description

The changes to the software can be divided into three parts:

- Remove combination (DCT-8, DCT-8) from consideration in the MTS tool. As this combination is rarely used, there is no good reason to keep the combination in the tool. Removing it allows the combination (DST-7, DCT-8) to be coded with one less bin which increases compression efficiency.

- Replace size-32 DCT-8 with DCT-2. The size-32 DCT-8 can be replaced by the existing DCT-2 without a significant impact on compression efficiency. Furthermore, no transform coefficients need to be stored for the size-32 DCT-8, which reduces required storage.

- Dynamic sorting of (DST-7, DCT-8) and (DCT-8, DST-7). The order in which (DST-7, DCT-8) and (DCT-8, DST-7) are evaluated and coded are switched based on the intra prediction mode (for intra-predicted blocks) or the block shape (for inter-predicted blocks).

While the first two parts are also part of the software tested in CE6-2.3 (see document JVET-Nxxxx [2]), the third part differs. In this proposal, the order in which the two candidates are evaluated is determined by the intra prediction mode or, for inter-predicted blocks, the block shape. Adjusting the order in which candidates are evaluated matters mostly in cases where the encoder is not configured to test all combinations, but only the first N combinations (with N being set in the configuration). In this case it is possible that only one of the combinations (DST-7, DCT-8) or (DCT-8, DST-7) is evaluated, and the change ensures that the tested one is the one that is more likely to compress the block more efficiently.

A detailed overview of the order in which candidates are evaluated is shown in the following table.

| Prediction | Condition | First candidate | Second candidate |
| --- | --- | --- | --- |
| Intra MTS | Intra mode > 34 | (DCT-8, DST-7) | (DST-7, DCT-8) |
|  | Intra mode <= 34 | (DST-7, DCT-8) | (DCT-8, DST-7) |
| Inter MTS | Height > Width | (DCT-8, DST-7) | (DST-7, DCT-8) |
|  | Height <= Width | (DST-7, DCT-8) | (DCT-8, DST-7) |

# Test Results

Tests are based on VTM-4.0. Both anchor and test include a bugfix for ticket #196 [3] which affects MTS for inter-predicted blocks in class F.

The test results include several different tests which differ in the number of candidates tested. The software itself is identical in all tests. In the first test T1, both anchor and the test software evaluate up to 3 candidates for intra-coded blocks and 4 candidates for inter-coded blocks. These are the default values in the CTC configuration. In the second test T2, the test software evaluates only up to two candidates for both intra- and inter-coded blocks while the anchor is identical to T1. In the third test T3, the test software once again only evaluates 2 candidates for intra- and inter-coded blocks, but here the anchor also only evaluates 2 candidates.

Looking at the results shows that when both test software and anchor are configured to evaluate the same number of candidates (tests T1 and T3), the compression efficiency is increased. At the same time, there is only a small impact on the encoder run times. When looking at test T2, there is a small loss in compression efficiency, but a significant reduction in encoder run time.

The table shown in section 3.1 summarizes the results. Missing results and run times will be updated in a later revision.

## Summary

| ID | Cand. Anchor | Cand. Test | MTS only for intra (MTS = 1) |  |  |  |  |  | MTS for intra and inter (MTS = 3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | BDR-Y |  |  | EncT/DecT |  |  | BDR-Y |  | EncT/DecT |  |
|  | (num intra, num inter) |  | AI | RA | LD | AI | RA | LD | RA | LD | RA | LD |
| T1 | (3, 4) | (3, 4) | -0.04% | -0.01% | -0.04% | 98%/ 102% | 97%/ 95% | 98%/ 95% | 0.01% | 0.05% | 94%/ 96% | 95%/ 95% |
| T2 | (3, 4) | (2, 2) | 0.06% | 0.03% | 0.02% | 87%/ 101% | 98%/ 95% | 97%/ 94% | 0.14% | 0.15% | 92%/ 97% | 91%/ 93% |
| T3 | (2, 2) | (2, 2) | -0.07% | -0.02% | -0.03% | 97%/ 98% | 100%/ 98% | 99%/ 99% | -0.02% | -0.03% | 99%/ 99% | 98%/ 96% |

## Detailed Results

### T1 without inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (3, 4) [T1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.09% | 0.01% | 100% | 104% |
| Class A2 | -0.04% | 0.08% | 0.02% | 97% | 101% |
| Class B | -0.04% | 0.02% | 0.04% | 99% | 106% |
| Class C | -0.01% | -0.04% | -0.08% | 98% | 99% |
| Class E | -0.05% | -0.07% | 0.04% | 96% | 97% |
| Overall | -0.04% | 0.01% | 0.01% | 98% | 102% |
| Class D | -0.01% | 0.12% | -0.04% | 96% | 94% |
| Class F | 0.01% | -0.05% | -0.08% | 98% | 104% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (3, 4) [T1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.16% | 0.05% | 96% | 94% |
| Class A2 | -0.04% | -0.04% | -0.02% | 97% | 95% |
| Class B | 0.00% | 0.03% | -0.13% | 96% | 94% |
| Class C | 0.02% | 0.01% | -0.11% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.07% | 97% | 95% |
| Class D | 0.02% | -0.10% | -0.09% | 97% | 96% |
| Class F | -0.06% | -0.21% | -0.03% | 100% | 96% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (3, 4) [T1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.25% | -0.10% | 99% | 96% |
| Class C | -0.04% | 0.02% | -0.18% | 98% | 98% |
| Class E | -0.10% | 0.02% | 0.72% | 98% | 89% |
| Overall | -0.04% | -0.09% | 0.08% | 98% | 95% |
| Class D | 0.01% | 0.16% | 0.30% | 99% | 100% |
| Class F | -0.11% | -0.01% | 0.08% | 95% | 90% |

### T1 with inter MTS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3, (3, 4) [T1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | -0.10% | 91% | 91% |
| Class A2 | 0.04% | 0.05% | -0.12% | 93% | 96% |
| Class B | -0.02% | 0.03% | -0.12% | 96% | 96% |
| Class C | 0.03% | 0.11% | -0.03% | 95% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | -0.09% | 94% | 96% |
| Class D | -0.01% | -0.09% | -0.11% | 96% | 96% |
| Class F | -0.08% | -0.19% | -0.03% | 93% | 96% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3, (3, 4) [T1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.08% | -0.24% | -0.30% | 94% | 92% |
| Class C | 0.05% | -0.03% | -0.06% | 94% | 97% |
| Class E | 0.00% | 0.25% | 1.82% | 98% | 97% |
| Overall | 0.05% | -0.05% | 0.31% | 95% | 95% |
| Class D | 0.02% | 0.47% | 0.23% | 93% | 94% |
| Class F | 0.08% | 0.36% | -0.01% | 95% | 91% |

### T2 without inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (3, 4) [T2] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.10% | -0.05% | 87% | 101% |
| Class A2 | 0.09% | -0.02% | -0.06% | 86% | 100% |
| Class B | 0.06% | -0.08% | -0.04% | 86% | 103% |
| Class C | 0.10% | -0.14% | -0.14% | 87% | 101% |
| Class E | 0.02% | -0.06% | -0.20% | 87% | 100% |
| Overall | 0.06% | -0.08% | -0.09% | 87% | 101% |
| Class D | 0.12% | -0.10% | -0.25% | 87% | 98% |
| Class F | 0.05% | -0.13% | -0.04% | 92% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (3, 4) [T2] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.15% | -0.05% | 96% | 95% |
| Class A2 | -0.01% | 0.02% | -0.04% | 98% | 95% |
| Class B | 0.05% | -0.15% | -0.11% | 99% | 96% |
| Class C | 0.05% | -0.21% | -0.25% | 98% | 95% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.13% | -0.12% | 98% | 95% |
| Class D | 0.05% | -0.48% | -0.04% | 100% | 95% |
| Class F | 0.05% | -0.14% | -0.20% | 107% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (3, 4) [T2] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.29% | -0.40% | 98% | 95% |
| Class C | 0.00% | 0.52% | 0.10% | 96% | 95% |
| Class E | 0.07% | -0.53% | -0.12% | 97% | 91% |
| Overall | 0.02% | -0.08% | -0.16% | 97% | 94% |
| Class D | -0.02% | -0.07% | -0.59% | 99% | 97% |
| Class F | -0.16% | -0.28% | -0.91% | 94% | 90% |

### T2 with inter MTS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3 (3, 4) [T2] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | -0.04% | -0.08% | 89% | 94% |
| Class A2 | 0.18% | -0.02% | -0.28% | 91% | 97% |
| Class B | 0.15% | -0.22% | -0.09% | 93% | 98% |
| Class C | 0.12% | -0.26% | -0.11% | 94% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.14% | -0.15% | -0.13% | 92% | 97% |
| Class D | 0.12% | -0.41% | -0.04% | 90% | 96% |
| Class F | 0.07% | -0.08% | -0.18% | 89% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3 (3, 4) [T2] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -0.29% | -0.67% | 90% | 90% |
| Class C | 0.14% | 0.34% | -0.08% | 90% | 97% |
| Class E | 0.13% | -0.57% | 0.36% | 93% | 95% |
| Overall | 0.15% | -0.15% | -0.22% | 91% | 93% |
| Class D | 0.17% | 0.19% | -0.56% | 93% | 99% |
| Class F | -0.11% | -0.27% | -0.07% | 93% | 93% |

### T3 without inter MTS

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (2, 2) [T3] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.03% | -0.01% | 96% | 97% |
| Class A2 | -0.07% | -0.08% | 0.04% | 97% | 95% |
| Class B | -0.06% | 0.01% | 0.04% | 97% | 99% |
| Class C | -0.03% | 0.11% | -0.06% | 97% | 97% |
| Class E | -0.10% | -0.02% | -0.07% | 98% | 99% |
| Overall | -0.07% | 0.00% | -0.01% | 97% | 98% |
| Class D | -0.04% | -0.02% | 0.10% | 101% | 98% |
| Class F | -0.01% | -0.01% | 0.02% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (2, 2) [T3] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.08% | 98% | 99% |
| Class A2 | -0.02% | -0.01% | -0.03% | 99% | 93% |
| Class B | -0.01% | -0.12% | 0.02% | 101% | 100% |
| Class C | -0.05% | -0.23% | -0.08% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.11% | -0.04% | 100% | 98% |
| Class D | -0.02% | -0.46% | 0.19% | 101% | 99% |
| Class F | 0.01% | -0.02% | -0.03% | 94% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 MTS (2, 2) [T3] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.14% | -0.43% | 98% | 97% |
| Class C | -0.01% | 0.28% | 0.41% | 102% | 102% |
| Class E | -0.09% | -0.74% | -0.69% | 98% | 98% |
| Overall | -0.03% | -0.15% | -0.21% | 99% | 99% |
| Class D | 0.02% | -0.09% | 0.04% | 98% | 97% |
| Class F | 0.00% | -0.61% | -0.86% | 101% | 102% |

### T3 with inter MTS

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3 (2, 2) [T3] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | 0.05% | 98% | 99% |
| Class A2 | -0.03% | -0.09% | -0.14% | 97% | 98% |
| Class B | -0.02% | -0.13% | -0.06% | 99% | 99% |
| Class C | -0.03% | -0.20% | -0.03% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.11% | -0.05% | 99% | 99% |
| Class D | -0.01% | -0.21% | 0.31% | 98% | 101% |
| Class F | 0.02% | 0.09% | 0.06% | 94% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0, MTS = 3 (2, 2) [T3] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.04% | -0.21% | 97% | 93% |
| Class C | 0.01% | 0.29% | -0.01% | 98% | 98% |
| Class E | -0.18% | -0.63% | -0.85% | 99% | 100% |
| Overall | -0.03% | -0.04% | -0.31% | 98% | 96% |
| Class D | 0.07% | 0.17% | -0.49% | 103% | 103% |
| Class F | -0.20% | 0.20% | 0.13% | 97% | 95% |

# Recommendation

We propose adopting the changes described above into the next version of the VVC draft and the VVC test model. Furthermore, as there is only a minimal impact on compression efficiency while the run time is reduced, we propose setting the default value of MTSIntraMaxCand and MTSInterMaxCand to 2.

# References

[1] C. Hollmann, D. Saffar, P. Wennersten, J. Ström. CE-6 related: Transform Simplification, JVET-M0366, Marrakech, Morocco, January 2019

[2] C. Hollmann, D. Saffar, P. Wennersten, J. Ström. CE6: Transform Simplification (CE6-2.3a-c), JVET-N0490, Geneva, Switzerland, March 2019

[3] VVC Bug Tracker. Ticket #196: Enc/Dec mismatch for Inter MTS in class F, https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/196, February 2019

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).