| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0534 |
| --- | --- |

| Title: | Crosscheck of JVET-N0123 (AHG13/Non-CE6/CE8: Chroma Transform Skip) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-N0123, regarding the following two tests:

- Test#1 (TSM_1): Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 enabled

- Test#2 (TSM_2): Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including bugfix of ticket#185)

Simulations results which are partially available in this crosscheck reportedly match with the results provided by proponent of JVET-N0123.

# Crosscheck report

In JVET-N0123, additional tool off tests are carried out for AHG13 as described below:

- TSM_1: Restrict transform skip size up to 4x4 on top of VTM-4.0 (JVET-M0464 enabled)

  - Encoder option: --MTS=1 --TransformSkipLog2MaxSize=2

  - Chroma Transform skip: N/A

  - Configuration for TSM_1 are found in config_TSM_1.

- TSM_2: Restrict transform skip size up to 4x4 on top of VTM-4.0 with JVET-M0464 disabled (including ticket#185)

  - Encoder option: --EMT=1 --EMTFast=1 --TransformSkipLog2MaxSize=2 with JVET-M0464 macro disabled

  - Chroma Transform skip: 2x2/4x2/2x4/4x4 can be used

  - Configuration and source changes for TSM_2 are found in config_TSM_2.

The anchor used for both TSM_1 and TSM_2 tests are VTM-4.0 simulated using common test condition (CTC) [1]. Below figure summarizes the results.

Table 1: Coding performance of TSM_1.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.24% | 0.20% | 0.18% | 102% | 98% | 0.05% | -0.31% | -0.20% | 98% | 100% |  |  |  |  |  |
| Class A2 | 0.14% | 0.40% | 0.24% | 100% | 99% | 0.04% | 0.12% | 0.12% | 100% | 101% |  |  |  |  |  |
| Class B | 0.23% | 0.38% | 0.39% | 100% | 97% | 0.17% | -0.08% | -0.17% | 99% | 99% |  |  |  |  |  |
| Class C | 0.32% | 0.35% | 0.38% | 100% | 101% | 0.35% | -0.04% | 0.01% | 98% | 103% | 0.37% | 0.06% | -0.21% | 95% | 100% |
| Class E | 0.27% | 0.53% | 0.41% | 99% | 98% |  |  |  |  |  | -0.23% | 0.38% | 0.13% | 99% | 101% |
| Overall | 0.24% | 0.37% | 0.33% | 100% | 99% | 0.17% | -0.07% | -0.07% | 99% | 101% |  |  |  |  |  |
| Class D | 0.30% | 0.27% | 0.24% | 102% | 101% | 0.54% | -0.19% | -0.58% | 97% | 101% | 0.53% | 0.10% | 0.32% | 95% | 101% |
| Class F | 1.96% | 1.42% | 1.39% | 98% | 102% | 1.80% | 1.04% | 1.05% | 98% | 102% | 2.03% | 0.85% | 0.78% | 95% | 102% |

Table 2: Coding performance of TSM_2.

|  | All Intra Main10 |  |  |  |  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.05% | 0.00% | 99% | 97% | -0.09% | -0.27% | -0.58% | 102% | 98% |  |  |  |  |  |
| Class A2 | -0.06% | 0.04% | -0.03% | 102% | 99% | -0.04% | -0.03% | -0.22% | 101% | 98% |  |  |  |  |  |
| Class B | 0.00% | 0.03% | 0.09% | 100% | 96% | -0.01% | -0.02% | -0.37% | 102% | 98% | 0.00% | -0.49% | -0.60% | 101% | 99% |
| Class C | 0.05% | 0.02% | -0.01% | 100% | 101% | 0.12% | -0.11% | -0.50% | 102% | 102% | 0.29% | -0.03% | 0.05% | 101% | 99% |
| Class E | 0.06% | -0.07% | -0.10% | 93% | 99% |  |  |  |  |  | -0.23% | 0.35% | -0.61% | 101% | 100% |
| Overall | 0.02% | 0.02% | 0.00% | 99% | 98% | 0.00% | -0.10% | -0.42% | 102% | 99% | 0.04% | -0.12% | -0.39% | 101% | 99% |
| Class D | 0.06% | 0.03% | -0.09% | 103% | 102% | 0.39% | -0.45% | -0.66% | 101% | 103% | 0.48% | 0.07% | 0.31% | 98% | 103% |
| Class F | 1.68% | 1.09% | 1.04% | 93% | 103% | 1.58% | 0.82% | 0.79% | 101% | 102% | 1.76% | 0.98% | 0.90% | 98% | 98% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-M1010, 13th meeting, Marrakesh, MA, 9–18 Jan. 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).