| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0724-v1 |
| --- | --- |

| Title: | BoG report on general high level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jill Boyce | Email: | jill.boyce@intel.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met on 19 Mar 2019 from 17:30 to 20:00. The BoG met again on 20 Mar 2019 from 09:00 to 13:00. The BoG expects to meet again.

The BoG recommends the following:

- Adopt a Decoder Parameter Set (DPS), from JVET-N0349 and previously proposed in JVET-M0101

- Adopt signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header, from JVET-N0100

# Contribution Review

### General high-level syntax (31)

#### NAL unit header and general aspects of parameter sets (6)

JVET-N0050 AHG17: On Forbidden-Zero Bit in NAL Unit Header [S. Wenger, B. Choi, S. Liu (Tencent)]

The authors propose removing the “forbidden_zero_bit” from the NAL unit header and re-allocating that bit for other purposes, such as an extension of the NAL unit type numbering space, unless compelling reasons are found and documented that continue to justify the bit’s existence.  We also wonder whether the restriction of the values for the temporal ID in the HEVC NAL unit header, and the complex language regarding SODB/RBSP (section 7.4.2.3 of JCTVC-M1001-v5) can be released/removed, as the authors’ admittedly dim recollection is that such restriction was introduced for essentially the same purpose.

It was suggested that avoiding PES start code emulation was a motivation for the existence of the forbidden_zero_bit in earlier codecs.

JVET-N0067 is also related.

JVET-N0051 AHG17: On NAL Unit header design for VVC [S. Wenger, B. Choi, S. Liu (Tencent)]

This contribution proposes an update to the NAL unit header design for VVC.  At the expense of an—the authors assert insignificant—increase in processing power demands, and certain limitations in extensibility, the NAL Unit header length can be kept at 8 bits.  Extensibility concerns can be addressed by using an extension mechanism similar to the one proposed in JVET-M0520 of the Marrakech meeting.

The reduction of the NAL unit header size from 16 bits to 8 bits results in bitrate savings.  Quantifying those bitrate savings is difficult as they depend highly on the number of tile groups per coded picture, number of SEI messages and marker NAL units per picture, GOP structure and resulting parameter set NAL units for GOP, and so forth.

It was suggested that this proposal could be combined with JVET-N0067 to free up some additional NUT values.

It was suggested that a reserved NUT value could be used to enable an extension of the NUH length, such as to be able to carry layer_id, but it was noted that it couldn’t be used in combination with another NUT value.

JVET-N0067 AHG17: On the first byte of the NAL unit header [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)] [late]

The contribution proposes to remove forbidden_zero_bit from the NAL unit header. It is asserted that MPEG-2 Program Elementary Stream (PES) start code emulation is avoided with the proposed syntax element arrangement in the first byte of the NAL unit header. The proposal can be summarized as follows:

- The following first byte of the NAL unit header syntax is proposed:

| nal_unit_header( ) { | Descriptor |
| --- | --- |
| zero_tid_required_flag | u(1) |
| nuh_temporal_id_plus1 | u(3) |
| nal_unit_type_lsb | u(4) |
| … // remaining part of the NAL unit header |  |
| } |  |

- NalUnitType is set equal to ( zero_tid_required_flag << 4 ) + nal_unit_type_lsb.

- The assignment of NAL unit type values to types is rearranged. zero_tid_required_flag shall be equal to 1 for DPS, SPS, EOS, EOB, and IRAP NAL units, which therefore have NalUnitType in the range of 16 to 31, inclusive. The proposed number and assignment of reserved NAL unit types differs from that in VVC draft 4.

- When zero_tid_required_flag is equal to 1, nuh_temporal_id_plus1 shall be equal to 1 (i.e., TemporalId shall be equal to 0).

The contribution leaves the discussion of the content of the remaining part of the NAL unit header beyond the first byte, if any, to other contributions. If no decision on the remaining part of the NAL unit header is reached, it is proposed to add one additional reserved bit to nuh_reserved_zero_7bits, i.e. make it nuh_reserved_zero_8bits.

Avoid values of 188 to 255, to avoid PES start code values.

It was noted that with this proposal, all unspecified values would require temporal id equal to 0. It was suggested that using a value as an escape to add extra bytes eliminates the possibility of using the types.

Is it possible to use a bit as an extension? Could an extension use a partial bit instead of the full bit? It was suggested that new NUT values could use the extension.

This proposal allows reclaiming of the temporal id bits for NUT values that don’t need temporal id, e.g. because they are required to have temporal id equal to 0.

The question of temporal sub-bitstream extraction for this contribution was raised.

It was suggested to review the proposals on layer id before making any recommendations.

The basic philosophy of N0067 and N0051 is to be more efficient in the use of NUH values by not requiring simple assignment of contiguous integer bits to syntax elements. There was support for this general concept.

Revisit.

JVET-N0278 AHG17: On VVC HLS relevant to MPEG requirements on immersive media delivery and access (N18134) [J. Boyce (Intel)] [late]

TBP

JVET-N0511 AHG17: Signalling leading picture information in the NAL unit header structure [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

This contribution is a continuation of JVET-N0072 to signal the leading picture related information in the NAL unit header structure. The information can be used by file writer, middle box, or system applications. The suggested syntax and semantics are provided.

Syntax elements in the NUH are proposed to replace reserved bits: num_leading_pictures u(4), and leading_picture_poc_cnt u(5).

The need for including this information in the NUH was questioned. It was suggested that the tile group header might be a more suitable location. The sps_max_dec_pic_buffering syntax element might serve a similar purpose already.

No action.

JVET-N0349 AHG17: On Parameter Set Design for VVC [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]

This contribution proposes to keep following the design principle of using the same picture parameter set for all NAL units of a picture. It also proposes to adopt the decoder parameter set (DPS) that was proposed in JVET-M0101 and discussed in the BoG on high level syntax, but was accidentally missed in track review.

This is relevant to the coded picture region category of combinations. The current VVC Draft 4 design already requires that the same PPS be used for all NAL units of the picture.

The DPS applies to an entire bitstream, beyond a single CVS. It was suggested that the two reserved syntax elements in the syntax could be combined.

It was suggested that syntax elements in the DPS could be included in VUI or SEI. It was noted that contributions at this meeting propose conditional syntax in other NAL units based on constraint flags in the DPS.

BoG recommends to adopt the DPS, as proposed here and in M0101.

#### Reference picture management (4)

JVET-N0058 AHG17: On decoded picture buffer management [B. Choi, S. Wenger, S. Liu (Tencent)] [late]

The contribution proposes the following items to improve the decoded picture buffer management, by addressing the temporal sub-layer adaptation:

Proposal 1: TRAIL_NUT, STSA_NUT, RASL_NUT and RADL_NUT are re-specified as (TRAIL_N, TRAIL_R), (STSA_N, STSA _R), (RASL_N, RASL_R) and (RADL_N, RASL_R) to indicate non-referenced pictures in the same sub-layer. Accordingly, non-referenced pictures having the temporal ID equal to that of the current picture are included in the RefPicList[ i ].

Proposal 2: sps_max_dec_pic_buffering_minus1 is signaled for each highest temporal id in SPS.

Proposal 3: The list of unused reference pictures for each highest temporal id is signaled in tile group header.

Proposal 4: All reference pictures with TemporalId greater than HighestTid are marked as "unused for reference", when the value of HighestTid is not equal to sps_max_sub_layers_minus1.

It was noted that the proposed definition of non reference pictures differs from what HEVC used, where it was a sub-layer non-reference picture.

It was suggested that it is limiting to restrict all references, rather than just sub-layer non-reference.

It was suggested to only provide separate NUT values for TRAIL.

It was suggested to use a fixed position in the tile group header to indicate non-reference, such as the discardable_flag in HEVC’s slice header.

Should a non-reference indication be per sub-layer or global?

It was suggested that if tile group header signaling were used, there would be room to include indication for both global and per sub-layer.

So far, we are using 16 of the 32 NUT values in the design (counting a new one for DPS).

Revisit Proposal 1 after contributions for new NUT values are reviewed.

Proposal 2 is HRD related, and should be considered in that discussion.

An issue with the syntax of Proposal 3 was raised.

For Proposal 4, it isn’t clear that the specification would need to change to realize the asserted benefit.

JVET-N0100 AHG17: On Reference Picture List Signalling [S. Deshpande (Sharp)]

Modifications are proposed to reference picture list signalling. Following is proposed:

- Proposal 1: It is proposed to allow signalling information about long-term reference picture POC LSB in reference picture list syntax structure or collocated with corresponding long-term reference picture’s delta POC MSB related syntax elements in the tile group header.

- Proposal 2: A conditional signaling is proposed for signaling of num_ref_idx_active_override_flag. An inference rule is proposed when this flag is not signaled.

- Proposal 3: A bug-fix is proposed for calculation of FullPocLt[ i ][ RplsIdx[ i ] ][ j ]. It is asserted that without this bug-fix, the incorrect value of poc_lsb_lt signalled in ref_pic_list_struct( listIdx, rplsIdx ) is used for the calculation of FullPOCLt – which provides full POC for a long term-reference picture.

Proposal 1 saves for long term reference picture list signaling for some conditions. It was questioned if long term reference pictures are even needed anymore. It was questioned if the VTM software encoder has the capability to use long term reference pictures.

It was suggested to consider if signaling in the SPS is necessary.

BoG recommends adoption of Proposal 1.

Proposal 2 saves a bit in the tile group header in some cases, when override flag isn’t useful.

It was suggested that the condition could be simplified.

Revisit proposal 2 after offline study to simplify the condition if possible.

Proposal 3 is included within Proposal 1.

JVET-N0135 AHG17: On reference picture list [Y. Fujimoto, T. Suzuki (Sony)] [late]

In the current VVC design, ref_pic_list_idx and ref_pic_list_struct are signaled in tile_group_header. However, when more than one tile_group exist, the same value must be duplicated in each tile group. When ref_pix_list_struct is signaled, only long term reference is updated in many cases. However, in the current design, both long term and short term references must be updated. When more than one tile_group exist, whole set of long term and short term must be duplicated in all tile_group. It is redundant.

This contribution proposes 1) new NAL unit to send parameters frequently changed, e.g. ref_pic_list_idx and ref_pic_list_struct. The parameters in this NAL unit are not changed within AU. 2) the syntax change to update long term reference picture list, which is defined in ref_pic_list_struct in SPS. Only long term reference can be updated in AU.

It was suggested that there are some advantages to allowing the flexibility of list orders in different tile group.

It was suggested that the second identified problem can be addressed by JVET-N0100 Proposal 1.

JVET-N0136 AHG17: On non-referenced picture [Y. Fujimoto, T. Suzuki (Sony)] [late]

In the current VVC design, there is no syntax to indicate whether the current picture is reference picture or non-reference picture. If there is no such syntax element at higher layer of syntax, it is hard to identify if the current picture can be discarded. This contribution proposes to include a flag to identify whether current picture is non-reference picture. Three methods are proposed.

It was suggested that if the second option of adding ref_pic_flag to the AUD, were used with multi-layer coding, wouldn’t allow different values for different layers.

It was commented that the HEVC design of nal_ref_idc indicated referencing or not within a temporal sub layer. HEVC has NUT values that distinguish between reference and non-reference.

It was suggested that a fixed position in the tile group header could be used for this purpose.

It was suggested that temporal_id could be used for this purpose.

It was suggested that the two trailing picture NUT values be used to differentiate between reference and non-reference.

The contribution asserts that in HEVC if temporal sub-layers are used, the HRD parameters must be sent per sub-layer. This could be changed for VVC.

JVET-N0058 was suggested to be related.

#### APS and tile group header (8)

JVET-N0065 AHG17 & AHG9: Comments on carriage of coding tool parameters in Adaptation Parameter Set [M. Li, P. Wu (ZTE)]

In 13th JVET meeting, Adaptation Parameter Set (APS) has been introduced into Versatile Video Coding (VVC) standard text. A few years ago, the APS was once adopted into High Efficiency Video Coding (HEVC) standard for carrying coding tool parameters, such as Adaptive Loop Filter (ALF) parameters; but the APS was removed from HEVC standard along with the removal of ALF as a coding tool at a final HEVC standardisation stage. The APS was designed to carry coding tool parameters as a picture level adaptive nature and alternatively the APS data can also be maintained as unchanged for the whole video sequence for avoiding resending them unnecessarily. This contribution is for information only and it is commented and recommended to use the APS to carry the coding tool data, such as neural networks parameters and affine linear weighted intra prediction parameters, etc., if some associated coding tools are indeed to be adopted into VVC standard. If more coding tool data need to be carried by APS in the future, it commented that further study on updating parameters of multiple tools using APS would be needed.

The contribution suggests that the APS would be an appropriate place to signal parameters for a CNN loop filter, if such a tool were adopted.

Contribution is for information.

JVET-N0069 AHG16/AHG17: Proposed Cleanup for Reshaper High Level Syntax [B. Heng, M. Zhou, W. Wan (Broadcom)]

The following document discusses potential issues with the high-level syntax for the reshaper tool. It is asserted that the current syntax has undefined behavior, and that it would cause issues for random access, temporal scalability, and trick-modes (fast-forward, reverse, etc…).

This contribution proposes to:

- Define the behavior when the reshaper is enabled but a model is not present in a tile-group.

- Add a default reshaper model in the SPS.

- Send additional reshaper syntax only if the reshaper is actually enabled for a tile-group.

The current design includes reshaper parameters in the tile group header. The reshaper parameters signaled in the tile group header are used by other pictures, which is not consistent with our usual HLS design principles, which would use a parameter set for signaling of parameters.

This contribution proposes including default reshaper parameters in the SPS. It was also suggested that the PPS might be a more appropriate location. It may be possible to include them in more than one parameter set location, with an override in the tile group header.

There is agreement that default reshaper parameters should be sent in a parameter set rather than propagated from tile group header.

The size of reshaper parameters is approx. 100 bits.

JVET-N0117 AHG17: Signalling of reshaper parameters in APS [Y.-K. Wang, Hendry, J. Chen (Huawei), P. Yin, T. Lu, F. Pu, S. McCarthy (Dolby)]

This contribution asserts that the properties of the reshaper parameters can be characterized as follows:

- The size of a set of reshaper parameters contained in the tile_group_reshaper_model( ) syntax structure is typically around 60 to 100 bits for video coded with a 10-bit internal bit depth.

- The reshaper model is typically updated by the encoder about once per second.

- Within a bitstream, many sets of reshaper parameters may be applied.

Based on the above asserted properties of the reshaper parameters, it is asserted that signalling of the reshaper parameters in the APS is more desirable than in any of the following: SPS; PPS; picture header (if it exists); and tile group header, and therefore it is proposed to signal the reshaper parameters in the APS.

In addition, to avoid the partial-update problem, it is also proposed that an APS NAL unit shall not contain more than one type of parameters; i.e., either ALF parameters or reshaper parameters, never both.

In the CTC for RA, the reshaper parameters are signaled once per random access period.

It is asserted that there would be a coding efficiency benefit to adjusting reshaper parameters more frequently, but no experimental results are available.

In the contribution, an APS type is proposed, which can be used for ALF and reshaper parameters. More than one aps_id can be signaled in the tile group header, based on which tools are enabled.

The design currently allow 32 values of aps_id. If this contribution were adopted, we might want to increase the number.

Some semantic constraint would be needed to disallow signaling of aps_id in the TGH for the wrong type of APS.

The proposal creates different APSs without the need to use additional NUT values.

This concept could be further extended for other APS types, such as for CNN loop filters, if they were adopted.

JVET-N0138 AHG17: Carriage of reshaper model parameters [W. Choi, K. Choi, K. Choi (Samsung)]

Similar in design to JVET-N0117. Proponent considers the JVET-N0117 design able to address the identified problem. No presentation of this contribution requested.

JVET-N0284 AHG17: On ALF and Reshaper Signaling [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

# This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters. In the latest VVC WD, the ALF parameters are signaled in the APS while the reshaper model is signaled in the tile group header. This contribution proposes to signal in-loop reshaper in the APS. It is considered beneficial to signal both these parameters in the APS.

Proposes signaling default reshaper parameters in SPS (in N0293), and update reshaper parameters in the APS. Conditionally signal a reshaper aps_id in the TGH based on a flag.

JVET-N0290 AHG17: Conditional Signaling of ALF and In-Loop Reshaper Model [S.Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

# This contribution proposes a design of the Adaptation Parameter Set (APS) for the carriage of Adaptive Loop Filter (ALF) and In-Loop reshaper model parameters, while still being present conditionally in the tile group header. This is advantageous because, it can provide the flexibility of partial updates of the ALF parameters and the in-loop reshaper model either through the APS or through the tile group header itself. A new flag proposed in the tile group header is used to control if the tile group should refer to an APS or would carry its own ALF and in-loop reshaper model.

JVET-N0293 AHG17: Reshaper Reset Signaling [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes a design for the reset of the In-Loop reshaper model at Intra Random Access Point (IRAP) picture. The In-Loop reshaper model is conditionally present in the tile group header. In the existing design, the reshaper parameters are controlled by two flags, i.e., tile_group_reshaper_model_present_flag and tile_group_reshaper_enable_flag, which indicate whether the reshaper model is present in the current tile group and whether it is to be used in the current tile group. These two flags work independently of one another. During an intra refresh, the model may not be present, however the model use may be enabled for the current and subsequent tile groups. This may not be a desirable option at a random access point and may cause dependency across random access point. Therefore, it is proposed to reset the reshaper to a default reshaper at IRAP picture if reshaper is not present. The default reshaper can be linear mapping or it can be signaled in the Sequence Parameter Set.

JVET-N0297 AHG17: Reshaper Model Signaling for Intra Coded Tile Group [S. Paluri, J. Zhao, J. Lim, S. Kim (LGE)]

This contribution proposes to signal the use of an In-loop reshaper model for an intra coded tile group. In particular it imposes the condition that a reshaper model be present in the intra coded tile group header and allows the reshaper model to be optionally present in other tile group headers.

Overall discussion:

APS has the same temporal id as the AU which it is sent in.

Need to have a restriction that can’t reference an aps_id which had a higher temporal id value than the current picture temporal id. Check to see if the spec text has any language for that.

It was noted that parameter set activation text is missing in VVC Draft 4.

It was noted that the current design uses a fixed length code for the aps_id, which may be sub-optimal.

Suggested solution:

- optionally send reshaper parameters in the SPS

- define APS types for ALF and reshaper parameters. Each APS has only one type.

- can send reshaper parameters in APS

- if reshaper tool enabled, have a flag in the TGH to indicate whether or not reshaper aps_id is present. If not signaled, the SPS parameters are used.

  - need to add semantic constraint to always have something valid that is referred to when tool enabled

Proponents are asked to submit a late contribution to implement this solution. Hendry to coordinate.

#### Interoperability and capability points definition and signalling (1)

JVET-N0276 AHG15: On interoperability point signalling [J. Boyce (Intel)]

#### High efficiency random access (4)

Contributions in this category were discussed XXday XX March XXXX–XXXX (chaired by XXXqq).

JVET-N0119 AHG17: EDR - external decoding refresh [Y.-K. Wang (Huawei)] [late]

JVET-N0244 AHG17: Video coding based on cross RAP referencing (CRR) [H. Yu, L. Yu (Zhejiang Univ.)] [miss] [late]

JVET-N0494 AHG17: Dependent random access point pictures in VVC [M. Pettersson, R. Sjöberg, M. Damghanian (Ericsson)]

JVET-N0072 AHG17: New NAL unit types for VVC [L. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

The DRAP part of this document belongs to this agenda item.

#### Miscellaneous general HLS topics (8)

JVET-N0061 AHG19: Signalling temporal IDs and levels for HFR backwards compatible bitstreams [V. Drugeon (Panasonic)]

JVET-N0063 AHG17: Separate essential display information from non-essential display information in the Video Usability Information [V. Drugeon (Panasonic)]

JVET-N0120 AHG17: Misc. HLS clean-ups [Y.-K. Wang (Huawei)] [late]

JVET-N0227 Maximum transform size signaling in HLS [L. Li, M. Koo, J. Nam, J. Heo, J. Lim (LGE)] [miss] [late]

JVET-N0234 [AHG12/AHG17] Signaling of virtual boundaries [A. DSouza, C. Pujara, R. Gadde, W. Choi, K. Choi, K.P. Choi (Samsung)] [late]

JVET-N0438 AHG12: Loop filter disabled across virtual boundaries [S.-Y. Lin, L. Liu, J.-L. Lin, Y.-C. Chang, C.-C. Ju (MediaTek), P. Hanhart, Y. He (InterDigital)]

JVET-N0288 AHG17: Comments on High-Level Syntax of VVC [S. Deshpande (Sharp)]

JVET-N0352 AHG17: Conformance Window [K. Sühring, R. Skupin, Y. Sanchez, T. Schierl (HHI)]