| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0762r1 |
| --- | --- |

| Title: | Crosscheck of JVET-N0485 (Non-CE6: Efficient separable Multi-Transform-Selection (MTS) without zero-out) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Pierrick Philippe | Email: | pierrick.philippe@orange.com |
| Source: | Orange, bcom |  |  |

_____________________________

# Abstract

This document reports the crosscheck of JVET-N0485. It is reported that the average BD-rates match those provided by the proponents. No discrepancies have been found between the provided software and the description.

# Description

This proposal presents a new implementation of the 64-point DCT-2, also with a new computation of the DCT-8 and DST-7 transforms (for size 32, & 64) which are based on the adjustment method of JVET-M0538. TrQuant_EMT.cpp reflects most of these changes, were the new DCT-2 and the adjustment method are implemented.

Also, the zero-out threshold JVET_C0024_ZERO_OUT_TH is set to 64, consequently the zero out for longer transforms is not applied anymore.

RomTr.cpp contains the adjustment matrix (one single 8b-8x8 matrix) and the new DCT-2 64 structure (373 8b coefficients) replacing the VTM-4.0 matrix.

Hence, the software was verified and the elements described above are consistently implemented.

# Experimental Results

Experiments were performed based on VTM4.0, in All Intra configuration, using the common test conditions described in JVET-M1010.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.15% | -0.22% | 101% | 97% |
| Class A2 | -0.27% | 0.30% | 0.17% | 101% | 98% |
| Class B | -0.15% | 0.12% | 0.17% | 101% | 97% |
| Class C | -0.06% | -0.14% | -0.09% | 101% | 99% |
| Class E | -0.28% | -0.05% | -0.15% | 101% | 98% |
| Overall | -0.25% | 0.02% | -0.01% | 101% | 98% |
| Class D | -0.08% | 0.02% | -0.08% | 102% | 99% |
| Class F | 0.02% | 0.02% | -0.10% | 101% | 97% |

The average BD-rates match those provided by the proponents.

# Patent rights declaration

Orange and b<>com may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).