| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0776-v4 |
| --- | --- |

| Title: | BoG report on CE2 sub-block based motion prediction related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Yuwen He | Email: | chunchic@qti.qualcomm.com, yuwen.he@interdigital.com |
| Source: | BoG |  |  |

_____________________________

# Summary

The sessions were held 1915~2345 on March 20 and 1430~1600 on March 21 for discussing 15 technical contributions in 4 categories:

- Shared merging candidate list region for sub-block merge modes (1)

- Memory bandwidth and complexity reduction (3)

- Affine motion compensation (10)

- Storage reduction for sub-block modes (1)

## Recommendations

### Recommended adoptions to VTM

Normative changes (1)

- JVET-N0481 (Method 3): BCW index inheritance for constructed affine merge candidate

- Divide BCW index into groups of {0}, {1, 2, 3} and {4}. If all the BCW indices are from the same group, method 1 will be used; otherwise, the default BCW index will be set. (Method 1: if at least 2 out of the 3 CPMVs (top-left, top-right and bottom-left) are sharing the same BCW index, this BCW index will be set to all three CPMVs; otherwise, default BCW index will be set).

Non-normative changes (0)

### Recommended CE2 tests from following contributions

#### Affine motion compensation (5)

- JVET-N0236: Prediction refinement with optical flow for affine mode

- Test 1: Sample-based optical flow for 4x4 Uni/Bi sub-blocks

- Test 2: Sample-based optical flow for 4x4x Uni and 8x8 Bi sub-blocks

- It was clarified in the plenary that test 2 was the intended variant, not test 5

- JVET-N0273: Affine motion compensation using 2x2 subblock

- Switch 2x2 interpolation and 4x4 interpolation for uni-prediction with bounding box size of 15x15 for each 8x8 region (equivalent to 8x8 MC) with MV storage at 4x4 unit. (1) Validate the bounding box calculation; (2) SIMD implementation for 2x2 sub-block.

- JVET-N0399: Simplifications of interweaved affine mode

- Use high-precision bi-prediction for interweave and apply 0.5 weight to non-boundary samples to achieve bi-prediction.

- JVET-N0504: Simplification of 2x2 sub-block based affine motion compensation

- Apply sample-based optical flow with 2x2 refinement offset for 4x4 sub-blocks

- JVET-N0510: Phase-variant affine subblock motion compensation

- Switch filters across rows based on affine model in horizontal interpolation and across columns in vertical interpolation.

## Open issues

### Revisit in track (3)

- JVET-N0737: Combination of affine MV clip (CE2-4.4 in JVET-N0398) and prediction refinement with optical flow (JVET-N0236)

- Combination of Affine MV clipping (CE2-4.4 in JVET-N0398) and optical-flow based refinement (JVET-N0236). (-0.35% for RA with Tx=Ty=0)

- Revisit to determine whether this proposal can be considered for further study in a CE.

- JVET-N0752: Supplementary results for CE2-5.5.c

- Test 1: At most 2 merge candidates can be inherited from the history table or neighbouring blocks. (-0.04% for RA)

- Test 2: 5 affine constructed merge candidates are removed. (-0.01% for RA)

  - Introduces 3 new potential candidates with removing 5 constructed candidates, so the total potential candidate is reduced from 8 to 6.

- Revisit for reviewing the supplemental results with CE2-5.5c (i.e. Parameter-based HMVP affine candidates + Replace spatial affine inheritance by affine HMVP + Affine inheritance from line buffer).

- JVET-N0268: Remove above-left affine inherited motion vector predictor and inter merge mode

- (Test 1) 0.01% for RA and -0.03% for LB

- ; (Test 2) 0.03% for RA and -0.01% for LDB

- Revisit: how much the memory storage saving in the proposed method in Track B.

### Contribution not reviewed yet (0)

# Shared merging candidate list region for sub-block merge modes

JVET-N0079 CE2-related: Applying SMR to subblock-based merging candidate list [Y.-C. Lin, C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, it is proposed to apply shared merging candidate list region (SMR) to subblock-based merging candidate list. The test results are reported below.

When SMR area threshold is set equal to 128:
VTM4.0-RA: {BD-rate=0.01% (Y), 0.01% (Cb), -0.07% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.01% (Y), 0.20% (Cb), -0.01% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 256:
VTM4.0-RA: {BD-rate=0.02% (Y), -0.05% (Cb), -0.08% (Cr); EncT=100%; DecT=102%}
VTM4.0-LB: {BD-rate=0.03% (Y), 0.12% (Cb), -0.16% (Cr); EncT=100%; DecT=101%}

When SMR area threshold is set equal to 512:
VTM4.0-RA: {BD-rate=0.06% (Y), -0.02% (Cb), -0.05% (Cr); EncT=100%; DecT=103%}
VTM4.0-LB: {BD-rate=0.05% (Y), 0.11% (Cb), -0.23% (Cr); EncT=100%; DecT=102%}

It is asserted that the proposed method can increase hardware decoder throughput when subblock-based merging candidates are enabled.

Q. Why the threshold value equal to 32 is chosen for SMR in the last meeting.

A. A participant responded that the decision was made based on less coding loss.

Whether there is really a bottleneck in affine candidate derivation is still questionable.

Test 1 (threshold=128): 0.01%/100%/103% for RA; 0.01%/100%/101% for LB.

Test 2 (threshold=256): 0.02%/100%/102% for RA; 0.03%/100%/101% for LB.

Test 3 (threshold=512): 0.06%/100%/103% for RA; 0.05%/100%/102% for LB.

2-3% decoding time increase is observed, due mainly to non-optimized software implementation.

An expert commented that 8x8 affine may not be a problem and has no throughput issue when smaller CU size for affine merge is 8x8.

Suggestion: No action.

JVET-N0599 Crosscheck of JVET-N0079 (CE2-related: Applying SMR to subblock-based merging candidate list) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

# Memory bandwidth and complexity reduction

JVET-N0199 CE2-related: combined test of affine memory bandwidth complexity reduction (Test2.4.6+Test2.4.8) [J. Li, C.-W. Kuo, C. S.Lim (Panasonic), M. Zhou (Broadcom)]

This contribution proposes a combination of CE2.4.6 and CE2.4.8 to reduce memory bandwidth and complexity of affine prediction. BD rate changes are reported as below:

Test 1 (CE2.4.6+CE2.4.8a): 0.11% and 0.15% BD luma BD rate change in RA and LDB configurations.

Test 2 (CE2.4.6+CE2.4.8b): 0.**% and 0.**% BD luma BD rate change in RA and LDB configurations.

Combination test of CE2.4.6 (6-tap IF for luma affine subblock) and CE2.4.8 (subblock affine MV conformance).

Test 1: encoder conformance.

Test 2: Test 1 + fallback mode.

This proposal incurs large coding loss (0.78% BQSquare, 0.4x% for PartyScene) for some sequences and the performance-wise impact on Class F from cross-checker seems moderate.From the coding performance results, it shows the performance-wise gain looks less stable across different testing sequences. Overal, 0.07% loss for RA.

As the decision for whether we should have 6-tap interpolation filter is still pending, the additional information provided in this proposal may worth considering as side information later in Track B when 6-tap filter topic is revisited.

The proponents stated that this proposal is just for information.

JVET-N0623 Crosscheck of JVET-N0199 (CE2-related: Combined test of affine memory bandwidth complexity reduction (Test2.4.6+Test2.4.8)) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0266 CE2-related: Disabling bi-prediction or inter prediction for small blocks [H. Liu, L. Zhang (Bytedance)]

Since this proposal was adopted during the discussion of CE2 in Track B, the proponents agreed to not present for this proposal.

JVET-N0645 Cross-chreck of JVET-N0266 (CE2-related: Disabling bi-prediction or inter prediction for small blocks) [J. Chen (Alibaba)] [miss] [late]

JVET-N0369 CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant) [M. Zhou (Broadcom)]

This contribution provides supplemental results for CE2-4.8 (JVET-N0068). Different from algorithm tested in JVET-N0068, in this variant the reference block bounding box size is computed for 2x2 sub-block vectors for both bi-directional and unidirectional affine mode.

For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and 2*17*17 for 2x2 sub-block vectors of unidirectional affine mode, the average BD-differences compared to the VTM4.0 are as follows:

- For Test1 in which the fallback mode is not triggered, the BD-rate difference is 0.01% in RA and 0.00% in LD_B, respectively.

- For Test2 in which the fallback mode is triggered, the BD-rate difference is -0.01% in RA and -0.01% in LD_B, respectively.

In CE2-4.8, bounding box is of size 17*17 for 2x2 sub-block vectors of bi-directional affine modes and  17*13 for 2x1 and 1x2 sub-block vectors of unidirectional affine mode.

In this proposal, the setting of bounding box for unidirectional affine mode becomes 2*17*17 for 2x2 subblock vectors.

Method 1: encoder control to ensure affine model parameter meet the memory bandwidth requirement.

Method 2: Test 1 + fallback mode (8x8 subblocks)

The proponent stated that this contribution is proposed just for information.

JVET-N0575 Crosscheck of JVET-N0369 (CE2-related: On restriction of memory bandwidth consumption of affine mode (2x2 variant)) [K. Zhang (Bytedance)] [late]

# Affine motion compensation

JVET-N0236 CE2-related: Prediction refinement with optical flow for affine mode [J. Luo, Y. He (InterDigital)]

This contribution proposes a method to refine the sub-block based affine motion compensated prediction with optical flow. After the sub-block based affine motion compensation is performed, prediction sample is refined by adding a difference derived by the optical flow equation, which is referred as prediction refinement with optical flow (PROF). The proposed method can achieve inter prediction in pixel level granularity without increasing the memory access bandwidth. The simulation result reportedly shows on average 0.48% and 0.37% BD rate saving for RA and LD, respectively, with 3% encoding time and 1% decoding time increase.

This proposal extends the notion of BDOF to Affine sub-block based motion compensation. It was later confirmed that that BDOF is not applied to sub-block modes in the current draft standard.

Boundary padding of BDOF for regular inter modes is applied only to CU boundaries, while this proposal applies it to subblock boundaries.

The proponent stated that text edges are improved in visual quality.

Test 1: 4x4 Uni/Bi sub-blocks (-0.48%/103%/101% for RA)

Boundary padding process is not aligned with the current BDOF’s design. This proposal takes the nearest integer samples, while BDOF clips MV and takes the corresponding integer samples for padding.

Test 2: Test 1 + Replace 4x4 Bi by 8x8 Bi (-0.33%/102%/100% for RA.)

Test 3: Same as Test 1 but used for Huawei’s sequences (-1.77% for RA; -1.02% for LB; Peak -2.62% for SpincalendarPart)

Test 4: 4x4 Uni/Bi sub-blocks + Alternative gradient method (-0.52%/105%/102% for RA)

The padding process is applied to each boundary of the 11x11 reference samples of each 4x4 subblock by 1 pixel at each side, resulting in 13x13.

Test 5: Test 4 + Replace 4x4 Bi by 8x8 Bi + Alternative gradient method (-0.38%/103%/101% for RA)

Test 6: Test 1 + Replace 4x4 Bi by 8x4 Bi (-0.40%/103%/101% for RA)

One non-proponent recommended Test 1 could be considered for adoption, while several other non-proponents recommend to further study this proposal in a context of CE.

As bandwidth can already be addressed in CE2-4.8, 8x8 Bi sub-block configuration as proposed may not be necessary for testing.

It was also commented that there are similar approaches (e.g. interweave prediction) that can achieve similar coding performance-wise benefit and are suggested to study them together.

Some participants showed interest to see combination testing results of this proposed method and other related proposals (e.g. interweave prediction)

Suggestion: Further study Test 1 (with at most 8x8 subblock size for Bi) and Test 2 in a CE.

It was clarified in the plenary that test 2 was the intended variant, not test 5

JVET-N0626 Crosscheck of JVET-N0236 (CE2-related: Prediction refinement with optical flow for affine mode) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0651 Crosscheck of JVET-N0236 (CE2-related: Prediction refinement with optical flow for affine mode) test 1-2 [G. Li (Tencent)] [miss] [late]

JVET-N0736 Crosscheck of JVET-N0236 test 6 (CE2-related: Prediction refinement with optical flow for affine mode) [T. Ikai (Sharp)] [late]

JVET-N0273 CE2-related: Affine motion compensation using 2x2 subblock [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

In this contribution, it’s proposed to adaptively use 2x2 or 4x4 subblock based motion compensation for affine uni-prediction.  The adaptation is based on the affine parameters and ensure that the proposed method doesn’t increase worst case memory bandwidth. The proposed method is only applied to uni-prediction, it also doesn’t increase the worst-case computational complexity for interpolation. The test results show that the BD-rate changes are -0.34%, -0.05%, 0.02% in RA and -0.18%, -0.01%, -0.19% in LDB

In current VTM, the worst-case computational complexity for motion compensation is 4x4 bi-prediction. For 4x4 uni-prediction, it requires 4*(4+7) interpolations in horizontal direction and 4*4 interpolations in vertical direction. Therefore, the total number of interpolations for 4x4 bi-prediction is (4*11+4*4) *2 = 120, i.e. 7.5 interpolations / sample. For 2x2 uni-prediction, it requires 2*(2+7) interpolations in horizontal direction and 2*2 interpolations in vertical direction, 22 in total, i.e. 5.5 interpolations / sample. A 4x4 subblock using 2x2 uni-prediction needs 88 (22*4) interpolations.

Each interpolation requires 8 multiplications and 7 additions. Therefore, 4x4 bi-prediction requires 960 multiplications and 840 additions. The 2x2 uni-prediction for a 4x4 block requires 704 multiplications and 616 additions.

(Chaired by Y. He)

2x2 uni-prediction is less complex than 4x4 bi-prediction.

Discussions:

The method will switch 2x2 interpolation and 4x4 interpolation for uni-prediction. The bounding box size is 15x15 for each 8x8 region (equivalent to 8x8 MC).

The storage for MV field is still 4x4. 4x4 sub-block MV is derived by averaging two diagonal 2x2 sub-block MVs. The method is only applied to luma component.

In order to pre-fetching memory for MC, the bounding box needs to be calculated before MC.

The MV used in the MC (2x2 sub-block) is not aligned with the MV (4x4 sub-block) used for MV prediction.

The MV used in the strength derivation of deblocking is not aligned with the MV used in MC.

2x2 MC is not friendly for SIMD implementation.

Suggestion: Further study in CE; (1) validate the bounding box calculation; (2) SIMD implementation.

JVET-N0547 Crosscheck of JVET-N0273 (CE2-related: Affine motion compensation using 2x2 subblock) [H. Chen (Huawei)] [miss] [late]

JVET-N0399 CE2-related: Simplifications of interweaved affine mode [X. Li, G. Li, X. Xu, S. Liu (Tencent)]

Interweaved affine mode was proposed to improve coding efficiency. It converts uni-predicted affine block to bi-predicted block by introducing a uni-predicted auxiliary affine block. The final prediction is obtained by weighted averaging the two uni-predicted blocks. In this contribution, it is proposed to regard interweaved affine mode as a special bi-predicted mode so that weighted bi-prediction process is reused. Furthermore, it is proposed to simplify the weighting process. It is reported that minor impact on coding performance is observed by the proposed methods.

Improvement to Interweave CE2-1.1b.

Method 1: Use high-precision bi-prediction for interweave

Method 2: Apply 0.5 weight to non-boundary samples to achieve bi-prediction

[Test 1]  Method 1

- Relative to VTM-4.0: -0.32%/103%/102% for RA; -0.12%/104%/104% for LB.

- Relative to CE2-1.1b: 0.00%/102%/103% for RA; 0.02%/100%/101% for LB.

[Test 2]  Method 1 + Method 2

- Relative to VTM-4.0: -0.30%/103%/101% for RA; -0.14%/104%/103% for LB.

- Relative to CE2-1.1b: 0.02%/102%/103% for RA; 0.00%/101%/100% for LB.

The proponent stated that the runtime is not reliable in this test.

A non-proponent and crosschecker suggest that this simplification could benefit the design of the interweave prediction (CE2-1.1) in terms of complexity.

Suggestion: Further study Test 1 and Test 2 in CE.

JVET-N0624 Crosscheck of JVET-N0399 (CE2-related: Simplifications of interweaved affine mode) [K. Zhang (Bytedance)] [miss] [late]

JVET-N0456 CE2-related: Affine motion model derivation method for affine merge mode [W. Chen, J. Luo, Y. He (InterDigital)]

Regression based motion vector field (RMVF) proposed in JVET-L0171 provides a new motion vector representation for merge mode. This tool derives the motion vectors of each sub-block in the CU based on the spatially neighboring motion vectors. The current contribution proposes a new affine motion model derivation method without using any least mean square error calculation. When affine merge mode is selected and the proposed model derivation method is enabled in VTM-4.0, the affine model estimated from MVs of neighboring coding blocks can be used as a predictor of the affine model to be used for current CU.

The proposed method provides on average -0.xx% and -0.xx% luma BD-Rate impact in RA and LDB configurations, respectively. The encoding runtimes of xxx% and xxx % and decoding runtimes of xxx% and xxx% in RA, LDB, and LDP configurations were observed, respectively.

Simplification of RMVF to avoid using the least-mean square for parameter derivation. This mode is signalled as an alternative merge mode with 4-paramenter and 6-parameters modes.

In VTM-3.0, it is mentioned from the proponent that the coding gain of RMVF (without simplification) is around 0.09%.

A non-proponent commented that the RMVF’s complexity characteristics should be evaluated and compared with that of affine merge (inherited and constructed modes) to justify the coding performance-wise improvement with the additionally increase of computational complexity.

-0.07%/-0.06% for Class B/C with +2%/+2% encoding/decoding time increase .

Suggestion: No action.

JVET-N0481 Non-CE2: Gbi inheritance for constructed affine merge candidate [J. Chen, R.-L. Liao, Y. Ye (Alibaba)]

In current VVC, Bi-prediction with CU weights (BCW, or GBi) and affine merge are both employed. With BCW, each inter-coded block is further assigned a bi-prediction weight index in addition to the motion information. In the current affine merge design, for inherited affine merge candidate, the BCW of neighboring block is inherited, while for constructed affine merge candidate, the BCW index is set to the default value (i.e. equal weight). To align the design of BCW index derivation between these two types of affine merge candidates, it is proposed to derive a BCW index for constructed affine merge candidate instead of always setting to the default value. Simulation results reportedly show that the harmonization achieves 0.02% and 0.05% BD rate reduction for RA Main10 and LDB Main 10 configurations, respectively.

BCW index inheritance alignment for constructed affine merge.

Method 1: if at least 2 out of the 3 CPMVs (top-left, top-right and bottom-left) are sharing the same BCW index, this BCW index will be set to all three CPMVs; otherwise, default BCW index will be set -> at most 3 comparisons.

RA: -0.02%/100%/99%; LB: 0.05%/101%/102%, where Class E is observed to be with 0.05% loss w/ peak loss 0.16%.

Method 2: if 2 of the CPMVs' BCW index forms a pair (e.g. index 0 and 4 which correspond to -1/4 and 5/4), then the default BCW index will be set; otherwise, Method 1 applies -> at most 3 additions and 3 comparisons + 3 comparison from Method 1.

RA: -0.02%/102%/101%; LB: -0.02%/101%/102%.

Method 3: divide BCW index into groups of {0}, {1, 2, 3} and {4}. If all the BCW indices are from the same group, method 1 will be used; otherwise, the default BCW index will be set.

The intention of BCW index grouping is to prevent BCW index pairs from falling into extreme cases.

RA: -0.02%/101%/101%; LB: -0.05%/101%/102%.

The proponent stated preference for this method and was later supported by several non-proponents.

It is stated this method achieves better design consistency between inheritance and constructed affine merge candidates.

Suggestion: Adopt Method 3.

JVET-N0655 Crosscheck of JVET-N0481 (Non-CE2: Gbi inheritance for constructed affine merge candidate) [H. Liu (Bytedance)] [miss] [late]

JVET-N0659 Crosscheck of JVET-N0481: Non-CE2: Gbi inheritance for constructed affine merge candidate [J. Luo (InterDigital)] [miss] [late]

JVET-N0504 CE2-related: simplification of 2x2 subblock based affine motion compensation [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

In this contribution, a 2x2 subblock based prediction refinement method is proposed for affine motion compensation. The prediction refinement is performed by using the optical flow equation to derive an offset for each pixel in a 4x4 subblock. The delta motion vectors are calculated in a 2x2 subblock basis, and the gradient can be calculated in parallel for each 4x4 subblock. The test results show that the BD-rate changes are -0.28%, -0.08%, -0.01% in RA and -0.18%, -0.06%, -0.11% in LDB.

(Chaired by Y. He)

Difference between this proposal and JVET-N0236: (1) 2x2 sub-block based optical flow; (2) the gradient calculation at 4x4 sub-block boundary.

Discussion:

It is commented that the refinement is still sample based operation though the optical flow is based on 2x2 sub-block level.

The gradient calculation is not aligned with that in BDOF at 4x4 sub-block boundary.

The reduction on optical flow calculation for each CU is from 32 additions to 8 additions.

Suggestion: Further study in CE; test 2x2 sub-block level optical flow.

JVET-N0597 Crosscheck of JVET-N0504 (CE2-related: Simplified prediction refinement for affine motion compensation) [R.-L. Liao (Alibaba)] [miss] [late]

JVET-N0510 CE2-related: Phase-variant affine subblock motion compensation [T.-D. Chuang, C.-Y. Chen, C.-Y. Lai, Y.-W. Huang, S.-M. Lei (MediaTek)] [late]

In this contribution, phase-variant affine subblock motion compensation (MC) is proposed. The conventional two stage horizontal-vertical interpolation is applied. However, unlike the phase-invariant block-based MC that uses the same horizontal filter for all sample rows and the same vertical filter for all sample columns, different phases of filters can be applied to different sample rows and different sample columns in the affine subblock. The used interpolation filter sets in the proposed method are the same as those in VTM4.0. The number of filtering operations for each affine subblock in the proposed method is also the same as that in VTM4.0. Simulation results reportedly show that the luma BD-rates are -0.33% and -0.23% in RA and LB cases, respectively.

This proposal applies different horizontal filters associated with different phases as suggested by affine parameters to generating intermediate filter results from horizontal filtering. Therefore, multiple interpolation filters (11) associated different phases are applied to respective rows on the (4+7)x(4+7) reference samples.

The same approach is further applied to vertical filtering to generate the resulting 4x4 affine sub-block prediction signals.

Subjective comparisons are provided in the proposal, showing better visual quality particularly in object edges.

It is noted that additional logics (one add, right-shift, clip operations) are required in MC stage, compared with current affine motion compensation.

It was asked whether loading multiple set of interpolation filters for 4x4 subblock is doable. It was later confirmed from the proponent that in hardware implementation, 11 sets of filter coefficients are loaded simultaneously (one for a row of 4x4 subblock‘s reference samples) and then horizontal or vertical interpolation can be done parallelly in a pipeline stage.

RA: -0.33%/102%/101%; LB: -0.23/101%/100%.

Suggestion: Further study in CE.

JVET-N0676 Crosscheck of JVET-N0510 (CE2-related: Phase-variant affine subblock motion compensation) [Y.-C. Sun (Alibaba)] [miss] [late]

JVET-N0709 CE2-related: Computation reductions of interweaved affine mode [K. Kawamura, S. Naito (KDDI)] [late]

This contribution proposes the computation reduction method for interweaved affine mode. The interweaved affine mode maintains the memory bandwidth but increases the motion compensation computation. The problem is the different incensement of MC computation for each block size. This contribution shows that the additional computation and gain are trade-off relation by using checker board pattern. The proposed method is implemented on the top of CE2-1.1 software. The result of Y BD-rate is -0.11% under the random access condition with shorter running time.

Improvement to Interweave mode (CE2-1.1a).

The supplemental predictor’s pattern is checkboard

Test 1: Applied to Affine Uni only (RA: -0.11%/101%/100%)

Test 2: Applied to Affine Uni and Bi (RA: -0.30%/101%/100%)

(CE2-1.1a: RA 0.30%/100%/100%; LB -0.14%/101%/101%)

Q. How to conduct bi-prediction to weighted-average the uni-prediction signals and the supplemental prediction signals?

A. The bi-prediction takes place only when the 4x4 blocks are available on the checkboard pattern.

In Test 2, although some 4x4 subblocks still can achieve the same computational complexity as regular bi-prediction, while some are not (i.e. in worst case, there are four 4x4 subblocks’ prediction signals are averaged.)

Suggestion: No action.

JVET-N0737 Non-CE2: Combination of affine MV clip and prediction refinement with optical flow [X. Li, G. Li, X. Xu, S. Liu (Tencent)] [late]

This contribution proposes a combination of Affine MV clip and Affine prediction refinement based with optical flow. It is reported that the combination brings additional coding gain over VTM-4 while keeping the worst-case memory bandwidth of bi-predicted affine the same as that of regular bi-predicted inter block.

Combination of Affine MV clipping (CE2-4.4 in JVET-N0398) and optical-flow based refinement (JVET-N0236).

Test 1: JVET-N0236 (the proponent re-implemented this technique)

Test 2: Test 1 + Tx=Ty=1

Test 3: Test 1+ Tx=Ty=0 (RA: -0.35%, runtime is not reliable)

It is stated the proposed tests are SIMD friendly; however, the proponent does not implement the proposed tests by using SIMD instructions.

Q. There is a concern whether the required MV clipping would impact the software SIMD implementation.

A. The proponent stated that even with clipping, the implementation is still SIMD-friendly; however, this may have to be confirmed from software crosscheck if a CE is intended.

Q. Is Test 3 a straightforward combination of JVET-N0236 and Affine MV clipping?

A. An expert mentioned that this is straightforward combination.

It was suggested to study PDOF + 8x8 subblock size. (A participant late shared that for RA, it is observed 0.23% loss from 8x8 PDOF.

It is suggested to quantify the number of compiled SIMD instructions.

Suggestion: Revisit in Track B to determine whether this proposal can be considered for further study in a CE.

JVET-N0752 CE2-related: Supplementary results for CE2-5.5.c [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang (Bytedance)] [late]

In this contribution, supplementary results for CE2-5.5.c are provided. Two tests are conducted based on CE2-5.5.c. In supplementary test 1, at most two merge candidates can be inherited from the history table or neighbouring blocks. In supplementary test 2, five affine constructed merge candidates are removed. Simulation results reportedly show that the BD-rate changes are -0.04% and -0.01% under the RA test configurations in test 1 and test 2, respectively.

Test 1: -0.04% / 100% / 100% for RA

Test 2: -0.01% / 101% / 101% for RA

This proposed method introduces 3 new potential candidates with removing 5 constructed candidates, so the total potential candidate is reduced from 8 to 6.

Suggestion: Revisit in Track B together with CE2-5.5c.

# Storage reduction for sub-block modes

JVET-N0268 CE2-related: Remove above-left affine inherited motion vector predictor [H. Huang, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution proposes to remove the above-left affine inherited motion vector predictor. In the memory optimized implementation, the proposed method can reduce one third of the memory requirement of affine motion inheritance. The Y, U, V BD-rate changes are 0.01%, -0.01%, 0.00% in RA and -0.03%, 0.10%, -0.26% in LDB.  When the proposed method is extended to normal inter-prediction mode and intra block copy, The Y, U, V BD-rate changes are 0.03%, 0.01%, -0.01% in RA and -0.01%, 0.13%, -0.09% in LDB. When the proposed method is combined with CE2-5.2.b, the Y, U, V BD-rate changes are 0.0x%, 0.0x%, -0.0x% in RA and -0.0x%, 0.0x%, -0.0x% in LDB. It’s also proposed to apply VPDU constrain for affine motion inheritance to further reduce the memory requirement. The memory requirement is 4,352 bits without memory-optimized implementation and is 432 bits with memory-optimized implementation. The Y, U, V BD-rate changes are 0.0x%, 0.0x%, -0.0x% in RA and -0.0x%, 0.0x%, -0.0x% in LDB.

(Chaired by Y. He)

Test1 applies the method to affine merge mode. 0.01% loss for RA (0.08% peak loss), 0.03% gain for LDB;

Test2 applies the method to both affine merge mode and inter merge mode.  0.03% loss for RA, 0.01% gain for LDB;

Test3: test1 + CE2-5.2b.  xx% for RA, xx% for LDB;

Test4: test2 + CE2-5.2b.  The loss is a little big (0.09% loss for Class B).

Suggestion: Revisit how much the memory storage saving in the proposed method in Track B

JVET-N0546 Crosscheck of JVET-N0268 (CE2-related: Remove above-left affine inherited motion vector predictor) [H. Chen (Huawei)] [miss] [late]