| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0780-v2 |
| --- | --- |

| Title: | BoG report on CE8-related contributions and updates for screen content CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Yu-Chen Sun | Tel: Email: | xiaozhongxu@tencent.com yuchen.s@alibaba-inc.com |  |
| Source: | BoG |  |  |

_____________________________

# Summary

The mandates of this BoG are 1) to make an initial review of CE related contributions and identify the aspects to be investigated in the upcoming CE. 2) discuss possible updates on screen content CTC.

There are 82 technical contributions in the CE-8 related category (some of them are cross-check reports). These contributions are classified into the following 5 categories:

- IBC (47)

- Palette (9)

- BDPCM (3)

- Transform skip (14)

- Others (7)

- Screen content test materials (2).

BoG sessions were held:

- 20:30 – 22:30 on 03/21 in Room H1;

- 13:00 –16:00 on 03/22 in Room A;

- 9:00–12:30 on 03/23 in Room H2;

- 15:00–17:00 on 03/23 in Room H2.

Section 1 of this document summarizes the BoG’s recommendations. Section 2 of this document contains detailed notes on BoG discussion. Section 3 of this document contains BoG discussed input test materials and class F updates.

Comment: record the overall benefit in addition to recommendation for track/plenary review.

## Recommendations

### Recommended adoptions to VTM (SW) and CTC

- JVET-N0247: Hash based ME improvement (0.65%/1.4% gain RA/LB for TGM).

- JVET-N0251: encoder only improvement for IBC, increase search range (0.2%/0.8% gain in AI/RA for TGM)

- JVET-N0318: encoder only improvement for IBC, increase block sizes (0.8% TGM AI gain)

- JVET-N0329: encoder only improvement for IBC, by more predictors, SAD/STAD evaluation and bug fixes (1.3% TGM AI gain).

- JVET-N0386: AOV in 4:4:4 format (for 4:4:4 support);

- Section 3 discussion: update class F with AOV and 4 TGM sequences (for CTC updates)

### Recommended adoptions to VVC/VTM

- JVET-N0175 M1/JVET-N0251/JVET- N0384 M2, increase IBC range for small CTUs (Bug fix, 13.4% gain TGM AI in non-CTC)

- JVET-N0316: default IBC compensation using value of 1<<(bitdepth - 1) when 0 vector is detected (Design consideration)

- JVET-N0317: put 0 vector as default vector in merge list to make spec complete (Bug fix)

- JVET-N0383/JVET-N0251: IBC search range constraint to avoid reading/writing memory conflict (Bug fix)

- JVET-N0413: BDPCM applied in quantized residual domain (2.1% TGM AI gain over CE8-4.4a)

- JVET-N0260: disable fractional search refinement part in DMVR from tile group header (Complexity reduction)

### Recommended to further study in CE

- JVET-N0095/JVET-N0289: on MVD/BVD unified coding (1.4% TGM AI gain)

- JVET-N0201/JVET-N0316: default IBC compensation mode in case of invalid vector (Design consideration)

- JVET-N0459: BVD binarization changes (1.1% TGM AI gain)

- JVET-N0316/JVET-N0466: subblock BV vs. CU based BV (Complexity reduction)

- JVET-N0472: IBC reference memory size reduction (Complexity reduction)

- JVET-N0258: PLT mode flag signaling changes, to be included into PLT software (0.4% TGM AI gain)

- JVET-N0259: PLT and IBC combination (1.0% TGM AI gain)

- JVET-N0550: CG (per 16 samples) based PLT mode (Complexity reduction)

- JVET-N0366: Syntax arrangement and context change for TS coding (0.4% TGM AI gain)

- JVET-N0094: Context modelling of transform skip mode (2.1% TGM AI gain, over VTM-4.0)

- JVET-N0357: Context Modelling of Sign for TS Residual Coding (0.2%~0.3% TGM AI gain)

- JVET-N0401: TS enabling for ISP mode (0.5% TGM AI gain, over VTM-4.0)

- JVET-N0455: simplification of TS mode (0.5% TGM AI gain)

- JVET-N0337: Disabling intra sample interpolation for SCC at tile group level (0.4% TGM AI gain)

## Open issues

### Revisit in track/plenary

- JVET-N0346: PLT in 4:4:4

- JVET-N0405: on PLT implementation simplification

- JVET-N0318/JVET-N0466: explicitly use syntax constraint for disabling 128x128, on top of current bitstream constraint

- JVET-N0123: Chroma TS mode in the context of 4:4:4

- JVET-N0093/JVET-N0174: Disallowing coexistence of regular merge and IBC merge in SMR

- JVET-N0843: combination of JVET-N0176/JVET-N0317/JVET-N0382, on simplification of block vector prediction. Consider adoption pending the combined results.

# Contributions related to CE8

## 2.1. IBC

JVET-N0093 CE8-related: Disallowing coexistence of regular merge and IBC merge in SMR [Y.-C. Lin, C.-C. Chen, M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

A shared merging candidate list is applied to regular merge in VTM4.0, and another different shared merging candidate list is applied to intra block copy (IBC) merge in the VTM4.0 fix of Ticket #193. Therefore, in one shared merging candidate list region (SMR), two shared merging candidate lists may have to be generated. This contribution proposes to disallow coexistence of regular merge and IBC merge in the SMR, so only one merging candidate list has to be generated for one SMR. It is reported that proposed method results in negligible BD-rate and runtime changes.

In current design, with the consideration of removing 4x4 uni-prediction, there exists the case that inside the shared merge list region, one merge list can be derived for IBC and the other is for regular inter merge.

The proposed constraint is a bitstream constraint to disallow such case from encoder. The first merge coded block in the region will be used to determine which list is allowed.

It was commented that with simplified merge list derivation, it might not be necessary to impose such constraint.

It was commented that the imposed bistream conformance constraint would require encoder to do the right choice by disallowing some blocks being coded in merge mode. Syntax constraint might be another solution.

JVET-N0174 proposes the same method.

Question: can IBC merge list and inter merge be derived in parallel? It is confirmed to be true at decoder.

It was commented that encoder may need extra efforts.

Recommendation: revisit on track or plenary for further discussion.

JVET-N0601 Crosscheck of JVET-N0093 (CE8-related: Disallowing coexistence of regular merge and IBC merge in SMR) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0095 CE8-related: Unified method for coding BVD and MVD [S.-T. Hsiang, S.-M. Lei (MediaTek)]

This contribution proposes a unified method for coding motion vector differences (MVDs) and block vector differences (BVDs) for coding screen contents. The proposed method corresponds to the same binarization as that of VVC Draft 4 while increasing the number of context-coded bins for coding the prefix part. Each vector component can use up to five context-coded bins. The proposed method reportedly provides -1.41%, -1.20% and -0.99% luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions (CTCs) while increasing the number of context variables by four.

Code MVD/BVD magnitude in the form of MSB index + refinement bins.

When more contexts are assigned, higher gain can be achieved for SCC but not natural content.

Minor gain (0.02~0.05%) for natural content, due to the increase of contexts.

The gain is -1.41%/-1.20%/-0.99% luma BD-rates for the AI/RA/LB for Class TGM with IBC on.

The increase in the number of context coded bins is 6 (it was 4).

It was commented that when further studied in a CE, this aspect should be studied (bin to bit ratio, etc. as studied in CE7). Low QP cases should also be tested.

Note that: JVET-N0289 test b is asserted to be the same method for binarization and context modeling.

Recommendation: further study in next CE.

JVET-N0677 Crosscheck of JVET-N0095 (CE8-related: Unified method for coding BVD and MVD) [Y.-C. Sun (Alibaba)] [miss] [late]

JVET-N0096 CE8-related: A fixed updating order for IBC reference memory [C.-Y. Lai, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution, a fixed updating order for intra block copy (IBC) reference memory is proposed to replace the two updating orders depending on the coding block (CB) order. Currently in VTM4.0, the IBC reference memory used to store reconstructed samples in the left 128×128-luma/64×64×2-chroma coding tree unit (CTU) is updated according to the CB order within the current CTU. The updating order can be replacing “top-left => top-right => bottom-left => bottom-right” or “top-left => bottom-left => top-right => bottom-right” 64×64-luma/32×32×2-chroma units of the left CTU. In this proposal, it is proposed to always apply only one updating order as replacing “bottom-left => top-left => top-right => bottom-right” 64×64-luma/32×32×2-chroma units of the left CTU. In addition, it is proposed to add an encoder-only normative constraint in the VVC specification to ensure the maximum CB size for IBC is “64×64 for luma and 32×32 for chroma” or smaller. It is reported that IBC with the proposed updating order for IBC reference memory achieves 0.00%, -0.64%, and -1.63% luma BD-rates in AI condition for common test condition (CTC), Class F, and TGM sequences, respectively, compared to VTM4.0 IBC. It is asserted that the process of updating IBC reference memory is simplified while achieving coding gains.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

X

Curr

The updating order of the four VPDUs in the CTU size memory will be from BL->TL->TR->BR

The gain for class F/TGM is 0.6%/1.6% in AI.

It was commented that this method will increase the bistream conformance checks for the available range of different positions, which may not be desirable.

It was commented that there are two updating orders of IBC memory. In this proposal it is asserted to be one.

It was commented that current IBC encoder in VTM only search up to 16x16 blocks; with the increase of block size of IBC, gains can be achieved.

It was suggested to have more study in terms of additional checks required in different platforms (software, hardware, etc.).

JVET-N0249 proposes a method of similar concept.

Recommendation: no action.

JVET-N0702 Crosscheck of JVET-N0096 (CE8-related: A fixed updating order for IBC reference memory) [T.-S Chang (Alibaba)] [miss] [late]

JVET-N0173 Non-CE8: IBC Reference Area Rearrange [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

In VVC Draft 4.0, IBC mode reference area is extended to left CTU. However, in some case the IBC reference area is not continues. An IBC reference Area rearrange method is proposed in this contribution. Compare with CTC and TGM sequences (IBC and HashME on for all classes), the proposed method achieves -0.06%/0.00%/xx% in AI/RA/LDB coding gain for CTC overall, -0.22%/-0.17%/-0.25% AI/RA/LDB coding gain for Class F and -0.51%/-0.56%/-0.24% AI/RA/LDB coding gain for TGM sequences.

The QT scan order is proposed to change, from TL->TR->BL-BR to TL->BL->TR-BR

It was commented that for multi-standard implementation, the change in scan order it would be a problem.

It was reported by the proponent that in JVET-0822, when this method is applied on top of the one in JVET-0249 (memory rearrangement), some additional 0.3% gain can be observed.

It was commented that the gain is relatively low to be considered for a change.

See notes in JVET-N0096 for more discussion.

Recommendation: no action.

JVET-N0611 Cross-check of JVET-N0173 [?? (??)] [miss] [late]

JVET-N0174 Non-CE8: Exclusive Regular Merge and IBC Merge in One Shared Merge List Area [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

See notes in JVET-N0093.

JVET-N0640 Crosscheck of JVET-N0174 (Non-CE8: Exclusive regular merge and IBC merge in one shared merge list area) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0175 Non-CE8: IBC Reference Memory for Arbitrary CTU Size [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

In VVC Draft 4.0, IBC mode reference area is extended to left CTU for any CTU sizes. However, the hardware reference memory is optimized with 128x128 samples size, if the CTU size is smaller than 128x128, the hardware reference memory is wasted. An IBC reference memory for arbitrary CTU size is proposed. Compare with CTC and TGM sequences (IBC is also on for nature classes), the proposed method 1 achieves xx%/xx%/xx% in AI/RA/LDB coding gain for CTC overall, -4.34%/-3.33%/xx% AI/RA/LDB coding gain for ClassF and -13.16%/-6.55%/xx% AI/RA/LDB coding gain for TGM sequences and the proposed for 64x64 CTU size method 2 achieves xx%/xx%/xx% in AI/RA/LDB coding gain for CTC overall, -4.07%/xx%/xx% AI/RA/LDB coding gain for ClassF and -12.40%/xx%/xx% AI/RA/LDB coding gain for TGM sequences for 64x64 CTU Size

Method1:

Instead of always left 1 CTU and current CTU can be used as IBC reference are for all CTU size, in the proposed method, if the CTU size is smaller than 128x128, the left ((128/CTUsize)2  - 1) th till the left one CTU and the current CTU can be used as IBC reference area.

It was reported the method 1 achieves -4.36%/-3.44% AI/RA coding gain for ClassF and -13.37%/-6.72% AI/RA coding gain for TGM sequences.

It was confirmed that there is no major problem in hardware for method 1. There might be some additional logic check, but it is minor.

JVET-N0251 item 2 and JVET- N0384 method 2 are the same to JVET-N0175.

In JVET- N0384 method 1, it was proposed to use one more reference block in addition to method 1 in JVET-N0175. About 0.2% additional gain for TGM sequences is reported.

It was commented that it makes area checking more complicated.

Method2:

Some of the CTU vertical edges are discontinues in the hardware reference memory, because the hardware reference memory is implemented as 128x128 sample area. Method 2 introduces the discontinuity between some CTUs to avoid double access of the hardware memory.

Then the position of the discontinues vertical edge is determined from a fixed position (for example a left picture boundary) and the Ctu size as following,

If NumofCtu % (128/Ctusize) is equal to 0, then the left vertical edge of the reference CTU is set as discontinues vertical edge. Where NumofCtu is define as the number of CTUs from the left picture boundary to the reference CTU.

A discontinues vertical edge means: when there is a reference block, the left coordinator of the reference block is in the left of the discontinues vertical edge and the right coordinator of the reference block is in the right of the discontinues vertical edge, then such a reference block is invalid.

It was commented that this depends on implementation. And, it is not a problem in some implementation.

Recommendation: consider adopt JVET-N0175 method 1/JVET-N0251/JVET- N0384 method 2

JVET-N0540 Crosscheck of JVET-N0175: Non-CE8: IBC Reference Memory for Arbitrary CTU Size [X. Xu (Tencent)] [miss] [late]

JVET-N0176 Non-CE8: IBC Merge List Simplification [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)]

In VVC Draft 4.0, the IBC merge list is constructed in a similar way as the regular merge with maximum 9 pruning. The proposed methods simplify the spatial candidate pruning and HMVP candidate pruning for IBC merge list construction. The proposed methods can reduce the pruning from 9 to 3 with minor coding efficiency different for both CTC ClassF and TGM sequences.

The best proposal from this contribution is as follows:

1) reduce the spatial merge candidates from 5 to 2 ( A1 and B1)

2) reduce the HMVP pruning to 2

- Limit HMVP pruning up to 2

- Only compare the first available spatial candidate with the 1st and 2nd last coded entries

For item 2) only, the impact of HMVP pruning reduction shows loss about 0.1% or less in TGM

See notes under JVET-N0382.

JVET-N0683 Crosscheck of JVET-N0176 (Non-CE8: IBC Merge List Simplification) [J. Nam, J. Lim, S. Kim (LGE)] [miss] [late]

JVET-N0201 Non-CE8: IBC modifications [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)]

This contribution describes three modifications to intra block copy (IBC) design: (1) Decoder fallback mode for illegal motion vectors. (2) Reference area extension for CTU size < 128x128. (3) Avoid prediction size and block vectors not fitting IBC local buffer.

Following average results of luma BD rate difference for All Intra, RA and LDB are reported relative to VTM-4:

(1) Decoder fallback mode for illegal motion vectors.

(2) disable 128x128 IBC, fixed MV for 128x64/64x128 IBC

No apparent impact on coding performance from either of the two aspects.

For (1), the asserted benefit is to have defined decoder behaviour when IBC vector points outside the current defined reference range. It was a failure of bitsteam conformance. And with the proposal, it becomes part of the valid bitstreams. This requires the decoder to perform all the existing bistream conformance checks (checking if IBC vector points outside the current defined reference range) in the current design.

It was commented that in some implementations it is still required to perform these checks.

It was also commended that in some other implementations (e.g. closed ecosystem), not all the decoders need to do this.

The proposed change would convert the encoder bitstream constraints into part of the decoding process.

Recommendation: to further study in the next CE the proposed fallback mode, in terms of the complexity aspects (operations required at decoder side) and potential simplification of design.

JVET-N0516 Crosscheck of JVET-N0201 (Non-CE8: IBC modifications) [S.-T. Hsiang (MediaTek)] [miss] [late]

JVET-N0202 AHG16/non-CE8: Report on conformance check failures of IBC block vectors [J. Li, C.-W. Kuo, C. S.Lim (Panasonic)] late]

The identified issue has been addressed by JVET-N0383 and JVET-N0251 item 3. No need to be presented here.

JVET-N0249 Non-CE8: An alternative search area for IBC [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document presents an alternative search area setting for intra block copy when CTU size is 128x128. For class F/4:2:0 TGM, the BD-rate numbers of the proposed setting are -0.90%/-2.12% for AI, -0.74%/-1.74% for RA.

Figure 2. Proposed reference area when the 1st partitioning for the current CTU is not vertical BT.

Figure 3. Proposed reference area when the 1st partitioning is for the current CTU vertical BT.

The reported gain includes an encoder only change to allow IBC local search goes beyond fixed 128x128 window.

The overall gain is 2.1%/1.7% in AI/RA for TGM.

The gain of the encoder only change is 0.2%/0.8% in AI/RA for TGM. The increase 101%/106% in AI/RA

See notes in JVET-N0096.

Recommendations: no action.

JVET-N0250 Non-CE8: Reference memory reduction for intra block copy [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document presents solutions to reduce on-chip memory for intra block copy. Solution A provides 20% memory reduction with BD-rate of -0.13% for 4:2:0TGM. Solution B provides 6.7% memory reduction with BD-rate of -2.57% for 4:2:0TGM.

Solution 1: reduce 3 additional 64x64 buffers to 8-bit, the saving is about 4.5KB local memory. The same logic of current design in terms of availability of search range applies. No performance loss compared to having 10-bit memory for the additional 3 64x64 buffer.

Solution 2: a separate IBC buffer with 7-bit precision, 128x128 in size. The saving in local memory is 1.5KB compared to current design. -2.57% AI gain in TGM is observed. The logic of current design in terms of availability of search range has been changed to allow CU based reused of the memory.

For solution 1, it is commented that if a reference block comes from across the 64x64 region boundary, samples in part of the reference block may have to be shifted while other parts of the reference block may not. This seems to be undesirable.

Recommendation: no action.

Note: solution 2 is the same as item 1) in JVET-N0472. See notes there.

JVET-N0684 Crosscheck of JVET-N0250 (Non-CE8: Reference memory reduction for intra block copy) [J. Nam, J. Lim, S. Kim (LGE)] [miss] [late]

JVET-N0251 Non-CE8: Intra block copy clean-up [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document presents several clean-ups for the current intra block copy (IBC) design to clarify reference buffer size, reference area restriction and reference area for small CTU size.

Item 1, clarify the reference buffer size is 128x128, not dependent of CTU size.

It is confirmed by actions taken from item 2.

Item 2, increase reference area for small CTU sizes.

See notes in JVET-0175

For item 3, bug fix for reference area definition to avoid read/write conflict.

see notes in JVET-N0383.

For item 4, an encoder only change to allow IBC local search goes beyond fixed 128x128 window. 0.2%/0.8% gain in AI/RA for TGM. The increase 101%/106% in AI/RA.

Recommendations: adopt the encoder only change in item 4 to VTM-5.0;

JVET-N0541 Crosscheck of JVET-N0251: Non-CE8: Intra block copy clean-up [X. Xu (Tencent)] [miss] [late]

JVET-N0289 CE8-Related: On MVD Coding [M. Salehifar, S. Paluri, S. Kim (LGE)]

This contribution proposes unified MVD coding to address the different characteristics of IBC MVD (BVD, Block Vector Difference). Method 1 is a MVD coding based on VTM MVD binarization with modified context model. Method 2 proposes more context coded bins and modified context modeling. Specifically, abs_mvd_prefix is introduced, which contains gt0, gt1, gt3, gt7, and gt15 and is context coded. Method 1 reportedly provides 0.24%, 0.26 % and 0.06 % luma BD-rates improvement for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions. Method 2 reportedly provides 1.03%, 0.88% and X.XX % luma BD-rates for the AI, RA, and LB settings, respectively, for Class SCC under the CE8 common test conditions.

Two methods are proposed.

The first one, separate context models between x and y components. ~0.3% gain for AI/RA in TGM class.

The second one is the same as in JVET-N0095.

It was commented that the gain is relatively low for low level change consideration.

Recommendation: no action for the first one. further study in next CE for the second one.

JVET-N0722 Crosscheck of JVET-N0289 (CE8-Related: On MVD Coding) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0316 CE8-related: Default Processing for IBC Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution presents a default processing method for IBC mode in order to prevent un-defined decoder behavior. There are 3 encoder constraints in VTM4.0 which are heavily relied on the quality of the encoder. First, the IBC merge list may not be fully filled. Therefore, encoder can only signal merge index which does not exceed the valid size of IBC merge list. Second, the encoder can only signal valid block vector whether it is skip/merge mode or AMVP mode. Third, the encoder can only signal chroma IBC mode when collocated luma blocks are all coded with IBC mode and the block vectors are all valid. In this contribution, a default processing method for IBC mode is designed which uses a default value as the prediction samples to prevent these un-defined decoder behaviors. The simulation result of “Test 1: Apply default processing method to IBC merge/skip mode” shows 0.00% gain in AI case, 0.03% gain in RA case and 0.04% gain in LB case in screen content coding materials. “Test 2: Test 1 + Apply default processing method to IBC chroma mode” shows 0.02% gain in AI case, 0.04% gain in RA case and 0.06% loss in LB case. “Test 3: Test 1 + Apply default processing method to CU based IBC chroma mode” shows 0.18% gain in AI case, 0.02% loss in RA case and 0.04% loss in LB case.

- The IBC merge list may not be fully filled. Therefore, encoder can only signal merge index which does not exceed the valid size of IBC merge list.

- The encoder can only signal valid block vector whether it is skip/merge mode or AMVP mode.

- The encoder can only signal chroma IBC mode when collocated luma blocks are all coded with IBC mode and the block vectors are all valid.

Recommendation: Adopt the proposal that if decoder receives (0, 0) block vector, a default process (filling with 1<<(bitdepth - 1)) is used.

For item 3, JVET-0466 proposes similar solutions.

Recommendation: study the impact of subblock based vs. CU based chroma BV in the next CE, in terms of performance, visual quality and complexity. (together with JVET-0466)

Recommendation: to further study in the next CE of the proposed default mode, in terms of the complexity aspects (operations required at decoder side) and simplification of the current design.

JVET-N0558 Crosscheck of JVET-N0316 (CE8-related: Default Processing for IBC Mode) [T. Zhou (Sharp)] [miss] [late]

JVET-N0317 CE8-related: Simplification on IBC Merge/Skip Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution presents a simplification method for IBC merge/skip mode by reducing the number of potential IBC merge candidates. On top of VTM4.0, the simulation results show 0.02% loss in AI case, 0.00% loss in RA case and 0.00% loss in LB case in screen content coding materials.

Modification on IBC merge list

In this contribution, the spatial candidates of B0, A0, B2 and pairwise candidate are removed.

The order to generate an IBC merge candidate list is modified as below:

- Spatial candidates for blocks A1, B1 (1 pruning required)

- 5 HMVP candidates (4 pruning required).

IBC merge/skip mode sharing list

Two related fixed applied in the proposal:

1) In order to enable parallel processing small IBC merge/skip CUs, sharing list method is integrated in VTM4.0.1. In this method, the 32 and below pixels region shares the same IBC merge list.

2) The IBC HMVP table is reset at the beginning of each CTU line.

Compared with JVET-N0176, it has two more pruning.

See notes under JVET-N0382

JVET-N0593 Cross-check of JVET-N0317 (CE8-related: Simplification on IBC Merge/Skip Mode) [F. Le Léannec, A. Robert (Technicolor)] [miss] [late]

JVET-N0318 CE8-related: Block Size Limitation for IBC Mode [Y. Han, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution presents 6 tests to evaluate the performance and complexity of IBC mode with different thresholds of block size. The thresholds defined by MxN means that the width and height of the IBC block must be both equal to or smaller than M and N. In these tests, the IBC mode is only enabled for the blocks whose size is under the threshold. In test 5 and 6, encoder constraint has been added and tested. For screen content coding materials, the simulation results of “Test 3: Threshold 64x64 in both encoder side and decoder side” achieves 0.79% gain in AI case, 0.87% gain in RA case and 0.80% gain in LB case. “Test 6: Threshold 16x16 in encoder side, 64x64 in decoder side” shows no run time increase.

The encoder side constraint removal (which allows IBC mode to be checked for larger blocks) shows some benefits (0.8%~0.9% gain across configurations for TGM with 5%~8% runtime increase).

Recommendation: adopt this encoder only change into VTM-5.0

Block size constraints from syntax: disallow IBC block size to be 128x128.

It was commented that this has been fixed by bistream conformance check.

It was also commented that having syntax constraint would be more clear to reflect the constraint.

Recommendation: Revisit at track or plenary.

JVET-N0642 Crosscheck of JVET-N0318 (CE8-related: block size limitation for IBC mode) [Y.-C. Lin (MediaTek)] [miss] [late]

JVET-N0329 CE8-related: Encoder improvements on IBC search [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

In this contribution, several encoder improvements are proposed to enhance the intra block copy (IBC) search in the VTM-4.0, including 1) enhanced IBC local search by using block vector (BV) candidates from previously IBC coding units (CUs); 2) enhanced predictor-based IBC search by considering the BV candidates derived by history-based BV prediction (HBVP) and extended BV derivation; 3) adaptive SAD and SATD for encoder rate-distortion (R-D) calculation; 4) simplified IBC hash table generation in original sample domain. Compared to CE8 anchors, for Class SCC sequences, the proposed encoder improvements reportedly provide 1.26%, 0.84% and 0.34% BD-rate reductions for AI, RA and LD configurations, respectively, without increasing encoding and decoding time.

1) 8 best BV candidates of current block will replace 8 old ones in the 64-entry history table for IBC prediction.

2) If a reference block is coded in IBC, the reference block of the first reference block will also be checked for IBC search.

3) SAD/SATD adaptively used for distortion measurement

4) complexity reduction by using only the original pixel for hash generation (instead of using reshaped samples)

5) Bug fix

Recommendation: adopt the proposed encoder changes to VTM-5.0

JVET-N0543 Crosscheck of JVET-N0329: CE8-related: Encoder improvements on IBC search [X. Xu (Tencent)] [miss] [late]

JVET-N0382 CE8-related: unified IBC block vector prediction [X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, a unified block vector prediction list is constructed for both merge mode and AMVP mode in intra block copy mode. It is asserted to be simpler than either of the two modes in their original processes.

Simulation results report that, by using a unified candidate list for both merge and AMVP mode in IBC:

- -0.05%/0.00% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -0.13%/0.06% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

Simulation results report that, by using the proposed simpiflied list for merge mode in IBC:

- -0.02%/0.00% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -0.12%/-0.13% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

Compared with JVET-N0176 and JVET-N0317, two differences were proposed:

(1) some default candidates were proposed

(2) it was proposed to apply the proposed list to both merge list and AMVP list.

All related proposals show similar and minor BD-rate change. It was commented that the simplest method is preferred, which might be a combination of JVET-N0176, JVET-N0317 and JVET-N0382. That is: two spatial candidates (A1 and B1), 2 HMVP pruning, 0 vector padding and predictor list applied to both merge and AMVP lists.

Recommendation: put (0, 0) as default vector(s) to merge list.

Recommendation: Revisit: consider adopting the combination pending on the combined results are available.

JVET-N0520 Crosscheck of JVET-N0382 (CE8-related: unified IBC block vector prediction) [C.-C. Chen (MediaTek)] [miss] [late]

JVET-N0383 Non-CE8: IBC search range adjustment for implementation consideration [X. Xu, X. Li, S. Liu (Tencent)]

This contribution has been discussed in plenary for its technical aspect.

Same solution has been provided in JVET-0251 item 3.

Recommendation: adopt into VVC/VTM-5.0.

JVET-N0610 Cross-check of JVET-N0383 [?? (??)] [miss] [late]

JVET-N0384 Non-CE8: IBC search range increase for small CTU sizes [X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, a modification is made on intra block copy search range when the CTU size is smaller than 128x128. In this case, the size of local memory for IBC prediction is larger than one CTU size so more reference area can be stored in local memory and accessed for IBC prediction. It is asserted that by allowing fully utilzing the local memory for IBC prediction, coding efficiency improvement can be achieved. In the simulation, both anchor and tested methods are with the CTU size equal to 64x64.

In test A, N-1 left CTUs plus current CTU can be used for IBC prediction. In addition, parital data of left Nth CTU are available, the same availablity logic of VTM-4.0 applies to this CTU. Simulation results report that:

- -4.36%/-3.44% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -13.37%/-6.72% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

In test B, only N-1 left CTUs plus current CTU can be used for IBC prediction. The left Nth (left most) CTU in the reference area is marked not available. Simulation results report that:

- -4.34%/-3.33% BD rate changes are observed for AI/RA configurations for class F when compared to VTM-4.0.1 anchor.

- -13.16%/-6.55% BD rate changes are observed for AI/RA configurations for class TGM when compared to VTM-4.0.1 anchor.

See notes in JVET-N0175.

JVET-N0459 CE8-related: Modified block vector coding for IBC [J. Nam, H. Jang, J. Choi, J. Heo, J. Lim, S. Kim (LGE)]

This contribution proposes some improvements on block vector coding for IBC mode. Syntaxes for block vector coding are used by separating those of motion vector coding. Especially, context modeling and binarization method are designed for block vector coding. In addition, block vector predictors are clipped based on position of current coding block. From experiment results, following BD rate changes are observed over VTM-4.0 with IBC:

- AI: -0.49% (CTC), -0.44% (Class F), -1.09% (TGM)

- RA: -0.01% (CTC), -0.34% (Class F), -0.50% (TGM)

1) Sperate contexts for mvd and bvd coding modules. So two syntax tables with the same structure.

2) On top of that, some elements, such as abs_bvd_minus2 are with different binarization and bypass coded. Change is made to abs_bvd_greater0_flag in terms of context modeling.

3) BVP is clipped if it is out of valid range.

The above results are from all three changes.

It was commented that the decoder needs to perform checking of BVP validity. Additional cycles will be needed to generate the predictor list.

It was commented that for different position inside the CTU, the clipped position would be different, which makes the clipping more complex.

The proponent claimed that the benefit of clipping is about 0.1%. Most of the overall gain (~0.8%) comes from changes in binarization.

Recommendation: further study in next CE.

JVET-N0460 CE8-related: Default candidates for IBC merge mode [J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

This contribution proposes to insert default candidates for IBC merge mode. A default position is iteratively added at the last position until merge list is fully filled. It prevents from accessing an empty candidate list.

Some minor coding efficiency improvement (up to 0.2% gain in some case) is observed by using default position other than (0, 0) to fill up the merge candidate list.

Some supports from experts for having (0, 0) as default candidates always, as in JVET-N0317.

It seems that no default values can solve the validity problem always.

Recommendation: no action.

JVET-N0461 CE8-related: Signaling on maximum number of candidates for IBC merge mode [J. Nam, H. Jang, J. Lim, S. Kim (LGE)]

In VVC Draft 4, maximum number of candidates for IBC merge mode and normal merge mode is defined by one syntax element (six_minus_max_num_merge_cand), which is originally signalled when current tile is not intra. Considering that IBC mode is activated for intra tile, the syntax is also signalled to define the maximum number of candidates for IBC even if intra tile. Therefore, this contribution proposes to signal a syntax element indicating the maximum number of candidates for IBC merge mode at tile group header. By the syntax element, it could be supported to have different maximum number of candidates between normal and IBC merges.

Currently we have separate size signaling for regular merge list and subblock merge list. We also have an ongoing discussion for a separate size signaling of TPM. Do we want a separate size signaling of IBC merge mode as well?

It was commented that the reason for TPM separate signaling is for resolving a defect when merge list size < 2. What is the benefit of having a separate merge list size for IBC?

It is encouraged to consider testing a shorter IBC merge candidate list size. Without too much penalty, it could help to reduce the complexity of IBC list derivation.

Recommendation: no action.

JVET-N0630 Crosscheck of JVET-N0461: CE8-related: Signaling on maximum number of candidates for IBC merge mode [L. Pham Van (Qualcomm)] [miss] [late]

JVET-N0466 Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

See notes in JVET-0316.

JVET-N0603 Crosscheck of JVET-N0466 (Non-CE8: The corner case handling regarding mv derivation for Chroma IBC in dual tree structure) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0467 Non-CE8: Experimental result for various size of IBC block with optimized syntax signaling [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

See notes in JVET-N0318

JVET-N0604 Crosscheck of JVET-N0467 (Non-CE8 : Experimental result for various size of IBC block with optimized syntax signaling) [Y. Han, W.-J. Chien (Qualcomm)] [miss] [late]

JVET-N0472 Non-CE8: On IBC reference buffer design [J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document analyzes several issues in the current IBC reference buffer design and presents a different design to address the issues. An independent IBC reference buffer is proposed instead of mixing with decoding memory.

A Dedicated IBC memory buffer with different bit depths.

Remove most bitstream conformance constrains for IBC

1) With 6.7% memory reduction (7-bit), BD-rate is -0.99% classF, -2.57% TGM for AI

2) With 6.7% memory increase(8-bit), BD-rate is -1.31% classF, -3.23 % TGM for AI

3) With 20% memory reduction (8-bit but 96x128 in size), BD-rate is -0.43% classF, -0.93% TGM for AI

IBC reference buffer status

Before decoding current CU

After decoding current CU

It is commented that a dedicated memory for IBC with bit shifts may require additional buffer in some implementation, where otherwise the reuse of reconstruction sample buffer is possible.

Recommendation: further study in the next CE in 1) and 3).

JVET-N0822 Non-CE8: Combination of JVET-0173 and JVET-N0249 for improved IBC buffer utilization [H. Gao, S. Esenlik, B. Wang, A. M. Kotra, J. Chen (Huawei)] [late]

For information only. Do not need to take action on this.

## 2.2. Palette

JVET-N0258 CE8-related: Palette Mode Coding [W. Zhu, L. Zhang, J. Xu, K. Zhang, H. Liu, Y. Wang (Bytedance)]

In this proposal, the palette mode is coded as a prediction mode for a coding unit. Therefore, the prediction modes for a coding unit can be MODE_INTRA, MODE_INTER, MODE_IBC and MODE_PLT. The binarization of prediction modes is changed accordingly. The simulation is conducted follow the CE 8 test condition described in M1028. The proposed method is evaluated on top of CE8.2.1. Results reportedly show that, when IBC is enabled, on class F, the proposed method achieves -0.09%, -0.08% and -0.01% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On 4:2:0 TGM, the proposed method achieves -0.40%, -0.14% and -0.21% BD rate changes on luma component under RA, AI and LDB configurations respectively. When IBC is turned off, on class F, the proposed method achieves -0.01%, -0.02% and 0.02% BD rate changes on luma component under RA, AI and LDB configurations respectively.  On 4:2:0 TGM, the proposed method achieves -0.31%, -0.31% and -0.19% BD rate changes on luma component under RA, AI and LDB configurations respectively.

Improvement/fix of existing PLT mode design.

Coding of MODE_INTRA, MODE_INTER, MODE_IBC and MODE_PLT flags.

In intra tile group, signal IBC flag and PLT flag only; in P/B tile groups, signal pred_mode_flag before IBC and PLT flags.

It is asserted that in current PLT anchor software, it is possible that when IBC flag is signaled, the PLT flag still needs to be signaled. So it appears to be some redundancy there.

Recommendation: include this syntax change into the base PLT software, in next CE.

JVET-N0701 Crosscheck of JVET-N0258 (CE8-related: Palette Mode Coding) [T.-S. Chang (Alibaba)] [miss] [late]

JVET-N0259 CE8-related: compound palette mode [W. Zhu, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document proposes a new coding mode to combine palette mode and IBC prediction. On top of CE8.2.1, on Class F, in AI/RA configuration, the results show -0.12%/-0.24%/ BD-rate luma gain; on TGM class, the results show -1.03%/-0.68% BD-rate luma gain for AI/RA.

PLT and IBC combination.

IBC block vector is derived from the first available merge candidate, so no need to signal; PLT index being 0 means the predictor sample at this location is from IBC predictor; otherwise it will come from the palette predictor.

It was asked if IBC predictor will need to be ready before doing the proposed PLT mode. It is asserted that the proposed method requires a copy of pixel from local memory, which is simpler than MC.

Recommendation: study this method in next CE.

JVET-N0346 Non-CE8: Palette Mode in HEVC for YUV4:4:4 format [Y.-H. Chao, H. Wang, W.-J. Chien, V. Seregin, M. Karczewicz (Qualcomm)]

This document reports the results of palette base code (CE8-2.1: Palette Mode in HEVC) on top of VTM-4.0 for sequences of YUV4:4:4 format.  The results show that for dual tree disabled, where palette mode is applied on luma and chroma components jointly, palette mode provides additional -20.26% AI, -13.71% RA gain on top of IBC for TGM 444 sequences. For dual tree enabled on I slice, palette mode provides additional -8.16% AI, -7.57% RA gain on top of IBC.

Anchor is 4:4:4 extension of VTM (from Qualcomm).

The same thing from CE8-2.1 except for changes fixing variable for 4:4:4.

Tested on RExt CTC sequences and four TGM class sequences in 4:4:4.

Several tests have been conducted, with dual-tree turn on or off.

When dual-tree is turn off for VTM-4 with 4:4:4 support (JVET-N0414), about 1.0%/1.7% loss is observed for natural content in AI/RA config. However, about 5.5%/1.6% gain is observed for TGM class in AI/RA config. This may suggest some issues with dual-tree in 4:4:4.

Palette mode as in CE8-2.1 is tested in the software with both dual-tree on and off. In the following results, both IBC and HashME are turned on.

- When dual-tree is off, 20.3%/13.7% gain is observed for TGM class in AI/RA configurations.

- When dual-tree is on, 8.2%/7.6% gain is observed for TGM class in AI/RA configurations.

It was commented that comparing the proposed VTM-4 with 4:4:4 support in JVET-N0414 with the SCM-8.7 software, there were around 30% loss for TGM in AI configuration.

Recommendation: Revisit based on the decision made to 4:4:4 support in VVC/VTM

JVET-N0690 Crosscheck of JVET-N0346 (Non-CE8: Palette Mode in HEVC for YUV4:4:4 format) [C.-Y. Lai (MediaTek)] [miss] [late]

JVET-N0405 CE8-related: Palette Mode Simplification [Y.-C. Sun, T.-S. Chang, J. Lou (Alibaba), Y.-H. Chao, V. Seregin, M. Karczewicz (Qualcomm)]

This document proposes methods to simplify the palette mode of CE8-2.1. Three modifications are proposed:

- In order to limit buffer size storing palette_index_idc to 256, it is proposed to interleave palette_index_idc and copy_above_palette_index_flag/palette_run for every 256 index runs.

- In order to limit peak bin rate, it is proposed to make an decoder constraint on the palette size of a CU depend on the CU size.

- In order to avoid dual pass reconstruction, it is proposed to interleave the escaped samples with other syntax elements.

The proposed methods are evaluated according to the CE8 description. The experimental results show no BD-rate difference between the proposed methods and CE8-2.1 while the proposed methods can reduce the implementation complexity.

For 1), It is asserted that without this constraint, maximumly 64x64 PLT indices need to be stored prior to the PLT map decoding. This seems to be a storage issue. With the proposed constraint, PLT map decoding is divided into groups, process one at a time (up to 256 indices). After the entire PLT map is processed, the reconstruction of pixels will start.

For 2), It is asserted that the worst case will be we have 4x4 blocks everywhere in the CTU, while for each of these blocks (if PLT coded), if the maximum PLT size is set to be 31, the paring of these indices will become a problem. It is proposed to limit the maximum PLT size to be at most the number of samples in the block or 31, whichever is smaller. The proposed constraint is implemented as a bitstream constraint, which the current encoder also does the same thing. So there is no performance change due to this added constraint.

For 3), escape coding is no longer grouped at the end, instead, it is decoded each time an escape index is parsed.

There is no BD rate change compared to the anchor as the changes are asserted to be lossless.

Recommendation: Revisit. To be discussed in a broader audience if these changes can address implementation issues raised in this proposal, if any. So that we can reach agreement within the group which way to design PLT (the original or the proposed), not just from a fraction of people in CE8.

JVET-N0608 Crosscheck of JVET-N0405 (CE8-related: Palette Mode Simplification) [W. Zhu (Bytedance)] [miss] [late]

JVET-N0550 CE8-related: Subblock based Palette Mode [Y.-H. Chao, T. Hsieh, M. Karczewicz (Qualcomm)] [late]

In this proposal, a subblock based approach is proposed to simplify the palette mode in CE8-2.1. Similar to the coefficient group (CG) used in transform coefficient coding, a CU is divided into multiple segments, each consists of m samples, where index runs, palette index values, and quantized colors for escape mode are encoded/parsed. The results show only minor loss compared to CE8-2.1: 0.11% AI, 0.00% RA for class F and 0.29% AI, 0.16% RA for class TGM.

It is asserted to 1) address the buffer issue (PLT index values) in worst case, i.e., 64x64 CU; 2) align the design with transform coefficient coding; 3) group ctx coded bins

Scan order is the same in PLT design, each CG has 16 samples along the scan direction. New syntax elements are designed to accommodate this change. Escape pixels are grouped.

Small loss observed for TGM, no changes for CTC.

Recommendation: further study in next CE

JVET-N0580 Cross-check of JVET-N0550: CE8-related: Line-based CG Palette Mode [X. Xiu (Kwai Inc.)] [miss] [late]

## 2.3. BDPCM

Xxx

JVET-N0366 CE8-related: Modified limitation on context coded bins for CE8-3.1a and CE8-5.1a [X. Zhao, X. Li, X. Xu, S. Liu (Tencent)] [placeholder] [late]

In CE8-4.4a and CE8-5.1a, the number of context coded bins for entropy coding Block-DPCM and Transform Skip residuals is limited to be average 2 per coefficient. In this contribution, an alternative approach for achieving same number of context coded bins is proposed. The proposed method relocates the signaling of parity flag to be after gt2 flag and allows more context-coded bins for gt2 flag given the total budget of context coded bins.

The proposed method shows 0.39%/0.10% further gain in AI/RA in TGM class on top of CE8-4.4a, which was adopted in this meeting.

The gain is reported from the observation that parity bin show less coding gain than other syntax.

Recommendation: further study in the next CE.

JVET-N0413 CE8-related: Quantized residual BDPCM [M. Karczewicz, M. Coban (Qualcomm)]

This document proposes to apply BDPCM in quantized residual domain instead of reconstructed sample domain. BDPCM signalling and prediction directions are same as the methods utilized in sample domain BDPCM. The simulations results show Class F: -3.91% AI, -2.68% RA, BD-Rate, and Class SCC: -5.98% AI, -3.57% RA, -2.73% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions for quantized residual BDPCM case. For both proposed methods combined, the simulations results show Class F: -4.04% AI, -2.92% RA, -2.09% LB BD-Rate, and Class SCC: -6.33% AI, -3.83% RA, -2.79% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions.

Prediction is done like normal intra prediction (vertical/horizontal) first, BDPCM applies in quantized residual domain. No division of 4xN or Nx4 into sub regions in the proposal. Simulation results show similar performance as the BDPCM scheme in CE8-3.1x. It is asserted to be aligned with existing reconstruction process with intra coding.

The reported gain on top of CE8-4.4a is 1.4%/1.0% AI/RA in class F; 2.1%/1.1% AI/RA for TGM

Remove the context dependency at CG boundary shows only minor BD rate changes (~0.05% to 0.1% for TGM) compared with the first aspect (quantized residual domain BDPCM).

Recommendation: consider adoption into VVC (discuss in the track/plenary).

JVET-N0703 Crosscheck of JVET-N0413 (CE8-related: Quantized residual BDPCM) [T.-S Chang (Alibaba)] [miss] [late]

## 2.4. Transform Skip

JVET-N0094 CE8-related: Context modelling of transform skip mode [M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this proposal a context modelling method for coefficient coding of the transform skip mode is introduced. It is proposed to derive the context models for coefficient coding from three previously decoded neighboring coefficients located at the right, bottom, and bottom-right positions of the current to-be-decoded coefficient. A separate context set is defined for transform skip mode. It is reported that under RA configuration this proposal can achieve -0.04%/0.00%/-0.08% Y/Cb/Cr BD-rate for VTM4.0 under common test conditions (CTC).

1: reduced number of neighbors: using 3 neighbors instead of 5.

With the reduction of neighbors, the gain on top of VTM-4.0 is 1.0%/0.7%/0.6% in AI/RA/LB for TGM class.

2: increase #of context coded bins for transform skip syntax

With 63 additional contexts added, the gain on top of VTM-4.0 is 2.1%/1.4%/1.1% in AI/RA/LB for TGM class.

Recommendation: further study in next CE.

JVET-N0768 Crosscheck of JVET-N0094 (CE8-related: Context modelling of transform skip mode associated Resources) [B. Wang (Huawei)] [miss] [late]

JVET-N0122 CE6/CE8-related: Alternative implementation of residual rearrangement for transform skipped blocks [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)] [placeholder] [late]

This contribution proposes an alternative implementation of transform skip and residual rearrangement CE8-4.1b using MTS framework: 1D transform skip by Identity Transform and 1D residual reorder. It is asserted that the proposal can unify MTS and residual rearrangement with on -0.82% and -0.67% of bdrate changes in ClassF for AI and RA, respectively; -1.02% and -0.76% of bdrate changes in SCC sequences, respectively

Rotation is applied to residues before transform. The proposal tries to apply identity transform to TS residues in the spirit of unification between MTS and TS.

The reported gains are: 1.02%/0.76% AI/RA in SCC TGM sequences, respectively About 0.4% of which is from an encoder only tool in JVET-N363.

It was commented that the CE8-4.1b (rotation) can achieve similar gain without using additional memory for rotation. The CE test also presents similar gain.

The proponent asserted that the memory used for MTS residue rotation can be reused.

This proposal is closely related to another one JVET-N0121.

Recommendation: Bring the whole package of both proposals in the next meeting for consideration. No action.

JVET-N0123 Non-CE6/CE8: Chroma Transform Skip [T. Tsukuba, M. Ikeda, T. Suzuki (Sony)]

Chroma TS is now not available in the current design. In this proposal it is proposed to enable it up to 4x4 chroma blocks.

Simulation results shows that when luma TS size is up to 4x4, by enable chroma TS up to 4x4 additionally, some luma BD rate gain in the range of 0.2%~0.3% for TGM class can be observed.

It was commented that Chroma TS may be more relevant for 4:4:4, where chroma will have higher impact for overall BD rates.

Recommendation: Revisit. After the general discussion of 4:4:4 support in VVC.

JVET-N0534 Crosscheck of JVET-N0123 (AHG13/Non-CE6/CE8: Chroma Transform Skip) [X. Zhao (Tencent)] [miss] [late]

JVET-N0357 CE8-related: Context Modelling of Sign for TS Residual Coding [B. Bross, T. Nguyen, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

The compression efficiency of screen content can be improved when using a modified residual coding (transform coefficient level coding) for the transform skip mode (TSM) as proposed in the input document JVET-M0464. One part of the design is the coding of the sign syntax element using context models instead of the bypass coding mode as can be found in the residual coding for regular transformed signals. This document describes a simple context modelling for the sign syntax element of the approach in the input document JVET-M0464. An improvement in terms of BD-rate relative to the approach in JVET-M0464 is about -0.26 for Class F and -0.42 for TGM for the AI configuration, and -0.25 for Class F and -0.20 for TGM in the RA configuration, respectively, in the IBC enabled configuration (CTC).

To track the statistics of coded signs to switch to additional contexts when the ratio of signs fall into different range.

Around 0.2%~0.3% additional improvements for class F and TGM

Note: JVET-N0455 is related to this proposal.

Recommendation: further study in next CE

JVET-N0401 Non-CE3/Non-CE8: Enable Transform Skip in CUs using ISP [S. De-Luxán-Hernández, T. Nguyen, B. Bross, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

This contribution enables the transform skip mode on CUs using ISP at the TU and CU level. The experimental results are the following:

- Test 1: gains of 0.01 (CTC), 0.29% (class F) and 0.44% (TGM) for the AI configuration. The corresponding encoder run-times are 101% (CTC), 102% (class F) and 103% (TGM). In the case of RA, the gains are 0.02% (CTC), 0.20% (class F) and 0.22% (TGM) and the associated encoder run-times are 100% (CTC), 100% (class F) and 101% (TGM).

- Test 2: gains of 0.00% (CTC), 0.26% (class F) and 0.52% (TGM) for the AI configuration. The corresponding encoder run-times are 100% (CTC), 101% (class F) and 104% (TGM). In the case of RA, the gains are 0.00% (CTC), 0.17% (class F) and 0.33% (TGM) and the associated encoder run-times are 100% (CTC), 100% (class F) and 101% (TGM).

Allow TS in ISP. In test 1, signaled per TU; in test 2, signaled per CU (if signaled, each of TU in the CU, if cbf=1, will use TS). Additional RD checks are needed at encoder for the selection of TS mode in ISP, which causes some runtime increase.

0.4%/0.2% gain for TGM in AI/RA in test 1; 0.5%/0.3% gain for TGM in AI/RA in test 2;

It is noted that in JVET-N0475, the same method is proposed as in Test 1 here (TU based).

Recommendation: further study in next CE

JVET-N0430 CE8-related: Transform skip restriction [J. Choi, J. Heo, S. Yoo, J. Choi, J. Lim, S. Kim (LGE)]

Current VVC text specification allows high QP up to 63 (from 51), and very large quantization step sizes prevent from occurring any non-zero coefficients for transform skip block. Therefore this contribution proposes to remove unnecessary transform_skip_flag coding in the low bit-rate environment. Since transform skip block does not occur in the low bit-rate environment (in high quantization parameter (QP)), we remove transform_skip_flag in this case. As a result, we can reduce redundant context-coded bin coding.

Propose to disable TS when QP is high.

| if(init_qp_minus26 <= 32) |  |
| --- | --- |
| transform_skip_enabled_flag | u(1) |
| if( transform_skip_enabled_flag ) |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| if(init_qp_minus26 <= 32) |  |
| cu_qp_delta_enabled_flag | u(1) |

It is asserted that when QP is high enough, all residues will be zero therefore transform skip mode will only deal with all zeros. There is no need to have TS mode in that case. One argument is that # of context coded bins can be saved.

It is commented that this case is not the worst case, rather it is an easier case to handle.

It seems that there is no critical problems to be solved by disallowing TS in high QPs.

Recommendation: no action.

JVET-N0455 CE8-related: Sign context modelling and level mapping for TS residual coding [M. Karczewicz, M. Coban (Qualcomm)]

This document proposes a local neighbor-based context derivation for sign coding and a level mapping scheme for TS residual coding proposed in JVET-M0464 input document. The simulations results show Class F: -3.27% AI, -2.37% RA, -1.94% LB BD-Rate, and Class SCC: -5.20% AI, -3.44% RA, -2.84% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions for sign context modelling case. For both proposed methods combined, the simulations results show Class F: -3.92% AI, -2.83% RA, -2.22% LB BD-Rate, and Class SCC: -5.83% AI, -4.06% RA, -3.15% LB BD-Rate, versus the VTM-4.0 anchor under common test conditions. Additionally, bitplane coding of TS residual elements are proposed. For bitplane coding, the simulations results show Class F: -2.60% AI, -1.78% RA, BD-Rate, and Class SCC: -3.94% AI, -2.71% RA, BD-Rate, versus the VTM-4.0 anchor under common test conditions.

1) Sign coding: 3 context depending the signs of top/left neighbors

In the context of CE8-4.4b, the additional gain is about 0.8%/0.5% in AI/RA for TGM class.

2) Level map coding: predict the levels before coding

On top of sign coding, additional 0.5%/0.5% gain in AI/RA for TGM class.

3) Coding pass of CE8-4.4x uses multiple passes for syntaxes. Propose to use 1-pass coding with rearrange of syntax and using bitplane.

Comparing with CE8-4.4a anchor, very similar results can be achieved. Claimed to be simplification, not coding efficiency improvement.

Recommendation: further study in next CE

JVET-N0594 Cross-chreck of JVET-N0455 (CE8-related: Sign context modelling and level mapping for TS residual coding) [F. Le Léannec, T. Poirier (Technicolor)] [miss] [late]

JVET-N0609 Crosscheck of JVET-N0455: CE8-related: Sign context modelling and level mapping for TS residual coding [T. Nguyen (HHI)] [miss] [late]

JVET-N0475 Non-CE3/6: Enabling Transform Skip for ISP [T.-C. Ma, X. Xiu, Y.-W. Chen, X. Wang (Kwai Inc.)]

See notes in JVET-N0401.

JVET-N0742 CE8-related: Joint test of JVET-N0094 and CE8-4.1 [M. G. Sarwer, O. Chubach, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek), S. Yoo, J. Choi, J. Heo, J. Choi, L. Li, J. Lim (LGE)] [late]

For information only. Do not need to take action on this.

JVET-N0763 CE8-related: Joint proposal for transform skip (CE8-4.1c, JVET-M0279 and JVET-N0094) [S. Yoo, J. Choi, J. Heo, J. Choi, J. Lim, S. Kim (LGE), M. G. Sarwer, O. Chubach, C. -W. Hsu, Y. -W. Huang, S.-M. Lei (MediaTek)] [late]

For information only. Do not need to take action on this.

## 2.5. Others

JVET-N0247 Non-CE: An improvement to hash-based motion estimation [J. Li, J. Xu, L. Zhang, K. Zhang, H. Liu, Y. Wang (Bytedance)]

This document presents an improvement to hash-based motion estimation. In the proposed method, all rectangle block can perform hash-based motion estimation without building corresponding hash map. In addition, we reuse the MV of hash mode to initialize the MV in the normal motion estimation. For TGM sequences, experimental results report 1.01% and 2.04% coding gain for RA and LD_B, with encoding time reduced to 92% and 92%. The anchor is VTM-4.0 and IBC is off. If IBC is on, 0.65% and 1.43% coding gain is reported, with encoding time reduced to 91% and 95%.

Extend the hash based search to rectangular blocks, in addition to existing design (only square blocks).

1.0%/2.0% gain RA/LB for TGM class, with IBC off

0.65%/1.4% gain RA/LB for TGM class, with IBC on

Encoder runtime reduction is also observed as 91% and 95% with IBC on.

Recommendation: adopt into VTM-5.0.

JVET-N0260 Non-CE8: Disabling fractional MVD search in DMVR for SCC [W. Zhu, H. Liu, K. Zhang, L. Zhang, J. Xu, Y. Wang (Bytedance)]

In DMVR mode, fractional motion vector difference (MVD) search is conditionally performed after the integer MVD refinement.  However, fractional MVD refinement may be inefficient for screen contents. To address this issue, this contribution proposes to disable fractional MVD refinement adaptively in DMVR mode.

The simulation followed the CE8 test condition. When IBC is turned on, simulation results reportedly show that the proposed method has 0.00%, 0.01% and 0.00% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.03% and -0.01% BD rate changes. When IBC is turned off, simulation results reportedly show that the proposed method has 0.00%, 0.00% and 0.01% BD rate changes compared with CE8 anchor. On class F and class SCC, the proposed method has -0.01% and -0.03% BD rate changes respectively.

Propose to disable fractional search refinement part in DMVR for SCC content, using a flag in tile group level to control. Encoder uses similar method as in fractional-pel offset disable flag for MMVD to determine if this flag should be on or off.

It was commented that this may help to reduce software complexity.

It was commented that by disabling fractional refinement, the subjective quality may be improved.

It was commented that there are some other aspects in the current design that it can be helpful to screen content by doing similar handling as this one.

Recommendation: adopt this tile group level flag to disable fractional search refinement part in DMVR.

JVET-N0739 Crosscheck of JVET-N0260 (Non-CE8: Disabling fractional MVD search in DMVR for SCC) [S. Ye (Hikvision)] [miss] [late]

JVET-N0337 CE8-related: Adaptive disabling of intra sample interpolation for screen content coding [X. Xiu, Y.-W. Chen, T.-C. Ma, X. Wang (Kwai Inc.)]

In this contribution, it is proposed to adaptively disable the intra interpolation at fractional sample positions by introducing control flags in SPS and tile group header. Experimental results reportedly show that when the IBC is enabled, the proposed method provides 0.42%, 0.42% and 0.45% BD-rate savings for Class SCC sequences for AI, RA and LD configurations, respectively. When the IBC is disabled, the corresponding BD-rate savings are 3.26%, 1.70% and 0.32% for AI, RA and LD configurations, respectively.

Propose to use a tile group level flag to disable intra sample interpolation.

When the IBC is off, the corresponding BD-rate savings are 3.26%, 1.70% and 0.32% for AI, RA and LD configurations.

When the IBC is on, the proposed method provides 0.42%, 0.42% and 0.45% BD-rate savings for TGM class.

It is shown that when using a SPS level flag to decide the choice, some performance loss is observed.

The gain in this case drops to -2.17%/-0.85% AI/RA for TGM class. That suggests that for screen content video, sometimes it is better to have fractional sample positions.

It was questioned the gain is relatively low for SCC, and required additional logic at pixel level.

Recommendation: further study in next CE

JVET-N0661 Crosscheck of JVET-N0337 (CE8-related: Adaptive disabling of intra sample interpolation for screen content coding) [H. Yang (InterDigital)] [miss] [late]

JVET-N0464 Non-CE8: MMVD Motion vector rounding for SCC [H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)]

In JVET-M0255, MMVD without Fractional Distances for SCC was introduced. MMVD distances are multiplied by 4 and it is interpreted as integer precision when fractional distance MMVD offset is disabled. However the predicted motion vector from merge candidate still has 1/16 precision, then the final MMVD motion vector can be fractional-Pel. In this proposal, it is proposed to round MMVD motion vector to integer precision when the fractional distance disable flag is enabled. Simulation results reportedly show 0.14% and 0.03% luma BD-rate saving on SCC sequences in Random Access configuration and Low Delay B configuration respectively over VTM-4.0. With the proposed method, motion compensation of MMVD does not need to interpolate the reference picture for fractional pel.

If integer offset of MMVD is used, propose to round the based MV to integer.

It was commented that the benefit is small for SCC content by introducing low level changes.

Recommendation: no action.

JVET-N0576 Crosscheck of JVET-N0464: Non-CE8: MMVD Motion vector rounding for SCC [X. Xu (Tencent)] [miss] [late]

## 2.6 Contribution related to test materials

JVET-N0386 AOV YUV4:4:4 sequence for VVC standardization and suggestions for class F [X. Xu, J. Ye, X. Li, S. Liu, L. Wu, C. Xie, K. Liu, B. Wang, P. Liu, K. Dong, Y. Kuang, W. Feng (Tencent)]

This contribution proposes to include the AOV test sequence in YUV4:4:4 format, whose 4:2:0 version is currently in Class F test set, as part of the 4:4:4 test set in VVC.

- It was proposed to include the AOV test sequence in YUV4:4:4 format into 4:4:4 test set for VVC.

A question was raised that: do we need 4:4:4 RGB class? The question should be discussed in plenary.

For item (1),

It was discussed in Track A that we likely need new Class F in 4:4:4 pending on the discussion of 4:4:4 support in VVC in plenary. It is reasonable on have the AOV test sequence in YUV4:4:4 format into 4:4:4 test set for VVC if AOV in YUV4:2:0 format is included in 4:2:0 test set. But, supporting of 4:4:4 in VVC has not been clear yet.

Recommendation: adopt AOV in 4:4:4: format for 4:4:4 test set for VVC pending on the plenary discussion for 4:4:4 support in VVC.

(2) It was proposed to replace some sequences in Class F of the current CTC with the SCC TGM 4:2:0 sequences, in particular:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| SCC TGM | 1920x1080 1920x1080 1920x1080 1920x1080 1920x1080 | AOV FlyingGraphics* Desktop Console ChineseEditing | 60 60 60 60 60 |

All sequences have 4:2:0 and 4:4:4 format so that the sequences of Class F in 4:2:0 and potential Class F in 4:4:4 could be aligned.

It’s confirmed that all sequences are 8-bits.

A question was raised that: do we need 10-bits class? It was commented most applications of SCC use 8 bits.

It was commented that the behavior of gaming sequences and other SCC sequences are different. It was suggest to have more class rather than one SCC class to reflect the applications.

If the RGB tests is decided later, the AOV in RGB 4:4:4 can be provided.

Recommendation: it could be discussed in Section 4 on updating Class F.

JVET-N0502 Five SCC TGM sequences of both YUV4:4:4 and YUV4:2:0 formats [T. Lin, K. Zhou, S. Wang (Tongji), L. Zhao (Shaoxing)] [late]

This contribution provides five screen content test sequences for VVC standardization activity. These test sequences can be categorized as TGM (text and graphics with motion) content. It is reported that the five test sequences representing a wide range of typical screen content commonly seen in SCC applications. It is also reported that this new content exhibits different coding characteristics when compared with the existing SCC TGM test sequences and thus are useful to evaluate SCC tools. It is reported that the five sequences are difficult to code and using the HEVC-SCC CTC AI lossy coding configurations and HM16.6-SCM5.2 reference software, the total compression ratio and average PSNR of the five sequences are 46.7 and 45.26dB, respectively. On the other hand, the total compression ratio and average PSNR of the eight TGM sequences in HEVC-SCC CTC are 69.6 and 46.96dB, respectively, which are 49% and 1.7dB higher than that of the five proposed sequences. It is also reported that the five sequences have coding mode distribution statistics quite different from the eight TGM sequences in HEVC-SCC CTC. Actually, four of the five proposed sequences were used in HEVC-SCC verification test reported in JCTVC-AA1006.

It was highlighted that sub-pixel anti-aliasing should be included in the sequences. It was confirmed that the sequences in TGM class already have that.

It was reported the proposed test sequences exhibit very different coding characteristics when compared with the existing SCC test sequences and thus are quite useful to evaluate new SCC tools. It was commented the difficult sequences is preferred for the tests.

It was commented that the background of document editing related sequences should be further clarified.

Generally, updating class F in the current CTC is encouraged.

After discussion, experts showed interests in the proposed sequences.

Further viewing tests, making the sequences public available, and RD performance are required for the proposed sequences.

Recommendation: no action.

# Discussion on updates of screen content CTC (class F)

1) make natural classes optional for LDB test in CE8

2) test IBC=0 case optionally with only class F (SCC content) for non-IBC tools in CE8, in addition to the CTC condition (IBC=0 for natural content, IBC=1 for class F)

It was encouraged to bring high resolution (e.g. 4K) to the meeting.

For current test condition, we have eight sequences for Class F and Class TGM:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| Class F | 1920x1080 832x480 1280x720 1280x720 | AOV* BasketballDrillText SlideEditing SlideShow | 60 50 30 20 |

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| SCC TGM | 1920x1080 1920x1080 1920x1080 1920x1080 | FlyingGraphics* Desktop* Console* ChineseEditing* | 60 60 60 60 |

There are five sequences proposed by JVET-N0502. (viewing test has not been performed)

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
|  | 1920x1080 1920x1080 1920x1080 1920x1080 1920x1080 | ClearTypeSpreadsheet BitstreamAnalyzer EnglishDocumentEditing ChineseDocumentEditing CircuitLayoutPresentation | 30 30 30 30 30 |

*both 4:2:0 and 4:4:4 formats are available.

Option (1):

In JVET-N0386, it was suggested to update class F as:

| Category | Resolution | Sequence name | Fps |
| --- | --- | --- | --- |
| Class F | 1920x1080 1920x1080 1920x1080 1920x1080 1920x1080 | AOV FlyingGraphics Desktop Console ChineseEditing | 60 60 60 60 60 |

It was commented that the updated Class F might introduce more difficulty to finish CTC test since Class F is mandatory now. It was commented that the setting of new added sequences are the same to the setting in CE8.

Recommendation: update Class F by option (1) and keep class F mandatory.