| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0806 |
| --- | --- |

| Title: | Suggested luma mapping with chroma scaling modifications in N0220/N0389/N0477 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Edouard François, Christophe Chevance (Technicolor) Yan Ye, Jie Chen, Ruling Liao (Alibaba) Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen (Dolby) | Tel: Email: | edouard.francois@technicolor.comyan.ye@alibaba-inc.com pyin@dolby.com |
| Source: | Technicolor, Alibaba Group, Dolby Laboratories Inc. |  |  |

_____________________________

# Abstract

This contribution uses JVET-N0220 as the base package and combines the following chroma residual scaling related aspects from JVET-N0389 and JVET-N0477:

- From JVET-N0389: when dual tree partitioning is on, the scaling factor is derived using the top-left reconstruction sample neighboring the luma block containing the luma sample collocated with the top-left sample of the chroma block;

- From JVET-0477: 1) when single tree partitioning is on, and when the block is intra coded, the scaling factor is derived using the top-left reference sample used in intra prediction; 2) chroma scaling factor derivation is conditioned upon chroma cbf’s.

Further, it is proposed to disable chroma residual scaling if the chroma block contains less than or equal to 8 samples.

For SDR content, reported {Y, U, V} BD-rate variations are {0.03%, -0.61%, -0.56%} in RA. For HDR content, reported BD-rate variations are {0.40%, -6.81%, -6.71%} in RA, in terms of wPSNR.

# Introduction

JVET-N0220, JVET-N0389 and JVET-N0427 all propose changes to the luma mapping with chroma scaling (LMCS) tool. This contribution is a combination of these three proposals. In this combined proposal, JVET-N0220 is used as the basis package, and some elements from JVET-N0389 and JVET-N0427 are incorporated.

JVET-N0220 proposes the following elements targeted at simplifying LMCS. It is reported in JVET-N0220 that the proposed design simplifications do not negatively impact coding performance.

- In chroma scaling factor derivation, instead of averaging the entire luma prediction block, the top-left mapped luma sample of the luma prediction block collocated with the chroma block is used to reduce pipeline delay

- Add clipping on chroma residual signal before scaling to reduce storage size

- Unification of the fixed-point precision used in luma mapping and chroma residual scaling to 11

- Unification of the method of calculating chroma residual scale with the method of calculating the luma inverse scale, and removal of the pre-computed LUT

JVET-N0389 proposes to enable the chroma residual scaling process for dual-tree partitions, but derives the chroma scaling factor using reconstructed samples on the top and/or to the left of the top-left corner of the luma block that contains the luma sample collocated with the top-left sample of the chroma block. Several variants are proposed and tested in JVET-N0389 and are reported to better balance luma/chroma BD-rate variations compared to VTM4.0 (where chroma scaling is disabled for dual-tree partition).

JVET-N0477 proposes to simplify the luma averaging process used to derive the chroma scaling factor in the following way: for intra coded blocks, the top-left reference sample used to generate the intra prediction is used to derive the chroma scaling factor; for inter coded blocks, up to 4x4 luma prediction samples are used to calculate the average, which is then used to derive the chroma scaling factor.  JVET-N0477 further proposes to bypass the chroma scaling factor derivation process if tu_cb_cbf and tu_cr_cbf are both equal to 0. JVET-N0477 reports complexity and latency reduction with negligible impact on BD-rate performance.

In this contribution, the following process for chroma scaling factor derivation is proposed:

- When dual tree partitioning is on, the scaling factor is derived using the reconstructed samples on to the top-left, top or to the left of the luma block containing the luma sample collocated with the top-left sample of the chroma block. Specifically, one of the top-left, top or left samples is used depending on their availability. If none is available, chroma residual scaling is not applied.

- When dual tree partitioning is off, the scaling factor for intra coded chroma block is derived using the top-left reference sample used to generate the intra prediction of the luma block collocated with the chroma block.

- For inter coded chroma blocks, the scaling factor is derived from the luma prediction sample collocated with the top-left sample of the chroma block, i.e., the top-left prediction sample is used.

In addition, it is proposed to bypass the chroma scaling factor derivation process if tu_cb_cbf and tu_cr_cbf are both equal to 0. Further, based on recommendation of hardware implementation experts, it is proposed to disable chroma residual scaling for chroma blocks containing less than or equal to 8 samples.

# Simulation results

SDR content

BD-rate variations relative to VTM4.0 using Option2 are reported in the table below. Results are in the attached xls file “N0220_N0389_N0477_vs_VTM4.0_SDR”.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | 0.36% | 0.14% | 101% | 98% |
| Class A2 | -0.16% | 1.35% | 1.32% | 98% | 97% |
| Class B | 0.06% | -0.51% | -0.67% | 99% | 90% |
| Class C | 0.22% | -1.67% | -1.62% | 98% | 91% |
| Class E | 0.24% | -2.31% | -2.42% | 98% | 90% |
| Overall | 0.06% | -0.61% | -0.71% | 99% | 93% |
| Class D | 0.15% | -1.11% | -1.23% | 102% | 91% |
| Class F | 0.30% | -1.41% | -1.32% | 97% | 97% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.38% | 0.23% | 95% | 98% |
| Class A2 | 0.04% | -0.41% | -0.46% | 90% | 94% |
| Class B | 0.03% | -0.72% | -0.59% | 97% | 93% |
| Class C | 0.04% | -1.35% | -1.20% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.61% | -0.56% | 96% | 94% |
| Class D | 0.09% | -1.46% | -1.41% | 101% | 91% |
| Class F | 0.15% | -1.20% | -0.98% | 101% | 95% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 4.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.09% | -0.22% | 102% |  |
| Class E | 0.06% | -0.45% | -0.66% | 92% |  |
| Overall |  |  |  |  |  |
| Class D | -0.02% | -0.14% | -1.36% | 102% |  |
| Class F |  |  |  |  |  |

HDR content

BD-rate variations relative to VTM4.0 using Option2 are reported in the table below. Results are in the attached xls file “N0220_N0389_N0477_vs_VTM4.0_HDR”.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4 HDR reshaping |  |  |  |  |
|  | BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | -6.85% | 0.55% | 0.54% | -14.49% | -14.79% |
| SunRise | -7.32% | 0.76% | 0.82% | -8.49% | -16.15% |
| ShowGirl2 | 0.33% | -0.20% | -0.28% | -1.08% | 3.72% |
| BalloonFestival | -2.08% | 0.70% | 0.73% | -2.15% | -2.27% |
| Hurdles | -16.17% | 1.83% | 1.87% | -21.77% | -25.21% |
| Starting | -10.59% | 1.37% | 1.37% | -8.89% | -11.49% |
| Cosmos1 | 3.63% | -1.26% | -1.29% | 15.38% | 23.06% |
| Average HDR-B | -5.58% | 0.54% | 0.54% | -5.93% | -6.16% |
| Enc Time[%] | 96% |  |  |  |  |
| Dec Time[%] | 96% |  |  |  |  |

|  | Random Acess |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4 HDR reshaping |  |  |  |  |
|  | BD-rate |  |  |  |  |
|  | DE100 | PSNRL100 | wPsnrY | wPsnrU | wPsnrV |
| Market | -7.10% | 0.47% | 0.50% | -15.61% | -15.63% |
| SunRise | -4.39% | 0.61% | 0.61% | -7.00% | -13.08% |
| ShowGirl2 | -2.01% | -0.10% | -0.05% | -5.24% | -3.64% |
| BalloonFestival | -1.80% | 0.53% | 0.36% | -2.33% | -2.06% |
| Hurdles | -10.04% | 0.66% | 0.64% | -14.64% | -14.34% |
| Starting | -8.80% | 1.09% | 1.07% | -8.56% | -9.25% |
| Cosmos1 | 1.85% | -0.41% | -0.33% | 5.68% | 11.05% |
| Average HDR-B | -4.61% | 0.41% | 0.40% | -6.81% | -6.71% |
| Enc Time[%] | 98% |  |  |  |  |
| Dec Time[%] | 97% |  |  |  |  |

In addition, the table below present results for HDR using as anchor the VTM4.0 with reshaping off and adaptive luma-based dQP on, in RA configuration. Performance of the VTM4.0 with reshaping on, and of the current, are presented. It is observed that using this proposal, the chroma is improved (5.2% improvement in DE100, 8% in wPSNR-U/V), while the luma gain is around 1.4% - 1.5%.

|  | DE100 | L100 | wY | wU | wV |
| --- | --- | --- | --- | --- | --- |
| VTM4.0 reshap ON LQP OFF vs VTM4.0 reshap OFF LQP ON | 7.70% | -1.93% | -1.83% | 10.98% | 12.76% |
| proposal vs VTM4.0 reshap off LQP on | 2.54% | -1.54% | -1.44% | 3.06% | 4.40% |

# Conclusion

The proposed combination results in a simplified and more unified design of LMCS. The design addresses a number of hardware implementation concerns. It also provides a better luma/chroma quality balance, especially in HDR PQ case.

# Patent rights declaration

Technicolor may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).