| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0829 |
| --- | --- |

| Title: | Crosscheck of JVET-N0742 (CE8-related: Joint test of JVET-N0094 and CE8-4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma | Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution reports a crosscheck of JVET-N0742 (CE8-related: Joint test of JVET-N0094 and CE8-4.1). The partial result of cross-checker match with the experimental result provided by proponent. The finished result will be uploaded when it is done.

# Experimental results

The crosscheck was performed under common test conditions [1].

Table 1. Experimental results for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.03% | 0.03% | 102% | 100% |
| Class A2 | 0.00% | -0.04% | -0.01% | 102% | 99% |
| Class B | -0.04% | -0.02% | -0.05% | 102% | 100% |
| Class C | -0.33% | -0.19% | -0.29% | 102% | 99% |
| Class E | -0.07% | -0.06% | -0.06% | 101% | 100% |
| Overall | -0.11% | -0.06% | -0.09% | 102% | 100% |
| Class D | -0.29% | -0.33% | -0.20% | 101% | 98% |
| Class F | -1.84% | -1.25% | -1.42% | 101% | 98% |
| TGM | -2.46% | -1.56% | -1.57% | 101% | 99% |

Table 2. Experimental results for RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.25% | 0.13% | 100% | 95% |
| Class A2 | -0.02% | -0.03% | 0.00% | 100% | 99% |
| Class B | -0.01% | -0.17% | -0.29% | 100% | 96% |
| Class C | -0.16% | 0.33% | -0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -0.02% | -0.08% | 100% | 98% |
| Class D | -0.12% | -0.08% | 0.15% | 101% | 97% |
| Class F | -1.45% | -0.98% | -1.14% | 100% | 96% |
| TGM | -1.65% | -0.75% | -0.79% | 100% | 100% |

Table 3. Experimental results for LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0 (enable CPR) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | 101% | 100% |
| Class C | -0.09% | 0.09% | 0.13% | 101% | 99% |
| Class E | -0.11% | -0.18% | 0.45% | 101% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | 101% | 100% |
| Class D | -0.06% | 2.58% | 1.42% | 100% | 99% |
| Class F | -1.00% | -0.72% | 0.05% | 100% | 100% |
| TGM | -1.16% | -0.76% | -0.71% | 100% | 101% |

# Reference

- F. Bossen, et al., “JVET common test conditions and software reference configurations for SDR video”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-M1010, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.