| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0830 |
| --- | --- |

| Title: | AHG14 related: low-level design directions for progressive intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81-80-5066-9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes design directions in video coding layer for the progressive intra refresh.

Introduction

A progressive intra refresh (PIR) is one of the key idea to realize an ultra-low latency transmission under the constant and limited bandwidth [1]. For instance, intra coded blocks are distributed for every picture. In the past video codecs likes MPEG-2, AVC, and HEVC, such mechanism is realized in the closed system by an encoder-decoder set. From the interoperability viewpoint, hypothetical-reference decoder models and conformance points of the legacy video codecs are insufficient.

One of the implementation approach of PIR is to use the tile mechanism since the tile boundary can avoid to refer to the dirty region. Four boundaries of a region are affected by tile functionality while only one boundary between refreshed and dirty regions are really required.

The tile mechanism also provides encoding / decoding parallelism functionality, which is required to implement real codecs. At the same time, tile scheme can provide bistream extraction functionality at the boundary between refreshed and dirty regions but such the functionality is not required for the PIR use-case.

Proposed directions

From the above consideration in the introduction, this contribution proposes the following design directions to consider / develop the progressive intra refresh mechanism. This contribution focuses on the low-level, which is video coding layer.

- Restriction flags in video coding layer should be common for another application. It means any flag is specified not only PIR but also another object.

- Bitstream extraction on the intra coded block boundary is not desirable. It means that tile functionality may be decomposed into small parts.

- Intra coded region is aligned with CTU boundary. It means that CTU granularity is guaranteed by the specification but CU granularity is done by encoder design.

References

- J.-M. Thiesse, A. Duenas, K. Kazui, R. Sjöberg, A. Tourapis, " JVET AHG report: Progressive intra refresh (AHG14)," JVET-N0014, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.