| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0846 |
| --- | --- |

| Title: | Crosscheck of JVET-N0330 (Non-CE3: Intra sub-partition coding without thin partitions) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Eiichi Sasaki | Tel: Email: | +81-50-5213-6277 eiichi.sasaki@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-N0330 (Non-CE3: Intra sub-partition coding without thin partitions). The crosscheck was carried out under the JVET common test condition (CTC). Experimental results match with those provided by the proponent.

Introduction

In JVET-N0330 [1], 1xN, 2xN and/or Nx1 subblock partitions are disabled for intra sub-partition (ISP). Following 3 options are provided.

- Removing ISP blocks that introduce 1xN subblocks.

- Removing ISP blocks that introduce 1xN or 2xN subblocks.

- Removing IPS blocks that introduce 1xN or Nx1 subblocks

Simulation results

The software on top of VTM-4.0 was provided by the proponents [1]. Crosscheck experiments were performed under the JVET common test conditions. Following table show the results of the cross-check experiments under VTM-4.0 configuration.

Table1: Simulation results of Option 1

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.03% | 100% | 100% |
| Class A2 | -0.01% | 0.12% | -0.03% | 99% | 100% |
| Class B | 0.01% | -0.02% | 0.01% | 100% | 100% |
| Class C | 0.05% | -0.01% | -0.03% | 99% | 100% |
| Class E | 0.10% | 0.16% | 0.03% | 98% | 98% |
| Overall | 0.03% | 0.03% | 0.00% | 99% | 99% |
| Class D | 0.05% | 0.07% | -0.01% | 99% | 100% |
| Class F | 0.08% | 0.11% | 0.01% | 99% | 99% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.05% | 101% | 101% |
| Class A2 | -0.03% | 0.04% | 0.02% | 101% | 101% |
| Class B | 0.00% | -0.07% | -0.08% | 100% | 102% |
| Class C | -0.01% | -0.06% | -0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.03% | -0.06% | 100% | 101% |
| Class D | 0.03% | 0.03% | -0.20% | 100% | 96% |
| Class F | 0.05% | -0.01% | -0.04% | 100% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.10% | 0.02% | 101% | 100% |
| Class C | -0.01% | 0.28% | 0.07% | 101% | 109% |
| Class E | 0.08% | 0.38% | 1.09% | 101% | 102% |
| Overall | 0.02% | 0.15% | 0.30% | 101% | 103% |
| Class D | -0.04% | 0.02% | 0.17% | 101% | 105% |
| Class F | -0.13% | -0.15% | -0.21% | 101% | 104% |

Table2: Simulation results of Option 2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.02% | -0.02% | 98% | 101% |
| Class A2 | 0.01% | 0.15% | -0.02% | 98% | 100% |
| Class B | 0.06% | 0.03% | 0.09% | 97% | 99% |
| Class C | 0.24% | 0.07% | 0.15% | 97% | 100% |
| Class E | 0.47% | 0.23% | 0.18% | 96% | 99% |
| Overall | 0.15% | 0.08% | 0.08% | 97% | 100% |
| Class D | 0.27% | 0.27% | 0.14% | 97% | 96% |
| Class F | 0.35% | 0.28% | 0.27% | 98% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.04% | -0.07% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | -0.02% | 100% | 101% |
| Class B | 0.03% | 0.03% | -0.11% | 100% | 99% |
| Class C | 0.13% | 0.16% | -0.11% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.06% | -0.09% | 100% | 100% |
| Class D | 0.09% | -0.04% | 0.18% | 100% | 97% |
| Class F) | 0.18% | -0.02% | 0.18% | 100% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.09% | -0.36% | 100% | 100% |
| Class C | 0.01% | 0.21% | 0.05% | 100% | 102% |
| Class E | 0.09% | 0.85% | -0.14% | 100% | 97% |
| Overall | 0.03% | 0.24% | -0.17% | 100% | 100% |
| Class D | 0.01% | 0.12% | -0.03% | 100% | 105% |
| Class F | -0.04% | 0.16% | -0.12% | 101% | 100% |

Table3: Simulation results of Option 3

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | 0.13% | -0.05% | 99% | 101% |
| Class B | 0.05% | 0.10% | 0.10% | 99% | 101% |
| Class C | 0.10% | 0.02% | 0.05% | 99% | 100% |
| Class E | 0.12% | 0.08% | -0.02% | 98% | 100% |
| Overall | 0.06% | 0.07% | 0.03% | 99% | 100% |
| Class D | 0.08% | 0.20% | -0.10% | 100% | 100% |
| Class F | 0.15% | 0.10% | 0.07% | 99% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.09% | -0.01% | 100% | 101% |
| Class A2 | -0.01% | -0.06% | 0.00% | 100% | 100% |
| Class B | 0.03% | 0.02% | -0.13% | 100% | 102% |
| Class C | 0.06% | 0.07% | -0.12% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | -0.08% | 100% | 101% |
| Class D | 0.04% | -0.13% | -0.12% | 99% | 98% |
| Class F | 0.11% | 0.01% | 0.04% | 100% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-4.0_SHARP |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.08% | -0.41% | 100% | 99% |
| Class C | -0.02% | 0.51% | -0.01% | 101% | 104% |
| Class E | 0.15% | 0.51% | 0.34% | 101% | 101% |
| Overall | 0.03% | 0.27% | -0.09% | 101% | 101% |
| Class D | -0.04% | 0.10% | 0.20% | 100% | 106% |
| Class F | -0.07% | 0.10% | -0.16% | 101% | 103% |

Conclusion

It was confirmed that the the simulation results of bitrate and PSNR match with those provided by the proponent.

References

[1] T. Ma, X. Xiu, Y. Chen, X. Wang, “Non-CE3: Intra sub-partition coding without thin partitions” ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting: Geneva, CH, 19–27 March 2019