| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 14th Meeting: Geneva, CH, 19–27 March 2019 | Document: JVET-N0847-v1 |
| --- | --- |

| Title: | CE7-related: Support of quantization matrices for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Tadamasa Toma, Kiyofumi Abe Sung-Chang Lim, Jungwon Kang | Tel: Email: | +886-3-5670766 {olena.chubach, jenny.lai, chingyeh.chen, peter.chuang, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com {toma.tadamasa, abe.kiyo}@ jp.panasonic.com {sclim, jungwon}@etri.re.kr |
| Source: | MediaTek Inc., Panasonic, Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

# Abstract

This contribution is related to JVET-N0090 and JVET-N0204, containing proposed changes to the default scaling matrices and procedure of defining scaling matrices for rectangular transform blocks (TBs). In this contribution, it is proposed to add support for signalling the default and user-defined scaling matrices on top of VTM4.0. The proposal is conforming to the bigger size range for the blocks (from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma), rectangular TBs, dependent quantization, multiple transform selection (MTS), large transform with zeroing-out high frequency coefficients (aligned to the one-step definition procedure of scaling matrices for TBs), intra subblock partitioning (ISP), and intra block copy (IBC, also known as current picture referencing, CPR). The proposed method of signalling scaling matrices has been implemented on top of VTM4.0.

# Introduction

At the last two JVET meetings, it has been acknowledged by the experts that scaling matrices should be supported in VVC. At the same time, the scaling matrix support in VTM4.0, being based on HEVC, does not conform to the current stage of the VVC development. For example, only DCT-II transforms are supported for only square TBs of sizes from 4×4 to 32×32. This means that scaling matrices do not conform to neither of the following: dependent quantization, MTS, ISP, IBC, a wider range of TB sizes (from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma), and rectangular TBs.

In this contribution, it is proposed to add corresponding syntax elements to support signalling of the default and user-defined scaling matrices for VVC. The proposed signalling supports all the above mentioned adoptions, resulting in conformance with the current state of the VVC development.

# Proposed method

## Scaling matrices in HEVC

The scaling matrices are disabled in VTM4.0 by default and can be enabled using macro HEVC_USE_SCALING_LISTS. Encoding and signalling of scaling matrices in HEVC implies three modes: OFF, DEFAULT, and USER_DEFINED.

In case of the USER_DEFINED mode, scaling matrices are characterized by the following:

- MatrixType: 20 = 2 (2 for intra/inter × 1 Y component) × 4 (TB size: 4×4, 8×8, 16×16, 32×32) + 4 (2 for intra/inter × 2 Cb/Cr components) × 3 (TB size: 4×4, 8×8, 16×16)

- MatrixType_DC: 8 = 2 (2 for intra/inter × 1 Y component) × 2 (TB size: 16×16, 32×32) + 4 (2 for intra/inter × 2 Cb/Cr components) × 1 (TB size: 16×16)

For TB sizes larger than 8×8 (i.e., 16×16, 32×32), the scaling matrix is obtained from the 8×8 scaling matrix by upsampling to a bigger size (duplication of elements). An additional DC value has to be signalled for scaling matrices of TBs bigger than 8×8.

The maximum number of coded values for one scaling matrix in HEVC is equal to 64.

The DC value for DEFAULT mode is equal to 16 for all TB sizes.

## Proposed scaling matrices for VVC

In VTM4.0, the same macro HEVC_USE_SCALING_LISTS is used to enable scaling matrices, and the same three modes for scaling matrices, as in HEVC, are supported: OFF, DEFAULT, and USER_DEFINED. In the case of transform skip (TS) mode or secondary transform (ST) mode, signalling of scaling matrices shall be disabled for all TB sizes.

In the current proposal, the DEFAULT mode scaling matrices are all flat, with elements equal to 16 for all TB sizes. It is proposed that IBC and intra coding modes currently share the same scaling matrices. Thus, for the case of USER_DEFINED matrices in this proposal, the number of MatrixType and MatrixType_DC are updated as follows:

- MatrixType: 30 = 2 (2 for intra&IBC/inter) × 3 (Y/Cb/Cr components) × 5 (square TB size: from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma)

- MatrixType_DC: 14 = 2 (2 for intra&IBC/inter × 1 for Y component) × 3 (TB size: 16×16, 32×32, 64×64) + 4 (2 for intra&IBC/inter × 2 for Cb/Cr components) × 2 (TB size: 16×16, 32×32)

In the proposed design, the DC values are separately coded for following scaling matrices: 16×16, 32×32, and 64×64. For TBs of size smaller than 8×8, all elements in one scaling matrix are signalled. If the TBs have size greater than or equal to 8×8, only 64 elements in one 8×8 scaling matrix are signalled as a base scaling matrix.

For obtaining square matrices of size greater than 8×8, the 8×8 base scaling matrix is up-sampled (by duplication of elements) to the corresponding square size (i.e. 16×16, 32×32, 64×64).

When generating a scaling matrix for rectangular TBs, two cases are considered:
1. Height of the rectangular matrix H is greater than width W, then scaling matrix ScalingMatrix for rectangular TB of size WxH is defined from the reference scaling matrix of size baseL × baseL as follows, where baseL is equal to min ( log2(H), 3 ):

ScalingMatrixi,j=ScalingList[baseL∙intjratioH+inti∙ratioHWratioH] 			(1)

for i=0:W-1, j=0:H-1 and ratioH=HbaseL, ratioHW=HW.

2. Height of the rectangular matrix H is less than width W, then scaling matrix ScalingMatrix for rectangular TB of size WxH is defined from the reference scaling matrix of size baseL × baseL as follows, where baseL is equal to min ( log2(W), 3 ):

ScalingMatrixi,j=ScalingList[basel∙intj∙ratioWHratioW+intW] 			(2)

for i=0:W-1, j=0:H-1, and ratioW=WbaseL, ratioWH=WH.

Here intx is modifying value of x by truncating the fractional part.

In the current proposal, signalling of the default and user-defined scaling matrices for MTS is as well supported. Since at the current stage the default scaling matrices for MTS are not defined, the default transform scaling matrices for DCT-II are used.

In addition, when the zeroing-out of the high frequency coefficients for 64-point transform is applied, corresponding high frequencies of the scaling matrices are also zeroed out. That is, if the width or height of the TB is greater than or equal to 32, only left or top half of the coefficients is kept, and the remaining coefficients are assigned to zero, as shown in Figure 1. For this, a check is performed when obtaining rectangular matrices according to equations (1) and (2) and corresponding elements in ScalingMatrixi,j are assigned to 0.

Moreover, the number of elements signalled for the 64×64 scaling matrix is also reduced from 8×8 to three 4×4 submatrices, since the bottom-right 4×4 elements are never used.

8

64

0

Figure 1.  The right 32×8 part of the 64×8 scaling matrix is zeroed out and used for scaling of the 64×8 TB where only left 32×8 coefficients are kept.

In the current proposal, the scaling matrices are implemented in a way that their size is aligned to the TB size. Therefore, in case of ISP coding mode, when TBs of sizes 2×8, 8×2, 1×16, and 16×1 are generated, the corresponding scaling matrices are used as well. These scaling matrices are obtained in the same way as those generated for the rectangular TBs (i.e., from the corresponding square scaling matrix).

In order to make the proposed signalling of scaling matrices (for both, USER_DEFINED and DEFAULT modes) conforming to the latest stage of the VVC development, scaling matrices are modified to be compatible with other coefficient coding, i.e. dependent quantization [1].

# Conclusion

This document proposes to add syntax for signalling the DEFAULT and USER_DEFINED scaling matrices which support the new coding tools adopted in VVC. The DEFAULT scaling matrices are assigned to flat, with all elements equal to 16. The proposal is conforming to the bigger size range for the blocks (from 4×4 to 64×64 for luma, from 2×2 to 32×32 for chroma), a one-step definition procedure for defining scaling matrices for rectangular TBs, dependent quantization, MTS, large transform with zeroing-out high frequency coefficients (aligned to the one-step definition procedure of scaling matrices for TBs), ISP, and IBC.

# References

[1] H. Schwarz, T. Nguyen, D. Marpe, and T. Wiegand, “CE7: Transform coefficient coding and dependent quantization,” Document of Joint Video Experts Team, JVET-K0071.

[2] F. Bossen, J. Boyce, K. Sühring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Electronics and Telecommunications Research Institute (ETRI) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).