| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-P0007 |
| --- | --- |

| Title: | JVET AHG report: Coding of HDR/WCG Material (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | A. Segall E. Francois W. Husak S. Iwamura D. Rusanovskyy | Email: | asegall@sharplbas.com Edouard.Francois@InterDigital.com WJH@dolby.com iwamura.s-gc@nhk.or.jp> dmytror@qti.qualcomm.com |
| Source: | AHG7 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG7: Coding of HDR/WCG Material between the 15th meeting in Gothenburg, SE (3 – 12 July 2019) and the 16th meeting in Geneva, CH (1 – 11 October 2019).

# Mandates

The AHG was established with the following mandates:

- Study and evaluate available HDR/WCG test content.

- Study objective metrics for quality assessment of HDR/WCG material, including investigation of the correlation between subjective and objective results.

- Compare the performance of the VTM and HM for HDR/WCG content.

- Prepare for expert viewing of HDR content at the next JVET meeting if feasible.

- Investigate the implications of chroma sampling location.

- Coordinate implementation of HDR anchor aspects in the test model software with AHG3.

- Study additional aspects of coding HDR/WCG content.

# Activities

The AHG used the main JVET reflector, jvet@lists.rwth-aachen.de, with an [AHG7] indication on message headers.  The primary activity of the AhG was related to the mandates of (i) comparing the performance of the VTM for HDR/WCG content; (ii) preparing for expert viewing of HDR content at the 16th JVET meeting; (iii) investigating the implications of chroma sampling locations, and (iv) coordinating implementation of HDR anchor aspects in the test model software. This work is described in the following subsection.

## Performance Comparison

The AhG performed experiments comparing the performance of the VTM 6.0 on HDR content.  The first part of this work included determining the performance of VTM 6.0 relative to the VTM 5.0 and HM 16.18.  A summary of the performance is provided below, and more detailed information may be found in the included XLS data.

It was observed that the bit-allocation between luma and chroma in the HDR configuration changed between the two releases of the VTM – especially for Class H1.  This is likely due to the change in the chroma table mapping function adopted into VTM 6, as well as the merge of request 857 (discussed later).

### VTM 6.0 versus VTM 5.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -9.90% | 1.15% | 1.19% | -13.94% | -28.44% | 1.14% | -13.57% | -26.92% | 86% | 125% |
| Class H2 |  |  |  |  |  | -0.40% | 1.30% | 0.05% | 80% | 92% |
| Overall | -9.90% | 1.15% | 1.19% | -13.94% | -28.44% | 0.58% | -8.17% | -17.11% | 84% | 112% |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -8.65% | 1.17% | 1.11% | -18.02% | -32.70% | 1.03% | -18.39% | -32.42% | 106% | 100% |
| Class H2 |  |  |  |  |  | -1.69% | -2.59% | -2.17% | 98% | 78% |
| Overall | -8.65% | 1.17% | 1.11% | -18.02% | -32.70% | 0.04% | -12.64% | -21.42% | 103% | 91% |

### VTM 6.0 versus HM 16.18

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -46.38% | -23.60% | -23.40% | -58.05% | -49.74% | -21.83% | -53.54% | -43.10% | - | - |
| Class H2 |  |  |  |  |  | -21.15% | -37.56% | -39.83% | - | - |
| Overall | -46.38% | -23.60% | -23.40% | -58.05% | -49.74% | -21.58% | -47.73% | -41.91% | - | - |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over HM-16.18 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -43.94% | -28.23% | -28.04% | -59.64% | -68.05% | -26.01% | -55.82% | -61.81% | - | - |
| Class H2 |  |  |  |  |  | -28.23% | -45.86% | -49.57% | - | - |
| Overall | -43.94% | -28.23% | -28.04% | -59.64% | -68.05% | -26.82% | -52.20% | -57.36% | - | - |

## Tool-On / Tool-Off Comparisons

In addition to evaluating the performance of VTM 6.0, the AhG also studied the performance of individual coding tools in the context of HDR content.  This was accomplished by conducting a Tool-On/Tool-Off test according to the methodology established in AhG13.  The only exception was for the LMCS tool.  Currently, the HDR CTC defines the tool to be off when coding Class H1 content.  So, to account for this configuration, the AhG performed a Tool-On test (instead of a Tool-Off test) for the tool instead.

Results are summarized in the tables below.  Additionally, more detailed results are provided in the included XLS data.

The AhG would like to thank the following companies for contributing to the Tool-On tests: Alibaba, Dolby, InterDigital, LG, MediaTek, NHK, and Sharp.

### Class H1 (PQ)

Simulation Results for AI (Class H1)

|  |  |  |  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acronym | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| CST | 19.02% | 0.77% | 0.71% | 22.69% | 50.33% | 155% | 100% | 151% | 101% |
| DQ | -1.52% | 1.58% | 1.68% | -4.79% | -8.80% | 100% | 101% | 98% | 105% |
| CCLM | 26.79% | 2.94% | 2.74% | 96.24% | 199.89% | 101% | 100% | 101% | 100% |
| MTS | 0.86% | 1.11% | 1.23% | 0.68% | 0.59% | 86% | 100% | 86% | 101% |
| ALF | 3.73% | 2.75% | 2.24% | 6.88% | 7.26% | 99% | 86% | 94% | 90% |
| MRLP | 0.27% | 0.39% | 0.36% | 0.40% | 0.11% | 99% | 101% | 98% | 101% |
| IBC on | -0.28% | -0.34% | -0.34% | 0.09% | -0.01% | 138% | 103% | 148% | 101% |
| ISP | 0.29% | 0.55% | 0.61% | 0.05% | -0.29% | 90% | 97% | 87% | 100% |
| LMCS on* | 2.36% | -1.46% | -1.01% | 6.36% | 14.44% | 104% | 101% | 100% | 101% |
| RDPCM on | 0.09% | 0.00% | -0.04% | 0.26% | 0.10% | 105% | 100% | 106% | 101% |
| MIP | 0.51% | 0.59% | 0.50% | 0.63% | -0.10% | 86% | 99% | 91% | 101% |
| LFNST | -0.58% | 0.86% | 0.75% | -1.04% | -0.83% | 103% | 100% | 101% | 102% |
| JCCR | -0.05% | 0.66% | 0.66% | 4.05% | -3.31% | 95% | 99% | 97% | 102% |
| SAO | 0.64% | 0.07% | 0.18% | 0.67% | 2.29% | 101% | 98% | 100% | 99% |

* Note: Unlike in AhG13, LMCS is evaluated with a Tool-On test here

Simulation Results for RA (Class H1)

| Acronym | DE100 | PSNR-L | BDR-wY | BDR-wU | BDR-wV | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CST | 13.05% | 0.37% | 0.32% | 15.55% | 25.44% | 103% | 102% | 104% | 100% |
| DQ | -0.73% | 1.66% | 1.57% | -3.50% | -6.64% | 101% | 100% | 100% | 101% |
| CCLM | 18.66% | 1.41% | 1.31% | 91.24% | 190.73% | 99% | 101% | 101% | 100% |
| MTS | 1.00% | 0.88% | 0.96% | 1.45% | 1.38% | 94% | 98% | 96% | 100% |
| ALF | 3.96% | 2.23% | 2.00% | 6.11% | 11.62% | 94% | 85% | 91% | 89% |
| AFF | 0.80% | 0.97% | 0.98% | 0.70% | 0.30% | 80% | 97% | 80% | 99% |
| SbTMVP | 0.74% | 0.45% | 0.45% | 0.50% | 0.52% | 100% | 103% | 101% | 102% |
| AMVR | 1.11% | 0.70% | 0.73% | 2.39% | 4.72% | 80% | 89% | 83% | 101% |
| TPM | 0.31% | 0.17% | 0.20% | 1.10% | 1.16% | 98% | 100% | 99% | 100% |
| BDOF | 0.75% | 0.92% | 0.97% | 0.24% | -0.03% | 97% | 97% | 97% | 98% |
| PROF | 0.07% | 0.10% | 0.11% | 0.13% | -0.11% | 99% | 100% | 100% | 99% |
| CIIP | -0.15% | 0.10% | 0.13% | -0.25% | -0.49% | 97% | 99% | 97% | 100% |
| MMVD | 0.49% | 0.29% | 0.27% | 0.71% | 0.96% | 89% | 102% | 89% | 100% |
| BCW | 0.61% | 0.24% | 0.22% | 0.69% | 1.00% | 92% | 102% | 91% | 102% |
| MRLP | 0.21% | 0.20% | 0.18% | 0.02% | 0.35% | 99% | 101% | 99% | 100% |
| IBC on | 0.10% | -0.05% | 0.06% | 0.72% | 0.78% | 105% | 103% | 105% | 100% |
| ISP | 0.26% | 0.36% | 0.35% | 0.14% | 0.82% | 98% | 100% | 98% | 100% |
| DMVR | 1.13% | 0.99% | 0.84% | 1.37% | 1.66% | 100% | 96% | 101% | 97% |
| SBT | 0.08% | 0.13% | 0.33% | 0.28% | 0.47% | 97% | 101% | 98% | 100% |
| LMCS on* | 0.20% | -1.12% | -0.92% | 2.62% | 11.66% | 103% | 102% | 100% | 102% |
| SMVD | 0.21% | 0.14% | 0.16% | 0.30% | 0.58% | 95% | 101% | 91% | 100% |
| RDPCM on | 0.16% | -0.01% | -0.01% | 0.13% | 0.79% | 101% | 102% | 100% | 100% |
| MIP | 0.42% | 0.42% | 0.33% | 0.68% | 0.60% | 93% | 97% | 98% | 100% |
| LFNST | 0.17% | 0.49% | 0.41% | 0.66% | 1.47% | 94% | 100% | 97% | 100% |
| JCCR | -0.61% | 0.49% | 0.52% | 2.88% | -3.91% | 97% | 99% | 100% | 101% |
| SAO | 0.72% | -0.03% | 0.01% | 0.79% | 2.21% | 99% | 101% | 100% | 99% |
| SIF | 0.25% | 0.13% | 0.10% | 0.43% | 1.04% | 97% | 100% | 97% | 100% |

Figure 1. wPSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 2. wPSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H1)

Figure 3. wPSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H1)

### Class H2 (HLG)

Simulation Results for AI (Class H2)

| Acronym | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CST | 0.60% | 13.59% | 21.01% | 171% | 104% | 162% | 101% |
| DQ | 1.86% | -1.29% | -1.52% | 99% | 103% | 99% | 105% |
| CCLM | 1.72% | 40.73% | 20.54% | 102% | 100% | 101% | 99% |
| MTS | 1.85% | 2.59% | 1.90% | 86% | 97% | 84% | 98% |
| ALF | 2.83% | 2.16% | 3.20% | 97% | 84% | 94% | 86% |
| MRLP | 0.06% | -0.12% | 0.08% | 99% | 101% | 97% | 101% |
| IBC on | -0.11% | -0.06% | 0.03% | 163% | 101% | 148% | 100% |
| ISP | 0.32% | -0.59% | -0.15% | 87% | 99% | 86% | 100% |
| LMCS | 0.05% | -0.83% | -0.59% | 93% | 96% | 97% | 93% |
| RDPCM on | 0.00% | -0.05% | 0.03% | 108% | 101% | 108% | 100% |
| MIP | 0.62% | 0.85% | 0.55% | 92% | 101% | 92% | 101% |
| LFNST | 0.49% | -0.61% | -0.94% | 106% | 101% | 103% | 100% |
| JCCR | 0.27% | 0.38% | 5.19% | 99% | 100% | 97% | 100% |
| SAO | 0.05% | 0.25% | 0.59% | 100% | 96% | 98% | 97% |

Simulation Results for RA (Class H2)

| Abbreviation | BDR-Y | BDR-U | BDR-V | Tester EncTime | Tester DecTime | XChecker EncTime | XChecker DecTime |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CST | 0.26% | 8.07% | 15.98% | 103% | 100% | 102% | 100% |
| DQ | 1.81% | -0.41% | -0.86% | 103% | 102% | 103% | 102% |
| CCLM | 0.76% | 47.97% | 25.56% | 101% | 100% | 100% | 101% |
| MTS | 1.19% | 1.43% | 0.81% | 95% | 100% | 95% | 100% |
| ALF | 3.53% | 3.41% | 3.04% | 97% | 84% | 94% | 89% |
| AFF | 0.70% | 0.45% | 0.56% | 80% | 98% | 81% | 98% |
| SbTMVP | 0.40% | 0.28% | 0.27% | 102% | 103% | 100% | 102% |
| AMVR | 0.72% | 1.12% | 1.59% | 82% | 100% | 81% | 101% |
| TPM | 0.34% | 0.63% | 0.57% | 99% | 100% | 99% | 100% |
| BDOF | 0.60% | 0.21% | 0.17% | 96% | 98% | 96% | 98% |
| PROF | 0.14% | 0.04% | 0.04% | 100% | 99% | 99% | 100% |
| CIIP | 0.18% | -0.32% | -0.45% | 98% | 100% | 97% | 100% |
| MMVD | 0.19% | 0.29% | 0.56% | 89% | 100% | 89% | 100% |
| BCW | 0.21% | 0.18% | 0.23% | 92% | 102% | 92% | 102% |
| MRLP | 0.06% | 0.05% | 0.06% | 100% | 100% | 97% | 100% |
| IBC on | 0.11% | 0.13% | -0.13% | 106% | 100% | 102% | 100% |
| ISP | 0.21% | 0.29% | 0.17% | 99% | 100% | 98% | 100% |
| DMVR | 0.83% | 1.06% | 0.91% | 101% | 96% | 101% | 98% |
| SBT | 0.30% | -0.20% | -0.34% | 97% | 100% | 97% | 100% |
| LMCS | 0.95% | 1.35% | 1.10% | 97% | 98% | 98% | 98% |
| SMVD | 0.20% | 0.05% | 0.15% | 92% | 100% | 90% | 100% |
| RDPCM on | 0.02% | 0.12% | 0.05% | 102% | 100% | 101% | 100% |
| MIP | 0.46% | 0.95% | 0.08% | 98% | 100% | 97% | 99% |
| LFNST | 0.41% | -0.26% | -0.78% | 95% | 99% | 96% | 101% |
| JCCR | 0.21% | 0.12% | 7.06% | 99% | 99% | 100% | 101% |
| SAO | 0.06% | 0.32% | 1.81% | 100% | 97% | 101% | 100% |
| SIF | 0.12% | 0.14% | 0.23% | 96% | 99% | 97% | 100% |

Figure 4. PSNR-Y vs encoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 5. PSNR-Y vs decoding runtime ratio of VTM with VTM tool tests (Class H2)

Figure 6. PSNR-Y vs weighted runtime ratio (a = 6) of VTM with VTM tool tests (Class H2)

## Preparations for Expert Viewing at the 16th JVET meeting

The AhG coordinated the availability of an HDR display for the 16th meeting in order to better support the visual evaluation of HDR content.  This includes the planned viewing of HDR content in CE5.  A SONY BVM X 300 will be available for this activity.  Currently, it is planned to be located at the EBU, to ensure that the necessary playback equipment is available to drive the display.

It is noted that the BVM X 300 is able to show HLG content natively.  However, tone mapping will be required to display PQ content.

The AhG also wishes to thank Frans De Jong and Paola Sunna from EBU for providing support for the activity.

## Investigate the implications of chroma sampling process

During the 15th meeting, it was decided to change the HDR common test conditions for Class H1 (PQ) to reflect that the chroma type of the content was Type 2.  This resulted in changing the configuration file to include the following parameter “CclmCollocatedChroma: 1”

During the AhG period, the AhG investigated if a similar change was needed for the Class H2 (HLG) content.  Simulations were performed measuring the improvement in coding efficiency by setting the chroma type to Type 2 for the CCLM coding tool.  The results are provided below, and the AhG concluded that a change was not needed for Class H2.

VTM-5.0 with CclmCollocatedChroma:1 versus VTM-5.0

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | #VALUE! | -0.23% | -0.19% | -5.14% | -10.59% | -0.19% | -4.32% | -8.33% | - | - |
| Class H2 |  |  |  |  |  | 0.14% | 3.55% | 4.28% | - | - |
| Overall | #VALUE! | -0.23% | -0.19% | -5.14% | -10.59% | -0.07% | -1.46% | -3.75% | - | - |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | #VALUE! | -0.17% | -0.11% | -4.82% | -7.39% | -0.13% | -3.80% | -5.60% | - | - |
| Class H2 |  |  |  |  |  | 0.09% | 2.73% | 6.97% | - | - |
| Overall | #VALUE! | -0.17% | -0.11% | -4.82% | -7.39% | -0.05% | -1.43% | -1.03% | - | - |

## Coordinate implementation of HDR anchor aspects in the test model software with AHG3

### Integration of HDR metric calculation into the VTM

Prior to this AhG period, the calculation of HDR specific metrics required use of an external HDRMetrics application.  This also required the management of additional configuration files to control the conversion of decoded YUV sequences to float point representations in linear light.  Software to address this issue was provided at the 15th meeting in JVET-O0756.

During the AhG period, the AhG worked to coordinate and support the integration of the HDRMetrics functionality into the VTM software.  The functionality was provided as part of the VTM6.0.  The timeline for the release was as follows:

  - 2019/08/09

    - VTM 6.0 released including support for calling external HDRLib for metric calculation

    - HDRTools 0.19-dev branch available to support linking HDRLib into VTM 6.0

  - 2019/09/24

    - HDRTools 0.19 released with official support for linking HDRLib into VTM.  However, it was later determined that the release included a change in the API that effected the linking process.

  - 2019/09/27

    - HDRTools 0.19.1 released with bugfix to support linking HDRLib into VTM

# Enabling the HDRLib functionality in the VTM software is accomplished by doing the following:

- Create a local copy of the VTM Version 6 or newer software.

- Create a local copy of the HDRTools v0.19.1 or newer utility available at https://gitlab.com/standards/HDRTools.  More recent versions are encouraged where applicable.

- Copy the HDRTools/common directory from the HDRTools utility into the source/Lib/HDRLib directory of the VTM software.

Example: On a Linux system, this could be accomplished with the command:

ln -s <HDRTools_dir>/HDRTools/common <VTM_dir>/souce/Lib/HDRLib

- Append “-DEXTENSION_HDRTOOLS=on” when building the VTM software using CMake.

Example: If the VTM is normally built using the command:

cmake -DCMAKE_BUILD_TYPE=Release

The HDRLib functionality is enabled by using the command:

cmake -DCMAKE_BUILD_TYPE=Release -DEXTENSION_HDRTOOLS=on

The class specific parameters and sequence specific parameters provided with the VTM will then enable and configure the metric calculation parameters appropriately. Note that these instructions are also available in the HDR CTC.

It was observed that most participants interested in performing HDR experiments used the integrated HDRTool functionality to calculate the HDR metric.  However, as the functionality was only officially released a week before the meeting, it is possible that some proponents chose to use the previous, standalone approach for calculating the metric.  The AhG compared the performance of using the integrated metric calculation and the (previously defined) external metric calculation to determine the significance of using the different approaches.

Results are provided below and show that the objective difference between the two approaches is less than .025%.

VTM-6.0 External Metric Calculation vs Integrated Metric Calculation

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM 6.0 (with external metric calculation) |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | 0.010% | 0.020% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |
|  | Random Access |  |  |  |  |  |  |  |  |  |
|  | Over VTM 6.0 (with external metric calculation) |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.006% | 0.024% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |  |  |

### Addressing chroma loss observation in previous AhG period

As part of the AhG report provided to the 15th meeting of JVET, it was observed that there was a rather large chroma loss between VTM 4.0 and VTM 5.0.  It was identified that the integration of JVET-M0091 resulted in the loss (commit: 65c9bacdf38).

During this AhG activity, merge request 857 addressed this chroma loss issue.  The merge request was integrated into the VTM (commit: 95246223) and provided as part of the VTM 6.0 release.

# Contributions

There are two contributions related to HDR video coding.  While not listed here, it is noted that responses to CE5-1 and CE5-2 sub-categories consider the performance of in-loop filtering on HDR content:

| JVET-P0335 | AHG15: Chroma QP mapping table for HDR | T. Lu, F. Pu, P. Yin, S. McCarthy, W. Husak, T. Chen (Dolby) |
| --- | --- | --- |
| JVET-P0371 | AHG7/AHG15: Signalling of corrective values for chroma residual scaling | E. François, F. Galpin, K. Naser, P. de Lagrange (InterDigital) |
| JVET-P0623 | Crosscheck report of JVET-P0335 on Chroma QP mapping table for HDR | E. François (InterDigital) |
| JVET-P0711 | Crosscheck of JVET-P0371 (AHG7/AHG15: signalling of corrective values for chroma residual scaling) | T. Lu (Dolby) |

# Recommendations

The AHG recommends the following:

- Identify HDR visual testing to be performed at the 16th meeting

- Review all input contributions

The HDRTools utility and VTM software are maintained separately and not released according to the same schedule.  Thus, it is possible that a version of the utilities newer than v0.19.1 may not be compatible with a given version of the VTM software (and, equivalently, a newer version of the VTM software may not be compatible with a given version of the HDRTools utility). It is recommended to use the most recent version of the HDRTools utility when possible.  Otherwise, if there are compatibility issues between the most recent version of the HDRTools and the VTM software, it is then recommended to use v0.19.1.