| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0025-v3 |
| --- | --- |

| Title: | CE5: Summary Report on Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrey Norkin Ching-Yeh Chen | Tel: Email: | anorkin@netflix.com chingyeh.chen@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Core Experiment 5 on Loop Filtering. There are three categories of the tests: deblocking filter (CE5-1), adaptive loop filter (CE5-2), and bilateral filter, Hadamard filter, and SAO (CE-3). The subjective testing is planned to be performed at the meeting.

The corresponding compression performance of each coding tool evaluated in CE5 is summarized in this contribution. In addition, analysis of complexity of the proposals and cross-checking results are provided.

The software basis for this CE was VTM-6.0. Configurations and test conditions in JVET-N1010 [1] for SDR sequences are used. Results for additional configuration with ALF turned off and SAO turned off are also reported where applicable. For the subjective viewing of deblocking proposals, additional encodes with different QPs have been generated, including two sequences in 4:4:4 chroma format, and two HDR sequences.

# CE5-1: Deblocking filter

## Description of tests

## CE5-1.1: Deblocking at DMVR boundaries

Proposal in this sub-CE investigates effect of using DMVR refined motion for deblocking decisions and also the effect of turning off DMVR.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.1.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com #### JVET-P0062 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE5-1.1.2 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-P0062 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE5-1.1.3 DMVR off | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-P0062 | Jeeva Raj jeeva.raj@ittiam.com |

## Description of technologies in CE5-1.1

#### CE5-1.1.1 Deblocking based on DMVR refined motion JVET-P0062

It has been shown in JVET-O0158 that block artifacts can occur for CU and subblock boundaries of DMVR. The reason is that VVC uses unrefined motions for deblocking decisions.

The test investigates the effect of using the DMVR refined motion for deblocking decisions to make deblocking aware of actual differences in motion of DMVR predicted subblocks.

#### CE5-1.1.2 Restriction of CE5-1.1.1 JVET-P0062

This test is the same as CE5-1.1.1 with the restriction to not use refined motion across horizontal CTU boundary.

#### CE5-1.1.3 DMVR off JVET-P0062

The test disables DMVR for comparison with tests CE5-1.1.1 and CE5-1.1.2.

## CE5-1.2: Deblocking simplifications

Proposal in this sub-CE investigates changes to simplify the chroma deblocking filter.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.2 | Jizheng Xu xujizheng@bytedance.com JVET-P0061 | Daniel Luo jiancong.luo@alibaba-inc.com |

## Description of technologies in CE5-1.2

#### CE5-1.2 Aligning deblocking chroma decisions JVET-P0061

In the current VVC design, when one side of the chroma blocks is of intra, the boundary strength is set to 2. Otherwise, when one chroma block has non-zero transform coefficients, the boundary is set to 1. Thus, it is possible that the bS=1 for Cb and bS=0 for Cr. The test sets bS=1 for both Cb and Cr when one component has non-zero transform coefficients at least.

Performance of the proposal on HDR material is also to be tested.

## CE5-1.3 Longer tap deblocking filter at chroma CTB boundaries.

Proposal in this sub-CE investigates changes to longer tap filter application at Chroma CTB boundaries

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-1.3 | Anand Meher Kotra anand.meher.kotra@huawei.com JVET-P0081 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

## Description of technologies in CE5-1.2

#### CE5-1.3 Longer tap filter application at Chroma CTB boundaries JVET-P0081

In VTM-5.0, longer tap filter is turned off at the Chroma CTB boundaries, unlike the Luma CTB boundaries where a longer tap asymmetric filter (3+7) is applied.

This test proposes unification of deblocking design by also applying a longer tap asymmetric filter (1+3) at the Chroma CTB boundaries as shown in the figure below.

At the horizontal CTU boundaries the modified longer tap filter decisions are as follows: (p2,0, p2,1  are simply replaced with p1,0  and  p1,1)

dp0 = Abs( p21,0 − 2 * p1,0 + p0,0 )

dp1 = Abs( p21,1 − 2 * p1,1 + p0,1 )

dq0 = Abs( q2,0 − 2 * q1,0 + q0,0 )

dq1 = Abs( q2,1 − 2 * q1,1 + q0,1 )

At the horizontal CTU boundaries the modified longer tap deblocking equations are as follows:

p0′ = Clip3( p0 − tC, p0 + tC, ( p3 + p2 + 3 * p1 + 2 * p0 + q0 + q1 + q2  + 4 )  >>  3 )

q0′ = Clip3( q0 − tC, q0 + tC, ( p2 + 2 * p1 + p0 + 2 * q0 + q1 + q2 + q3  + 4 )  >>  3 )

q1′ = Clip3( q1 − tC, q1 + tC, ( p1 + p0 + q0 + 2 * q1 + q2 + 2 * q3  + 4 )  >>  3 )

q2′= Clip3( q2 − tC, q2 + tC, ( p0 + q0 + q1 + 2 * q2 + 3 * q3 + 4 )  >>  3 )

## CE5-1 Tests results

The number of YUV BD-rate is calculated by using the same formula in AHG13 (YUV BD-rate = (8*Y-BD-rate + U-BD-rate + V-BD-rate) / 10 ).

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-1.1.1 | Deblocking based on DMVR refined motion | JVET-P0062 |
| CE5-1.1.2 | Restriction of CE5-1.1.1 | JVET-P0062 |
| CE5-1.1.3 | DMVR off | JVET-P0062 |
| CE5-1.2 | Aligning deblocking chroma decisions | JVET-P0061 |
| CE5-1.3 | Longer tap filter application at Chroma CTB boundaries | JVET-P0081 |

### VTM6.0 CTC (ALF on)

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.05% | 0.05% | 0.03% | 0.02% | 101% | 102% |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.05% | 0.06% | 0.02% | 0.02% | 100% | 109%* |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.87% | 0.81% | 1.11% | 1.11% | 101% | 97% |
| CE5-1.2 | -0.02% | 0.00% | -0.07% | -0.09% | 99% | 99% | -0.01% | 0.00% | -0.02% | -0.09% | 100% | 97% |
| CE5-1.3 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% | 0.00% | 0.00% | 0.03% | -0.01% | 100% | 101% |

|  | Low Delay B |  |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV |  | Y | U | V | EncT | DecT |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |  | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |  | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |  | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.2 | 0.03% | -0.01% | 0.12% | 0.25% | 101% | 99% | 0.07% |  | 0.06% | 0.01% | 0.17% | 98% | 98% |
| CE5-1.3 | -0.01% | -0.01% | 0.10% | -0.16% | 100% | 100% | -0.02% |  | 0.00% | -0.13% | -0.03% | 100% | 100% |

### VTM6.0 CTC + ALF off

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.02% | 0.03% | 0.00% | 0.00% | 98% | 103% |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.03% | 0.04% | -0.01% | 0.00% | 99% | 109%* |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | 0.82% | 0.75% | 1.09% | 1.12% | 100% | 93% |
| CE5-1.2 | 0.00% | 0.00% | 0.01% | -0.04% | 100% | 99% | 0.00% | 0.01% | 0.02% | -0.06% | 100% | 99% |
| CE5-1.3 | 0.00% | 0.00% | -0.03% | -0.02% | 100% | 100% | 0.00% | 0.01% | 0.01% | -0.04% | 100% | 100% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 |
| CE5-1.2 | -0.02% | -0.02% | -0.06% | 0.05% | 101% | 100% | -0.01% | -0.03% | 0.13% | 0.02% | 99% | 101% |
| CE5-1.3 | -0.06% | -0.01% | -0.15% | -0.33% | 100% | 100% | -0.07% | -0.03% | -0.15% | -0.24% | 100% | 100% |

## Complexity Analysis

### Luma deblocking complexity

| Tests | Samples from block boundary modified | Samples from block boundary for deblocking decision | Max number of operations for filtering per line (add/mult/compar/shift) | Max number of operations for filtering per *sample* (add/mult/compar/shift) | Max. num. oper., filtering and decsion (Per *Sample*) | Max oper. for decis. per line (add/mult/compar/shift) | Number of line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |

### Chroma deblocking complexity

| Tests | samples from block boundary modified | samples from block boundary for deblocking decision | Max number of operations for filtering per line (add/mult/compar/shift) | Max number of operations for filtering per *sample* (add/mult/compar/shift) | Max number of operations including filter and decision per *sample* (add/mult/compar/shift) | Max number of oper. for decision for 8-sample boundary (add/mult/compar/shift) | Number of line buffers | Worst case complexity increased (Y/N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1.1 | VTM6 | VTM6 | VTM6 | VTM6 | #### VTM6 | VTM6 | VTM6 | N |
| CE5-1.1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.1.3 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.2 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |
| CE5-1.3 | VTM6, except at chroma CTB boundaries where (1 + 3) filter may be applied | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | VTM6 | N |

### Decoding complexity on additional sequences (deblocking proposals)

Decoding times on subjective test sequences in ALF-off configuration have been measured. This allows to evaluate the complexity increase on 4:4:4 content among the proposal in CE5-1. Note that the decoding time may be higher than on average since the high QP (i.e. low bitrate) encodes have been used.

Geometric average of decoding time anchor versus test on same machine in randomized order without yuv output and MD5 for CE5-1.1.1 to CE5-1.1.3. For CE5-1.1.2 a bit-exact speedup version provided in the contribution was used to get fair decoding times.

| Sequence | CE5-1.1.1 | CE5-1.1.2 | CE5-1.1.3 | CE5-1.2 | CE5-1.3 |
| --- | --- | --- | --- | --- | --- |
| SDR_01 QP37 | 107 | 109 | 92 | 100 | 100 |
| SDR_01 QP32 | 107 | 109 | 89 | 100 | 100 |
| SDR_02 QP37 | 106 | 104 | 99 | 99 | 99 |
| SDR_02 QP32 | 102 | 104 | 98 | 98 | 100 |
| SDR_03 QP37 | 99 | 104 | 98 | 106 | 100 |
| SDR_03 QP32 | 101 | 102 | 95 | 96 | 100 |
| SDR_04 QP37 | 105 | 107 | 91 |  |  |
| SDR_04 QP32 | 109 | 107 | 92 |  |  |
| SDR_05 QP37 | 101 | 104 | 93 |  |  |
| SDR_05 QP32 | 108 | 104 | 91 |  |  |
| SDR_06 QP37 |  |  |  | 105 | 100 |
| SDR_06 QP32 |  |  |  | 99 | 100 |
| SDR_07 QP37 |  |  |  | 101 | 100 |
| SDR_07 QP32 |  |  |  | 95 | 100 |
| SDR_08 QP37 |  |  |  | 94 | 100 |
| SDR_08 QP32 |  |  |  | 92 | 101 |
| HDR_01 QP37 | 107 | 108 | 97 | 100 |  |
| HDR_01 QP32 | 107 | 107 | 95 | 95 |  |
| HDR_02 QP37 | 106 | 106 | 92 | 100 |  |
| HDR_02 QP32 | 105 | 107 | 90 | 103 |  |
| HDR_03 QP37 | 104 | 111 | 98 |  |  |
| HDR_03 QP32 | 103 | 105 | 95 |  |  |
| 444_01 QP37 | 108 | 105 | 92 |  | 100 |
| 444_01 QP32 | 103 | 106 | 92 |  | 100 |
| 444_02 QP37 |  |  |  |  | 100 |
| 444_02 QP32 |  |  |  |  | 101 |

## CE5-1 Cross-checkers report

| Test # | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Visual quality observations | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-1.1.1 | C.-M. Tsai | Y | Y | Y | N | Y | Blocking artifact removal for CUs coded with DMVR can be observed when comparing with VTM6. |  |
| CE5-1.1.2 | C.-M. Tsai | Y | Y | Y | N | Y | Blocking artifact removal for CUs coded with DMVR can be observed when comparing with VTM6. |  |
| CE5-1.1.3 | J. Raj | Y | Y | Y | N | Y | As per subjective tests carried out at Ittiam with 6 video experts, the scores indicated that the anchor (VTM6.0) was marginally felt better than CE5-1.1.3. The tests involved ALF ON and ALF OFF for both anchor (VTM6.0) and CE-5.1.1.3, for Qp 32 and 37, across sequences SDR_01 to SDR_05. On an average, 81% of the scores favored the anchor (VTM 6.0) for the ALF OFF cases. 71% of the scores favored the anchor (VTM 6.0) for ALF ON cases. |  |
| CE5-1.2 | D. Luo | Pending | Pending | Y | N | N |  |  |
| CE5-1.3 | C.-M. Tsai | Y | Y | Y | N | N |  |  |

# CE5-2: Adaptive Loop Filter

## Description of tests

## CE5-2: Cross-component adaptive loop filter for chroma

Proposal in this sub-CE investigates effect of cross-component adaptive loop filter for chroma, including in-loop filtering and post-loop filtering.

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-2.1 | Kiran Misra misrak@sharplabs.com JVET-P0080 | Nan Hu nanh@qti.qualcomm.com |
| CE5-2.1 | Kiran Misra misrak@sharplabs.com JVET-P0080 | Nan Hu nanh@qti.qualcomm.com |

## Description of technologies in CE5-2

#### CE5-2.1 Cross-component adaptive loop filter for chroma JVET-P0080

The contribution proposes a tool called the Cross-Component Adaptive Loop Filter (CC-ALF). CC-ALF operates as part of the adaptive loop filter process and makes use of luma sample values to refine each chroma component.

#### CE5-2.2 Cross-component adaptive post-processing filter for chroma JVET-P0080

This is the same proposal as in the CR5-2.1 implementing as post-processing filter.

## CE5-2 Tests results

The number of YUV BD-rate is calculated by using the same formula in AHG13 (YUV BD-rate = (8*Y-BD-rate + U-BD-rate + V-BD-rate) / 10 ).

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-2.1 | Cross-component adaptive loop filter for chroma | JVET-P0080 |
| CE5-2.2 | Cross-component adaptive post-processing filter for chroma | JVET-P0080 |

### VTM6.0

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-2.1 | -1.26% | 0.13% | -7.58% | -6.01% | 103% | 103% | -1.95% | 0.16% | -10.69% | -10.07% | 101% | 100% |
| CE5-2.2 | -1.23% | 0.15% | -7.54% | -5.93% | 100% | 107% | -0.84% | 0.78% | -7.50% | -7.10% | 101% | 105% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-2.1 | -2.60% | 0.17% | -15.62% | -11.75% | 101% | 99% | -2.59% | 0.21% | -15.89% | -11.72% | 102% | 101% |
| CE5-2.2 | -0.27% | 1.83% | -10.59% | -6.71% | 100% | 107% | -0.39% | 1.79% | -11.02% | -7.15% | 101% | 107% |

## CE5-2 Complexity Analysis

### CE5-2 Decoding complexity on additional sequences

Decoding times on subjective test sequences in ALF-off configuration have been measured. This allows to evaluate the complexity increase among the proposal in CE5-2. Note that the decoding time may be higher than on average since the high QP (i.e. low bitrate) encodes have been used.

| Sequence | CE5-2.1 | CE5-2.2 |
| --- | --- | --- |
| SDR_03 QP37 |  |  |
| SDR_03 QP32 |  |  |
| SDR_06 QP37 |  |  |
| SDR_06 QP32 |  |  |
| SDR_07 QP37 |  |  |
| SDR_07 QP32 |  |  |
| SDR_09 QP37 |  |  |
| SDR_09 QP32 |  |  |
| 444_01 QP37 |  |  |
| 444_01 QP32 |  |  |

## CE5-2 Cross-checkers report

| Test | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Visual quality observations | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-2.1 | N. Hu | Y | Y | Y | N | N | N/A | No |
| CE5-2.2 | N. Hu | Y | Y | Y | N | N | N/A | No |

# CE5-3: Bilateral/Hadamard/SAO loop filters

## Description of tests

This CE would evaluate and compare the bilateral and Hadamard loop filters (CE5-3.1 and CE5-3.2). It would also evaluate SAO version that allows sharpening (CE5-3.3).

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-3.1 | Jacob Strom jacob.strom@ericsson.com JVET-P0073 | Sergey Ikonin sergey.ikonin@huawei.com Kiho Choi kiho14.choi@samsung.com Jennifer Rasch Jennifer.Rasch@hhi.fraunhofer.de |
| CE5-3.2 | Sergey Ikonin sergey.ikonin@huawei.com JVET-P0078 | Kiho Choi kiho14.choi@samsung.com |
| CE5-3.3 | Jenny Lai jenny.lai@mediatek.com JVET-P0045 | Elena Alshina elena.alshina@huawei.com Kiho Choi kiho14.choi@samsung.com |
| CE5-3.4 | Sergey Ikonin sergey.ikonin@huawei.com JVET-P0078 | Kiho Choi kiho14.choi@samsung.com Nan Hu nanh@qti.qualcomm.com |

In this sub-CE, the bilateral in-loop filter and the Hadamard in-loop filter are investigated under the same conditions. The sample adaptive offset (SAO) without edge offset (EO) sign constraint is also tested. The following tests are performed.

- Test 1: Filtering in combination with SAO. This will be the base-line implementation similar to test3 in JVET-O0548 and test3 in JVET-O1120 but implemented in VTM-6.0. Hence it will be allowed to apply the filtering on the reconstructed block in the RDO (mode selection part) of the encoder. It will also include the possibility to turn off the filter at the CTU level.

- Test 2: Filtering without SAO. In this implementation, SAO is turned off. Otherwise it is similar to test 1.

- Test 3: Filtering in combination with SAO without RDO. In this implementation, it will not be allowed to apply filtering on the reconstructed block in the RDO (mode selection part) of the encoder. Otherwise it is similar to test 1.

- Test 4: Filtering as a post filter. In this implementation, the filtering will be applied outside the coding loop, as if it was used as a post filter. Hence the sample input to the filter should be the same as the output yuv samples of the decoded anchor. Apart from the sample input, the filter may use information available to the decoder, such as QP and/or transform size. Only parameters available in the decoder anchor bit streams are allowed.

## CE5-3.1 Bilateral filter JVET-P0073

This sub-CE will contain following tests based on JVET-O0548.

- CE5-3.1.1 This will follow the test 1 restrictions outlined in the previous section

- CE5-3.1.2 This will follow the test 2 restrictions outlined in the previous section

- CE5-3.1.3 This will follow the test 3 restrictions outlined in the previous section

- CE5-3.1.4 This will follow the test 4 restrictions outlined in the previous section

### Detailed description of the bilateral filter

The proposed filter is carried out in the sample adaptive offset (SAO) loop-filter stage, as shown in Figure 3.2.1. Both the proposed bilateral filter (BIF) and SAO are using samples from deblocking as input. Each filter creates an offset, and these are added to the input sample and then clipped, before proceeding to ALF.

Fig. 3.2.1: Both the proposed filter (BIF) and SAO uses samples from the deblocking stage as input. Both create an offset, and these are added to the input sample and clipped.

In detail, the output sample IOUT is obtained as

IOUT=clip3IC+ΔIBIF+ΔISAO,                 (Eq. 3.2.1)

where IC is the input sample from deblocking, ΔIBIF is the offset from the bilateral filter and ΔISAO is the offset from SAO.

### Calculating the bilateral filter offset

The offset ΔIBIF for a bilateral filter is calculated using the surrounding 8-neighbors. As is shown in Figure 3.2.2, this means that the same sample values are used as for SAO.

Figure 3.2.2: The bilateral filter and the SAO filters access the same 3×3 pixels

At the picture border, where samples are unavailable, the bilateral filter uses extension (sample repetition) to fill in unavailable samples. For virtual boundaries, the behavior is the same as for SAO, i.e., no filtering occurs.

The samples surrounding the center sample IC are denoted according to Figure 3.2.3, where A, B, L and R stands for above, below, left and right and where NW, NE, SW, SE stands for north-west etc.

| INW | IA | INE |
| --- | --- | --- |
| IL | IC | IR |
| ISW | IB | ISE |

Figure 3.2.3: Naming convention for samples surrounding the center sample, IC.

Each surrounding sample IA, IR etc will contribute with a corresponding modifier value μΔIA, μΔIR, etc. These are calculated the following way: Starting with the contribution from the sample to the right, IR, we calculate the difference

ΔIR=IR-IC+4≫3,         (Eq. 3.2.2)

where |⋅| denotes absolute value. For data that is not 10-bit, we instead use ΔIR= IR-IC+2n-6≫n-7, where n = 8 for 8-bit data etc. The resulting value is now clipped so that it is smaller than 16:

sIR=min15,ΔIR.          (Eq. 3.2.3)

The modifier value is now calculated as

μΔIR=LUTROWsIR,  if IR-IC≥0,-LUTROWsIR           otherwise        (Eq. 3.2.4)

where LUTROW[ ] is an array of 16 values determined by the value of qpb = QP + bilateral_filter_qp_offset:

If qpb ≤ 22, then LUT(x) = (0, 4, 4, 4, 3, 2, 1 ,2, 1, 1, 1, 1, 0, 1, 1, -1), otherwise

if qpb ≤ 27, then LUT(x) = (0, 8, 11, 11, 7, 5, 5, 4 , 5, 4, 4, 2, 2 , 2, 2, -2), otherwise

if qpb ≤ 32, then LUT(x) = (0, 9, 16, 19, 22, 22, 20, 15, 12, 12, 11, 9, 9, 7, 8, -3), otherwise

if qpb ≤ 37, then LUT(x) = (0, 12, 21, 28, 33, 36, 40, 40, 40, 36, 29, 22, 19,17, 15, -3), otherwise

LUT(x) = (0, 17, 23, 33, 37, 41, 44, 44, 45, 44, 42, 27, 22, 17, 15, -3).

The modifier values for μΔIL, μΔIA and  μΔIB are calculated from IL, IA and IB in the same way. For diagonal samples, the calculation also follows Equations 2 and 3, but uses a value shifted by 1. Using the diagonal sample ISE as an example, we get

μΔISE=LUTROWsISE≫1,  if ISE-IC≥0,-(LUTROWsISE≫1)           otherwise        (Eq. 3.2.5)

and the other diagonal samples are calculated likewise. The modifier values are summed together

msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE.        (Eq.3.2.6)

Note that μΔIR equals -μΔIAfor the previous sample. Likewise, μΔIA equals -μΔIBfor the sample above, and similar symmetries can be found also for the diagonal modifier values. This means that in a hardware implementation, it is sufficient to calculate four values μΔIR, μΔIB, μΔISW and μΔISE, and the remaining four values can be obtained from previously calculated values.

The msum value is now multiplied either by c = 1, 2 or 3, which can be done using a single adder and logical AND gates in the following way:

cv=k1&msum≪1+k2&msum,        (Eq. 3.2.7)

where & denotes logical and and k1 is the most significant bit of the multiplier c and k2 is the least significant bit. The value to multiply with is obtained using the minimum block dimension D=min⁡(width, height) as shown in Table 1:

| Block type | D≤4 | 4<D<16 | D≥16 |
| --- | --- | --- | --- |
| Intra | 3 | 2 | 1 |
| Inter | 2 | 2 | 1 |

Table 1: Obtaining the c parameter from the minimum size D=min(width,height) of the block.

Finally, the bilateral filter offset ΔIBIF is calculated. For full strength filtering, we use

ΔIBIF=cv+16≫5,          Eq. 3.2.8

whereas for half-strength filtering, we instead use

ΔIBIF=cv+32≫6.        (Eq. 3.2.9)

A general formula for n-bit data is to use

radd=214-n-bilateral_filter_strengthrshift=15-n-bilateal_filter_strength ΔIBIF=cv+radd≫rshift,           (Eq. 3.2.10)

where bilateral_filter_strength can be 0 or 1 and is signalled in the pps.

### Hardware complexity of the bilateral filter

In this section, we count the hardware complexity needed to decode a sample. As is usual in core experiments in JVET, we calculate the number of adds, shifts and checks, where a check is a min- max- or abs operation. Table 3.2.2 shows the number of such operations for calculating the modifier value of a sample below or to the right. Rounding additions are calculated separately in brackets, since these can be implemented with a carry bit in the nearby addition or check. No adder is needed to negate the LUT value, since it will be used in an addition later and it is possible to invert all the bits and set the carry to 1.

| Operation | add | shift | check | LUT |
| --- | --- | --- | --- | --- |
| ΔIR=IR-IC+4≫3 | 1 (1) | 1 | 1 |  |
| sIR=min15, ΔIR |  |  | 1 |  |
| μΔIR=LUTROWsIR, if IR-IC≥0,-LUTROWsIR otherwise |  |  |  | 1 |
| in total | 1 (1) | 1 | 2 | 1 |

Table 3.2.2: Operations per modifier value.

Table 3.2.2 is also valid for diagonal samples, except that one more shift is needed. Table 3.2.3 describes the total number of operations per sample. In total 13 additions and 5 rounding additions are needed per sample, as well as 8 shifts, 8 checks and 4 look-ups.

| Variable calculated | add | shift | check |
| --- | --- | --- | --- |
| μΔIR and μΔIB | 2 (2) | 2 | 4 |
| μΔISW and μΔISE | 2 (2) | 4 | 4 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE | 7 |  |  |
| cv=k1&msum≪1+k2&msum | 1 | 1 |  |
| ΔIBIF=cv+16≫5 | (1) | 1 |  |
| IOUT=clip3IC+ΔIBIF+ΔISAO | 1 |  |  |
| in total | 13 (5) | 8 | 8 |

Table 3.2.3: Operations per sample

The widths of the additions are reported in Table 3.2.4, where the addition being investigated is marked by ⊕ and ⊖. There is one 11-bit adder, four 10-bit adders, one 9-bit adder, one 8-bit adder, three 7-bit adders and three 6- bit adders.

| Operation | bit width | number |
| --- | --- | --- |
| ΔIR=IR⊖IC+4≫3 | 10 | 4 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR+μΔINW⊕μΔINE+μΔISW⊕μΔISE | 6 | 2 |
| msum= μΔIA⊕μΔIB+μΔIL⊕μΔIR+μΔINW+μΔINE⊕μΔISW+μΔISE | 7 | 3 |
| msum= μΔIA+μΔIB⊕μΔIL+μΔIR+μΔINW+μΔINE+μΔISW+μΔISE | 8 | 1 |
| msum= μΔIA+μΔIB+μΔIL+μΔIR⊕μΔINW+μΔINE+μΔISW+μΔISE | 9 | 1 |
| cv=k1&msum≪1⊕k2&msum | 11 | 1 |
| IOUT=clip3IC+ΔIBIF⊕ΔISAO | 6 | 1 |

Table 3.2.4: Adder widths

The LUT values can be stored in 6 bits, which means that 16×6×5 = 480 bits or 60 bytes are needed. Normally 7 bits would be required for signed values, but the sign bit can be uniquely recovered by ANDing the two next most significant bits, and thus does not need to be stored.

## CE5-3.2 Hadamard filter JVET-P0078

This sub-CE will contain following tests based on JVET-O1120.

- CE5-3.2.1 This will follow the test 1 restrictions outlined in the previous section

- CE5-3.2.2 This will follow the test 2 restrictions outlined in the previous section

- CE5-3.2.3 This will follow the test 3 restrictions outlined in the previous section

- CE5-3.2.4 This will follow the test 4 restrictions outlined in the previous section

### Detailed description of the Hadamard filter

Hadamard transform domain filter was proposed in JVET-M0468. The filter parameters are explicitly derived from the coded information and filtering process is as follows.

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

F(i,QP)=Ri   ,  AbsRi≥128LUTRi,QP, R(i)>0-LUT-Ri,QP, R(i)≤0

F(i,QP)=Ri   ,  AbsRi≥128LUTRi,QP, R(i)>0-LUT-Ri,QP, R(i)≤0

Spectrum filtering using a look-up table (LUT):

,wherein (i) is an index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to the index, QP is quantization parameter of the current block.

The LUT has 16 entries of 7-bit values for certain QP value. For whole QP range five LUT are pre-calculated having overall size 5x16x7-bit = 560 bits = 70 bytes.

The first spectrum component corresponding to DC value is bypassed without filtering.

- Inverse 4 point Hadamard transform of filtered spectrum

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of pixels the accumulated values are normalized by number of processing groups used for each pixel filtering. Due to use of padding of one sample around the block number of processing groups is equal to 4 for each pixel in the block and normalization is performed by right shifting on 2 bits.

- To fill one-sample padding around the current block only samples of current CTU are used. If block is on CTU boundary padding is filled by extending sample of current CTU, so cross-CTU sample access is avoided.

Equivalent filter shape is 3x3 pixels as depicted on figure below.

Equivalent filter shape

It can be seen that all pixels in the block can be processed independently in case of maximum parallelism is required. It should also be noted that results of 2x2 groups filtering are reused for spatial collocated samples. Generally each new pixel in the block requires one 2x2 filtering, rest three are reused.

When SAO is enabled results of Hadamard and SAO filtering is averaged:

The following code illustrates the inner processing loop to get one sample filtered:

for (int r = 0; r < height - 1; ++r)
{

for (int c = 0; c < width - 1; ++c, pIn++, pAcc++)

{

const int x0 = pIn[p0];

const int x1 = pIn[p1];

const int x2 = pIn[p2];

const int x3 = pIn[p3];

// forward transform

const int y0 = x0 + x2;

const int y1 = x1 + x3;

const int y2 = x0 - x2;

const int y3 = x1 - x3;

const int t0 = y0 + y1;

const int t1 = y0 - y1;

const int t2 = y2 + y3;

const int t3 = y2 - y3;

// filtering

const int z0 = t0;  // skip DC

const int z1 = RdTbl(t1, tbl, thr, tblShift, tblRnd);

const int z2 = RdTbl(t2, tbl, thr, tblShift, tblRnd);

const int z3 = RdTbl(t3, tbl, thr, tblShift, tblRnd);

// backward transform

const int iy0 = z0 + z2;

const int iy1 = z1 + z3;

const int iy2 = z0 - z2;

const int iy3 = z1 - z3;

pAcc[p0_acc] += ((iy0 + iy1) >> HTDF_BIT_RND4);

pAcc[p1_acc] += ((iy0 - iy1) >> HTDF_BIT_RND4);

pAcc[p2_acc] += ((iy2 + iy3) >> HTDF_BIT_RND4);

pAcc[p3_acc] += ((iy2 - iy3) >> HTDF_BIT_RND4);

// normalization

pIn[p0] = ClipPel((pAcc[p0_acc] + HTDF_CNT_SCALE_RND) >> HTDF_CNT_SCALE, clpRng);

}

}

inline int RdTbl(const int z, const uint8_t *tbl, const int thr, const int tblShift, const int tblRnd)

{

return  (z > 0 ?  (z < thr ?  tbl[(  z + tblRnd) >> tblShift] : z) : 
                    (-z < thr ? -tbl[((-z)+ tblRnd) >> tblShift] : z));

}

### Hardware complexity analysis of Hadamard filter

Detailed complexity analysis was conducted during previous study in CE and can be found in JVET-L0326 including RTL simulation results. From code example above the total amount of operations required per each sample is estimated as:

- Additions: 8 on forward, 8 on backward Hadamard transform, and 4 on averaging, totally: 20 + 4 of 1-bit additions for rounding, most of additions can be done in parallel

- LUT access: 3

- Shifts: 8,

- Checks: 6.

It can be further observed that 2 operations can be saved due to reusing results of forward Hadamard transform calculation in shared area of overlapping 2x2 blocks. To illustrate this let’s consider calculation of Hadamard transform for first 2x2 block:

x0x1x2x3=>t0t1t2t3, where t0=x0+x2+(x1+x3)t1=x0+x2-(x1+x3)t2=x0-x2+(x1-x3)t3=x0-x2-(x1-x3)

Then for second 2x2 block shifted on one pixel right:

x1x4x3x5=>t0t1t2t3, where t0=x1+x3+(x4+x5)t1=x1+x3-(x4+x5)t2=x1-x3+(x4-x5)t3=x1-x3-(x4-x5)

It can be seen that 2 components x1+x3 and x1-x3 are part of both transforms. Thus it can be shared between two overlapping blocks and 2 adding operations can be saved.

Sharing calculation results in overlapped area.

After taking into account shared calculation amount of operations required per each sample is:

- Additions: 18 + 4 of 1-bit additions for rounding,

- LUT access: 3

- Shifts: 8,

- Checks: 6.

When SAO is enabled one more addition is required to do an averaging.

## CE5-3.3 Sample adaptive offset JVET-P0045

In current VVC draft, the signs of SAO EO are inferred in order to allow smooth filtering only. In JVET-K0233, this constraint is removed. That is, for each non-zero EO offset, one flag is coded to indicate the sign is positive or negative, so both of smooth filtering and sharp filtering are allowed.

- CE5-3.3.1: Remove the sign constraint of SAO EO

- CE5-3.3.2: Remove SAO

## CE5-3.4 Bilateral and Hadamard filter combination JVET-P0078

This sub-CE will evaluate whether combining CE5-3.1 and CE5-3.2 can provide additional improvement. Two filters are evaluated at encoder side then the best filter to be combined with SAO is signalled at CTU level. This sub-CE will contain following tests:

- CE5-3.4.1 This will follow the test 1 restrictions outlined in the previous section

- CE5-3.4.2 This will follow the test 2 restrictions outlined in the previous section

- CE5-3.4.3 This will follow the test 3 restrictions outlined in the previous section

- CE5-3.4.4 This will follow the test 4 restrictions outlined in the previous section

## CE5-3 Tests results

The number of YUV BD-rate is calculated by using the same formula in AHG13 (YUV BD-rate = (8*Y-BD-rate + U-BD-rate + V-BD-rate) / 10). The reported run time data are inaccurate in CE5-3.3.1 and no SIMD optimization is used in CE5-3.3.

| Test# | Description | Document# |
| --- | --- | --- |
| CE5-3.1 | Bilateral filter | JVET-P0073 |
| CE5-3.2 | Hadamard filter | JVET-P0078 |
| CE5-3.3 | Sample adaptive offset | JVET-P0045 |
| CE5-3.4 | Bilateral and Hadamard filter combination | JVET-P0078 |

### VTM6.0 CTC (ALF on) Test 1 – filtering in combination with SAO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.1 | -0.28% | -0.40% | 0.20% | 0.20% | 106% | 104% | -0.30% | -0.44% | 0.29% | 0.23% | 103% | 102% |
| CE5-3.2.1 | -0.19% | -0.33% | 0.39% | 0.37% | 107% | 108% | -0.24% | -0.37% | 0.30% | 0.31% | 104% | 103% |
| CE5-3.4.1 | -0.21% | -0.35% | 0.35% | 0.32% | 109% | 105% | -0.19% | -0.34% | 0.45% | 0.39% | 104% | 102% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.1 | -0.12% | -0.32% | 0.67% | 0.71% | 102% | 102% | -0.19% | -0.39% | 0.52% | 0.67% | 103% | 102% |
| CE5-3.2.1 | 0.07% | -0.08% | 0.68% | 0.64% | 103% | 103% | -0.01% | -0.16% | 0.47% | 0.73% | 104% | 103% |
| CE5-3.4.1 | 0.02% | -0.17% | 0.78% | 0.80% | 103% | 102% | -0.05% | -0.23% | 0.53% | 0.82% | 104% | 102% |

### VTM6.0 CTC (ALF on) Test 2 – filtering without SAO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.2 | -0.24% | -0.40% | 0.38% | 0.41% | 109% | 104% | -0.19% | -0.38% | 0.56% | 0.55% | 103% | 103% |
| CE5-3.2.2 | -0.26% | -0.42% | 0.39% | 0.36% | 108% | 105% | -0.34% | -0.46% | 0.11% | 0.16% | 104% | 101% |
| CE5-3.4.2 | -0.21% | -0.35% | 0.36% | 0.34% | 109% | 106% | -0.22% | -0.36% | 0.37% | 0.35% | 104% | 103% |

|  | Low Delay B |  |  |  |  |  |  |  | Low Delay P |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT |  | DecT |  | YUV | Y |  | U |  | V |  | EncT |  | DecT |  |
| CE5-3.1.2 | 0.09% | -0.22% | 1.03% | 1.63% |  | 105% |  | 101% | 0.06% |  | -0.27% |  | 0.91% |  | 1.84% |  | 105% |  | 101% |  |
| CE5-3.2.2 | -0.12% | -0.25% | 0.47% | 0.31% |  | 102% |  | 101% | -0.14% |  | -0.32% |  | 0.59% |  | 0.62% |  | 103% |  | 101% |  |
| CE5-3.4.2 | -0.04% | -0.17% | 0.61% | 0.32% |  | 103% |  | 103% | -0.08% |  | -0.27% |  | 0.62% |  | 0.76% |  | 104% |  | 103% |  |

### VTM6.0 CTC (ALF on) Test 3 – Filtering in combination with SAO without RDO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.3 | -0.03% | -0.03% | -0.02% | -0.03% | 101% | 105% | -0.04% | -0.04% | 0.02% | -0.05% | 100% | 102% |
| CE5-3.2.3 | 0.00% | 0.00% | 0.02% | 0.02% | 100% | 107% | -0.03% | -0.03% | 0.00% | -0.01% | 99% | 103% |
| CE5-3.4.3 | -0.03% | -0.04% | 0.01% | 0.00% | 100% | 106% | 0.05% | 0.03% | 0.16% | 0.08% | 99% | 102% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.3 | -0.03% | -0.07% | 0.14% | 0.09% | 103% | 105% | -0.09% | -0.09% | -0.14% | -0.05% | 101% | 101% |
| CE5-3.2.3 | 0.01% | -0.02% | 0.20% | 0.09% | 99% | 103% | 0.00% | -0.02% | 0.00% | 0.15% | 99% | 103% |
| CE5-3.4.3 | 0.09% | 0.03% | 0.25% | 0.38% | 99% | 102% | 0.03% | -0.04% | 0.22% | 0.40% | 99% | 102% |

### VTM6.0 CTC (ALF on) Test 4 – filtering as a post filter

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.4 | 0.48% | 0.60% | 0.00% | 0.00% | 100% | 109% | 0.18% | 0.23% | 0.00% | 0.00% | 100% | 106% |
| CE5-3.2.4 | 0.06% | 0.08% | 0.00% | 0.00% | 100% | 111% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 108% |
| CE5-3.4.4 |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.4 | 0.18% | 0.23% | 0.00% | 0.00% | 100% | 107% | 0.22% | 0.28% | 0.00% | 0.00% | 100% | 106% |
| CE5-3.2.4 | -0.02% | -0.02% | 0.00% | 0.00% | 100% | 108% | -0.01% | -0.01% | 0.00% | 0.00% | 100% | 108% |
| CE5-3.4.4 |  |  |  |  |  |  |  |  |  |  |  |  |

### VTM6.0 CTC (ALF on) tests of SAO (CE5-3.3)

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.3.1 | 0.00% | 0.00% | 0.01% | 0.02% | 67%* | 80%* | 0.00% | 0.00% | 0.02% | 0.00% | 72%* | 78%* |
| CE5-3.3.2 | 0.04% | 0.01% | 0.14% | 0.17% | 100%* | 99%* | 0.13% | 0.08% | 0.28% | 0.35% | 100%* | 98%* |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.3.1 | -0.02% | -0.03% | -0.02% | 0.07% | 73%* | 77%* | 0.06% | 0.02% | 0.13% | 0.29% | 72%* | 79%* |
| CE5-3.3.2 | 0.24% | 0.07% | 0.58% | 1.24% | 101%* | 101%* | 0.27% | 0.14% | 0.41% | 1.14% | 100%* | 100%* |

*: the reported run time data are inaccurate in CE5-3.3.1. No SIMD optimization is used in CE5-3.3.

CE5-3 results with ALF off

### VTM6.0 CTC (ALF off) Test 1 – filtering in combination with SAO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.1 | -0.40% | -0.56% | 0.23% | 0.22% | 106% | 104% | -0.46% | -0.61% | 0.20% | 0.13% | 103% | 103% |
| CE5-3.2.1 | -0.28% | -0.46% | 0.44% | 0.42% | 108% | 110% | -0.42% | -0.60% | 0.28% | 0.32% | 104% | 105% |
| CE5-3.4.1 | -0.34% | -0.52% | 0.39% | 0.37% | 109% | 106% | -0.46% | -0.67% | 0.39% | 0.40% | 105% | 103% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.1 | -0.19% | -0.42% | 0.69% | 0.74% | 105% | 103% | -0.32% | -0.57% | 0.66% | 0.66% | 107% | 102% |
| CE5-3.2.1 | -0.26% | -0.49% | 0.70% | 0.61% | 103% | 104% | -0.28% | -0.58% | 0.97% | 0.91% | 105% | 105% |
| CE5-3.4.1 | -0.32% | -0.58% | 0.70% | 0.73% | 104% | 103% | -0.43% | -0.75% | 0.84% | 0.86% | 106% | 103% |

### VTM6.0 CTC (ALF off) Test 2 – filtering with SAO off

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.2 | -0.21% | -0.47% | 0.64% | 0.98% | 106% | 106% | -0.09% | -0.35% | 1.02% | 0.86% | 102% | 101% |
| CE5-3.2.2 | -0.29% | -0.61% | 0.81% | 1.13% | 108% | 103% | -0.27% | -0.56% | 0.93% | 0.87% | 104% | 101% |
| CE5-3.4.2 | -0.12% | -0.38% | 0.78% | 1.11% | 109% | 104% | 0.01% | -0.29% | 1.25% | 1.13% | 105% | 102% |

|  | Low Delay B |  |  |  |  |  |  |  |  | Low Delay P |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT |  | DecT |  | YUV |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| CE5-3.1.2 | 0.55% | -0.14% | 2.56% | 4.06% |  | 105% |  | 101% |  | 1.13% |  | 0.33% |  | 3.77% |  | 4.84% |  | 108% |  | 100% |  |
| CE5-3.2.2 | 0.35% | -0.39% | 2.78% | 3.82% |  | 103% |  | 100% |  | 0.85% |  | -0.05% |  | 3.90% |  | 4.97% |  | 105% |  | 100% |  |
| CE5-3.4.2 | 0.43% | -0.33% | 2.75% | 4.18% |  | 105% |  | 102% |  | 1.02% |  | 0.18% |  | 3.83% |  | 4.93% |  | 107% |  | 101% |  |

### VTM6.0 CTC (ALF off) Test 3 – Filtering in combination with SAO without RDO

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.3 | -0.11% | -0.13% | -0.02% | -0.02% | 100% | 105% | -0.13% | -0.13% | -0.16% | -0.14% | 99% | 102% |
| CE5-3.2.3 | -0.06% | -0.08% | 0.01% | 0.01% | 100% | 109% | -0.04% | -0.06% | 0.04% | 0.00% | 99% | 104% |
| CE5-3.4.3 | -0.17% | -0.21% | 0.01% | 0.01% | 100% | 107% | -0.14% | -0.18% | 0.05% | 0.02% | 99% | 103% |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.3 | -0.15% | -0.17% | 0.01% | -0.10% | 103% | 104% | -0.25% | -0.29% | -0.06% | -0.15% | 103% | 102% |
| CE5-3.2.3 | -0.09% | -0.13% | 0.05% | 0.05% | 100% | 104% | -0.05% | -0.11% | 0.18% | 0.16% | 100% | 105% |
| CE5-3.4.3 | -0.15% | -0.23% | 0.13% | 0.18% | 99% | 103% | -0.33% | -0.43% | 0.08% | 0.03% | 99% | 103% |

### VTM6.0 CTC (ALF off) Test 4 – filtering as a post filter

|  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.4 | 0.06% | 0.08% | 0.00% | 0.00% | 100% | 109% | 0.02% | 0.02% | 0.00% | 0.00% | 100% | 107% |
| CE5-3.2.4 | -0.10% | -0.13% | 0.00% | 0.00% | 100% | 113% | -0.06% | -0.08% | 0.00% | 0.00% | 100% | 109% |
| CE5-3.4.4 |  |  |  |  |  |  |  |  |  |  |  |  |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.1.4 | 0.01% | 0.01% | 0.00% | 0.00% | 100% | 107% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 107% |
| CE5-3.2.4 | -0.07% | -0.09% | 0.00% | 0.00% | 100% | 109% | -0.08% | -0.10% | 0.00% | 0.00% | 100% | 110% |
| CE5-3.4.4 |  |  |  |  |  |  |  |  |  |  |  |  |

### VTM6.0 CTC (ALF off) tests of SAO (CE5-3.3)

|  | All Intra |  |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV |  | Y | U | V | EncT | DecT |
| CE5-3.3.1 | 0.02% | 0.02% | 0.03% | 0.04% | 100%* | 92%* | 0.00% |  | 0.03% | -0.09% | -0.14% | 100%* | 89%* |
| CE5-3.3.2 | 0.37% | 0.32% | 0.37% | 0.72% | 100%* | 90%* | 0.79% |  | 0.80% | 0.81% | 0.73% | 100%* | 98%* |

|  | Low Delay B |  |  |  |  |  | Low Delay P |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | YUV | Y | U | V | EncT | DecT | YUV | Y | U | V | EncT | DecT |
| CE5-3.3.1 | -0.06% | -0.05% | -0.19% | 0.02% | 100%* | 87%* | 0.09% | 0.05% | 0.27% | 0.19% | 100%* | 86%* |
| CE5-3.3.2 | 1.24% | 0.96% | 1.77% | 2.98% | 101%* | 98%* | 2.83% | 2.68% | 3.19% | 3.65% | 100%* | 97%* |

*: the reported run time data are inaccurate in CE5-3.3.1. No SIMD optimization is used in CE5-3.3.

## Complexity Analysis (Bilateral/Hadamard filter/SAO)

### Computational complexity of filters

| Test | Filter shape | Comp. complexity per sample (add/shifts/compar./look-ups) | Memory requirements (bytes) |
| --- | --- | --- | --- |
| CE5-3.1 | 3x3 | 18 adds (incl 5 rounding) / 8 shifts / 8 comp / 4 look-ups | 60 |
| CE5-3.2 | 3x3 | 19 adds +5 l-bit for rounding / 8 shifts / 6 comp / 3 look-up | 70 (14 bytes per QP group) |
| CE5-3.3 | Four 1D patterns in 3x3 | 2 adds / 0 shift / 4 comp / 1 LUT | 3.75 |

### Decoding complexity on additional sequences, Test 1 configuration, no ALF

Decoding times on subjective test sequences in the random-access, ALF-off configuration have been measured. Note that the decoding time may be higher than on average since the high QP (i.e. low bitrate) encodes have been used. Note that no SIMD optimization is used in CE5-3.3.

Test

| Sequence | CE5-3.1 | CE5-3.2 | CE5-3.3 | CE5-3.4 |
| --- | --- | --- | --- | --- |
| SDR_06 QP37 | 103% | 107% |  | 101% |
| SDR_06 QP32 | 104% | 108% |  | 102% |
| SDR_10 QP37 | 104% | 108% |  | 101% |
| SDR_10 QP32 | 103% | 109% |  | 102% |
| SDR_11 QP37 | 101% | 107% |  | 102% |
| SDR_11 QP32 | 103% | 107% |  | 103% |
| SDR_12 QP37 | 101% | 106% |  | 102% |
| SDR_12 QP32 | 103% | 107% |  | 104% |
| SDR_13 QP37 | 108% | 108% |  | 107% |
| SDR_13 QP32 | 105% | 108% |  | 104% |
| SDR_14 QP37 | 103% | 109% |  | 106% |
| SDR_14 QP32 | 104% | 109% |  | 104% |
| SDR_15 QP37 | 104% | 108% |  | 104% |
| SDR_15 QP32 | 101% | 101% |  | 100% |

## CE5-3 Cross-checkers report

| Test | Cross-checker | PSNR&BR Match (Y/N) | RunT matched? (Y/N) | SW studied? (Y/N) | Significant inconsistencies between description and SW? (Y/N) | Studied visual quality? (Y/N) | Visual quality observations | Other comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CE5-3.1 | S. Ikonin | Y | Y | Y | N | N | N/A | - CTU level on/off flag is always set to 1 at encoder side without cost or distortion checking - When SAO is off there is no CTU level flag, whole picture is filtered. - Cross-CTU pel accessed to fill the padding around the current block - Additional logic is required to extract sing for negative values of the LUT, otherwise the size required will be 70 bytes |
|  | K. Choi |  |  |  |  |  |  |  |
|  | J. Rasch |  |  |  |  |  |  |  |
| CE5-3.2 | K. Choi |  |  |  |  |  |  |  |
| CE5-3.3 | E. Alshina | Y | Y | Y | N | Y | Visual quality improvement examples provided by proponent have been investigated and quality improvement is confirmed. | Supplementary test results for EO with sign coded as equal probability are provided. |
|  | K. Choi |  |  |  |  |  |  |  |
| CE5-3.4 | K. Choi |  |  |  |  |  |  |  |
|  | N. Hu | Y | Y | Y | N | N | N/A | New HLS in BIF (BIFStrengh and BIFQpOffset). In T1 and T2, BIFStrength is 1. In T3, BIFStrength is 0. This is also mentioned in JVET-P0073 |

# Subjective test(s)

A number of subjective test sequences have been generated for an expert viewing test in Geneva meeting proposals will be performed for proposals in CE5-1, CE5-1, and CE5-3.

The following test sequences and conditions are used in the subjective tests. For most sequences, the sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1].

The subjective test sequences have been uploaded to the JVET ftp folder /ce/JVET-O2025_LoopFilter). For CE5-1 and CE5-3, coding configuration with ALF ON and ALF OFF shall be used. For CE5-2, coding configuration with ALF ON shall be used.

The anchors for the subjective viewing of deblocking (CE5-1) and SAO/Bilateral/Hadamard (CE5-3) proposals will use the ALF ON and ALF OFF configurations. Anchors have been provided by volunteers:

- Kiran Misra: SDR_06, SDR_07, SDR_01 and 444_1 anchors for CE5-2

- Kenneth Andersson: SDR_01 to SDR_05 and HDR_01 to HDR_03

- Jenny Lai: SDR_06 to SDR_15

- Anand Meher Kotra: 444_1 and 444_2

The following SDR sequences and QP are used for subjective evaluation in CE5-1.1 sub-category, where the five first are SDR sequences and the remaining sequences are HDR PQ sequences:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| SDR_02 | Campfire | 3840x2160p | 30fps | 300 | RA | 37, 32 |
| SDR_03 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 37, 32 |
| SDR_04 | Kimono | 1920x1080p | 24fps | 240 | RA | 37, 32 |
| SDR_05 | ParkJoy | 1920x1080p | 50fps | 500 | RA | 37, 32 |

* First 300 frames of the RedKayak sequence shall be used.

The following HDR PQ sequences will be used for subjective evaluation in CE5-1.1 sub-category.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR_01 | ShowGirl | 1920x1080p | 25fps | 339 | RA | 37, 32 |
| HDR_02 | Cosmos1_TreeTrunk | 1920x856p | 24fps | 240 | RA | 37, 32 |
| HDR_03 | EBU_Hurdles | 1920x1080p | 50fps | 500 | RA | 37, 32 |

The following sequences and QP are used for subjective evaluation in CE5-1.2 and CE5-1.3 sub- category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| SDR_02 | Campfire | 3840x2160p | 30fps | 300 | RA | 37, 32 |
| SDR_03 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| SDR_06 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| SDR_07 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| SDR_08 | Johnny | 1280x720p | 60fps | 600 | LDB | 32, 37 |

* First 300 frames of the RedKayak sequence shall be used.

Additionally, the following HDR PQ sequences are planned to be evaluated subjectively for CE5-1.2 to study its potential effect on the HDR material.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR_1 | ShowGirl | 1920x856p | 25fps | 339 | LDB | 32, 37 |
| HDR_2 | Cosmos1_TreeTrunk | 1920x1080p | 24fps | 240 | LDP | 32, 37 |

The following 4:4:4 sequences are planned to be evaluated subjectively for CE5-1.3 to study the subjective improvement of CE5-1.3 on 4:4:4 content.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| 444_1 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |
| 444_2 | CrowdRun YCbCr 4:4:4 | 1920x1080p | 50fps | 500 | LDP | 32, 37 |

The following sequences and QP are used for subjective evaluation (with ALF ON only) in CE5-2 category:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_03 | RedKayak | 1920x1080p | 30fps | 300 | RA | 32, 37 |
| SDR_06 | Cactus | 1920x1080p | 50fps | 500 | LDB | 32, 37 |
| SDR_07 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| SDR_09 | FourPeople | 1280x720p | 60fps | 600 | LDP | 32, 37 |
| 444_1 | Kimono YCbCr 4:4:4 | 1920x1080p | 24fps | 240 | RA | 32, 37 |

The following sequences and QP are used for subjective evaluation in CE5-3 category. A pre-selection of the subjective test material in CE5-3 should be done at the Geneva meeting.

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_06 | Cactus | 1920x1080p | 50fps | 500 | RA/LDB/LDP | 32, 37 |
| SDR_07 | KristenAndSara | 1280x720p | 60fps | 600 | LDB/LDP | 32, 37 |
| SDR_10 | ArenaOfValor | 1920x1080p | 60fps | 600 | RA/LDB/LDP | 32, 37 |
| SDR_11 | BasketballDrive | 1920x1080p | 50fps | 500 | RA/LDB/LDP | 32, 37 |
| SDR_12 | BQTerrace | 1920x1080 | 60fps | 600 | RA/LDB/LDP | 32, 37 |
| SDR_13 | CatRobot | 3840x2160p | 60fps | 600 | RA | 32, 37 |
| SDR_14 | SlideEditing | 1280x720p | 30fps | 300 | RA/LDB/LDP | 32, 37 |
| SDR_15 | BasketballDrillText | 832x480p | 50fps | 500 | RA/LDB/LDP | 32, 37 |

All three categories will be prepared by the proponents and the meeting will decide which mode to use (RA/LDB/LDP) for each sequence (except the CatRobot, which will only be generated according to the RA test conditions).

The test material at resolution lower than 4K will be expanded to full screen by the player and evaluated together with the 4K test material.

The HDR material will be shown at the HDR-capable display.

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring , “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010.

- J. Chen, Y. Ye, S. Kim , “Algorithm description for Versatile Video Coding and Test Model 6 (VTM 6)”, JVET-O1002.