| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0027-v1 |
| --- | --- |

| Title: | CE 7: Summary report on quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cauyeung@tencent.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

The CE report summarizes the test results and crosscheck reports for CE7 on quantization and coefficient coding.  The CE includes 4 sub-CEs on the following topics:

- CE7-1:	Coding order and bypass switch for transform skip residual coding

- CE7-2:	Context modelling and binarization for transform skip residual coding

- CE7-3:	Context reduction for regular transform coefficient coding

- CE7-4:	Chroma cbf coding

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient/residual coding.

# Core Experiment Results

## CE7-1:  Coding order and bypass switch for transform skip residual coding

CE7-1: Summary of tests

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-1.1 | S.-T. Hsiang (Mediatek) | Re-positioning of parity flag (JVET-O0295) | M. Sarwer (Alibaba) |
| CE7-1.2a | Y.-H- Chao (Qualcomm) | 2-pass variant (JVET-O0623) | T. Nguyen (HHI) |
| CE7-1.2b | Y.-H- Chao (Qualcomm) | re-positioning of the parity flag after gtx scan (JVET-O0623) | T. Nguyen (HHI) |
| CE7-1.2c | Y.-H- Chao (Qualcomm) | Rice-Golomb coding after bypass switch (JVET-O0623) for 2 pass scan | T. Nguyen (HHI) |
| CE7-1.2d | Y.-H- Chao (Qualcomm) | Rice-Golomb coding after bypass switch (JVET-O0623) for 3 pass scan | T. Nguyen (HHI) |
| CE7-1.3a | M. Sarwer (Alibaba) | Swap position of parity_flag and abs_gt3_flag (JVET-O0619) | S.-T. Hsiang (Mediatek) |
| CE7-1.3b | M. Sarwer (Alibaba) | Simplified bypass coding / unification of bypass switch for transform skip and regular transform coefficient coding (JVET-O0619) | S.-T. Hsiang (Mediatek) |
| CE7-1.3c | M. Sarwer (Alibaba) | Combination of CE7-1.3a and CE7-1.3b | S.-T. Hsiang (Mediatek) |

Remarks:

- One CE participant considered the test CE7-1.2d as new proposal and objected to include this test in the core experiment. There was no agreement among the CE participants regarding this point.

- One CE participant considered the alternative bypass coding for tests CE7-1.3b/c as new proposal and objected to include this alternative method in the core experiment. There was no agreement among the CE participants regarding this point.

- The starred versions “*” in the result tables represent alternatives for the corresponding CE tests.

- One CE participant remarked that the CABAC initialization tables in CE tests CE7-1.2a/c/d were modified relative to VTM-6, which may have an impact on comparing the BD rate results.

Table 1:  Average test results for CE7-1 and Common Test Conditions (CTC).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 | 0.00% | -0.01% | 0.01% | 100% | 99% | 0.01% | 0.06% | -0.04% | 100% | 100% | -0.02% | -0.14% | -0.23% | 101% | 106% |
| CE7-1.2a | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.06% | 0.05% | 101% | 100% | -0.03% | 0.07% | -0.22% | 101% | 99% |
| CE7-1.2b | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.01% | 0.04% | 0.02% | 101% | 100% | -0.02% | -0.01% | -0.23% | 100% | 99% |
| CE7-1.2c | 0.00% | -0.01% | -0.01% | 100% | 101% | 0.00% | 0.03% | 0.00% | 101% | 100% | -0.06% | 0.07% | 0.01% | 100% | 100% |
| CE7-1.2c* | 0.00% | 0.00% | -0.02% | 101% | 101% | 0.00% | 0.05% | 0.02% | 101% | 100% | -0.04% | 0.13% | -0.11% | 100% | 99% |
| CE7-1.2d | 0.00% | -0.01% | 0.00% | 101% | 101% | 0.00% | 0.03% | 0.01% | 101% | 99% | -0.01% | 0.09% | -0.24% | 100% | 100% |
| CE7-1.2d* | 0.00% | -0.01% | 0.00% | 101% | 100% | 0.00% | 0.04% | -0.04% | 101% | 100% | 0.00% | 0.17% | -0.22% | 100% | 99% |
| CE7-1.3a | 0.00% | -0.02% | -0.01% | 101% | 102% | -0.01% | 0.04% | 0.01% | 101% | 100% | -0.02% | -0.12% | -0.05% | 101% | 102% |
| CE7-1.3b | 0.00% | 0.01% | 0.00% | 101% | 102% | 0.01% | 0.06% | 0.04% | 101% | 101% | 0.01% | 0.00% | -0.06% | 101% | 102% |
| CE7-1.3b* | 0.00% | 0.00% | -0.02% | 101% | 102% | 0.00% | 0.10% | 0.07% | 101% | 101% | 0.01% | 0.07% | -0.13% | 101% | 102% |
| CE7-1.3c | 0.00% | -0.02% | 0.01% | 100% | 102% | 0.00% | 0.03% | 0.07% | 101% | 100% | -0.06% | 0.16% | -0.11% | 101% | 102% |
| Class F | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 | -0.06% | -0.09% | -0.07% | 102% | 100% | -0.02% | -0.08% | -0.03% | 100% | 95% | -0.02% | -0.07% | 0.30% | 99% | 98% |
| CE7-1.2a | 0.02% | 0.00% | -0.13% | 100% | 98% | -0.01% | 0.02% | 0.13% | 102% | 101% | 0.06% | -0.25% | 0.28% | 102% | 99% |
| CE7-1.2b | -0.05% | -0.05% | 0.07% | 101% | 101% | -0.02% | 0.01% | 0.02% | 102% | 100% | 0.03% | 0.11% | 0.90% | 101% | 98% |
| CE7-1.2c | -0.03% | 0.04% | -0.05% | 99% | 98% | -0.07% | -0.10% | -0.06% | 101% | 100% | 0.08% | 0.07% | 0.08% | 100% | 100% |
| CE7-1.2c* | -0.01% | 0.05% | -0.07% | 101% | 99% | 0.00% | -0.11% | 0.10% | 101% | 100% | -0.08% | 0.33% | 0.25% | 100% | 98% |
| CE7-1.2d | -0.05% | -0.05% | -0.15% | 99% | 99% | -0.03% | -0.11% | -0.04% | 101% | 100% | -0.15% | -0.03% | 0.15% | 100% | 98% |
| CE7-1.2d* | -0.06% | -0.04% | -0.14% | 101% | 98% | -0.10% | -0.08% | -0.15% | 101% | 101% | 0.05% | -0.12% | 0.29% | 100% | 98% |
| CE7-1.3a | 0.00% | -0.06% | -0.01% | 102% | 101% | 0.02% | 0.01% | -0.05% | 101% | 100% | 0.08% | 0.05% | 0.54% | 101% | 102% |
| CE7-1.3b | 0.00% | -0.07% | -0.16% | 102% | 101% | -0.04% | 0.02% | 0.01% | 101% | 100% | 0.07% | 0.13% | 0.55% | 102% | 103% |
| CE7-1.3b* | 0.00% | -0.05% | -0.02% | 102% | 101% | 0.01% | -0.04% | 0.14% | 101% | 101% | 0.10% | 0.26% | 0.54% | 102% | 103% |
| CE7-1.3c | 0.00% | -0.03% | -0.09% | 101% | 101% | -0.06% | 0.04% | 0.01% | 101% | 100% | 0.05% | -0.11% | 0.30% | 101% | 102% |
| TGM | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 | -0.04% | -0.03% | -0.05% | 100% | 99% | -0.04% | -0.10% | -0.07% | 100% | 98% | -0.04% | 0.00% | 0.07% | 101% | 100% |
| CE7-1.2a | 0.01% | 0.06% | 0.05% | 101% | 99% | 0.02% | 0.02% | 0.01% | 102% | 100% | -0.02% | -0.02% | 0.06% | 99% | 101% |
| CE7-1.2b | -0.05% | -0.05% | -0.05% | 101% | 98% | -0.09% | -0.03% | -0.09% | 102% | 99% | -0.04% | -0.02% | 0.04% | 98% | 102% |
| CE7-1.2c | -0.04% | 0.02% | 0.00% | 99% | 100% | -0.07% | -0.05% | -0.05% | 102% | 98% | -0.06% | -0.02% | -0.05% | 99% | 98% |
| CE7-1.2c* | 0.06% | 0.12% | 0.09% | 101% | 97% | 0.00% | 0.02% | 0.05% | 102% | 100% | -0.02% | 0.01% | 0.03% | 99% | 100% |
| CE7-1.2d | -0.15% | -0.11% | -0.12% | 99% | 100% | -0.10% | -0.07% | -0.12% | 101% | 99% | -0.10% | -0.06% | -0.04% | 99% | 100% |
| CE7-1.2d* | -0.12% | -0.11% | -0.11% | 101% | 100% | -0.09% | -0.11% | -0.09% | 102% | 101% | -0.06% | -0.01% | -0.04% | 98% | 102% |
| CE7-1.3a | -0.04% | -0.03% | -0.03% | 101% | 101% | -0.05% | -0.07% | -0.08% | 101% | 100% | -0.04% | -0.08% | -0.03% | 102% | 100% |
| CE7-1.3b | -0.02% | 0.02% | 0.01% | 101% | 101% | -0.04% | -0.06% | -0.02% | 101% | 100% | -0.06% | -0.01% | 0.03% | 103% | 104% |
| CE7-1.3b* | 0.04% | 0.08% | 0.05% | 102% | 101% | -0.03% | -0.01% | 0.03% | 101% | 101% | -0.09% | -0.07% | -0.05% | 103% | 101% |
| CE7-1.3c | -0.02% | -0.01% | 0.01% | 101% | 101% | -0.04% | -0.04% | -0.04% | 101% | 100% | -0.11% | -0.09% | -0.01% | 102% | 100% |

Table 2:  Average test results for CE7-1 and low QP (2, 7, 12, 17).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-1.2a | 0.00% | 0.00% | 0.01% | 100% | 100% |  |  |  |  |  | -0.01% | -0.01% | -0.01% | 100% | 99% |
| CE7-1.2b | 0.00% | 0.00% | 0.00% | 99% | 98% |  |  |  |  |  | 0.00% | 0.01% | -0.01% | 99% | 98% |
| CE7-1.2c | -0.01% | -0.01% | 0.00% | 99% | 98% |  |  |  |  |  | -0.01% | 0.00% | -0.01% | 99% | 98% |
| CE7-1.2c* | -0.03% | -0.03% | -0.01% | 99% | 98% |  |  |  |  |  | -0.03% | 0.00% | -0.01% | 99% | 99% |
| CE7-1.2d | -0.02% | -0.01% | 0.00% | 99% | 98% |  |  |  |  |  | -0.01% | 0.00% | -0.01% | 99% | 98% |
| CE7-1.2d* | -0.03% | -0.02% | -0.02% | 100% | 98% |  |  |  |  |  | -0.01% | -0.01% | -0.03% | 99% | 98% |
| CE7-1.3a | 0.00% | 0.00% | 0.00% | 100% | 101% | -0.01% | -0.02% | 0.00% | 100% | 100% | -0.01% | 0.01% | -0.01% | 101% | 102% |
| CE7-1.3b | -0.01% | -0.02% | -0.01% | 100% | 98% | -0.01% | -0.01% | 0.00% | 100% | 100% | -0.02% | -0.01% | 0.00% | 100% | 98% |
| CE7-1.3b* | -0.02% | -0.02% | 0.00% | 99% | 98% | -0.01% | -0.01% | -0.01% | 100% | 100% | -0.02% | 0.01% | -0.03% | 100% | 98% |
| CE7-1.3c | -0.01% | -0.02% | -0.01% | 99% | 99% | -0.01% | -0.01% | 0.00% | 100% | 100% | -0.02% | 0.01% | 0.01% | 100% | 98% |
| Class F | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-1.2a | -0.10% | -0.07% | -0.06% | 100% | 99% | -0.11% | -0.16% | -0.13% | 99% | 100% | -0.16% | -0.19% | -0.17% | 100% | 97% |
| CE7-1.2b | -0.06% | -0.08% | -0.07% | 99% | 96% | -0.13% | -0.07% | -0.01% | 98% | 99% | -0.13% | -0.04% | -0.04% | 99% | 97% |
| CE7-1.2c | -1.02% | -0.54% | -0.49% | 98% | 95% | -0.99% | -0.63% | -0.50% | 98% | 98% | -0.48% | -0.41% | -0.38% | 98% | 96% |
| CE7-1.2c* | -1.47% | -0.65% | -0.64% | 99% | 94% | -1.83% | -0.60% | -0.54% | 98% | 97% | -0.67% | -0.47% | -0.51% | 99% | 98% |
| CE7-1.2d | -1.33% | -0.71% | -0.71% | 97% | 94% | -1.28% | -0.78% | -0.80% | 98% | 97% | -0.50% | -0.41% | -0.54% | 98% | 98% |
| CE7-1.2d* | -1.56% | -0.78% | -0.80% | 99% | 95% | -2.00% | -0.78% | -0.67% | 98% | 97% | -0.68% | -0.62% | -0.57% | 99% | 97% |
| CE7-1.3a | -0.04% | -0.04% | -0.06% | 100% | 100% | -0.08% | -0.11% | -0.10% | 100% | 99% | -0.15% | -0.20% | -0.13% | 101% | 100% |
| CE7-1.3b | -0.68% | -0.33% | -0.34% | 99% | 96% | -0.73% | -0.41% | -0.38% | 99% | 99% | -0.43% | -0.32% | -0.29% | 100% | 91% |
| CE7-1.3b* | -1.21% | -0.50% | -0.51% | 99% | 96% | -1.51% | -0.57% | -0.67% | 100% | 99% | -0.49% | -0.28% | -0.27% | 100% | 91% |
| CE7-1.3c | -0.68% | -0.32% | -0.33% | 99% | 96% | -0.81% | -0.51% | -0.50% | 99% | 100% | -0.28% | -0.28% | -0.26% | 100% | 91% |
| TGM | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-1.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-1.2a | -0.31% | -0.19% | -0.19% | 99% | 102% | -0.20% | -0.21% | -0.26% | 100% | 100% | -0.30% | -0.20% | -0.16% | 100% | 98% |
| CE7-1.2b | -0.20% | -0.18% | -0.18% | 99% | 99% | -0.27% | -0.16% | -0.16% | 98% | 96% | -0.14% | -0.18% | -0.08% | 99% | 100% |
| CE7-1.2c | -1.82% | -1.47% | -1.47% | 97% | 98% | -2.26% | -1.29% | -1.28% | 98% | 96% | -1.33% | -1.06% | -1.00% | 97% | 99% |
| CE7-1.2c* | -1.53% | -1.15% | -1.14% | 99% | 99% | -2.15% | -0.97% | -0.98% | 98% | 97% | -1.05% | -0.68% | -0.62% | 98% | 99% |
| CE7-1.2d | -2.03% | -1.67% | -1.66% | 97% | 98% | -2.56% | -1.32% | -1.30% | 98% | 97% | -1.33% | -1.17% | -1.15% | 97% | 99% |
| CE7-1.2d* | -1.97% | -1.61% | -1.61% | 98% | 99% | -2.63% | -1.31% | -1.35% | 98% | 97% | -1.31% | -1.01% | -0.94% | 98% | 98% |
| CE7-1.3a | -0.20% | -0.15% | -0.15% | 100% | 99% | -0.17% | -0.17% | -0.18% | 100% | 99% | -0.20% | -0.11% | -0.05% | 101% | 98% |
| CE7-1.3b | -1.27% | -0.95% | -0.96% | 98% | 96% | -1.50% | -0.83% | -0.84% | 99% | 99% | -0.87% | -0.72% | -0.59% | 100% | 86% |
| CE7-1.3b* | -1.41% | -1.03% | -1.02% | 98% | 96% | -2.02% | -0.90% | -0.90% | 100% | 99% | -1.00% | -0.57% | -0.47% | 100% | 85% |
| CE7-1.3c | -1.25% | -0.93% | -0.93% | 98% | 96% | -1.43% | -0.78% | -0.82% | 100% | 99% | -0.96% | -0.70% | -0.64% | 100% | 85% |

Table 3: Bin-to-bit ratio analysis for CE7-1 and Common Test Conditions (CTC).

(no data available)

Table 4: Bin-to-bit ratio analysis for CE7-1 and low QP (2, 7, 12, 17).

(no data available)

## CE7-2:  Context modelling and binarization for transform skip residual coding

CE7-2: Summary of tests

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-2.1 | S.-T. Hsiang (Mediatek) | CE7-1.1 + Improved context modelling for abs_gtx_flags and par_level_flag (JVET-O0295) | M. Sarwer (Alibaba) |
| CE7-2.2a | Y.-H. Chao (Qualcomm) | QP dependent binarization (JVET-O0559) | Y. Chen (InterDigital) |
| CE7-2.2b | Y.-H. Chao (Qualcomm) | QP-dependent binarization + varying QP offset (JVET-O0559) | Y. Chen (InterDigital) |

Remarks:

- The starred versions “*” in the result tables represent alternatives for the corresponding CE tests.

Table 5:  Average test results for CE7-2 and Common Test Conditions (CTC).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 | 0.00% | -0.02% | -0.01% | 102% | 101% | 0.00% | 0.02% | 0.07% | 103% | 99% | -0.06% | 0.19% | -0.13% | 101% | 100% |
| CE7-2.2a | 0.00% | -0.01% | 0.00% | 101% | 101% | 0.01% | 0.06% | 0.06% | 101% | 100% | -0.05% | 0.26% | 0.18% | 100% | 99% |
| CE7-2.2a* | 0.00% | -0.01% | 0.01% | 101% | 101% | 0.00% | -0.01% | 0.04% | 101% | 100% | -0.05% | 0.16% | 0.01% | 100% | 99% |
| CE7-2.2b | 0.00% | 0.00% | -0.01% | 100% | 99% | 0.00% | -0.05% | 0.04% | 99% | 100% | 0.08% | 0.05% | -0.05% | 99% | 100% |
| CE7-2.2b* | 0.00% | 0.00% | -0.02% | 100% | 98% | -0.01% | -0.07% | -0.03% | 100% | 99% | 0.01% | 0.12% | 0.14% | 99% | 101% |
| Class F | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 | -0.10% | -0.05% | -0.14% | 100% | 97% | -0.08% | -0.11% | -0.06% | 101% | 106% | -0.07% | -0.06% | 0.45% | 100% | 102% |
| CE7-2.2a | -0.03% | -0.18% | -0.23% | 102% | 99% | -0.22% | -0.20% | -0.22% | 103% | 100% | -0.13% | 0.12% | 0.40% | 102% | 97% |
| CE7-2.2a* | -0.08% | -0.23% | -0.19% | 102% | 102% | -0.32% | -0.29% | -0.14% | 102% | 100% | -0.07% | 0.28% | 0.32% | 101% | 100% |
| CE7-2.2b | -0.14% | -0.28% | -0.05% | 101% | 98% | -0.28% | -0.23% | -0.31% | 100% | 100% | -0.13% | -0.20% | -0.58% | 100% | 101% |
| CE7-2.2b* | -0.20% | -0.24% | -0.07% | 101% | 99% | -0.36% | -0.26% | -0.36% | 99% | 98% | -0.24% | -0.38% | -0.21% | 99% | 96% |
| TGM | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 | -0.19% | -0.16% | -0.14% | 101% | 100% | -0.14% | -0.16% | -0.16% | 102% | 93% | -0.17% | -0.07% | -0.08% | 100% | 100% |
| CE7-2.2a | -0.01% | -0.09% | -0.08% | 102% | 103% | -0.11% | -0.14% | -0.13% | 103% | 100% | -0.09% | 0.01% | -0.05% | 98% | 101% |
| CE7-2.2a* | -0.15% | -0.22% | -0.24% | 102% | 100% | -0.24% | -0.21% | -0.26% | 103% | 100% | -0.18% | -0.12% | 0.03% | 99% | 100% |
| CE7-2.2b | -0.26% | -0.24% | -0.26% | 102% | 101% | -0.30% | -0.31% | -0.33% | 100% | 100% | -0.14% | -0.14% | -0.10% | 100% | 99% |
| CE7-2.2b* | -0.37% | -0.41% | -0.40% | 101% | 98% | -0.39% | -0.40% | -0.46% | 100% | 99% | -0.25% | -0.19% | -0.10% | 101% | 99% |

Table 6:  Average test results for CE7-2 and low QP (2, 7, 12, 17).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-2.2a | 0.07% | 0.02% | 0.03% | 100% | 99% |  |  |  |  |  | 0.05% | -0.03% | -0.04% | 100% | 99% |
| CE7-2.2a* | 0.03% | 0.01% | 0.01% | 99% | 98% |  |  |  |  |  | 0.03% | -0.01% | -0.01% | 99% | 98% |
| CE7-2.2b | 0.08% | 0.03% | 0.02% | 100% | 99% |  |  |  |  |  | 0.04% | 0.00% | -0.01% | 100% | 100% |
| CE7-2.2b* | 0.03% | 0.01% | 0.01% | 100% | 98% |  |  |  |  |  | 0.02% | 0.00% | -0.01% | 99% | 98% |
| Class F | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-2.2a | -1.01% | -1.57% | -1.60% | 101% | 96% | -1.04% | -1.71% | -1.65% | 100% | 101% | -1.06% | -1.00% | -1.05% | 101% | 100% |
| CE7-2.2a* | -2.06% | -1.57% | -1.54% | 99% | 96% | -1.76% | -1.68% | -1.55% | 99% | 99% | -1.13% | -1.05% | -1.06% | 99% | 98% |
| CE7-2.2b | -1.46% | -1.57% | -1.53% | 101% | 98% | -1.64% | -1.67% | -1.71% | 101% | 102% | -1.10% | -1.10% | -1.18% | 100% | 102% |
| CE7-2.2b* | -1.72% | -1.61% | -1.57% | 99% | 97% | -1.92% | -1.74% | -1.77% | 100% | 100% | -1.04% | -1.08% | -1.13% | 99% | 100% |
| TGM | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-2.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE7-2.2a | -5.60% | -4.88% | -4.86% | 101% | 98% | -5.28% | -3.92% | -3.94% | 101% | 99% | -3.56% | -2.89% | -2.85% | 102% | 101% |
| CE7-2.2a* | -5.43% | -4.74% | -4.74% | 100% | 97% | -5.36% | -3.67% | -3.66% | 100% | 97% | -3.41% | -2.94% | -2.90% | 100% | 99% |
| CE7-2.2b | -5.69% | -4.96% | -4.97% | 102% | 97% | -4.62% | -3.90% | -3.88% | 102% | 100% | -3.51% | -3.10% | -3.14% | 101% | 99% |
| CE7-2.2b* | -5.51% | -4.82% | -4.80% | 101% | 94% | -4.49% | -3.74% | -3.72% | 100% | 97% | -3.30% | -2.93% | -2.94% | 101% | 102% |

Table 7: Bin-to-bit ratio analysis for CE7-2 and Common Test Conditions (CTC).

(no data available)

Table 8: Bin-to-bit ratio analysis for CE7-2 and low QP (2, 7, 12, 17).

(no data available)

## CE7-3:  Context reduction for regular transform coefficient coding

CE7-3: Summary of tests

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-3.1a | S.-T. Hsiang (Mediatek) | Context reduction for sig_coeff_flag (JVET-O0928) on 6 context subsets | C. Auyeung (Tencent) |
| CE7-3.1b | S.-T. Hsiang (Mediatek) | Context reduction for sig_coeff_flag (JVET-O0928) on 9 context subsets | M. Sarwer (Alibaba) |
| CE7-3.2 | F. Le Léannec (InterDigital) | Context reduction by reducing the template for chroma blocks (JVET-O1098) | Y.-H. Chao (Qualcomm) |
| CE7-3.3a | S.-T. Hsiang (Mediatek), F. Le Léannec (InterDigital) | Combination of CE7-3.1a and CE7-3.2 | Y.-H. Chao (Qualcomm) |
| CE7-3.3b | S.-T. Hsiang (Mediatek), F. Le Léannec (InterDigital) | Combination of CE7-3.1b and CE7-3.2 | Y.-H. Chao (Qualcomm) |

Remarks:

- For the starred versions “*” in the result tables, the CABAC initialization tables have been retrained (despite of that nothing was modified).

Table 9:  Average test results for CE7-3 and Common Test Conditions (CTC).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-3.1a | 0.01% | -0.04% | 0.00% | 102% | 103% | 0.02% | 0.02% | 0.07% | 103% | 100% | -0.05% | 0.43% | -0.25% | 102% | 101% |
| CE7-3.1b | 0.02% | -0.05% | -0.03% | 103% | 102% | 0.01% | 0.02% | 0.06% | 103% | 100% | 0.00% | 0.05% | 0.10% | 101% | 99% |
| CE7-3.2 | 0.01% | 0.17% | 0.21% | 103% | 103% | 0.02% | 0.22% | 0.20% | 99% | 103% | -0.02% | 0.17% | 0.03% | 106% | 108% |
| CE7-3.2* | 0.00% | 0.26% | 0.24% | 103% | 104% | 0.01% | 0.32% | 0.26% | 99% | 103% | -0.03% | 0.39% | 0.26% | 100% | 100% |
| CE7-3.3a | 0.02% | 0.18% | 0.14% | 105% | 104% | 0.00% | 0.19% | 0.21% | 101% | 102% | -0.03% | 0.30% | 0.15% | 101% | 101% |
| CE7-3.3a* | 0.01% | 0.22% | 0.27% | 101% | 101% |  |  |  |  |  |  |  |  |  |  |
| CE7-3.3b | 0.03% | 0.12% | 0.16% | 104% | 104% | 0.01% | 0.15% | 0.16% | 101% | 103% | -0.02% | 0.12% | 0.09% | 103% | 102% |
| CE7-3.3b* | 0.02% | 0.22% | 0.25% | 102% | 101% | 0.00% | 0.30% | 0.29% | 101% | 103% |  |  |  |  |  |

Table 10:  Average test results for CE7-3 and low QP (2, 7, 12, 17).

| Overall | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-3.1a | 0.01% | -0.01% | -0.01% | 101% | 100% | 0.00% | -0.03% | -0.03% | 102% | 100% | 0.01% | -0.01% | -0.01% | 99% | 98% |
| CE7-3.1b | 0.02% | -0.01% | 0.01% | 102% | 102% | 0.04% | -0.09% | -0.06% | 102% | 101% | 0.03% | -0.09% | -0.02% | 99% | 97% |
| CE7-3.2 | 0.09% | 0.59% | 0.61% | 96% | 99% | 0.04% | 0.25% | 0.23% | 98% | 98% | 0.02% | 0.16% | 0.18% | 102% | 102% |
| CE7-3.2* | 0.08% | 0.58% | 0.62% | 99% | 101% |  |  |  |  |  | 0.02% | 0.23% | 0.18% | 93% | 92% |
| CE7-3.3a | 0.10% | 0.59% | 0.61% | 99% | 100% | 0.05% | 0.23% | 0.23% | 104% | 101% | 0.03% | 0.17% | 0.17% | 94% | 94% |
| CE7-3.3a* | 0.08% | 0.60% | 0.63% | 101% | 103% |  |  |  |  |  |  |  |  |  |  |
| CE7-3.3b | 0.12% | 0.55% | 0.61% | 101% | 103% | 0.08% | 0.17% | 0.19% | 104% | 102% | 0.06% | 0.08% | 0.13% | 93% | 94% |
| CE7-3.3b* | 0.10% | 0.57% | 0.63% | 100% | 103% |  |  |  |  |  |  |  |  |  |  |

## CE7-4:  Chroma Cbf Coding

CE7-4: Summary of tests

| CE7-4.1 | S. Esenlik (Huawei) | Coding of chroma cbf’s and joint chroma residual coding flag (JVET-O0231) | C. Helmrich (HHI) |
| --- | --- | --- | --- |

Remarks:

- For the starred versions “*” in the result tables, the Lagrange parameter was modified (no changes on normative aspects).

Table 11:  Average test results for CE7-4, Common Test Conditions (CTC) and low QP (2, 7, 12, 13).

| CTC | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-4.1 | 0.08% | -0.67% | -0.48% | 103% | 103% | 0.14% | -1.15% | -1.39% | 103% | 102% | 0.03% | -0.41% | -2.08% | 103% | 102% |
| CE7-4.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| lowQP | All Intra (AI) |  |  |  |  | Random Access (RA) |  |  |  |  | Low Delay B (LB) |  |  |  |  |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE7-4.1 | 0.04% | 0.03% | -0.01% | 101% | 99% |  |  |  |  |  | 0.05% | 0.22% | -0.40% | 101% | 99% |
| CE7-4.1* |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 12:  Average test results for CE7-4 and HDR.

(no data available)

# Crosscheck Reports

The following tables summarize the crosscheck reports.

Table 13: Crosscheck reports for CE7-1.

| CE7-1: Coding order and bypass switch for transform skip residual coding |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE7-1.1 (M. Sarwer) | results: code verification: further comments: none |
| CE7-1.2a (T. Nguyen) | results: Full runs for CTC; all results match; similar encoding/decoding times. code verification: Careful checking of the provided source code; modifies coeff predictor (insignificant coeff untouched), modified context inits, implementation matches description, i.e., two, pass coding with the first pass is all context coded bins further comments: none |
| CE7-1.2b (T. Nguyen) | results: Full runs for CTC; all results match; similar encoding/decoding times. code verification: Careful checking of the provided source code; only parity syntax element is moved by the implementation as described further comments: none |
| CE7-1.2c (T. Nguyen) | results: Full runs for CTC; all results match; similar encoding/decoding times (same for alternative version). code verification: Careful checking of the provided source code; rice parameter derivation modified, additional template evaluation for rice code, matches the description Alternative version: smooth transition, matches description further comments: none |
| CE7-1.2d (T. Nguyen) | results: Full runs for CTC; all results match; similar encoding/decoding times (same for alternative version). code verification: Careful checking of the provided source code; same as 7-1.2c but for three passes, matches description Alternative version: same as 7-1.2c but for three passes, matches description further comments: none |
| CE7-1.3a (S.-T. Hsiang) | results: All results for CTC were verified; the results for lowQP were partially verified. code verification: tbd further comments: none |
| CE7-1.3b (S.-T. Hsiang) | results: All results for CTC were verified; the results for lowQP were partially verified. code verification: tbd further comments: none |
| CE7-1.3c (S.-T. Hsiang) | results: All results for CTC were verified; the results for lowQP were partially verified. code verification: tbd further comments: none |

Table 14: Crosscheck reports for CE7-2.

| CE7-2: Context modelling and binarization for transform skip residual coding |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE7-2.1 (M. Sarwer) | results: code verification: further comments: none |
| CE7-2.2a (Y. Chen) | results: code verification: further comments: none |
| CE7-2.2b (Y. Chen) | results: code verification: further comments: none |

Table 15: Crosscheck reports for CE7-3.

| CE7-3: Context reduction for regular transform coefficient coding |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE7-3.1a (C. Auyeung) | results: Simulations of the cross check are not completed yet. The partial results match those of the tester, except for one bitstream “Foodmarket”, RA, QP7. Here the following difference was observed: ΔR=1210.985kbit, ΔPSNR(Y,Cb,Cr)=-0.005,-0.0078,-0.0018 The crosschecker measured no BD-rate saving for this sequence at lowQp, while the tester reported a BD rate of -0.18%. code verification: Source code matched the description. further comments: Observed miss-matches in EncT and DecT by up to 3% in EncT and 4% in DecT. - Proponent reported increased in AI, RA, LB EncT by 102%, 103%, 102% for standard QP, and 101%, 102%, 99% for low QPs. - Cross-check timing for AI, RA, LB EncT is 100%, 100%, 100% for standard QP, and 101%, xxx%, 101% for low QPs. - Proponent reported increased in AI, RA, LB DecT by 103%, 100%, 101% for standard QP, and 100%, 100%, 98% for low QPs. - Cross-check timing for AI, RA, LB DecT is 99%, 100%, 98% for standard QP, and 100%, xxx%, 100% for low QP |
| CE7-3.1b (M. Sarwer) | results: code verification: further comments: none |
| CE7-3.2 (Y.-H. Chao) | results: code verification: further comments: none |
| CE7-3.3a (Y.-H. Chao) | results: code verification: further comments: none |
| CE7-3.3b (Y.-H. Chao) | results: code verification: further comments: none |

Table 16: Crosscheck reports for CE7-4.

| CE7-4: Chroma cbf coding |  |
| --- | --- |
| test (crosschecker) | crosscheck report |
| CE7-4.1 (C. Helmrich) | results: So far, SDR CTC and low-QP CTC simulations have been performed, with encoding run-times close to 100% (even closer to 100% than those of the proponent). These simulation results match those of the proponent. For SDR CTC, CE7-4.1 causes BD-rate shifts from luma to chroma. To assess the amount of chroma BD-rate gain present when compensating for any BD-rate losses in luma, the Intra and Inter quantization-related Lagrange constants (lambdas) were retuned and another simulation was run for SDR CTC. For this simulation the following average results are obtained (detailed results are contained in the attached Excel document for the cross check): AI: 0.02%, -0.10%, 0.02% (Y, Cb, Cr) RA: 0.02%, -0.33%, -0.74% (Y, Cb, Cr) LB: 0.02%, 0.42%, -1.35% (Y, Cb, Cr) HDR CTC simulations are in progress. code verification: The proponent's software was studied and found to be consistent with the CE description and syntax definition. Overall, CE7-4.1 requires around 10 additional lines in both bit-stream reading source code and syntax text (~2 lines replaced with ~12 lines), plus some additional descriptive text. further comments: none |

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Geert Van der Auwera <geertv@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> Yung-Hsuan Chao <yunghsua@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> Benjamin Bross <benjamin.bross@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> Xiaoyu Xiu <xiaoyuxiu@kwai.com> Tsung-Chuan Ma <tsung-chuanma@kwai.com> Jhuhong Jheng <jhuhong-jheng@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| MediaKind | Julien Le Tanou <julien.letanou@mediakind.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |
| InterDigital | Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> Fabien Racape <fabien.racape@interdigital.com> Ya Chen <Ya.chen@interdigital.com> Edouard Francois <Edouard.francois@interdigital.com> Philippe de Lagrange <Philippe.delagrange@interdigital.com> Le Léannec Fabrice <Fabrice.Leleannec@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Jonathan Gan <jonathan.gan@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Yusuke Kato <kato.yusuke003@jp.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> Jane Zhao <jie.zhao@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Xin Zhao <xinzzhao@tencent.com> Xiang Li <xlxiangli@tencent.com> Cheung Auyeung <cauyeung@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |
| NHK | Shunsuke Iwamura <iwamura.s-gc@nhk.or.jp > |
| Tongji University | Tao Lin <lintao@tongji.edu.cn> |
| Alibaba | Jie Chen <jiechen.cj@alibaba-inc.com> Ru-Ling Liao <ruling.lrl@alibaba-inc.com> Mohammed Golam Sarwer <m.sarwer@alibaba-inc.com> |
| Apple | Krishna Rapaka <krapaka@apple.com> |

# List of Input Contributions

| document | authors | title |
| --- | --- | --- |
| JVET-P0046 | S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-1.1 and CE7-2.1: Improving transform skip residual coding |
| JVET-P0047 | S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-3.1: Context reduction for sig_coeff_flag |
| JVET-P0049 | S. Esenlik, B. Wang, A. M. Kotra, H. Gao, E. Alshina (Huawei) | CE7-4.1: Coding of chroma cbf flags |
| JVET-P0066 | Y. Chen, F. Le Léannec, T. Poirier, F. Galpin (InterDigital) | CE7-3.2: Context reduction by reduced usage of local neighborhood |
| JVET-P0067 | Y. Chen, F. Le Léannec, T. Poirier, F. Galpin (Intedigital), S.-T. Hsiang, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-3.3: Context reduction for sig_coeff_flag, par_level_flag, abs_level_gtx_flag |
| JVET-P0072 | M. G. Sarwer, R. -L. Liao, J. Luo, Y. Ye (Alibaba) | CE7-1.3: Simplification of transform-skip residual coding |
| JVET-P0076 | M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm) | CE7-2.2: QP dependent binarization in TS residual coding |
| JVET-P0079 | M. Karczewicz, H. Wang, Y.-H. Chao, M. Coban (Qualcomm) | CE7: Modifications to transform-skip residual coding (CE7-1.2) |
| CE related contributions |  |  |
| JVET-P0042 | B. Heng, M. Zhou (Broadcom) | Non-CE7: A cleanup for inter_pred_idc coding |
| JVET-P0050 | M. Zhou (Broadcom) | AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0 |
| JVET-P0167 | O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek) | CE7-related: Improved coding of user defined quantization matrices |

# Test Conditions

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-6.0 software (outcome of the Gothenburg meeting).  Results have to be reported for the VTM-6.0 software and the common test conditions (standard configuration files).

All tools are tested against the VTM-6.0 anchor (same set of tools, except the tool to be tested).

Results have to be provided for two ranges of QP values:

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

For category 1 and 2 tests (transform skip residual coding), additional results for screen coding content sequences (class TGM with YUV 4:2:0 format) have to be provided (using the same configuration as for class F CTC).

For category 4 tests (related to joint chroma coding), additional HDR results have to be provided (for HDR common test conditions). For HDR, only tests for the standard QP set (22, 27, 32, 37) are required (visual testing is not required).

## Category 1 and 2 test bin to bit ratio measurements

Bin to bit ratio measurements for context coded as well as bypass coded bins of transform coefficient coding have to be provided for Category 1 and 2 tests using VTM-6.0 decoder SW with

#define RExt__DECODER_DEBUG_BIT_STATISTICS                1

setting. The following stats have to be collected (JVET-N0049).

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

JVET-N0049 SW’s maximum context coded bins for transform coefficient syntax elements as well as overall peak context coded bins rate feature will be integrated into VTM-6.0 decoder SW. The stats for the following syntax elements for transform coding have to be used for reporting transform coefficient coding stats.

- Context-coded bins: sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag

- Bypass-coded bins: abs_remainder, dec_abs_level

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results. For Category 4 tests with HDR, proponents have to provide results for the HDR metrics specified in JVET-N1011.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

# Detailed Description of Test

## Subblock processing flow and context modelling (JVET-O0295)

In CE7-1.1, the syntax elements abs_level_gtx_flag[n][j], j = 1..4, is signalled in a single pass. The syntax par_level_flag is signaled in a separate pass after signalling abs_level_gtx[n].

In CE7-2.1, abs_level_gtx_flag is entropy coded conditioned on the two neighbouring level values. par_level_flag is entropy coded conditioned on the partially reconstructed absolute level value of the current coefficient. This test is conducted on top of CE7-1.1.

## Context reduction for sig_coeff_flag (JVET-O0928)

In the proposed tests, the number of the context variables in a context subset associated with a TB zone is reduced to 2 for some context subsets. The 1st context variable is selected when locSumAbsPass1 is equal to 0. The 2nd context variable is selected, otherwise. Context reduction is applied to 6 context subsets in CE7-3.1 and to 9 context subset in CE7-3.1b, as illustarted in Figure 1.

Figure 1: Illustration of the tests for context reduction for sig_coeff_flag.

## Context reduction by reduced usage of local neighbourhood (JVET-O1098)

This CE test proposes to remove the usage of the local neighbourhood for the context derivation for syntax elements par_level_flag and abs_level_gtx_flag in some regions of the chroma transform blocks (TB).

As shown in Table 1, the local neighbourhood is replaced by a single CABAC context for each of these bins, in each chroma TB region.

Table 1: proposed context model index of par_level_flag and abs_level_gtx_flag

| Chroma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 2 |
| d>0 | 1 |
| Last non-zero coefficient | 0 |

Therefore, the index of the contexts used for coding par_level_flag and abs_level_gtx_flag is determined as follows:

ctxInc = (diag = = 0 ? 1 : 0)

where diag is determined as follows based on the scan location (xC,yC) of the considered coefficient:

diag=xC+yC .

As a consequence, the sum of partially reconstructed coefficient in the local neighbourhood of current coefficient, denoted locSumAbsPass1 in VVC draft specification, needs no more being computed for chroma TBs. Additionally, 24 contexts are removed compared to the current VVC draft specification with proposed method.

Figure 2: proposed modification in the context derivation for par_level_flag and abs_level_gtx_flag in chroma TBs

## Modified chroma coded block flag signalling (JVET-O0231)

The current contribution proposes an alternative way of coding the coded block flags for Cb and Cr blocks. Based on the statistical observation, a first flag (tu_cbf_cb_or_cr) indicating whether Cb or Cr  blocks have residual, and a second flag (tu_cbf_cb_and_cr) indicating that both Cb and Cr blocks have residual are included in the syntax. The joint CbCr residual flag will be signalled when tu_cbf_cb_or_cr flag is true.

According to the proposal, the following is the modified syntax table applicable to VVC draft 6:

| tu_cbf_cb_or_cr[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| if( tu_cbf_cb_or_cr[ x0 ][ y0 ] ) { |  |
| if( sps_joint_cbcr_enabled_flag ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA) |  |
| tu_cbf_cb_and_cr[ x0 ][ y0 ] | ae(v) |
| if( !tu_cbf_cb_and_cr[ x0 ][ y0 ] ) |  |
| tu_cbf_cb [ x0 ][ y0 ] | ae(v) |

tu_joint_cbcr_residual and tu_cbf_cb_and_cr flags are inferred to be equal to zero and tu_cbf_cb_or_cr, respectively, when not present in the bitstream. The difference between the modified syntax and the original proposal is as follows:

For non-intra CU: No difference. The tu_cbf_cb_and_cr is present in bitstream only if tu_joint_cbcr_residual is false. tu_joint_cbcr_residual indicates that both Cb and Cr blocks have residual and coded jointly.

For intra CU: tu_cbf_cb_and_cr is present in bitstream even if tu_joint_cbcr_residual is true, due to the adoption of JVET-O0105.

## Interleaved coefficient coding, parity flag re-ordering and bypass binarization in TS residual coding (JVET-O0623)

This contribution proposes to reduce the number of passes when encoding transform-skip coefficients as well as re-positioning of the parity flag and different binarization after switch to bypass coding. The following 3 aspects are proposed and tested.

- 2 scan passes: In the first pass, sig flag, sign flag, gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded. In the second pass, the remainder is coded.  If the context bin limit is reached in the first pass, all the syntax elements will be bypass coded.

- Re-positioning of parity flag: encode/parse parity flag after gtx flags.

- Binarization after switch to bypass coding:

  - For 2 pass scan: sig flag, sign flag, gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded in the first pass. If no available regular coded bins are left before coding of gt1 flag at CoefficientX, all remaining sig and sign flags are bypass coded. Starting from this coefficient (coefficient position >= CoefficientX position), gt par, gt3, gt7, gt9 flags are not coded (Case 1). If no available regular coded bins are left at CoefficientY after coding of any of gt1, par, gt3, gt5, gt7 or gt9 flags, all remaining flags after reaching the regular bin count limit are bypass coded for CoefficientY (Case2). Following this coefficient (coefficient position > CoefficientY position) all sig and sign flags are bypass coded and gt1, par, gt3, gt5, gt7, gt9 flags are not coded. Rice Golomb coding is applied to to coding of all remainder part of all coefficient levels if the gt9 flag is set to 1 or for all significant coefficients at position >= CoefficientX position in bypass (BP) switching (Case 1) above and for all significant coefficients at position > CoefficientY position in BP switching (Case 2). Rice Golomb coding applies to coding of all remainder part of all coefficient levels. The Rice Golomb parameter derivation for all significant coefficients at position >= Coefficient X position in BP switching (Case 1) above and for all significant coefficients at position > CoefficientY position in BP switching (Case 2) uses the table auiGoRicePars[32]={1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3}. Alternative table/derivation for Rice parameter will be investigated.

  - For 3 pass scan: sig flag, sign flag are coded in the first pass, and gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded in the second pass. If the regular bin count is reached after coding of a gt1 flag, the remaining present par, gtX flags for that coefficient are bypass coded. After the coefficient where the regular bin count is reached, the gt1, par, gtx (gt3, gt5, gt7, gt9) flags will not be coded. In the third pass, for the coefficient with gt9 equal to 1, the remainder is coded. For significant coefficients after (including) reaching the regular bin count, abs(coeff) – 1 will be encoded with Rice-Golomb code.

Subtests:

- 2 scan passes

- Re-positioning the parity flag

- Binarization after bypass switch for 2 pass scan

- Binarization after bypass switch for 3 pass scan

## QP dependent binarization in TS residual coding (JVET-O0559)

In this proposal, a new binarization method for transform skip residual coding is proposed, where the binarization of absolute coefficient levels is adjusted based on the dynamic range of the quantized residuals. In addition, a variable QPoffset is introduced which modifies QP to be QP – QPoffset. The maximum dynamic range (which depends on the QP value), is divided into m intervals. The following binarization is used: The syntax elements sig_coeff_flag, abs_level_gt1_flag and coeff_sign_flag are transmitted. When abs_level_gt1_flag is equal to 1, the interval index and the remainder inside the interval is transmitted.

Subsets:

- QP dependent binarization

- Varying QP offset across TS coded blocks for anchor and proposed QP dependent binarization (delta QP offset signaling for both anchor and proposed binarization). Note: this is to demonstrate the performance of proposed binarization.

## Modified coding order and bypass switch for transform skip residual coding (JVET-O0619)

In the test 1, the position of parity flag is moved to second pass and the position of the greater than 3 flag is moved to the first pass.

- Pass 1: following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - greater than 3 flag (abs_level_gtx_flag[1])

- Pass 2: following flags are signalled

  - parity (par_level_flag) flag

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding of remaining absolute level (abs_remainder) using Golomb-Rice coding

In order to unify the by-pass coding process of transform and TS residual coding, test 2 proposes to modify the by-pass coding method of TS residual coding.

Below is the pseudo code:

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 4; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

iFirstPassBypassPos = n;

Pass 2: for(n = 0; n < iFirstPassBypassPos && remainingCtxBin >= 4; n++ )

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] // gt5

decode abs_level_gtx_flag[3] // gt7

decode abs_level_gtx_flag[4] // gt9

iSecondPassBypassPos = n;

Pass 3 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

baseLevel = (n >= iSecondPassBypassPos ? 2 : 10);

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

Pass 3 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

baseLevel = 0

decode_bypass_sig_flag

if(decode_bypass_sig_flag) {

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

}

The Rice parameter is derived based on locSumAbs and baseLevel.

locSumAbs  += (10 – baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)

As an alternative method, in the pass 3(b) the coefficient coding without sending separate decode_bypass_sig_flag will be also investigated.  The alternative approach of Rice parameter derivation process and look-up table will be also investigated in this CE.