| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0041 |
| --- | --- |

| Title: | CE6-1: LFNST with one mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Jani Lainema Nokia Technologies Tampere, Finland | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com first.lastname@nokia.com |
| Source: | MediaTek Inc., Nokia |  |  |

_____________________________

# Abstract

This document provides results for core experiment CE6-1 which was set up to study effectiveness of different simplifications to LFNST transform selection process. It is asserted that the results would justify adopting the normative change of either CE6-1.1a or CE6-1.1d as both provide around 10 % encoder runtime decrease for around 0.1 % BD-rate impact in the AI configuration.

# Introduction

The following tests were performed as part of the Core Experiment, where tests 6-1.1a and 6-1.1d reflect the original proposals suggested for adoption and remaining tests provide extra information relating to possible encoder side optimizations.

| Test # | Description |
| --- | --- |
| 6-1.1a | LFNST kernel selection according to the method in JVET-O0292 with normative change |
| 6-1.1b | LFNST kernel selection according to the method in JVET-O0292 with encoder-only change |
| 6-1.1c | LFNST kernel selection according to the method in JVET-O0292 with encoder-only change and the last bit used to signal LFNST index is context coded. |
| 6-1.1d | LFNST kernel selection according to the method in JVET-O0350 with normative change |
| 6-1.1e | LFNST kernel selection according to the method in JVET-O0350 with encoder-only change |
| 6-1.1f | LFNST kernel selection according to the method in JVET-O0350 with encoder-only change and the last bit used to signal LFNST index is context coded. |

Both normative 6-1.1a and 6-1.1d suggest signalling only one flag that enables LFNST for a CU. In the case the flag, lfnst_idx, is true, the LFNST index, lfnstIdx, for transform matrix selection is derived from the intra prediction mode as follows:

Method described in JVET-O0292:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

If predModeIntra is smaller than or equal to 34, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 1 : 2.”

Otherwise, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 2 : 1.”

Method described in JVET-O0350:

lfnst_idx 0: LFNST is not used

lfnst_idx 1: The LFNST kernel index lfnstIdx, where lfnstIdx is 1 or 2, is calculated using the variable predModeIntra specifying the intra prediction mode for LFNST set selection as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

# Coding efficiency results

The proposed methods have been implemented on top of VTM6.0. The tests were conducted under the CTC for evaluation. Table 1 to Table 6 show the simulation results of CE6-1.1a to CE6-1.1f, respectively.

Table 1.  Results of CE6-1.1a

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.86% | 1.15% | 90% | 101% |
| Class A2 | 0.04% | 0.70% | 0.79% | 90% | 101% |
| Class B | 0.09% | 0.80% | 0.86% | 90% | 102% |
| Class C | 0.11% | 0.35% | 0.65% | 90% | 100% |
| Class E | 0.13% | 0.61% | 0.66% | 91% | 102% |
| Overall | 0.11% | 0.66% | 0.82% | 90% | 101% |
| Class D | 0.02% | 0.58% | 0.63% | 90% | 101% |
| Class F | 0.12% | 0.25% | 0.32% | 94% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.49% | 0.51% | 96% | 101% |
| Class A2 | 0.06% | 0.50% | 0.40% | 98% | 102% |
| Class B | 0.05% | 0.66% | 0.64% | 97% | 101% |
| Class C | 0.07% | 0.27% | 0.40% | 97% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.49% | 0.50% | 97% | 102% |
| Class D | -0.03% | 0.46% | 0.44% | 98% | 102% |
| Class F | 0.06% | 0.09% | 0.37% | 98% | 105% |

Table 2.  Results of CE6-1.1b

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.81% | 1.39% | 1.54% | 92% | 101% |
| Class A2 | 0.37% | 0.99% | 1.12% | 91% | 100% |
| Class B | 0.42% | 0.95% | 0.85% | 91% | 101% |
| Class C | 0.51% | 0.68% | 1.03% | 92% | 100% |
| Class E | 0.60% | 1.04% | 0.89% | 92% | 102% |
| Overall | 0.53% | 0.99% | 1.06% | 92% | 101% |
| Class D | 0.37% | 0.85% | 0.82% | 92% | 100% |
| Class F | 0.46% | 0.52% | 0.45% | 94% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.52% | 0.58% | 0.79% | 97% | 101% |
| Class A2 | 0.25% | 0.41% | 0.51% | 98% | 101% |
| Class B | 0.22% | 0.84% | 0.88% | 98% | 102% |
| Class C | 0.23% | 0.20% | 0.27% | 98% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.29% | 0.53% | 0.63% | 98% | 103% |
| Class D | 0.12% | 0.38% | 0.54% | 98% | 105% |
| Class F | 0.24% | 0.27% | 0.41% | 98% | 104% |

Table 3.  Results of CE6-1.1c

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.68% | 1.33% | 1.58% | 91% | 102% |
| Class A2 | 0.33% | 0.97% | 1.16% | 91% | 102% |
| Class B | 0.38% | 0.98% | 1.02% | 91% | 102% |
| Class C | 0.50% | 0.69% | 1.04% | 92% | 100% |
| Class E | 0.57% | 1.07% | 0.86% | 92% | 102% |
| Overall | 0.48% | 0.99% | 1.11% | 91% | 101% |
| Class D | 0.35% | 0.81% | 0.84% | 92% | 101% |
| Class F | 0.42% | 0.52% | 0.50% | 94% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.44% | 0.38% | 0.77% | 97% | 101% |
| Class A2 | 0.22% | 0.49% | 0.59% | 98% | 102% |
| Class B | 0.20% | 0.67% | 0.68% | 98% | 102% |
| Class C | 0.27% | 0.39% | 0.24% | 98% | 105% |
| Class E |  |  |  |  |  |
| Overall | 0.27% | 0.50% | 0.56% | 98% | 103% |
| Class D | 0.16% | 0.20% | 0.52% | 99% | 104% |
| Class F | 0.24% | 0.27% | 0.33% | 97% | 106% |

Table 4.  Results of CE6-1.1d

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18 % | 0.80 % | 1.08 % | 90 % | 99 % |
| Class A2 | 0.03 % | 0.61 % | 0.73 % | 89 % | 100 % |
| Class B | 0.08 % | 0.73 % | 0.82 % | 90 % | 100 % |
| Class C | 0.11 % | 0.45 % | 0.73 % | 90 % | 98 % |
| Class E | 0.13 % | 0.65 % | 0.67 % | 91 % | 100 % |
| Overall | 0.11 % | 0.64 % | 0.80 % | 90 % | 99 % |
| Class D | 0.05 % | 0.49 % | 0.63 % | 91 % | 100 % |
| Class F | 0.10 % | 0.29 % | 0.32 % | 94 % | 99 % |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.18 % | 0.34 % | 0.53 % | 97 % | 99 % |
| Class A2 | 0.07 % | 0.47 % | 0.40 % | 98 % | 99 % |
| Class B | 0.04 % | 0.67 % | 0.67 % | 97 % | 100 % |
| Class C | 0.04 % | 0.37 % | 0.30 % | 98 % | 102 % |
| Class E |  |  |  |  |  |
| Overall | 0.07 % | 0.48 % | 0.49 % | 98 % | 100 % |
| Class D | -0.04 % | 0.59 % | 0.39 % | 98 % | 102 % |
| Class F | 0.06 % | 0.10 % | 0.12 % | 98 % | 101 % |

Table 5.  Results of CE6-1.1e

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.82 % | 1.33 % | 1.53 % | 91 % | 98 % |
| Class A2 | 0.38 % | 0.94 % | 1.10 % | 90 % | 98 % |
| Class B | 0.42 % | 0.98 % | 0.94 % | 91 % | 99 % |
| Class C | 0.51 % | 0.71 % | 1.02 % | 91 % | 98 % |
| Class E | 0.62 % | 0.99 % | 1.03 % | 91 % | 103 % |
| Overall | 0.53 % | 0.97 % | 1.10 % | 91 % | 99 % |
| Class D | 0.37 % | 0.81 % | 0.88 % | 91 % | 99 % |
| Class F | 0.46 % | 0.56 % | 0.57 % | 94 % | 97 % |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.49 % | 0.47 % | 0.83 % | 97 % | 100 % |
| Class A2 | 0.25 % | 0.40 % | 0.56 % | 98 % | 100 % |
| Class B | 0.24 % | 0.71 % | 0.64 % | 98 % | 99 % |
| Class C | 0.25 % | 0.44 % | 0.41 % | 98 % | 98 % |
| Class E |  |  |  |  |  |
| Overall | 0.29 % | 0.53 % | 0.60 % | 98 % | 99 % |
| Class D | 0.10 % | 0.23 % | 0.42 % | 98 % | 98 % |
| Class F | 0.24 % | 0.40 % | 0.44 % | 98 % | 99 % |

Table 6.  Results of CE6-1.1f

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.66 % | 1.26 % | 1.51 % | 91 % | 98 % |
| Class A2 | 0.31 % | 0.89 % | 1.08 % | 90 % | 99 % |
| Class B | 0.36 % | 0.96 % | 0.94 % | 91 % | 99 % |
| Class C | 0.47 % | 0.68 % | 0.97 % | 91 % | 97 % |
| Class E | 0.56 % | 1.14 % | 1.01 % | 91 % | 101 % |
| Overall | 0.46 % | 0.97 % | 1.08 % | 91 % | 99 % |
| Class D | 0.32 % | 0.75 % | 0.85 % | 91 % | 99 % |
| Class F | 0.45 % | 0.58 % | 0.57 % | 94 % | 98 % |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.44 % | 0.47 % | 0.76 % | 97 % | 100 % |
| Class A2 | 0.24 % | 0.33 % | 0.50 % | 98 % | 99 % |
| Class B | 0.21 % | 0.78 % | 0.66 % | 98 % | 100 % |
| Class C | 0.24 % | 0.41 % | 0.33 % | 98 % | 101 % |
| Class E |  |  |  |  |  |
| Overall | 0.27 % | 0.53 % | 0.56 % | 98 % | 100 % |
| Class D | 0.08 % | 0.35 % | 0.57 % | 99 % | 103 % |
| Class F | 0.19 % | 0.28 % | 0.45 % | 98 % | 101 % |

# Specification text

## Common for both 6-1.1a and 6-1.1d

7.4.9.5	Coding unit semantics

…

lfnst_idx[ x0 ][ y0 ] specifies whether and which one of the two low frequency non-separable transform kernels in a selected transform set is used. lfnst_idx[ x0 ][ y0 ] equal to 0 specifies that the low frequency non-separable transform is not used. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left sample of the considered transform block relative to the top-left sample of the picture.

When lfnst_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

…

8.7.4	Transformation process for scaled transform coefficients

8.7.4.1	General

…

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, and the intra prediction mode for LFNST set selection predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnst_idx[ xTbY ][ yTbY ] as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

…

8.7.4.2	Low frequency non-separable transformation process

Inputs to this process are:

- a variable nonZeroSize specifying the transform input length,

- a variable nTrS specifying the transform output length,

- a list of scaled non-zero transform coefficients x[ j ] with j = 0..nonZeroSize − 1,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTrS − 1.

The transformation matrix derivation process as specified in clause 8.7.4.3 is invoked with the transform output length nTrS, and the intra prediction mode for LFNST set selection  predModeIntra, and the LFNST index for transform selection in the selected LFNST set lfnstIdx as inputs, and the (nTrS)x(nonZeroSize) LFNST matrix lowFreqTransMatrix as output.

…

Table 9-11 – Syntax elements and associated binarizations

| lfnst_idx[ ][ ] | FLTR | cMax = 1 2, cRiceParam = 0 |
| --- | --- | --- |

…

## Specific for 6-1.1a

8.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set is specified as follows:

If predModeIntra is smaller than or equal to 34, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 1 : 2.”

Otherwise, lfnst_idx is set to be “predModeIntra % 2 == 0 ? 2 : 1.”

…

## Specific for 6-1.1d

8.7.4.3	Low frequency non-separable transformation matrix derivation process

Inputs to this process are:

- a variable nTrS specifying the transform output length,

- a variable predModeIntra specifying the intra prediction mode for LFNST set selection,

- a variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set.

Output of this process is the transformation matrix lowFreqTransMatrix.

The variable lfnstIdx specifying the LFNST index for transform selection in the selected LFNST set is specified as follows:

lfnstIdx = ( predModeIntra % 2 ) + 1

…

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).