| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0043-v1 |
| --- | --- |

| Title: | AHG16/Non-CE5: A clean-up for de-blocking in the affine and TPM mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Minhua Zhou | Tel: Email: | +1 858 521 5838 brian.heng@broadcom.com minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

It is asserted that the current de-blocking design is not consistent for affine mode and TPM mode in some of corner cases. In the affine mode, two 4x4 subblock edges can be treated differently even if they have the exactly same conditions (i.e. having no non-zero coefficients and the same amount of large enough MV differences that can trigger de-blocking along the edge); in the TPM mode, an inner TU edge with no non-zero coefficients can still be de-blocked if the edge is located along the diagonal TPM boundary and the triangle block MV difference is large enough to trigger de-blocking. In order to resolve this inconsistency, it is proposed to separate the de-blocking filtering decision for a TU edge and from that of an MV edge by making the following modifications: 1) using CBFs to make decision only for TU edges; 2) using MV differences to make decisions only for MV edges and 3) if an edge is both a TU edge and an MV edge, de-blocking is on if either 1) or 2) is true. Experimental results revealed that the proposed changes did not change the BD-rate difference in RA and LD_B configurations when compared to VTM6.0.

# Introduction

The affine mode and triangle partition merge (TPM) mode are two new modes added to the VVC. Those two modes do not exist in any of previous MPEG/ITU-T video compression standards. It is asserted that the current de-blocking design for those two modes is not consistent in some of corner cases.

# Problem statement

The following examples will be used to elaborate the de-blocking design inconsistency in the affine mode and in the TPM mode.

Figure 1.  De-blocking filtering example in affine mode

Shown in figure 1 is an example for affine mode. In this example,

- The CU size is 16x32.

- The CU is coded in affine mode. Therefore, there are three inner sub-block boundaries marked 1, 2, 3 in the figure above.

- The CU uses quad SBT, therefore, sub-block boundary 1 is a TU boundary. However, there are no non-zero coefficients in the current component.

- The sub-block MV difference is large enough along the sub-block boundaries to trigger de-blocking.

Deblocking is currently only applied to sub-blocks of an affine-coded PU on an 8x8 basis, so normally only boundary 2 would be considered for MV filtering purposes.

According to the current de-blocking decision process, sub-block boundary 1 is marked as an edge only because it aligns with a TU edge.  And, because it has no coefficients it wouldn’t normally be filtered for TU filtering purposes.  However, since it was already marked as an edge, the MV diff are also checked, and that will cause it to be filtered.

Note that sub-block boundary 3 is not de-blocked.  In this example, sub-block boundary 3 has absolutely the same conditions as sub-block boundary 1, i.e. no non-zero coefficients along the edge, and MV difference is large enough to trigger de-blocking.

Similar behavior is happening in the TPM case along the diagonal TPM boundaries.  Those boundaries that happen to align with TU edges are filtered even if the TU edge has no coefficients, because of MV differences in the triangle block MV field.  And these artificial triangle block MV differences don’t line up with the actual diagonal MV edge anyway, so we believe this is not only inconsistent, but most likely unintentional.

Figure 2.  De-blocking filtering example in TPM mode

A de-blocking example of TPM mode is shown in Figure 2. In this example, the TU size may be smaller than the CU size, which leads to inner TU edges for de-blocking. In the transform units, the grey TUs do not have non-zero coefficients while the black ones have.  In the stored 4x4 vectors, the sub-block motion vector differences along the diagonal boundary are large enough to trigger de-blocking. In the actually filtered edges, the blue edges are filtered because they are either PU edges with large enough MV differences or TU edges with non-zero coefficients, while the orange edges are only filtered because of artificial triangle MV differences along the diagonal boundaries despite non-zero coefficients along those edges.

# Proposed solution

It is proposed to modify the de-blocking decision process as follows:

- Use CBFs to make decision only for TU edges.

- Using MV differences to make decisions only for MV edges.

- If an edge is both a TU edge and an MV edge, de-blocking is on if either (1) or (2) is true.

Here an MV edge here is defined as a PU edge or a sub-block edge which is aligned with the filtering grid.

# Experimental results

Tests were carried out by using the VTM6.0. The CTC (common test conditions) defined in [1] was followed. The summary results are provided in Table 1. Note that the run time may not be accurate.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 92% | 92% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 107% |
| Class B | 0.00% | 0.00% | 0.00% | 91% | 81% |
| Class C | 0.00% | 0.00% | 0.00% | 91% | 90% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 85% |
| Overall | 0.00% | 0.00% | 0.00% | 94% | 89% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 96% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 103% | 86% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.02% | 99% | 96% |
| Class A2 | 0.01% | 0.03% | 0.02% | 94% | 96% |
| Class B | 0.00% | 0.01% | -0.04% | 95% | 87% |
| Class C | 0.00% | 0.00% | 0.00% | 96% | 82% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.01% | 96% | 89% |
| Class D | 0.00% | 0.00% | 0.00% | 89% | 78% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 103% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.04% | -0.18% | 90% | 99% |
| Class C | 0.02% | 0.05% | -0.03% | 92% | 85% |
| Class E | -0.03% | 0.33% | -0.10% | 82% | 76% |
| Overall | 0.00% | 0.12% | -0.11% | 89% | 88% |
| Class D | 0.06% | 0.44% | -0.07% | 74% | 63% |
| Class F (optional) | 0.08% | -0.07% | 0.28% | 101% | 95% |

Table 1. Experimental results

# Discussions

# It is recommended to resolve the above mentioned design inconsistency in the de-blocking of the affine and TPM mode.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH.

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, the 15th JVET meeting, Gothenburg, SE.

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.