| `Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0045-v1 |
| --- | --- |

| Title: | CE5-3.3: Signalling EO signs in SAO |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, CE5-3.3 tests to explicitly signal edge offset (EO) signs in sample adaptive offset (SAO) and to disable SAO are reported. In VTM6.0, when SAO EO mode is selected, the signs of offsets are pre-defined in a way that allows sample smoothing and disallows sample sharpening. In CE5-3.3.1, for each nonzero offset, the sign will be explicitly signalled to allow both sample smoothing and sharpening. In CE5-3.3.2, the results of disabling SAO are reported. BD-rates are summarized as follows.

Anchor: VTM6.0
Test: anchor + signalling EO signs
AI: 0.00% (Y), 0.01% (Cb), 0.02% (Cr)
RA: 0.00% (Y), 0.02% (Cb), 0.00% (Cr)
LB: -0.03% (Y), -0.02% (Cb), 0.07% (Cr)
LP: 0.02% (Y), 0.13% (Cb), 0.29% (Cr)

Anchor: VTM6.0 + disabling ALF
Test: anchor + signalling EO signs
AI: 0.02% (Y), 0.03% (Cb), 0.04% (Cr)
RA: 0.03% (Y), -0.09% (Cb), -0.14% (Cr)
LB: -0.05% (Y), -0.19% (Cb), 0.02% (Cr)
LP: 0.05% (Y), 0.27% (Cb), 0.19% (Cr)

Anchor: VTM6.0
Test: anchor + disabling SAO
AI: 0.01% (Y), 0.14% (Cb), 0.17% (Cr)
RA: 0.08% (Y), 0.28% (Cb), 0.35% (Cr)
LB: 0.07% (Y), 0.58% (Cb), 1.24% (Cr)
LP: 0.14% (Y), 0.41% (Cb), 1.14% (Cr)

Anchor: VTM6.0 + disabling ALF
Test: anchor + disabling SAO
AI: 0.32% (Y), 0.37% (Cb), 0.72% (Cr)
RA: 0.80% (Y), 0.81% (Cb), 0.73% (Cr)
LB: 0.96% (Y), 1.77% (Cb), 2.98% (Cr)
LP: 2.68% (Y), 3.19% (Cb), 3.85% (Cr)

Objective and subjective coding gains of SAO are significantly overlapped with those of ALF. Small coding losses are observed by disabling SAO when ALF is on, but large coding losses are observed by disabling SAO when ALF is off. Given that SAO has very low complexity and is already deployed in more than one billion HEVC devices per year, it is asserted that keeping SAO in VVC is beneficial, especially for real-time low complexity encoding applications. It is additionally observed that signalling EO signs can let encoder have more choices to achieve better visual quality because of supporting not only sample smoothing but also sample sharpening.

# Introduction

In VTM6, for SAO EO mode, only the magnitudes of offsets are signalled and the signs of offsets are pre-defined based on sample classification results. As shown in Fig. 1, there are four EO classes: 1-D 0-degree EO, 1-D 90-degree EO, 1-D 135-degree EO, and 1-D 45-degree EO. For each EO class, two neighbouring samples, a and b, of a current to-be-processed sample are defined and used to perform sample classification of the current sample. The pre-defined sign for each offset is derived according to the sample classification result, edgeIdx, as shown in Fig. 2. If (a>c && b>c), edgeIdx = 0; else if ((a>c && b==c) || (a==c && b>c)), edgeIdx = 1; else if ((a==c && b<c) || (a<c && b==c)), edgeIdx = 3; else if (a<c && b<c), edgeIdx = 4; else edgeIdx = 2. The two edgeIdx values, 0 and 1, indicating the two neighbouring samples are larger than or equal to the current sample, are with positive offsets. The two edgeIdx values, 3 and 4, indicating the two neighbouring samples are smaller than or equal to the current sample, are with negative offsets. The edgeIdx value, 2, indicating none of the above conditions, are with zero offsets. The predefined signs result in sample smoothing but avoids sample sharpening.

Figure 1.  Four EO classes including 1-D 0-degree EO (top left), 1-D 90-degree EO (top right), 1-D 135-degree EO (bottom left), and 1-D 45-degree EO (bottom right), where c represents the current sample and a and b represent the two neighbouring samples that are used for classifying the current sample.

Figure 2.  Implicitly signalled signs for different sample classification results. edgeIdx0 and edgeIdx1 are with positive offsets. edgeIdx3 and edgeIdx4 are with negative offsets.

# Signalling EO signs

In CE5-3.3.1, to allow both sample smoothing and sharpening, for each nonzero offset, the sign is explicitly signalled. The following shows the related VVC specification text modification.

| sao( rx, ry ) { | Descriptor |
| --- | --- |
| if( rx > 0 ) { |  |
| leftCtbInBrick = BrickId[ CtbAddrInBs ] = = BrickId[ CtbAddrRsToBs[ CtbAddrInRs − 1 ] ] |  |
| if( leftCtbInBrick ) |  |
| sao_merge_left_flag | ae(v) |
| } |  |
| if( ry > 0 && !sao_merge_left_flag ) { |  |
| upCtbInBrick = BrickId[ CtbAddrInBs ] = = BrickId[ CtbAddrRsToBs[ CtbAddrInRs − PicWidthInCtbsY ] ] |  |
| if( upCtbInBrick ) |  |
| sao_merge_up_flag | ae(v) |
| } |  |
| if( !sao_merge_up_flag && !sao_merge_left_flag ) |  |
| for( cIdx = 0; cIdx < ( ChromaArrayType != 0 ? 3 : 1 ); cIdx++ ) |  |
| if( ( slice_sao_luma_flag && cIdx = = 0 ) ¦ ¦ ( slice_sao_chroma_flag && cIdx > 0 ) ) { |  |
| if( cIdx = = 0 ) |  |
| sao_type_idx_luma | ae(v) |
| else if( cIdx = = 1 ) |  |
| sao_type_idx_chroma | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] != 0 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| if( SaoTypeIdx[ cIdx ][ rx ][ ry ] = = 1 ) { |  |
| for( i = 0; i < 4; i++ ) |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] | ae(v) |
| sao_band_position[ cIdx ][ rx ][ ry ] | ae(v) |
| } else { |  |
| if( cIdx = = 0 ) |  |
| sao_eo_class_luma | ae(v) |
| if( cIdx = = 1 ) |  |
| sao_eo_class_chroma | ae(v) |
| for( i = 0; i < 4; i++ ) |  |
| if( sao_offset_abs[ cIdx ][ rx ][ ry ][ i ] != 0 ) |  |
| sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] |  |
| } |  |
| } |  |
| } |  |

sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] specifies the sign of the offset value of i-th category for the CTB at the location ( rx, ry ) for the colour component cIdx.

When sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is not present, it is inferred as follows:

- If sao_merge_left_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal to sao_offset_sign[ cIdx ][ rx − 1 ][ ry ][ i ].

- Otherwise, if sao_merge_up_flag is equal to 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal to sao_offset_sign[ cIdx ][ rx ][ ry − 1 ][ i ].

- Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2, the following applies:

- If i is equal to 0 or 1, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal 0.

- Otherwise (i is equal to 2 or 3), sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal 1.

- Otherwise, sao_offset_sign[ cIdx ][ rx ][ ry ][ i ] is inferred to be equal 0.

The list SaoOffsetVal[ cIdx ][ rx ][ ry ][ i ] for i ranging from 0 to 4, inclusive, is derived as follows:

SaoOffsetVal[ cIdx ][ rx ][ ry ][ 0 ] = 0
for( i = 0; i < 4; i++ )
SaoOffsetVal[ cIdx ][ rx ][ ry ][ i + 1 ] = rec_sao_offset_sign *	(7-125)
		sao_offset_abs[ cIdx ][ rx ][ ry ][ i ]

,where rec_sao_offset_sign = ((SaoTypeIdx[ cIdx ][ rx ][ ry ] !=2) || (i>1) ) ? ( 1 − 2 * sao_offset_sign[ cIdx ][ rx ][ ry ][ i ]) : ( 2 * sao_offset_sign[ cIdx ][ rx ][ ry ][ i ]-1)

# Objective results

Signalling EO signs was implemented on VTM6.0 [1]. The simulations were performed following the JVET common test conditions (CTC) [2]. According to CE5 description, tests with ALF off were also performed. Table 1 shows the BD-rates of signalling EO signs under CTC. Table 2 shows the BD-rate results of signalling EO signs under CTC with ALF off. Table 3 shows the BD-rate results of disabling SAO under CTC. Table 4 shows the BD-rate results of disabling SAO under CTC with ALF off. It should be noticed that the run time data are inaccurate.

Table 1.  CE5-3.3.1 results of signalling EO signs under CTC (inaccurate runtime data)

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.04% | 68% | 81% |
| Class A2 | 0.01% | 0.00% | 0.00% | 67% | 83% |
| Class B | 0.01% | 0.00% | 0.03% | 67% | 82% |
| Class C | 0.02% | 0.01% | 0.03% | 66% | 77% |
| Class E | -0.01% | 0.00% | 0.01% | 65% | 78% |
| Overall | 0.00% | 0.01% | 0.02% | 67% | 80% |
| Class D | 0.02% | 0.04% | 0.05% | 66% | 74% |
| Class F | 0.01% | 0.01% | 0.03% | 67% | 77% |
|  |  |  | RA |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.04% | 73% | 81% |
| Class A2 | 0.03% | 0.05% | 0.06% | 73% | 79% |
| Class B | -0.01% | 0.00% | -0.03% | 72% | 76% |
| Class C | 0.00% | 0.02% | 0.04% | 71% | 78% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.00% | 72% | 78% |
| Class D | 0.01% | -0.08% | 0.00% | 70% | 79% |
| Class F | -0.02% | 0.06% | 0.03% | 71% | 81% |
|  |  |  | LB |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | -0.06% | 71% | 75% |
| Class C | 0.00% | 0.04% | -0.07% | 73% | 80% |
| Class E | -0.10% | 0.06% | 0.50% | 74% | 78% |
| Overall | -0.03% | -0.02% | 0.07% | 73% | 77% |
| Class D | 0.04% | -0.09% | 0.59% | 72% | 79% |
| Class F | 0.09% | 0.28% | 0.60% | 72% | 80% |
|  |  |  | LP |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.24% | 0.11% | 72% | 79% |
| Class C | 0.06% | 0.01% | 0.17% | 71% | 79% |
| Class E | 0.00% | 0.10% | 0.75% | 72% | 80% |
| Overall | 0.02% | 0.13% | 0.29% | 72% | 79% |
| Class D | 0.06% | -0.07% | -0.53% | 71% | 77% |
| Class F | 0.00% | 0.09% | 0.15% | 71% | 78% |

Table 2.  CE5-3.3.1 results of signalling EO signs under CTC with ALF off (inaccurate runtime data)

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.04% | 101% | 90% |
| Class A2 | 0.02% | 0.02% | 0.01% | 99% | 90% |
| Class B | 0.03% | 0.02% | 0.00% | 100% | 93% |
| Class C | 0.03% | 0.07% | 0.09% | 100% | 96% |
| Class E | 0.01% | 0.03% | 0.04% | 100% | 92% |
| Overall | 0.02% | 0.03% | 0.04% | 100% | 92% |
| Class D | 0.04% | 0.09% | 0.14% | 98% | 93% |
| Class F | 0.03% | 0.08% | 0.11% | 101% | 92% |
|  |  |  | RA |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.08% | -0.01% | 98% | 87% |
| Class A2 | 0.02% | -0.07% | -0.16% | 100% | 89% |
| Class B | 0.03% | -0.13% | -0.30% | 100% | 88% |
| Class C | 0.04% | -0.18% | -0.02% | 101% | 91% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.09% | -0.14% | 100% | 89% |
| Class D | -0.03% | -0.05% | -0.12% | 100% | 93% |
| Class F | 0.04% | -0.02% | 0.13% | 101% | 84% |
|  |  |  | LB |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.15% | -0.03% | 101% | 89% |
| Class C | 0.03% | -0.20% | 0.15% | 99% | 90% |
| Class E | -0.24% | -0.23% | -0.06% | 101% | 80% |
| Overall | -0.05% | -0.19% | 0.02% | 100% | 87% |
| Class D | 0.09% | 0.20% | 0.66% | 100% | 91% |
| Class F | 0.06% | 0.28% | 0.59% | 101% | 78% |
|  |  |  | LP |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.13% | 0.18% | 101% | 89% |
| Class C | 0.06% | 0.21% | -0.05% | 99% | 89% |
| Class E | 0.00% | 0.56% | 0.51% | 100% | 76% |
| Overall | 0.05% | 0.27% | 0.19% | 100% | 86% |
| Class D | 0.07% | -0.16% | 0.02% | 99% | 89% |
| Class F | 0.06% | 0.16% | 0.93% | 100% | 79% |

Table 3.  CE5-3.3.2 results of disabling SAO under CTC (inaccurate runtime data)

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.20% | 0.10% | 102% | 107% |
| Class A2 | 0.02% | 0.27% | 0.17% | 102% | 105% |
| Class B | 0.02% | 0.05% | 0.16% | 102% | 105% |
| Class C | -0.02% | 0.28% | 0.40% | 100% | 101% |
| Class E | 0.03% | -0.09% | -0.05% | 99% | 104% |
| Overall | 0.01% | 0.14% | 0.17% | 101% | 104% |
| Class D | 0.00% | 0.17% | 0.21% | 99% | 98% |
| Class F | 0.36% | 0.36% | 0.56% | 98% | 100% |
|  |  |  | RA |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.15% | 0.17% | 104% | 112% |
| Class A2 | 0.22% | 0.38% | 0.31% | 103% | 110% |
| Class B | 0.09% | 0.30% | 0.51% | 102% | 106% |
| Class C | -0.02% | 0.27% | 0.33% | 99% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.28% | 0.35% | 102% | 108% |
| Class D | -0.16% | -0.03% | 0.06% | 98% | 113% |
| Class F | 0.51% | 0.32% | 0.45% | 99% | 122% |
|  |  |  | LB |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.81% | 1.60% | 99% | 106% |
| Class C | -0.02% | 0.45% | 0.94% | 103% | 110% |
| Class E | 0.02% | 0.35% | 1.04% | 101% | 121% |
| Overall | 0.07% | 0.58% | 1.24% | 101% | 111% |
| Class D | -0.11% | 0.02% | 0.26% | 100% | 110% |
| Class F | 0.74% | 0.44% | 1.55% | 99% | 121% |
|  |  |  | LP |  |  |
|  |  |  | VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.72% | 1.86% | 101% | 111% |
| Class C | 0.08% | 0.53% | 1.07% | 101% | 107% |
| Class E | 0.18% | -0.26% | 0.04% | 98% | 123% |
| Overall | 0.14% | 0.41% | 1.14% | 100% | 113% |
| Class D | -0.19% | -0.13% | 0.23% | 100% | 109% |
| Class F | 0.68% | 0.29% | 1.19% | 100% | 120% |

Table 4.  CE5-3.3.2 results of disabling SAO under CTC with ALF off (inaccurate runtime data)

|  |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.60% | 1.92% | 100% | 87% |
| Class A2 | 0.49% | 0.56% | 0.44% | 100% | 88% |
| Class B | 0.29% | 0.21% | 0.42% | 99% | 89% |
| Class C | 0.30% | 0.59% | 0.94% | 100% | 95% |
| Class E | 0.39% | -0.05% | 0.01% | 101% | 92% |
| Overall | 0.32% | 0.37% | 0.72% | 100% | 90% |
| Class D | 0.21% | 0.30% | 0.46% | 99% | 94% |
| Class F | 0.98% | 0.79% | 1.10% | 100% | 90% |
|  |  |  | RA |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | 0.47% | 0.54% | 99% | 100% |
| Class A2 | 1.42% | 1.30% | 0.92% | 100% | 97% |
| Class B | 1.02% | 0.90% | 0.70% | 100% | 98% |
| Class C | 0.40% | 0.58% | 0.76% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.80% | 0.81% | 0.73% | 100% | 98% |
| Class D | -0.05% | -0.09% | -0.07% | 100% | 100% |
| Class F | 1.30% | 0.91% | 1.34% | 101% | 101% |
|  |  |  | LB |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.00% | 1.99% | 2.75% | 102% | 96% |
| Class C | 0.98% | 2.63% | 4.83% | 100% | 97% |
| Class E | 0.88% | 0.26% | 0.92% | 102% | 103% |
| Overall | 0.96% | 1.77% | 2.98% | 101% | 98% |
| Class D | 0.15% | 1.33% | 1.39% | 101% | 98% |
| Class F | 2.56% | 3.53% | 4.04% | 100% | 96% |
|  |  |  | LP |  |  |
|  |  |  | VTM6.0 wo ALF |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3.29% | 3.59% | 3.71% | 99% | 98% |
| Class C | 2.13% | 3.51% | 5.18% | 102% | 99% |
| Class E | 2.42% | 2.08% | 2.28% | 99% | 94% |
| Overall | 2.68% | 3.19% | 3.85% | 100% | 97% |
| Class D | 0.90% | 1.20% | 1.48% | 99% | 97% |
| Class F | 2.71% | 3.61% | 4.13% | 103% | 98% |

# Subjective evaluation

Examples of subjective improvements by signalling EO signs are found when ALF is off. In Fig. 3, Fig. 4, and Fig. 5, the CTUs that SAO EO signs are different from the predefined signs are marked in red. The anchor is VTM6.0 with ALF off, and the test is anchor plus signalling EO signs. It is observed that signalling EO signs can result in sharper edges than the blurred edges in the anchors.

Test                                    Anchor

Figure 3.  BasketballDrive, QP=37, LDP, FrameNum=460; anchor: VTM6.0 with ALF off; test: anchor + signalling EO signs

Test                             Anchor

Figure 4.  Cactus, QP=37, LDP, FrameNum=294; anchor: VTM6.0 with ALF off; test: anchor + signalling EO signs

Test                                       Anchor

Figure 5.  BQTerrace, QP=37, LDP, FrameNum=247; anchor: VTM6.0 with ALF off; test: anchor + signalling EO signs

Examples of subjective improvements by SAO are found when ALF is off. In Fig. 6, Fig. 7, and Fig. 8, the CTUs that apply SAO are marked in red. The anchor is VTM6.0 with ALF off, and the test is anchor with SAO off. It is observed that enabling SAO (i.e., anchor) can result in better visual quality than disabling SAO in general.

Test                              Anchor

Figure 6.  BasketballDrive, QP=37, LDP, FrameNum=88; anchor: VTM6.0 with ALF off; test: anchor + SAO off

Test

Anchor

Figure 7.  Cactus, QP=32, LDP, FrameNum=60; anchor: VTM6.0 with ALF off; test: anchor + SAO off

Test                                     Anchor

Figure 8.  BQTerrace, QP=37, LDP, FrameNum=228; anchor: VTM6.0 with ALF off; test: anchor + SAO off

# References

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-M1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).