| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0047-v1 |
| --- | --- |

| Title: | CE7-3.1: Context reduction for sig_coeff_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of CE7-3.1 for reducing the number of the context variables for entropy coding the syntax element sig_coeff_flag. In the proposed tests, the numbers of the context variables of the context subsets for entropy coding sig_coeff_flag associated with dependent quantization states 2 and 3 are further reduced. In CE7-3.1a, the proposed method reportedly provides 0.01%, 0.02%, and -0.05% luma BD-rates for the AI, RA, and LB settings, respectively, versus the VTM6.0 anchor under the common test conditions (CTCs) while achieving reduction of context variables by 12 or 30% of the total number of context variables associated with dependent quantization states 2 and 3. In CE7-3.1b, the proposed method reportedly provides 0.02%, 0.01%, and 0.00% luma BD-rates for the AI, RA, and LB settings, respectively, versus the VTM6.0 anchor under the CTCs while achieving reduction of context variables by 18 or 45% of the total number of context variables associated with dependent quantization states 2 and 3.

# Introduction

In VVC Draft 6 [1], the total number of the context variables for entropy coding the syntax element sig_coeff_flag is significantly increased by allocating three separate sets of context variables for entropy coding transform coefficient levels corresponding to different dependent quantization states, indicated by the variable QState. This contribution reports the results of CE7-3.1 on the method proposed in [2] for reducing the number of the context variables for entropy coding sig_coeff_flag associated with dependent quantization states 2 and 3.

# CE description

In the proposed tests, the number of the context variables in the context sets associated with dependent quantization states 2 and 3 are reduced. It is motivated by the observation that the context variables associated with locSumAbsPass1 > 0 in a context subset corresponding to the same transform block (TB) zone often exhibit similar probability distribution, where the variable locSumAbsPass1 is the sum of absolute values of partialy reconstructed coefficient levels from a local template.

In the proposed method, the number of the context variables is reduced to 2 from 4 for some context subsets. The 1st context variable is selected when the reconstructed variable locSumAbsPass1 is equal to 0. The 2nd context variable is selected, otherwise. Context reduction is applied to 6 context subsets in CE7-3.1a and to 9 context subsets in CE7-3.1b, respectively, as illustrated in Figure 1, where each luma block (in black) and each chroma block (in green) are divided into three zones and two zones, respectively, and separate context sets may be employed for different dependent qantization states (indicated by QState).

Figure 1: Illustration of the tests for context reduction for coding the syntax element sig_coeff_flag in a transform block, where the number of the context variables is reduced from 4 to 2 for some context subsets.

# Experimental results

The proposed method was integrated into the VTM6.0 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM6.0 reference software under the CE7 test conditions [4]. Tables 1 and 2 summarize the experimental results for CE7-3.1a and CE7-3.1b, respectively, conducted under the CTCs over standard QPs 22, 27, 32, and 37. Tables 3 and 4 summarize the experimental results for CE7-3.1a and CE7-3.1b, respectively, conducted over low QPs 2, 7, 12, and 17. Please note the run time results may not be accurate in the tables. The detailed simulation results can be found in the attached spreadsheets.

Table 1: Summary of the average BD bitrate results for CE7-3.1a over standard QPs 22, 27, 32, and 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 103% | 103% |
| Class A2 | 0.02% | 0.01% | -0.04% | 104% | 106% |
| Class B | 0.01% | -0.07% | -0.05% | 103% | 104% |
| Class C | 0.01% | -0.09% | 0.03% | 101% | 101% |
| Class E | 0.01% | -0.03% | 0.04% | 100% | 100% |
| Overall | 0.01% | -0.04% | 0.00% | 102% | 103% |
| Class D | 0.01% | -0.14% | 0.06% | 101% | 99% |
| Class F | -0.01% | -0.09% | -0.04% | 98% | 98% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.15% | 0.04% | 105% | 99% |
| Class A2 | 0.04% | 0.00% | 0.04% | 104% | 101% |
| Class B | 0.00% | 0.09% | 0.12% | 102% | 100% |
| Class C | 0.04% | 0.08% | 0.07% | 99% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.02% | 0.07% | 103% | 100% |
| Class D | -0.08% | -0.25% | 0.00% | 100% | 105% |
| Class F | -0.04% | -0.06% | -0.01% | 100% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.13% | -0.57% | 100% | 98% |
| Class C | -0.04% | 0.07% | -0.20% | 103% | 104% |
| Class E | -0.17% | 1.42% | 0.21% | 102% | 101% |
| Overall | -0.05% | 0.43% | -0.25% | 102% | 101% |
| Class D | 0.02% | -0.31% | 0.24% | 100% | 104% |
| Class F | 0.03% | 0.10% | 1.13% | 98% | 102% |

Table 2: Summary of the average BD bitrate results for CE7-3.1b over standard QPs 22, 27, 32, and 37

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | 0.04% | 105% | 104% |
| Class A2 | 0.04% | -0.09% | -0.08% | 105% | 105% |
| Class B | 0.02% | -0.05% | -0.06% | 103% | 103% |
| Class C | 0.01% | -0.06% | -0.01% | 102% | 101% |
| Class E | 0.01% | 0.06% | -0.06% | 99% | 98% |
| Overall | 0.02% | -0.05% | -0.03% | 103% | 102% |
| Class D | 0.01% | -0.03% | 0.13% | 101% | 97% |
| Class F | 0.01% | -0.06% | -0.04% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.00% | 104% | 100% |
| Class A2 | 0.05% | 0.02% | 0.04% | 104% | 97% |
| Class B | 0.00% | 0.05% | 0.12% | 104% | 99% |
| Class C | 0.01% | 0.03% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.06% | 103% | 100% |
| Class D | -0.08% | -0.20% | -0.22% | 101% | 107% |
| Class F | -0.01% | -0.06% | -0.10% | 101% | 108% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.05% | -0.24% | 100% | 98% |
| Class C | 0.01% | 0.06% | -0.06% | 102% | 101% |
| Class E | -0.03% | 0.23% | 0.89% | 101% | 101% |
| Overall | 0.00% | 0.05% | 0.10% | 101% | 99% |
| Class D | 0.06% | -0.25% | 0.74% | 102% | 105% |
| Class F | 0.09% | 0.09% | 0.65% | 100% | 103% |

Table 3: Summary of the average BD bitrate results for CE7-3.1a over low QPs 2, 7, 12, and 17

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 98% | 94% |
| Class A2 | 0.00% | -0.01% | 0.00% | 99% | 97% |
| Class B | 0.01% | -0.02% | -0.01% | 101% | 102% |
| Class C | 0.01% | 0.00% | 0.00% | 101% | 102% |
| Class E | 0.01% | -0.04% | -0.03% | 105% | 106% |
| Overall | 0.01% | -0.01% | -0.01% | 101% | 100% |
| Class D | -0.01% | -0.01% | 0.05% | 101% | 101% |
| Class F | 0.02% | -0.04% | -0.03% | 103% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.08% | -0.08% | 103% | 100% |
| Class A2 | 0.01% | 0.00% | 0.00% | 102% | 102% |
| Class B | 0.01% | 0.00% | -0.02% | 102% | 100% |
| Class C | 0.01% | -0.04% | -0.04% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.03% | -0.03% | 102% | 100% |
| Class D | 0.02% | -0.10% | 0.02% | 101% | 99% |
| Class F | -0.07% | -0.21% | -0.23% | 100% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | -0.01% | 100% | 98% |
| Class C | 0.01% | -0.02% | -0.03% | 99% | 98% |
| Class E | 0.00% | -0.02% | 0.00% | 98% | 98% |
| Overall | 0.01% | -0.01% | -0.01% | 99% | 98% |
| Class D | 0.01% | 0.00% | 0.00% | 98% | 97% |
| Class F | -0.01% | -0.07% | -0.12% | 103% | 101% |

Table 4: Summary of the average BD bitrate results for CE7-3.1a over low QPs 2, 7, 12, and 17

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | -0.01% | 100% | 98% |
| Class A2 | 0.02% | -0.01% | 0.00% | 101% | 101% |
| Class B | 0.02% | -0.04% | 0.00% | 103% | 102% |
| Class C | 0.02% | 0.00% | 0.04% | 103% | 103% |
| Class E | 0.02% | -0.01% | -0.02% | 104% | 105% |
| Overall | 0.02% | -0.01% | 0.01% | 102% | 102% |
| Class D | 0.01% | -0.04% | 0.01% | 103% | 102% |
| Class F | 0.03% | -0.05% | 0.00% | 102% | 103% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.08% | -0.10% | 103% | 101% |
| Class A2 | 0.03% | -0.09% | -0.07% | 102% | 101% |
| Class B | 0.04% | -0.10% | -0.03% | 102% | 102% |
| Class C | 0.03% | -0.09% | -0.06% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.09% | -0.06% | 102% | 101% |
| Class D | 0.01% | -0.12% | -0.07% | 102% | 101% |
| Class F | -0.04% | -0.23% | -0.20% | 101% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.08% | -0.05% | 101% | 98% |
| Class C | 0.04% | -0.07% | -0.01% | 97% | 95% |
| Class E | 0.03% | -0.11% | 0.00% | 99% | 98% |
| Overall | 0.03% | -0.09% | -0.02% | 99% | 97% |
| Class D | 0.03% | -0.03% | -0.07% | 98% | 98% |
| Class F | 0.10% | -0.05% | -0.03% | 101% | 102% |

# Conclusions

This contribution proposes to reduce the number of context variables in the two additional context sets associated with dependent quantization states 2 and 3. The experimental results reportedly show that the proposed method has negligible impact on BD rate performance while significantly reducing the number of context variables for entropy coding the syntax element sig_coeff_flag.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 6),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2001, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- S.-T. Hsiang and S.-M. Lei, “CE7-related: Context reduction for entropy coding sig_coeff_flag,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O0928, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM6.0

- H. Schwarz and M. Coban, “Description of Core Experiment 7 (CE7): Quantization and coefficient coding,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-O2027, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).