| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0050-v1 |
| --- | --- |

| Title: | AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou 16340 West Bernardo DR San Diego, CA-92127, USA | Tel: Email: | +1 858 521 5838 minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution reports the bin to bit ratio of the VTM6.0 for both the CTC and low QP setting (QP =2, 7, 12, 17).  The summary results are as follows:

- On average for  CTC AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM6.0 is 19.89%/21.70% /17.14% higher than the HM16.19, up from 19.71%/20.88% /16.99%  in the VTM5.0, respectively.

  - the unweighted bin to bit ratio in the VTM6.0 is 12.91%/14.36%/11.57% higher than the HM16.19, down from 14.21%/15.25%/12.88% in the VTM5.0, respectively.

- On average for low-QP AI/RA/LD_B configurations,

  - the weighted bin to bit ratio in the VTM6.0 is 17.71%/15.15%/17.73% higher than the HM16.19, down  from 20.24%/19.00%/19.67% in the VTM5.0, respectively.

  - the unweighted bin to bit ratio in the VTM5.0  is 9.85%/6.87%/8.98% higher than the HM16.19, down from 11.40%/8.39%/10.28% in the VTM5.0, respectively.

As far as low-QP AI configuration is concerned, the weighted and un-weighted bin to bit ratio of the VTM6.0 is roughly 18% and 10% higher than that of VTM16.19, respectively.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1). All the percentage numbers are measured against the HM16.19.

# Introduction

It was reported in JVET-N0049 [3] that the bin to bit ratio of the VTM4.0 is double digits percentage higher than that of the HM16.19, for both the CTC and low QP coding (QP =2, 7, 12, 17).  This contribution reports the bin to bit ratio in VTM6.0 relative to HM16.19.

# Experimental results

Tests were carried out by using the VTM6.0 and HM16.19 reference software. The CTC (common test conditions) defined in [1] was followed. Additional tests were conducted for low QPs of 2, 7, 12 and 17 with a sequence length of 100 frames. The same test sequences and coding parameter settings were used for HM16.19.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements are computed based on the collected data:

- Weighted BD-binrate for the peak case.

- Weighted BD-binrate for the average case.

- Weighted BD-binrate for the transform coefficients coding part.

- Weighted bin to bit ratio for the peak case.

- Unweighted bin to bit ratio for the peak case, for the average case, and for the transform coefficients coding part.

In the weighted measurements, a bypass bin is counted as 0.25 context coded bins. In the unweighted measurements, a bypass bin and a context coded bin carry an equal weight (1:1).

It has been observed that the bin to bit ratio for the peak case, for the average case and for the transform coefficients coding part is roughly the same.

As far as low-QP AI configuration is concerned, the bin to bit ratio of the VTM6.0 is roughly same as that of VTM4.0 (see rows highlighted in yellow in Table 1 and Table 2 below). The weighted and un-weighted bin to bit ratio of the VTM6.0 is roughly 18% and 10% higher than that of VTM16.19, respectively.

| Bin2bit ratio (peak, weighted) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | VTM6.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff (%, VMT6.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | diff* (%, VMT6.0 vs. VTM5.0) | diff* (%, VMT6.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |
| AI | 0.95 | 1.10 | 1.13 | 1.14 | 15.72% | 19.71% | 19.89% | 3.99% | 0.18% | 4.17% |
| RA | 0.93 | 1.09 | 1.12 | 1.13 | 17.39% | 20.88% | 21.70% | 3.49% | 0.82% | 4.31% |
| LDB | 0.96 | 1.09 | 1.13 | 1.13 | 13.34% | 16.99% | 17.14% | 3.64% | 0.16% | 3.80% |
| LDP | 0.96 | 1.09 | 1.13 | 1.13 | 13.24% | 17.30% | 17.21% | 4.06% | -0.09% | 3.97% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |
| AI | 0.79 | 0.94 | 0.95 | 0.93 | 18.11% | 20.24% | 17.71% | 2.12% | -2.53% | -0.40% |
| RA | 0.74 | 0.87 | 0.88 | 0.86 | 17.17% | 19.00% | 15.15% | 1.83% | -3.85% | -2.02% |
| LDB | 0.78 | 0.92 | 0.93 | 0.92 | 17.67% | 19.67% | 17.73% | 2.00% | -1.94% | 0.06% |
| LDP | 0.78 | 0.92 | 0.93 | 0.92 | 17.46% | 19.46% | 17.52% | 2.00% | -1.94% | 0.06% |
| Note*: Percentage difference is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |

Table 1: Weighted bin to bit ratio from VTM4.0 to VTM6.0 (Overall)

| Bin2bit ratio (peak, unweighted) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HM16.19 | VTM4.0 | VTM5.0 | VTM6.0 | diff (%, VMT4.0 vs. HM16.19) | diff (%, VMT5.0 vs. HM16.19) | diff (%, VMT6.0 vs. HM16.19) | diff* (%, VMT5.0 vs. VTM4.0) | diff* (%, VMT6.0 vs. VTM5.0) | diff* (%, VMT6.0 vs. VTM4.0) |
|  | QP = (22, 27, 32, 37), CTC |  |  |  |  |  |  |  |  |  |
| AI | 1.23 | 1.38 | 1.40 | 1.38 | 12.38% | 14.21% | 12.91% | 1.82% | -1.30% | 0.53% |
| RA | 1.21 | 1.38 | 1.40 | 1.39 | 13.75% | 15.25% | 14.36% | 1.50% | -0.89% | 0.61% |
| LDB | 1.24 | 1.37 | 1.39 | 1.38 | 11.25% | 12.88% | 11.57% | 1.63% | -1.30% | 0.32% |
| LDP | 1.23 | 1.37 | 1.39 | 1.38 | 11.08% | 13.02% | 11.49% | 1.94% | -1.53% | 0.42% |
|  | QP = (2, 7, 12, 17), 100 frames |  |  |  |  |  |  |  |  |  |
| AI | 1.19 | 1.31 | 1.33 | 1.31 | 10.45% | 11.40% | 9.85% | 0.95% | -1.55% | -0.60% |
| RA | 1.18 | 1.28 | 1.28 | 1.27 | 7.63% | 8.39% | 6.87% | 0.76% | -1.52% | -0.76% |
| LDB | 1.18 | 1.30 | 1.31 | 1.29 | 9.54% | 10.28% | 8.98% | 0.74% | -1.30% | -0.56% |
| LDP | 1.18 | 1.30 | 1.31 | 1.29 | 9.50% | 10.24% | 8.94% | 0.74% | -1.30% | -0.56% |
| Note*: Percentage difference is measured against HM16.19 |  |  |  |  |  |  |  |  |  |  |

Table 2: Un-weighted bin to bit ratio from VTM4.0 to VTM6.0 (Overall)

The AI/RA/LD_B/LD_P summary results for the VVC common testing conditions (CTC) and the low-QP setting are provided in Table 3, and Table 4, respectively.

Table 3: Summary results for CTC (VTM6.0 vs. HM16.19)

Table 4: Summary results for low QPs (VTM6.0 vs. HM16.19)

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 6)”, JVET-O2001-vE, the 15th JVET meeting, Gothenburg, SE, 3–12 July 2019

[3] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049, the 14th JVET meeting, March 2019, Geneva, CH

[4] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0”, JVET-O0068, the 15th JVET meeting, July 2019, Gothenburg, SE

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.