| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0056-v3 |
| --- | --- |

| Title: | CE3-2: On rounding shift of MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki (Sony) Junyan Huo, Yanzhuo Ma, Fuzheng Yang (Xidian Univ.) Shuai Wan (NPU) Yuanfang Yu (OPPO) | Tel: Email: | kenji.kondo@sony.com, Masaru.Ikeda@sony.com, jyhuo@mail.xidian.edu.cn, yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn teruhiko.s@sony.com, swan@nwpu.edu.cn yuyuanfang@oppo.com |
| Source: | Sony Corporation, Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd. |  |  |

_____________________________

- Abstract

This contribution reports the test results for tests, CE3-2.1 and CE3-2.2. In these tests, a technique is tested to simplify an adaptive rounding shift used in a matrix-based intra prediction, and it removes a table that get the number of shifts in VVC specification text. The implementation was uploaded to GitlLab server, and simulations were carried out using test conditions of CE3. For CE3-2.1, the simulation results are reported that BD-rate Y, U and V are 0.03%, -0.01% and 0.02% for AI, 0.03%, -0.01% and 0.02% for RA. For CE3-2.2, the results are 0.03%, 0.01% and 0.00% for AI, 0.02%, 0.08% and 0.01% for RA. For encoding and decoding, the runtimes are almost same as anchor.

Introduction

This contribution tests a simplification as described in CE3 description [1]. In the test, it is considered to reduce the table size regarding weighted shift (sW). In the matrix calculation, the predicted sample predMip[ x ][ y ] is computed in the MIP using below equation as shown in JVET-O0925:

oW = ( 1 << ( sW − 1 ) ) – fO * (i = 0inSize - 1p i )

predMip[ x ][ y ] = ( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> sW ) + pTemp[ 0 ]

Where, the variable sW for downshift is found by referring a table. The table sizes are currently derived depending on MipSizeId and modeId of MIP.

To simplify the table size, two methods are tested. One is that the sW is fixed value. The other method is that the sW is derived depending on only MipSizeId.

| Test # | Description |
| --- | --- |
| 3-2.1 | The table of weight shift (sW) of MIP is removed. The variable (sW) is fixed value. It doesn’t depend on both MipSizeId and modeId. |
| 3-2.2 | The table of weight shift (sW) of MIP is simplified. The variable (sW) depends on only MipSizeId. It doesn’t depend on modeId. |

Proposed method

In the latest VVC specification draft [2], the decoding process for MIP is described in 8.4.5.2.1 Matrix-based intra sample prediction. These tests propose to modify the decoding process as shown in following sub-section. To simplify the spec, one table is removed, that is Table 8-5 – “Specification of weight shift sW depending on MipSizeId and modeId” in [2]. The modified text is included in this contribution.

CE3-2.1

In this method, a variable (sW), the number of shifts to compute a variable predMip is fixed to 5. The equations are changed as below. Also, the weighting parameters (mWeight) and the offset factor (fO) are modified because the number of shifts was changed from the original design. Those parameters are shown in the included VVC spec text in this contribution that is in section 8.4.5.2.3 “MIP weight matrix derivation process”.

oW = ( 1 << 4( sW − 1 ) ) −( (fO * (i = 0inSize - 1p i )+1) >> 1)	(8-64)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >> 5sW ) + pTemp[ 0 ]	(8-67)

CE3-2.2

In this method, the variable (sW) depends on MipSizeId. the sW is set to 6 when MipSizeId is equal to 1. Otherwise, it is set to 5. Thus, the equations are changed as below.

oW = ( 1 << ((MipSizeId==1)? 5:4) ) −( (fO * (i = 0inSize - 1p i )+1) >> 1)	(8-64)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i  ) + 
	oW ) >>  ((MipSizeId==1)? 6:5)sW ) + pTemp[ 0 ]	(8-67)

Complexity analysis

Above proposals can remove a memory storage because the table becomes unnecessary. In this section, the removed memory size is calculated. In current MIP design, it has a table as below. In the table, largest number is 7, and it can be expressed with 3 bits. The total number of elements is 34 (=18+10+6). The memory size is 102 bits (=3 bits * 34 elements).

Table 8-5 – Specification of weight shift sW depending on MipSizeId and modeId

|  | modeId |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MipSizeId | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| 0 | 6 | 6 | 6 | 5 | 6 | 5 | 5 | 6 | 5 | 6 | 6 | 6 | 6 | 6 | 5 | 6 | 5 | 5 |
| 1 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 6 | 7 |  |  |  |  |  |  |  |  |
| 2 | 7 | 5 | 6 | 6 | 6 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |

The both CE3-2.1 and CE3-2.2 can remove above table. But, CE3-2.2 needs to add a condition in equation (8-64).Test results

The proposed method has been implemented on VTM-6.0 [3]. For these tests. the CE3 description [1] and JVET common test condition [4] were followed. For the simulations, same platform machines as below in PC cluster were used.

- Encode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i7-7700K 4.2GHz

    - SIMD: AVX2.

  - Memory: 32 GB

- Decode

  - OS: CentOS 6.10

  - Compiler: g++ (GCC) 6.3.1

  - CPU: Intel Core i5-2500K 3.3 GHz

    - SIMD: AVX

  - Memory: 24 GB

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Test results for CE3-2.1

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.05% | 0.07% | 101% | 100% |
| Class A2 | 0.03% | 0.00% | -0.01% | 100% | 100% |
| Class B | 0.03% | -0.02% | -0.02% | 99% | 100% |
| Class C | 0.01% | -0.05% | 0.03% | 101% | 100% |
| Class E | 0.04% | 0.01% | 0.07% | 101% | 100% |
| Overall | 0.03% | -0.01% | 0.02% | 100% | 100% |
| Class D | 0.02% | 0.09% | 0.04% | 100% | 100% |
| Class F | -0.01% | 0.01% | -0.13% | 101% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.01% | 0.09% | 102% | 99% |
| Class A2 | 0.02% | 0.14% | 0.00% | 101% | 99% |
| Class B | 0.01% | 0.06% | -0.05% | 100% | 100% |
| Class C | 0.03% | 0.00% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.05% | 0.01% | 101% | 99% |
| Class D | 0.00% | -0.16% | -0.09% | 99% | 100% |
| Class F | 0.02% | -0.16% | 0.11% | 100% | 100% |

Test results for CE3-2.2.

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.08% | 0.05% | 101% | 100% |
| Class A2 | 0.03% | 0.02% | 0.00% | 101% | 100% |
| Class B | 0.02% | 0.00% | -0.06% | 99% | 100% |
| Class C | 0.02% | -0.07% | 0.02% | 100% | 100% |
| Class E | 0.02% | 0.08% | 0.05% | 100% | 100% |
| Overall | 0.03% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.06% | -0.06% | 100% | 100% |
| Class F | 0.01% | 0.01% | -0.12% | 101% | 100% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.07% | 0.07% | 101% | 101% |
| Class A2 | 0.05% | 0.06% | 0.05% | 100% | 100% |
| Class B | 0.01% | 0.10% | -0.12% | 100% | 100% |
| Class C | -0.01% | 0.10% | 0.10% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.21% | -0.07% | 100% | 100% |
| Class F | 0.01% | 0.13% | 0.05% | 100% | 100% |

Conclusion

This contribution proposes to fix a rounding shift to compute a sample value of MIP. This technique reduces complexity and simplify the VVC text by removing the table. Simulation results show that the BD-rate Y is 0.03% for RA with no impact runtime.

References

- Geert Van der Auwera, Ling Li and Alexey Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding”, Document of Joint Video Experts Team, JVET-O2023.

- Benjamin Bross, Jianle Chen and Shan Liu, “Versatile Video Coding (Draft 6)”, Output document approved by JVET, JVET-O2001-vE

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- JVET common test conditions, JVET-O2010 (It will be uploaded.)

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).