| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0060 |
| --- | --- |

| Title: | CE8: Palette mode with neighboring pixel copy (CE8-1.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Chen Sun, Tsui-Shan Chang, Jian Lou | Tel: Email: | +1-858-349-2818 yuchen.s@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd |  |  |

_____________________________

- Abstract

This document proposed palette mode with neighboring pixel copy (CE8-1.1) and reports the results on top of VTM-6.0. The CE results show that:

For 4:4:4 test sequences,

- when DualITree turned off, the proposed method achieves 1.66%, 1.13%, and 0.58% BD-rate gain in TGM 1080p sequences, and 0.45%, 0.44%, and 0.30% BD-rate gains overall for luma, with 101%, 100% and 100% in encoding time, and 100%, 99% and 98% in decoding time under AI, RA, and LD configurations, respectively.

- when DualITree turned on, the proposed method achieves 1.03%, 0.89%, and 0.55% BD-rate gain in TGM 1080p sequences, and 0.25%, 0.25%, and 0.20% BD-rate gains overall for luma with 100%, 98% and 101% encoding time, and 98%, 97% and 101% in decoding time under AI, RA, and LD configurations, respectively.

CE Description

In test CE8-1.1, the method combining palette mode and neighboring pixel copy is tested. When the palette mode is used for a CU and the number of pixels in the CU is greater than or equal to 32, the decoder will first decode a flag indicating whether neighboring pixel copy is enabled. For CUs in which number of pixels is less than 32, the proposed pixel copy disallowed to increase decoder throughput. If the proposed pixel copy is enabled, the decoder then decodes another direction flag to indicate whether horizontal or vertical intra prediction in used. Finally, the decoder decodes a palette and an index map. A specific index (0) is reserved to indicate the sample is reconstructed from intra prediction. When decoder reconstruct the palette coded block, samples with the index, 0, are reconstructed by the horizontal or vertical nearest samples across CU boundary, and samples with other indices are reconstructed by the colors in the palette. Figure 1 shows an example.

(a) vertical intra prediction combination 		 (b) horizontal intra prediction combination

Figure 1. An example of palette mode and horizontal/vertical intra prediction combination.

Simulation results

The proposed method is evaluated according to the CE8 description [2] for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. The Palette, IBC, HashME, and RDPCM are turned on for all classes. Table 1 shows the results with DualITree turned off while Table 2 shows the results with DualITree turned on.

Table 1: Results of the CE8-1.1 on test sequences 4:4:4, DualITree = 0

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.66% | -1.88% | -1.95% | 102% | 102% |
| TGM 720p | 0.08% | -0.07% | -0.32% | 102% | 100% |
| Animation | 0.06% | 0.13% | 0.18% | 101% | 100% |
| Mixed content | -0.36% | -1.04% | -1.06% | 100% | 97% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Overall | -0.45% | -0.66% | -0.73% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.13% | -1.24% | -1.24% | 99% | 96% |
| TGM 720p | -0.23% | -0.20% | -0.13% | 101% | 101% |
| Animation | -0.28% | -0.25% | -0.21% | 101% | 100% |
| Mixed content | -0.28% | -0.80% | -0.87% | 100% | 100% |
| Camera-Captured content | 0.00% | 0.00% | 0.01% | 100% | 99% |
| Overall | -0.44% | -0.56% | -0.54% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.58% | -0.87% | -0.73% | 101% | 101% |
| TGM 720p | -0.27% | -0.49% | 0.39% | 99% | 97% |
| Animation | -0.34% | -0.32% | -0.16% | 100% | 97% |
| Mixed content | -0.14% | 0.20% | -0.42% | 100% | 98% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Overall | -0.30% | -0.36% | -0.19% | 100% | 98% |

Table 2: Results of the CE8-1.1 on test sequences 4:4:4, DualITree = 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -1.03% | -1.44% | -1.53% | 101% | 99% |
| TGM 720p | 0.03% | 0.07% | 0.15% | 101% | 98% |
| Animation | 0.07% | 0.05% | 0.09% | 98% | 95% |
| Mixed content | -0.05% | -0.06% | -0.01% | 100% | 100% |
| Camera-Captured content | -0.01% | 0.05% | -0.01% | 100% | 99% |
| Overall | -0.25% | -0.34% | -0.33% | 100% | 98% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.89% | -1.44% | -1.20% | 99% | 96% |
| TGM 720p | -0.05% | -0.41% | -0.53% | 99% | 98% |
| Animation | -0.11% | -0.27% | -0.22% | 99% | 99% |
| Mixed content | 0.05% | -0.32% | 0.04% | 98% | 97% |
| Camera-Captured content | -0.02% | 0.01% | 0.08% | 97% | 97% |
| Overall | -0.25% | -0.57% | -0.46% | 98% | 97% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.55% | -0.99% | -0.73% | 102% | 101% |
| TGM 720p | 0.04% | -0.20% | 0.06% | 100% | 99% |
| Animation | -0.24% | -0.25% | -0.32% | 102% | 102% |
| Mixed content | -0.17% | -0.17% | -0.12% | 100% | 101% |
| Camera-Captured content | 0.01% | 0.05% | -0.11% | 101% | 101% |
| Overall | -0.20% | -0.37% | -0.26% | 101% | 101% |

- Reference

- Y.-S. Sun, T.-S. Chang, J. Lou, “CE8: Palette mode with neighboring pixel copy (CE8-2.2)”, JVET-O0059.

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028.

- Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).