| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET- P0063-v2 |
| --- | --- |

| Title: | AHG16: Fix on local dual tree |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao, Haitao Yang #360, Jiangshu Road, Hangzhou 310011, China Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +86-571-28163645 {yin.zhao, haitao.yang} @huawei.com {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | Huawei Technologies Co., Ltd MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes two fixes to the local dual tree technique of JVET-O0050. The first fix normatively disallows BT/TT split that creates 4x4 luma blocks in an Inter coding region, which removes redundant CU-split-related flags (e.g., split_cu_flag) and achieves -0.01% luma BD-rate in RA. The second fix harmonizes JVET-O0050 with other color formats beyond YUV4:2:0, i.e., monochrome, YUV4:2:2 and YUV4:4:4.

In addition, the contribution informatively introduces a way of maintaining luma delta qp at encoder to fit the local dual tree, which was not mentioned in the JVET-O0050.

Fix 1 on split condition

Problem and proposed method

In VVC draft 6, a coding tree node in an Inter coding region is allowed to split into 4x4 coding units (CU), as reported in Spec Ticket 470 (https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/470). However, in this case, neither Intra (including IBC) nor Inter coding mode is allowed for these coding tree according to the spec, because 4x4 blocks cannot use Inter mode and CUs in Inter coding region cannot use Intra mode.

The VTM6 always does not select these splits based on RDO cost, and thus does not produces bitstream including 4x4 Inter CUs, but VTM6 signals redundant split information, e.g., split_cu_flag as 0 for 8x4 node.

Therefore, it is proposed to disallow this kind of splits that introduces 4x4 CUs in Inter coding region, as illustrated in Fig. 1. More specifically, two split allowing conditions shall be added to BT and TT split conditions, respectively. Note the disallowing of TT split for a 16x4/4x16 node only triggers when maxMttDepth is greater than 3.

Fig. 1 Examples of BT and TT splits creating 4x4 CUs inside an Inter coding region

Draft text

Added text is heighted in yellow.

- 6.4.2 Allowed binary split process

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than maxBtSize

- cbHeight is greater than maxBtSize

- mttDepth is greater than or equal to maxMttDepth

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 16

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- cbWidth * cbHeight is equal to 32 and modeType is equal to MODE_TYPE_INTER

- 6.4.3 Allowed ternary split process

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowTtSplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than Min( MaxTbSizeY, maxTtSize )

- cbHeight is greater than Min( MaxTbSizeY, maxTtSize )

- mttDepth is greater than or equal to maxMttDepth

- x0 + cbWidth is greater than pic_width_in_luma_samples

- y0 + cbHeight is greater than pic_height_in_luma_samples

- treeType is equal to DUAL_TREE_CHROMA and ( cbWidth / SubWidthC ) * ( cbHeight / SubHeightC ) is less than or equal to 32

- treeType is equal to DUAL_TREE_CHROMA and modeType is equal to MODE_TYPE_INTRA

- cbWidth * cbHeight is equal to 64 and modeType is equal to MODE_TYPE_INTER

Fix 2 on color formats

Problem and proposed method

JVET-O0050 targeted at solving small intra prediction blocks for YUV4:2:0 format. However, how it applies to other color formats (monochrome, YUV4:2:2 and YUV4:4:4) was not mentioned in JVET-O0050. It was advised by the text editors to clarify this issue in a new contribution.

It is obvious that the JVET-O0050 method does not need for monochrome and YUV4:4:4, since there is no chroma block for monochrome and there is no chroma blocks smaller than 4x4 for YUV4:4:4.

In YUV4:2:2 case, only chroma blocks of 4x2 and 2x4 may appear. If it is desirable to apply JVET-O0050 to YUV4:2:2 format for removing 4x2 and 2x4 chroma intra blocks, the spec text should be revised to further consider color formats in the modeTypeCondition derivation process, as suggested in Section 2.2.

When a coding tree node contains 64 luma pixels (thus 32 chroma pixels in YUV4:2:2) and is split by a BT split, each child node still contains 16 chroma pixels, and modeTypeCondition should be 0.

Similarly, when a coding tree node contains 128 luma pixels (thus 64 chroma pixels in YUV4:2:2) and is split by a BT split, each child node still contains 16 or 32 chroma pixels, and modeTypeCondition should be 0.

Draft text

Added text is heighted in yellow.

The variable modeTypeCondition is derived as follows:

- If one of the following conditions is true, modeTypeCondition is set equal to 0

- slice_type = = I  and  qtbtt_dual_tree_intra_flag is equal to 1

- modeTypeCurr is not equal to MODE_TYPE_ALL

- chroma_format_idc is equal to 0 or 3

- Otherwise, if one of the following conditions is true, modeTypeCondition is set equal to 1

- cbWidth * cbHeight is equal to 64 and split_qt_flag is equal to 1

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- cbWidth * cbHeight is equal to 32 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- Otherwise, if chroma_format_idc is equal to 1 and one of the following conditions is true, modeTypeCondition is set equal to 1 + (slice_type  !=  I ? 1 : 0)

- cbWidth * cbHeight is equal to 64 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_BT_HOR or SPLIT_BT_VER

- cbWidth * cbHeight is equal to 128 and MttSplitMode[ x0 ][ y0 ][ mttDepth ] is equal to SPLIT_TT_HOR or SPLIT_TT_VER

- Otherwise, modeTypeCondition is set equal to 0

In addition, two fixes to the semantics of modeType and treeType are suggested.

- Since palette mode is only used in YUV4:4:4 and the local dual tree is not used in YUV4:4:4, the palette mode in the semantics of modeType should be removed.

- In the semantics of treeType, treeType specifies “whether a single tree or a dual tree is used to partition the CTUs”; it should be corrected as “coding tree node” instead of “CTUs”, because for either local dual tree or 64x64 dual tree in 128x128 CTU case, the dual tree does not start at a CTU, but starts at a coding tree node inside a CTU.

- 6.4.2 Allowed binary split process

Inputs to this process are:

- a binary split mode btSplit,

- …

- a variable treeType specifying whether a single tree (SINGLE_TREE) or a dual tree is used to partition the CTUs coding tree node and, when a dual tree is used, whether the luma (DUAL_TREE_LUMA) or chroma components (DUAL_TREE_CHROMA) are currently processed,

- a variable modeType specifying whether intra (MODE_INTRA), IBC (MODE_IBC), palette (MODE_PLT), and inter coding modes can be used (MODE_TYPE_ALL),  or whether only intra, and IBC, and palette coding modes can be used (MODE_TYPE_INTRA), or whether only inter coding modes can be used (MODE_TYPE_INTER) for coding units inside the coding tree node.

Output of this process is the variable allowBtSplit.

[Note: There are four places of the semantics of modeType and/or treeType that needs update. The related sections are 6.4.1, 6.4.2, 6.4.3, 8.7.1.]

Encoder Luma QP management for local dual tree (informative)

Before the adoption of JVET-O0050, in the single tree case of VTM5, after encoding CUs in a quantization group (QG) with a target QP (which is predQP + deltaQP), the QPs of CUs before the first CU with residual is revised to predQP. In principle, after encoding of a CU in a QG, if the CU does not have residual and all CUs in the QG before the CU do not have residual, the QP of the CU shall be revised to predQP, as at the decoder the QP of the CU is predQP.

In local dual tree case, a quantization group may contains several luma CUs, several chroma CUs and several luma&chroma CUs (i.e., single tree CUs), and the technique of revising QPs of CUs in QG after encoding all CUs in the QG does not work.

For example, consider a QG contains three luma CUs and a chroma CU, as shown in Fig 2. The first two luma CUs do not have residual, and the third luma CU and the chroma CU has residual. After encoding the QG and revising first two no-residual CU’s QP to predQP, the chroma CU still encoded with TargetQP. However, at decoder the chroma CU QP refers to the QP of luma CU1 and thus is still predQP. Therefore, when encoding the chroma CU, if its corresponding luma CU is actually in predQP, the chroma CU should be encoded with predQP (instead of the target QP).

Fig. 2 An example that revising CU QP after encoding a QG leads to decoder QP mismatch in local dual tree case.

Because luma delta QP is not allowed to be signaled at chroma CUs (only allowed at luma CUs and luma&chroma CUs), the principle to maintain encoding QP for local dual tree is as follows:

after encoding of a luma CU or a luma&chroma CU in a QG, if the CU does not have residual and all luma CUs and luma&chroma CUs in the QG before the CU do not have residual, the QP of the CU shall be revised to predQP.

To fix the after-QG-QP-revising scheme in VTM5, two encoder changes were made:

- after encoding of a luma CU in a QG, if the CU does not have residual and all luma CUs and luma&chroma CUs in the QG before the luma CU do not have residual, the QP of the luma CU is revised to predQP.

- after encoding all CUs in a QG, find the first luma CU or luma&chroma CU with residual, and revise QPs of the preceding luma CU and luma&chroma CU in the QG to predQP (i.e., not checking chroma CUs and not changing QP of no-residual chroma CUs).

This method of maintaining encoder luma QP was integrated into in VTM6, by merge request 848, link as follows:

(https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/848).

Experiment results

The proposed method on fix 1 has been implemented on top of VTM6.0, and tested under RA and LDB of the Common Test Conditions (CTC). The coding performance are provide in Table 1.

Table 3 Coding performance of the proposed method

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.05% | -0.01% | 100% | 100% |
| Class B | -0.01% | 0.05% | -0.08% | 100% | 100% |
| Class C | -0.04% | -0.05% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | -0.05% | 100% | 100% |
| Class D | -0.08% | -0.08% | -0.08% | 100% | 101% |
| Class F | -0.02% | -0.03% | -0.03% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | -0.04% | 100% | 101% |
| Class C | -0.06% | -0.01% | 0.03% | 100% | 100% |
| Class E | 0.00% | 0.87% | 0.58% | 100% | 101% |
| Overall | -0.02% | 0.15% | 0.14% | 100% | 101% |
| Class D | 0.00% | 0.36% | 0.43% | 100% | 100% |
| Class F | 0.10% | 0.04% | 0.25% | 99% | 100% |

Conclusions

It is suggested to adopt the proposed fixes to split condition and color format into VVC.

Acknowledgement

We would like to thank Canon for the cross-check.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).