| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0070 |
| --- | --- |

| Title: | CE4-1.10: CIIP with triangular partitions + GEO |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Saverio Blasi, Andre Seixas Dias, Gosala Kulupana 201 Wood Lane, The Lighthouse W12 7TQ, London, UK Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei) | Email: | saverio.blasi@bbc.co.uk andre.seixasdias@bbc.co.uk gosala.kulupana@bbc.co.uk han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com |
| Source: | British Broadcasting Corporation, Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution presents the results of test CE4-1.10. In this test, the usage of triangular partitions in Combined Inter-Intra Prediction (CIIP), tested in CE4-1.9, is combined with the usage of geometrical partitions for merge mode, specifically the configuration tested in CE4-1.2. Average BD-rates of -0.39% (Y), -0.48% (U) and -0.49% (V) are reported for RA with 104% encoding time and 100% decoding time with respect to VTM 6.0. For LD, the reported BD-rates are -0.77% (Y), -0.84% (U) and -0.62% (V) with 106% and 98% encoding and decoding times, respectively.

# Introduction

In the context of core experiment 4, different tests were defined to study the benefits of using geometrical partitions in merge mode and the combination of Intra and merge Inter predictions with triangular partitions. In test CE4-1.10, these two approaches are tested together to evaluate their joint performance.

# Proposed method

Test CE4-1.10 is a straightforward combination of tests CE4-1.9 and CE4-1.2. For the CE4-1.9 part, triangular partitions are allowed in Combined Inter-Intra Prediction (CIIP) [1]. The combination between an Intra and an Inter predictor is performed following the partitioning scheme used in the Triangle mode. The intra predictor is used in one of the two triangular partitions, and the merge inter-predictor is used in the other triangular partition. Three possible Triangle CIIP modes are considered with different triangle split and/or usage of inter or intra prediction in the two partitions. Usage of the specific Triangle CIIP mode is explicitly signalled in the bitstream as an alternative to conventional CIIP.

For the CE4-1.2 part, the flexibility of non-rectangular partitioning of inter-blocks is extended using geometrical partitioning (GEO) [2]. In the adopted configuration, the number of GEO split modes is 64, which constructed from 20 angle indices and 4 distance indices. The angle index φi is quantized between 0 and 360 degrees using fixed steps, near vertical direction angle indices and 45/135 degrees angles indices are removed. The distance index ρi is quantized from a maximal distance value ρmax with fixe steps. The blending operation and motion storage are derived from the parameter angle index φi and distance index ρi. The angle index φi and the distance index ρi.are jointly signaled using merge_geo_idx. The merge_geo_idx is signaled aside of the TPM mode using merge_geo_flag to indicate whether GEO is applied.

In test CE4-1.10, CIIP with triangle partitions and GEO are allowed independently from each other. This means that the combination of an Intra predictor with an Inter predictor for the same block is only allowed using the three different triangle splits defined in test CE4-1.9, in additional to the CIIP mode already in the VVC draft specification. Similarly, GEO only allows the combination of two Inter predictors for the same block with a given geometrical partition. Therefore, since these two tools are operating independently in test CE4-1.10, the signalling of each of these techniques remains the same as described for tests CE4-1.9 and CE4-1.2.

# Performance results

The performance of test CE4-1.10 is summarised in Table 1 and Table 2. The tests were performed using the JVET common test conditions [3] with parallel encoding/decoding used for RA. The software modifications were made on top of VTM 6.0 [4], compiled with gcc 4.8.2. All experiments were run using AVX.

Table 1 - Performance of test CE4-1.10 for RA.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.17% | -0.21% | -0.26% | 103% | 99% |
| Class A2 | -0.22% | -0.22% | -0.20% | 104% | 100% |
| Class B | -0.26% | -0.27% | -0.31% | 104% | 100% |
| Class C | -0.85% | -1.15% | -1.10% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.39% | -0.48% | -0.49% | 104% | 100% |
| Class D | -0.58% | -0.85% | -0.69% | 104% | 99% |
| Class F | -0.35% | -0.40% | -0.34% | 102% | 101% |

Table 2 - Performance of test CE4-1.10 for LD.

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | -0.66% | -0.55% | 106% | 99% |
| Class C | -0.93% | -1.12% | -1.09% | 106% | 98% |
| Class E | -1.08% | -0.78% | -0.10% | 105% | 96% |
| Overall | -0.77% | -0.84% | -0.62% | 106% | 98% |
| Class D | -0.83% | -0.84% | -1.28% | 107% | 101% |
| Class F | -0.45% | -0.66% | 0.74% | 105% | 101% |

# Conclusions

In this contribution, the results of test CE4-1.10 are presented. In test CE4-1.10, tests CE4-1.9 and CE4-1.2 are combined to evaluate the performance of triangular partitions in CIIP in combination with GEO partitions. From the performance results, these techniques do not seem to interfere with each other, as the gains of both techniques seem to be preserved when they are used together.

# References

- S. Blasi, A. Seixas Dias, G. Kulupana, “Non-CE4: CIIP using triangular partitions“, JVET-O0522, Gothenburg, July 2019.

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, Z. Zhao, E. Alshina, M. Bläser, J. Sauer, “Non-CE4: Geometrical partitioning for inter blocks“, JVET-O0489, Gotherburg, July 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

- VTM-6.0 software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git, February 2019

# Patent rights declaration(s)

The British Broadcasting Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).