| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0072-v3 |
| --- | --- |

| Title: | CE7-1.3: Simplification of transform-skip residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Ruling Liao, Daniel Luo, Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1 (408) 858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution reports the results of CE tests of CE7.1.3a, CE7.1.3b and CE7.1.3c. The proposed methods reduce the number of worst case checking of transform skip (TS) residual coding from 8 per coefficient to 2 per coefficient. The proposed method also unifies the syntax structure of a by-pass coding coefficient of TS residual coding and transform residual coding. Following results are reported.

- CE7-1.3a:

  - Overall: 0.00% (AI), -0.01% (RA), -0.02% (LB) luma BD-rate

  - Class SCC: -0.04% (AI), -0.05% (RA), -0.04% (LB) luma BD-rate

- CE7-1.3b:

  - Overall: 0.00% (AI), 0.01% (RA), 0.01% (LB) luma BD-rate

  - Class SCC: -0.02% (AI), -0.04% (RA), -0.06% (LB) luma BD-rate

- CE7-1.3b-alt:

  - Overall: 0.00% (AI), 0.00% (RA), 0.01% (LB) luma BD-rate

  - Class SCC: 0.04% (AI), -0.03% (RA), -0.09% (LB) luma BD-rate

- CE7-1.3c:

  - Overall: 0.00% (AI), 0.00% (RA), -0.06% (LB) luma BD-rate

  - Class SCC: -0.02% (AI), -0.04% (RA), -0.11% (LB) luma BD-rate

# Problem statement

In VVC 6, the transform skip (TS) residual levels of a coefficient group (CG) are coded in three passes over the scan positions as follows:

- Pass 1: Following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - parity (par_level_flag) flag

- Pass 2:

  - greater than 3 flag (abs_level_gtx_flag[1])

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: Rice-Golomb coding of remaining absolute level (abs_remainder)

This contribution solved following issues of VVC 6 TS residual coding.

- In TS residual coding, before coding each flag of each coefficient, it is required to check if the number of context coded bins are available or not. If the context coded bins are available, the flag is context coded. Otherwise, the flag is by-pass coded. Since there are total 8 flags, in worst case scenario, the number of checking is 8 per coefficient, whereas, in regular transform coding, the number of checking is 1 per coefficient.

- The syntax structure of the by-pass coded coefficient of the transform and TS residual coding is different. In transform residual coding, once the context coded bins exceed the limit, the rest of the coefficients are coded using only Rice-Golomb coding. However, in case of TS residual by-pass coding, the remaining flags are firstly by-pass coded followed by Rice-Golomb Coding of the remainder.  For instance, assume the number of context coded bins reached to maximum limit before coding of sig_coeff_flag of a coefficient. For that coefficient, following are the syntax of the transform and TS residual coding of VVC 6.

- VVC6 TS residual coding: sig sign  gt1 parity gt3 gt5 gt7  gt9 abs_remainder

- Transform residual coding: dec_abs_level sign

# Proposed method

## Test CE7-1.3a: Swapping the position of parity and greater than 3 flag

In this test, the position of parity flag is moved to second pass and the position of the greater than 3 flag is moved to the first pass.

- Pass 1: following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - greater than 3 flag (abs_level_gtx_flag[1])

- Pass 2: following flags are signalled

  - parity (par_level_flag) flag

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding of remaining absolute level (abs_remainder) using Golomb-Rice coding

## Test CE7-1.3b and CE7-1.3b-alt

Both CE-7.1.3b and CE-7.1.3b-alt solve following two issues:

- Reduce number of checks: As mentioned in section 1, in worst case scenario, the TS residual coding of VVC 6 checks 8 times per coefficient whether context coded bins are available or not whereas, in regular transform coding, the number of checking is 1 per coefficient.  Both CE-7.1.3b and CE-7.1.3b-alt reduces the number of checks from 8 per coefficient to 2 per coefficient.

- Unified Syntax structure of by-pass coding:  The syntax structure of the proposed method is more unified with transform residual coding. For instance, assume the number of context coded bin reached to maximum limit before coding of sig_coeff_flag of a coefficient. For that coefficient, following are the syntax of the transform and TS residual coding of VVC 6 and CE tests.

- VVC6 TS residual coding: sig sign  gt1 parity gt3 gt5 gt7  gt9 abs_remainder

- CE-7.1.3b TS residual coding: sigabs_remainder sign

- CE-7.1.3b-alt TS residual coding: abs_remainder sign

- Transform residual coding: dec_abs_level sign

### Test CE7-1.3b

Below is the pseudo code of CE7-1.3b. Here pass 1 and pass 2 are always context coded and pass 3 is by-pass coded.

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 4; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

iFirstPassBypassPos = n;

Pass 2: for(n = 0; n < iFirstPassBypassPos && remainingCtxBin >= 4; n++ )

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] // gt5

decode abs_level_gtx_flag[3] // gt7

decode abs_level_gtx_flag[4] // gt9

iSecondPassBypassPos = n;

Pass 3 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

baseLevel = (n >= iSecondPassBypassPos ? 2 : 10);

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

Pass 3 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

baseLevel = 0

decode_bypass_sig_flag

if(decode_bypass_sig_flag) {

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

}

The Rice parameter is derived based on locSumAbs and baseLevel.

locSumAbs  += (10 – baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)

The Rice parameter look-up table is same as VVC 6 TS residual coding look-up table.

const uint32_t auiGoRiceParr[32] =

{

0, 0, 0, 0,

0, 0, 0, 0, 0, 0,

0, 0, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1,

1, 1, 1, 2, 2, 2, 2, 2, 2, 2

};

### Test CE7-1.3b-alt

CE7-1.3b-alt is the same as CE7-1.3b, except a couple of differences listed below:

- The Rice parameter derivation

- Signal of decode_bypass_sig_flag

The syntax structure of CE7-1.3b-alt is more unified to that of the transform residual coding.

Following pseudo-code shows the detail of the proposed method.

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 4; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

iFirstPassBypassPos = n;

Pass 2: for(n = 0; n < iFirstPassBypassPos && remainingCtxBin >= 4; n++ )

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] // gt5

decode abs_level_gtx_flag[3] // gt7

decode abs_level_gtx_flag[4] // gt9

iSecondPassBypassPos = n;

Pass 3 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

baseLevel = (n >= iSecondPassBypassPos ? 2 : 10);

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

Pass 3 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

baseLevel = 0

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

The Rice parameter is derived based on locSumAbs and baseLevel.

locSumAbs  -= (2 * baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)

The Rice parameter look-up table is same as VVC 6 transform residual coding look-up table.

const uint32_t auiGoRicePars[32] =

{

0, 0, 0, 0,

0, 0, 0, 1, 1, 1,

1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2,

2, 2, 2, 2, 2, 2, 3, 3, 3, 3

};

Table 1 shows the comparison between VVC transform residual coding, VVC TS residual coding and the residual coding of the proposed CE. It shows that both CE7-1.3b and CE7-1.3b-alt reduce the number of checking points significantly (reduces 6 checks per coefficient). In addition to that, the syntax of the by-pass coding coefficient is similar to that of transform residual coding. It is worth to mention here that the Test CE7-1.3b-alt also saves one look-up table of size 8 bytes.

Table 1: Comparison between transform, VVC 6 TS residual coding and proposed CE tests

|  | Number of worst case checks/coefficient if context coded bins are available or not. | Total number of RG look-up tables | RG table selection | Syntax structure of fully by-pass coefficients |
| --- | --- | --- | --- | --- |
| VVC6 TS residual coding | 8 | 2 | locSumAbs | Sigsignpargt1 Gt3gt5 gt7 gt9 abs_remainder |
| Test CE7-1.3b | 2 | 2 | locSumAbs += 10 –baseLevel | Sig abs_remainder sign |
| Test CE7-1.3b-Alt | 2 | 1 | locSumAbs -= 2 * baseLevel | abs_reminder sign |
| VVC6 transform residual coding | 1 | 2 | locSumAbs -= 5 * baseLevel | dec_abs_level sign |

## Test CE7-1.3c

Test CE7-1.3c reports the combined method of CE7-1.3a and CE7-1.3b.

# Simulation results

The proposed methods are tested under both CTC and low QP (2, 7, 12, 17) settings. Table 2, Table 3, Table 4, and Table 5 show the results of CTC QP. Table 6, Table 7, Table 8, and Table 9   show the results of low QP setting.

Table 10 and Table 11 show the bin-to-bit ratio of CE7-1.3b and CE7-1.3b-alt under CTC QP. Table 12 and Table 13 show the bin-to-bit ratio of CE7-1.3b and CE7-1.3b-alt under low QP.

# Additional results

In VVC6, the maximum number of context coded bins for TS residual coding is 2 per coefficient. In this test, the maximum number of context coded bins is reduced to 1.75 per coefficient. Table 14 and Table 15 shows the results of CE7-1.3.b and CE7-1.3.b-alt under CTC test conditions. Table 16 and Table 17 shows the results of CE7-1.3.b and CE7-1.3.b-alt under low QP setting.

CE7-1.3b-alt is also implemented on top of VTM-6.0 lossless software released by AHG18 [1].Table 18 shows the results. In Table 18 both anchor and test included BDPCM bug fix described in JVET-P0463. BDPCM is on both anchor and test.

# Conclusion

This contribution reports results of 4 sub-tests in CE7-1.3. CE7-1.3a swap the position of parity and gt1 flag so that gt1 flag can get more context coded bins. CE7-1.3b and CE7-1.3b-alt are two alternatives that not only reduce the number of worst case checking of transform skip (TS) residual coding from 8 per coefficient to 2 per coefficients, but also unify the syntax structure of a by-pass coding coefficient of TS residual coding and transform residual coding without significant performance loss. CE7-1.3c is a combine test of CE7-1.3a and CE7-1.3b.

# References

VTM-6.0 lossless reference software, https://vcgit.hhi.fraunhofer.de/jvet-o-ahg-18/VVCSoftware_VTM

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 2: Results of CE7-1.3.a ( CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.03% | 101% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | -0.01% | -0.04% | 101% | 102% |
| Class C | 0.01% | -0.09% | 0.02% | 100% | 104% |
| Class E | 0.00% | 0.02% | -0.02% | 101% | 104% |
| Overall | 0.00% | -0.02% | -0.01% | 101% | 102% |
| Class D | 0.00% | 0.04% | -0.03% | 99% | 105% |
| Class F | 0.00% | -0.06% | -0.01% | 102% | 101% |
| Class SCC | -0.04% | -0.03% | -0.03% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.03% | 101% | 100% |
| Class A2 | 0.02% | 0.00% | 0.00% | 101% | 100% |
| Class B | -0.03% | 0.07% | -0.01% | 101% | 101% |
| Class C | -0.01% | 0.03% | 0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.01% | 101% | 100% |
| Class D | -0.03% | -0.31% | -0.23% | 101% | 100% |
| Class F | 0.02% | 0.01% | -0.05% | 101% | 100% |
| Class SCC | -0.05% | -0.07% | -0.08% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.12% | -0.26% | 101% | 100% |
| Class C | -0.03% | -0.23% | -0.22% | 101% | 103% |
| Class E | -0.05% | 0.04% | 0.53% | 102% | 102% |
| Overall | -0.02% | -0.12% | -0.05% | 101% | 102% |
| Class D | 0.01% | 0.08% | 0.51% | 101% | 106% |
| Class F | 0.08% | 0.05% | 0.54% | 101% | 102% |
| Class SCC | -0.04% | -0.08% | -0.03% | 102% | 100% |

Table 3: results of CE7-1.3.b ( CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 101% | 100% |
| Class A2 | 0.00% | 0.01% | 0.02% | 101% | 101% |
| Class B | 0.00% | 0.03% | -0.04% | 101% | 102% |
| Class C | 0.02% | -0.05% | 0.02% | 100% | 104% |
| Class E | 0.00% | 0.11% | 0.01% | 101% | 103% |
| Overall | 0.00% | 0.01% | 0.00% | 101% | 102% |
| Class D | 0.00% | 0.01% | 0.00% | 100% | 105% |
| Class F | 0.00% | -0.07% | -0.16% | 102% | 101% |
| Class SCC | -0.02% | 0.02% | 0.01% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class A2 | 0.01% | 0.13% | 0.05% | 101% | 100% |
| Class B | 0.00% | 0.07% | 0.01% | 101% | 102% |
| Class C | 0.02% | 0.05% | 0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.06% | 0.04% | 101% | 101% |
| Class D | -0.05% | -0.15% | -0.20% | 101% | 96% |
| Class F | -0.04% | 0.02% | 0.01% | 101% | 100% |
| Class SCC | -0.04% | -0.06% | -0.02% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.05% | -0.16% | 101% | 100% |
| Class C | 0.01% | -0.01% | 0.07% | 102% | 103% |
| Class E | 0.03% | 0.10% | -0.06% | 102% | 102% |
| Overall | 0.01% | 0.00% | -0.06% | 101% | 102% |
| Class D | 0.04% | 0.17% | 0.32% | 101% | 107% |
| Class F | 0.07% | 0.13% | 0.55% | 102% | 103% |
| Class SCC | -0.06% | -0.01% | 0.03% | 103% | 104% |

Table 4: results of CE7-1.3.b-alt (CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 101% | 101% |
| Class A2 | 0.00% | 0.01% | 0.01% | 101% | 101% |
| Class B | 0.00% | 0.01% | -0.05% | 101% | 102% |
| Class C | 0.01% | -0.06% | -0.02% | 100% | 104% |
| Class E | 0.00% | 0.06% | 0.00% | 101% | 104% |
| Overall | 0.00% | 0.00% | -0.02% | 101% | 102% |
| Class D | 0.00% | -0.05% | -0.09% | 99% | 105% |
| Class F | 0.00% | -0.05% | -0.02% | 102% | 101% |
| Class SCC | 0.04% | 0.08% | 0.05% | 102% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 101% | 100% |
| Class A2 | 0.01% | 0.08% | 0.08% | 101% | 100% |
| Class B | 0.00% | 0.18% | 0.07% | 101% | 101% |
| Class C | 0.00% | 0.09% | 0.09% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.10% | 0.07% | 101% | 101% |
| Class D | -0.05% | -0.33% | 0.02% | 101% | 100% |
| Class F | 0.01% | -0.04% | 0.14% | 101% | 101% |
| Class SCC | -0.03% | -0.01% | 0.03% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.11% | -0.15% | 101% | 101% |
| Class C | -0.02% | 0.00% | -0.16% | 101% | 104% |
| Class E | 0.03% | 0.09% | -0.06% | 102% | 103% |
| Overall | 0.01% | 0.07% | -0.13% | 101% | 102% |
| Class D | 0.02% | -0.30% | -0.02% | 101% | 106% |
| Class F | 0.10% | 0.26% | 0.54% | 102% | 103% |
| Class SCC | -0.09% | -0.07% | -0.05% | 103% | 101% |

Table 5: results of CE7-1.3.c ( CTC QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | -0.01% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.01% | 0.00% | 101% | 101% |
| Class C | 0.00% | -0.12% | 0.03% | 100% | 104% |
| Class E | 0.01% | 0.02% | -0.01% | 101% | 103% |
| Overall | 0.00% | -0.02% | 0.01% | 100% | 102% |
| Class D | 0.00% | 0.00% | -0.06% | 99% | 105% |
| Class F | 0.00% | -0.03% | -0.09% | 101% | 101% |
| Class SCC | -0.02% | -0.01% | 0.01% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.02% | 101% | 100% |
| Class A2 | 0.03% | -0.06% | 0.04% | 101% | 100% |
| Class B | -0.01% | 0.07% | 0.08% | 101% | 101% |
| Class C | -0.01% | 0.06% | 0.12% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.07% | 101% | 100% |
| Class D | -0.06% | -0.26% | -0.01% | 101% | 100% |
| Class F | -0.06% | 0.04% | 0.01% | 101% | 100% |
| Class SCC | -0.04% | -0.04% | -0.04% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.09% | -0.19% | 101% | 100% |
| Class C | -0.05% | 0.09% | -0.28% | 101% | 103% |
| Class E | -0.16% | 0.68% | 0.27% | 102% | 102% |
| Overall | -0.06% | 0.16% | -0.11% | 101% | 102% |
| Class D | -0.01% | -0.21% | 0.63% | 101% | 106% |
| Class F | 0.05% | -0.11% | 0.30% | 101% | 102% |
| Class SCC | -0.11% | -0.09% | -0.01% | 102% | 100% |

Table 6: results of CE7-1.3.a ( low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.02% | 0.00% | 98% | 102% |
| Class E | -0.01% | -0.02% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.02% | 0.00% | 100% | 105% |
| Class F | -0.04% | -0.04% | -0.06% | 100% | 100% |
| Class SCC | -0.20% | -0.15% | -0.15% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | -0.01% | 0.01% | 100% | 100% |
| Class C | -0.02% | -0.04% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.03% | 100% | 99% |
| Class F | -0.08% | -0.11% | -0.10% | 100% | 99% |
| Class SCC | -0.17% | -0.17% | -0.18% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.02% | 101% | 101% |
| Class C | 0.00% | -0.01% | -0.03% | 101% | 103% |
| Class E | -0.02% | 0.06% | 0.04% | 100% | 103% |
| Overall | -0.01% | 0.01% | -0.01% | 101% | 102% |
| Class D | -0.03% | -0.02% | -0.02% | 101% | 105% |
| Class F | -0.15% | -0.20% | -0.13% | 101% | 100% |
| Class SCC | -0.20% | -0.11% | -0.05% | 101% | 98% |

Table 7: results of CE7-1.3.b ( Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class B | -0.01% | -0.01% | -0.03% | 100% | 98% |
| Class C | -0.02% | -0.02% | -0.02% | 98% | 100% |
| Class E | 0.00% | -0.04% | -0.02% | 100% | 98% |
| Overall | -0.01% | -0.02% | -0.01% | 100% | 98% |
| Class D | -0.04% | 0.01% | -0.03% | 99% | 102% |
| Class F | -0.68% | -0.33% | -0.34% | 99% | 96% |
| Class SCC | -1.27% | -0.95% | -0.96% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.01% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class C | -0.02% | -0.02% | -0.02% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.02% | -0.12% | -0.04% | 99% | 99% |
| Class F | -0.73% | -0.41% | -0.38% | 99% | 99% |
| Class SCC | -1.50% | -0.83% | -0.84% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.02% | 100% | 97% |
| Class C | -0.01% | 0.01% | 0.02% | 100% | 100% |
| Class E | -0.03% | -0.02% | 0.01% | 100% | 98% |
| Overall | -0.02% | -0.01% | 0.00% | 100% | 98% |
| Class D | -0.01% | 0.08% | 0.02% | 100% | 100% |
| Class F | -0.43% | -0.32% | -0.29% | 100% | 91% |
| Class SCC | -0.87% | -0.72% | -0.59% | 100% | 86% |

Table 8: results of CE7-1.3.b-alt (Low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.01% | 100% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | -0.02% | -0.01% | 0.01% | 100% | 98% |
| Class C | -0.05% | -0.04% | -0.01% | 98% | 100% |
| Class E | -0.03% | -0.07% | -0.03% | 100% | 99% |
| Overall | -0.02% | -0.02% | 0.00% | 99% | 98% |
| Class D | -0.07% | -0.07% | -0.04% | 100% | 102% |
| Class F | -1.21% | -0.50% | -0.51% | 99% | 96% |
| Class SCC | -1.41% | -1.03% | -1.02% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class B | -0.01% | 0.00% | 0.01% | 100% | 100% |
| Class C | -0.03% | -0.03% | -0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.04% | -0.11% | -0.10% | 100% | 100% |
| Class F | -1.51% | -0.57% | -0.67% | 100% | 99% |
| Class SCC | -2.02% | -0.90% | -0.90% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.01% | -0.02% | 101% | 97% |
| Class C | -0.02% | 0.00% | -0.04% | 100% | 100% |
| Class E | -0.04% | 0.01% | -0.03% | 100% | 98% |
| Overall | -0.02% | 0.01% | -0.03% | 100% | 98% |
| Class D | -0.01% | -0.01% | -0.11% | 100% | 101% |
| Class F | -0.49% | -0.28% | -0.27% | 100% | 91% |
| Class SCC | -1.00% | -0.57% | -0.47% | 100% | 85% |

Table 9: results of CE7-1.3.c ( low QP)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.02% | 99% | 98% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class B | -0.01% | -0.01% | 0.00% | 99% | 98% |
| Class C | -0.02% | -0.03% | -0.02% | 98% | 100% |
| Class E | -0.02% | -0.04% | -0.03% | 100% | 99% |
| Overall | -0.01% | -0.02% | -0.01% | 99% | 99% |
| Class D | -0.02% | -0.04% | 0.00% | 100% | 103% |
| Class F | -0.68% | -0.32% | -0.33% | 99% | 96% |
| Class SCC | -1.25% | -0.93% | -0.93% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | -0.03% | 99% | 99% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class B | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class C | -0.01% | -0.03% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 100% |
| Class D | -0.01% | -0.08% | -0.01% | 100% | 100% |
| Class F | -0.81% | -0.51% | -0.50% | 99% | 100% |
| Class SCC | -1.43% | -0.78% | -0.82% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | 0.00% | 100% | 97% |
| Class C | -0.02% | -0.03% | 0.01% | 100% | 100% |
| Class E | -0.04% | 0.07% | 0.04% | 100% | 98% |
| Overall | -0.02% | 0.01% | 0.01% | 100% | 98% |
| Class D | -0.02% | 0.00% | 0.05% | 100% | 101% |
| Class F | -0.28% | -0.28% | -0.26% | 100% | 91% |
| Class SCC | -0.96% | -0.70% | -0.64% | 100% | 85% |

Table 10: bin-to-bit ratio of CE7-1.3.b ( CTC QP)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | -0.02% | 1.19 |  |  | 1.19 |  | 0.20% | 1.15 | 1.16 | 0.25% | 1.16 | 1.16 | 0.01% | 1.19 | 1.19 | 0.00% | 101% | 100% |
| Class A2 | 0.00% |  | 0.01% |  | 0.02% | 1.18 |  |  | 1.18 |  | 0.00% | 1.16 | 1.16 | -0.02% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 101% | 101% |
| Class B | 0.00% |  | 0.03% |  | -0.04% | 1.15 |  |  | 1.15 |  | 0.05% | 1.13 | 1.13 | 0.03% | 1.13 | 1.13 | 0.00% | 1.15 | 1.15 | 0.00% | 101% | 102% |
| Class C | 0.02% |  | -0.05% |  | 0.02% | 1.08 |  |  | 1.08 |  | -0.41% | 1.09 | 1.08 | -0.50% | 1.09 | 1.09 | -0.09% | 1.07 | 1.07 | -0.04% | 100% | 104% |
| Class E | 0.00% |  | 0.11% |  | 0.01% | 1.11 |  |  | 1.11 |  | -0.10% | 1.10 | 1.10 | -0.20% | 1.10 | 1.10 | -0.01% | 1.10 | 1.10 | -0.01% | 101% | 103% |
| Overall | 0.00% |  | 0.01% |  | 0.00% | 1.14 |  |  | 1.14 |  | -0.05% | 1.13 | 1.12 | -0.09% | 1.13 | 1.13 | -0.02% | 1.14 | 1.14 | -0.01% | 101% | 102% |
| Class D | 0.00% |  | 0.01% |  | 0.00% | 1.06 |  |  | 1.06 |  | 0.11% | 1.07 | 1.07 | 0.25% | 1.08 | 1.08 | -0.08% | 1.05 | 1.05 | -0.04% | 100% | 105% |
| Class F | 0.00% |  | -0.07% |  | -0.16% | 1.16 |  |  | 1.16 |  | -0.12% | 1.17 | 1.17 | -0.19% | 1.18 | 1.17 | -0.29% | 1.17 | 1.17 | -0.14% | 102% | 101% |
| Class TGM | -0.02% |  | 0.02% |  | 0.01% | 1.20 |  |  | 1.20 |  | -0.19% | 1.33 | 1.33 | -0.19% | 1.33 | 1.32 | -0.43% | 1.20 | 1.19 | -0.31% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | 0.00% | 1.18 |  | 1.18 |  | 0.01% |  | 1.15 | 1.15 | -0.03% | 1.17 | 1.17 | -0.01% | 1.22 | 1.22 | 0.00% | 101% | 100% |
| Class A2 | 0.01% |  | 0.13% |  | 0.05% | 1.17 |  | 1.17 |  | 0.00% |  | 1.14 | 1.14 | -0.02% | 1.20 | 1.19 | -0.02% | 1.22 | 1.22 | -0.01% | 101% | 100% |
| Class B | 0.00% |  | 0.07% |  | 0.01% | 1.14 |  | 1.14 |  | -0.01% |  | 1.10 | 1.10 | 0.04% | 1.16 | 1.16 | 0.00% | 1.19 | 1.19 | 0.00% | 101% | 102% |
| Class C | 0.02% |  | 0.05% |  | 0.08% | 1.05 |  | 1.05 |  | 0.15% |  | 1.03 | 1.03 | -0.13% | 1.11 | 1.11 | -0.01% | 1.10 | 1.10 | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% |  | 0.06% |  | 0.04% | 1.13 |  | 1.13 |  | 0.04% |  | 1.10 | 1.10 | -0.03% | 1.16 | 1.16 | -0.01% | 1.18 | 1.18 | 0.00% | 101% | 101% |
| Class D | -0.05% |  | -0.15% |  | -0.20% | 1.02 |  | 1.02 |  | -0.06% |  | 1.01 | 1.01 | 0.25% | 1.11 | 1.11 | -0.07% | 1.07 | 1.07 | -0.08% | 101% | 96% |
| Class F | -0.04% |  | 0.02% |  | 0.01% | 1.11 |  | 1.11 |  | -0.12% |  | 1.08 | 1.08 | -0.24% | 1.15 | 1.15 | -0.31% | 1.16 | 1.16 | -0.18% | 101% | 100% |
| Class TGM | -0.04% |  | -0.06% |  | -0.02% | 1.15 |  | 1.15 |  | -0.25% |  | 1.22 | 1.21 | -0.66% | 1.33 | 1.32 | -0.39% | 1.22 | 1.21 | -0.23% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% |  | -0.05% |  | -0.16% | 1.18 | 1.18 |  |  | -0.49% |  | 1.16 | 1.15 | -0.71% | 1.29 | 1.29 | 0.04% | 1.25 | 1.25 | 0.04% | 101% | 100% |
| Class C | 0.01% |  | -0.01% |  | 0.07% | 1.09 | 1.09 |  |  | 0.15% |  | 1.11 | 1.11 | -0.09% | 1.27 | 1.27 | -0.05% | 1.17 | 1.17 | -0.02% | 102% | 103% |
| Class E | 0.03% |  | 0.10% |  | -0.06% | 1.11 | 1.11 |  |  | 0.06% |  | 1.11 | 1.12 | 0.05% | 1.36 | 1.36 | 0.02% | 1.24 | 1.24 | 0.00% | 102% | 102% |
| Overall | 0.01% |  | 0.00% |  | -0.06% | 1.13 | 1.13 |  |  | -0.15% |  | 1.13 | 1.13 | -0.32% | 1.30 | 1.30 | 0.01% | 1.22 | 1.22 | 0.01% | 101% | 102% |
| Class D | 0.04% |  | 0.17% |  | 0.32% | 1.05 | 1.05 |  |  | -0.04% |  | 1.08 | 1.08 | 0.17% | 1.31 | 1.31 | 0.05% | 1.16 | 1.16 | 0.02% | 101% | 107% |
| Class F | 0.07% |  | 0.13% |  | 0.55% | 1.19 | 1.18 |  |  | -0.37% |  | 1.21 | 1.21 | -0.42% | 1.30 | 1.30 | -0.07% | 1.26 | 1.26 | 0.00% | 102% | 103% |
| Class TGM | -0.06% |  | -0.01% |  | 0.03% | 1.19 | 1.18 |  |  | -0.20% |  | 1.29 | 1.28 | -0.39% | 1.49 | 1.49 | -0.23% | 1.32 | 1.32 | -0.26% | 103% | 104% |

Table 11: bin-to-bit ratio of CE7-1.3b-alt ( CTC QP)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | -0.02% | 1.19 |  |  | 1.19 |  | 0.18% | 1.15 | 1.16 | 0.20% | 1.16 | 1.16 | 0.01% | 1.19 | 1.19 | 0.00% | 101% | 101% |
| Class A2 | 0.00% |  | 0.01% |  | 0.01% | 1.18 |  |  | 1.18 |  | 0.01% | 1.16 | 1.16 | -0.01% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 101% | 101% |
| Class B | 0.00% |  | 0.01% |  | -0.05% | 1.15 |  |  | 1.15 |  | 0.01% | 1.13 | 1.13 | -0.04% | 1.13 | 1.13 | 0.00% | 1.15 | 1.15 | 0.01% | 101% | 102% |
| Class C | 0.01% |  | -0.06% |  | -0.02% | 1.08 |  |  | 1.08 |  | -0.09% | 1.09 | 1.09 | -0.14% | 1.09 | 1.09 | -0.05% | 1.07 | 1.07 | -0.02% | 100% | 104% |
| Class E | 0.00% |  | 0.06% |  | 0.00% | 1.11 |  |  | 1.11 |  | 0.00% | 1.10 | 1.10 | -0.09% | 1.10 | 1.10 | -0.01% | 1.10 | 1.10 | 0.00% | 101% | 104% |
| Overall | 0.00% |  | 0.00% |  | -0.02% | 1.14 |  |  | 1.14 |  | 0.02% | 1.13 | 1.13 | -0.02% | 1.13 | 1.13 | -0.01% | 1.14 | 1.14 | 0.00% | 101% | 102% |
| Class D | 0.00% |  | -0.05% |  | -0.09% | 1.06 |  |  | 1.06 |  | -0.01% | 1.07 | 1.07 | -0.01% | 1.08 | 1.08 | -0.07% | 1.05 | 1.05 | -0.04% | 99% | 105% |
| Class F | 0.00% |  | -0.05% |  | -0.02% | 1.16 |  |  | 1.16 |  | -0.07% | 1.17 | 1.17 | -0.22% | 1.18 | 1.17 | -0.34% | 1.17 | 1.16 | -0.19% | 102% | 101% |
| Class TGM | 0.04% |  | 0.08% |  | 0.05% | 1.20 |  |  | 1.20 |  | -0.20% | 1.33 | 1.33 | -0.35% | 1.33 | 1.32 | -0.56% | 1.20 | 1.19 | -0.38% | 102% | 101% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | 0.01% | 1.18 |  | 1.18 |  | 0.01% |  | 1.15 | 1.15 | -0.06% | 1.17 | 1.17 | -0.01% | 1.22 | 1.22 | 0.00% | 101% | 100% |
| Class A2 | 0.01% |  | 0.08% |  | 0.08% | 1.17 |  | 1.17 |  | 0.03% |  | 1.14 | 1.14 | 0.04% | 1.20 | 1.20 | 0.00% | 1.22 | 1.22 | 0.00% | 101% | 100% |
| Class B | 0.00% |  | 0.18% |  | 0.07% | 1.14 |  | 1.14 |  | -0.04% |  | 1.10 | 1.10 | 0.05% | 1.16 | 1.16 | 0.00% | 1.19 | 1.19 | 0.01% | 101% | 101% |
| Class C | 0.00% |  | 0.09% |  | 0.09% | 1.05 |  | 1.05 |  | 0.13% |  | 1.03 | 1.03 | -0.01% | 1.11 | 1.11 | -0.02% | 1.10 | 1.10 | -0.01% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% |  | 0.10% |  | 0.07% | 1.13 |  | 1.13 |  | 0.03% |  | 1.10 | 1.10 | 0.01% | 1.16 | 1.16 | 0.00% | 1.18 | 1.18 | 0.00% | 101% | 101% |
| Class D | -0.05% |  | -0.33% |  | 0.02% | 1.02 |  | 1.02 |  | 0.01% |  | 1.01 | 1.01 | 0.20% | 1.11 | 1.11 | -0.07% | 1.07 | 1.07 | -0.06% | 101% | 100% |
| Class F | 0.01% |  | -0.04% |  | 0.14% | 1.11 |  | 1.11 |  | -0.28% |  | 1.08 | 1.08 | -0.47% | 1.15 | 1.14 | -0.45% | 1.16 | 1.16 | -0.29% | 101% | 101% |
| Class TGM | -0.03% |  | -0.01% |  | 0.03% | 1.15 |  | 1.14 |  | -0.67% |  | 1.22 | 1.21 | -0.84% | 1.33 | 1.32 | -0.52% | 1.22 | 1.21 | -0.32% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | 0.11% |  | -0.15% | 1.18 | 1.18 |  |  | -0.07% |  | 1.16 | 1.16 | -0.15% | 1.29 | 1.29 | 0.03% | 1.25 | 1.25 | 0.01% | 101% | 101% |
| Class C | -0.02% |  | 0.00% |  | -0.16% | 1.09 | 1.09 |  |  | 0.06% |  | 1.11 | 1.11 | -0.07% | 1.27 | 1.27 | -0.02% | 1.17 | 1.17 | -0.02% | 101% | 104% |
| Class E | 0.03% |  | 0.09% |  | -0.06% | 1.11 | 1.11 |  |  | 0.04% |  | 1.11 | 1.11 | 0.02% | 1.36 | 1.36 | 0.05% | 1.24 | 1.25 | 0.03% | 102% | 103% |
| Overall | 0.01% |  | 0.07% |  | -0.13% | 1.13 | 1.13 |  |  | 0.00% |  | 1.13 | 1.13 | -0.08% | 1.30 | 1.30 | 0.02% | 1.22 | 1.22 | 0.01% | 101% | 102% |
| Class D | 0.02% |  | -0.30% |  | -0.02% | 1.05 | 1.05 |  |  | -0.10% |  | 1.08 | 1.08 | -0.07% | 1.31 | 1.31 | 0.01% | 1.16 | 1.16 | 0.00% | 101% | 106% |
| Class F | 0.10% |  | 0.26% |  | 0.54% | 1.19 | 1.18 |  |  | -0.28% |  | 1.21 | 1.21 | -0.41% | 1.30 | 1.30 | -0.10% | 1.26 | 1.26 | -0.11% | 102% | 103% |
| Class TGM | -0.09% |  | -0.07% |  | -0.05% | 1.19 | 1.18 |  |  | -0.21% |  | 1.29 | 1.28 | -0.44% | 1.49 | 1.49 | -0.20% | 1.32 | 1.32 | -0.20% | 103% | 101% |

Table 12: bin-to-bit ratio of CE7-1.3.b ( low QP)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | 0.00% | 1.02 |  |  | 1.02 |  | -0.02% | 0.95 | 0.95 | -0.01% | 0.96 | 0.96 | -0.03% | 1.02 | 1.02 | -0.03% | 100% | 98% |
| Class A2 | 0.00% |  | -0.01% |  | 0.00% | 0.90 |  |  | 0.90 |  | -0.02% | 0.84 | 0.84 | -0.03% | 0.84 | 0.84 | 0.01% | 0.91 | 0.91 | 0.01% | 100% | 98% |
| Class B | -0.01% |  | -0.01% |  | -0.03% | 0.93 |  |  | 0.93 |  | -0.04% | 0.86 | 0.86 | -0.03% | 0.87 | 0.87 | 0.01% | 0.94 | 0.94 | 0.01% | 100% | 98% |
| Class C | -0.02% |  | -0.02% |  | -0.02% | 0.83 |  |  | 0.83 |  | -0.01% | 0.74 | 0.74 | 0.03% | 0.75 | 0.75 | 0.02% | 0.83 | 0.83 | -0.02% | 98% | 100% |
| Class E | 0.00% |  | -0.04% |  | -0.02% | 1.02 |  |  | 1.02 |  | 0.01% | 0.96 | 0.96 | 0.01% | 0.96 | 0.96 | -0.04% | 1.02 | 1.02 | -0.02% | 100% | 98% |
| Overall | -0.01% |  | -0.02% |  | -0.01% | 0.93 |  |  | 0.93 |  | -0.02% | 0.86 | 0.86 | -0.01% | 0.87 | 0.87 | 0.00% | 0.94 | 0.94 | -0.01% | 100% | 98% |
| Class D | -0.04% |  | 0.01% |  | -0.03% | 0.80 |  |  | 0.80 |  | 0.35% | 0.71 | 0.72 | 0.42% | 0.73 | 0.73 | 0.06% | 0.81 | 0.81 | 0.07% | 99% | 102% |
| Class F | -0.68% |  | -0.33% |  | -0.34% | 0.96 |  |  | 0.95 |  | -0.56% | 0.87 | 0.86 | -0.62% | 0.85 | 0.85 | -0.47% | 0.94 | 0.93 | -0.36% | 99% | 96% |
| Class TGM | -1.27% |  | -0.95% |  | -0.96% | 0.87 |  |  | 0.86 |  | -0.55% | 0.80 | 0.80 | -0.97% | 0.80 | 0.80 | -0.84% | 0.87 | 0.86 | -0.53% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | -0.01% | 0.96 |  | 0.96 |  | -0.04% |  | 0.89 | 0.89 | -0.02% | 1.09 | 1.09 | -0.02% | 1.15 | 1.15 | -0.02% | 100% | 99% |
| Class A2 | 0.00% |  | 0.00% |  | 0.01% | 0.84 |  | 0.84 |  | -0.01% |  | 0.77 | 0.77 | -0.01% | 1.03 | 1.03 | 0.00% | 1.09 | 1.09 | -0.01% | 100% | 100% |
| Class B | 0.00% |  | 0.00% |  | 0.02% | 0.87 |  | 0.87 |  | -0.02% |  | 0.80 | 0.80 | 0.01% | 1.03 | 1.03 | 0.00% | 1.11 | 1.11 | 0.00% | 100% | 100% |
| Class C | -0.02% |  | -0.02% |  | -0.02% | 0.77 |  | 0.77 |  | -0.06% |  | 0.68 | 0.68 | 0.04% | 0.96 | 0.96 | 0.03% | 1.03 | 1.03 | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% |  | -0.01% |  | 0.00% | 0.86 |  | 0.86 |  | -0.03% |  | 0.78 | 0.78 | 0.01% | 1.02 | 1.02 | 0.01% | 1.09 | 1.09 | -0.01% | 100% | 100% |
| Class D | -0.02% |  | -0.12% |  | -0.04% | 0.75 |  | 0.75 |  | -0.05% |  | 0.66 | 0.66 | -0.02% | 0.93 | 0.93 | 0.05% | 1.00 | 1.00 | 0.02% | 99% | 99% |
| Class F | -0.73% |  | -0.41% |  | -0.38% | 0.90 |  | 0.90 |  | -0.39% |  | 0.81 | 0.81 | -0.29% | 0.93 | 0.93 | -0.29% | 1.01 | 1.00 | -0.25% | 99% | 99% |
| Class TGM | -1.50% |  | -0.83% |  | -0.84% | 0.84 |  | 0.83 |  | -0.75% |  | 0.77 | 0.76 | -1.05% | 0.86 | 0.85 | -0.48% | 0.93 | 0.93 | -0.30% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% |  | -0.01% |  | -0.02% | 0.93 | 0.93 |  |  | -0.03% |  | 0.86 | 0.86 | 0.00% | 1.03 | 1.03 | 0.00% | 1.10 | 1.10 | 0.01% | 100% | 97% |
| Class C | -0.01% |  | 0.01% |  | 0.02% | 0.82 | 0.82 |  |  | -0.11% |  | 0.73 | 0.73 | -0.07% | 0.98 | 0.98 | 0.01% | 1.05 | 1.05 | 0.08% | 100% | 100% |
| Class E | -0.03% |  | -0.02% |  | 0.01% | 1.03 | 1.03 |  |  | -0.01% |  | 0.96 | 0.96 | 0.01% | 1.21 | 1.21 | -0.03% | 1.25 | 1.26 | 0.10% | 100% | 98% |
| Overall | -0.02% |  | -0.01% |  | 0.00% | 0.92 | 0.92 |  |  | -0.05% |  | 0.84 | 0.84 | -0.02% | 1.06 | 1.06 | -0.01% | 1.12 | 1.12 | 0.06% | 100% | 98% |
| Class D | -0.01% |  | 0.08% |  | 0.02% | 0.79 | 0.79 |  |  | -0.07% |  | 0.71 | 0.71 | 0.02% | 0.97 | 0.97 | 0.02% | 1.04 | 1.04 | 0.30% | 100% | 100% |
| Class F | -0.43% |  | -0.32% |  | -0.29% | 0.92 | 0.92 |  |  | -0.36% |  | 0.84 | 0.84 | -0.43% | 1.03 | 1.02 | -0.24% | 1.09 | 1.10 | 0.29% | 100% | 91% |
| Class TGM | -0.87% |  | -0.72% |  | -0.59% | 0.85 | 0.85 |  |  | -0.48% |  | 0.79 | 0.78 | -0.81% | 0.91 | 0.91 | -0.38% | 0.99 | 0.99 | 0.13% | 100% | 86% |

Table 13: bin-to-bit ratio of CE7-1.3b-alt ( low QP)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y |  | U |  | V | Ref |  |  | Test |  | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | 0.00% |  | 0.01% | 1.02 |  |  | 1.02 |  | -0.03% | 0.95 | 0.95 | -0.03% | 0.96 | 0.96 | -0.02% | 1.02 | 1.02 | -0.03% | 100% | 97% |
| Class A2 | 0.00% |  | 0.00% |  | 0.00% | 0.90 |  |  | 0.90 |  | 0.00% | 0.84 | 0.84 | -0.01% | 0.84 | 0.84 | 0.01% | 0.91 | 0.91 | 0.01% | 100% | 98% |
| Class B | -0.02% |  | -0.01% |  | 0.01% | 0.93 |  |  | 0.93 |  | -0.04% | 0.86 | 0.86 | -0.02% | 0.87 | 0.87 | 0.00% | 0.94 | 0.94 | 0.00% | 100% | 98% |
| Class C | -0.05% |  | -0.04% |  | -0.01% | 0.83 |  |  | 0.82 |  | -0.04% | 0.74 | 0.74 | 0.03% | 0.75 | 0.75 | 0.05% | 0.83 | 0.83 | -0.03% | 98% | 100% |
| Class E | -0.03% |  | -0.07% |  | -0.03% | 1.02 |  |  | 1.02 |  | 0.10% | 0.96 | 0.96 | 0.15% | 0.96 | 0.96 | -0.02% | 1.02 | 1.02 | -0.01% | 100% | 99% |
| Overall | -0.02% |  | -0.02% |  | 0.00% | 0.93 |  |  | 0.93 |  | -0.01% | 0.86 | 0.86 | 0.02% | 0.87 | 0.87 | 0.00% | 0.94 | 0.94 | -0.01% | 99% | 98% |
| Class D | -0.07% |  | -0.07% |  | -0.04% | 0.80 |  |  | 0.80 |  | 0.30% | 0.71 | 0.72 | 0.41% | 0.73 | 0.73 | 0.13% | 0.81 | 0.81 | 0.06% | 100% | 102% |
| Class F | -1.21% |  | -0.50% |  | -0.51% | 0.96 |  |  | 0.95 |  | -0.50% | 0.87 | 0.86 | -0.61% | 0.85 | 0.85 | -0.55% | 0.94 | 0.93 | -0.41% | 99% | 96% |
| Class TGM | -1.41% |  | -1.03% |  | -1.02% | 0.87 |  |  | 0.86 |  | -0.62% | 0.80 | 0.80 | -1.00% | 0.80 | 0.80 | -0.96% | 0.87 | 0.86 | -0.60% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 | 0.00% |  | -0.01% |  | -0.02% | 0.96 |  | 0.96 |  | -0.03% |  | 0.89 | 0.89 | -0.01% | 1.09 | 1.09 | -0.01% | 1.15 | 1.15 | -0.02% | 100% | 100% |
| Class A2 | 0.00% |  | 0.00% |  | 0.01% | 0.84 |  | 0.84 |  | 0.01% |  | 0.77 | 0.77 | 0.01% | 1.03 | 1.03 | 0.00% | 1.09 | 1.09 | 0.00% | 100% | 100% |
| Class B | -0.01% |  | 0.00% |  | 0.01% | 0.87 |  | 0.87 |  | -0.03% |  | 0.80 | 0.80 | 0.00% | 1.03 | 1.03 | 0.00% | 1.11 | 1.11 | 0.00% | 100% | 100% |
| Class C | -0.03% |  | -0.03% |  | -0.04% | 0.77 |  | 0.77 |  | -0.06% |  | 0.68 | 0.68 | 0.11% | 0.96 | 0.96 | 0.04% | 1.03 | 1.03 | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% |  | -0.01% |  | -0.01% | 0.86 |  | 0.86 |  | -0.03% |  | 0.78 | 0.78 | 0.02% | 1.02 | 1.02 | 0.01% | 1.09 | 1.09 | -0.01% | 100% | 100% |
| Class D | -0.04% |  | -0.11% |  | -0.10% | 0.75 |  | 0.75 |  | -0.12% |  | 0.66 | 0.66 | 0.03% | 0.93 | 0.93 | 0.08% | 1.00 | 1.00 | 0.02% | 100% | 100% |
| Class F | -1.51% |  | -0.57% |  | -0.67% | 0.90 |  | 0.90 |  | -0.36% |  | 0.81 | 0.81 | -0.28% | 0.93 | 0.93 | -0.28% | 1.01 | 1.00 | -0.22% | 100% | 99% |
| Class TGM | -2.02% |  | -0.90% |  | -0.90% | 0.84 |  | 0.83 |  | -0.47% |  | 0.77 | 0.77 | -0.73% | 0.86 | 0.85 | -0.46% | 0.93 | 0.93 | -0.22% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  | Max - Bin2Bit Ratio |  |  |  |  |  | TRF of Max - Bin2Bit Ratio |  |  | Avg TRF - Bin2Bit Ratio |  |  | Avg - Bin2Bit Ratio |  |  |  |  |
|  | Y | U |  | V |  | Ref | Test |  |  | Diff (%) |  | Ref | Test | Diff (%) | Ref | Test | Diff (%) | Ref | Test | Diff (%) | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% |  | 0.01% |  | -0.02% | 0.93 | 0.93 |  |  | 0.00% |  | 0.86 | 0.86 | 0.04% | 1.03 | 1.03 | 0.00% | 1.10 | 1.10 | 0.02% | 101% | 97% |
| Class C | -0.02% |  | 0.00% |  | -0.04% | 0.82 | 0.82 |  |  | -0.12% |  | 0.73 | 0.73 | 0.02% | 0.98 | 0.98 | 0.01% | 1.05 | 1.05 | 0.07% | 100% | 100% |
| Class E | -0.04% |  | 0.01% |  | -0.03% | 1.03 | 1.03 |  |  | 0.05% |  | 0.96 | 0.96 | 0.06% | 1.21 | 1.21 | -0.03% | 1.25 | 1.26 | 0.09% | 100% | 98% |
| Overall | -0.02% |  | 0.01% |  | -0.03% | 0.92 | 0.92 |  |  | -0.02% |  | 0.84 | 0.84 | 0.04% | 1.06 | 1.06 | 0.00% | 1.12 | 1.12 | 0.05% | 100% | 98% |
| Class D | -0.01% |  | -0.01% |  | -0.11% | 0.79 | 0.79 |  |  | -0.04% |  | 0.71 | 0.71 | 0.15% | 0.97 | 0.97 | 0.05% | 1.04 | 1.04 | 0.32% | 100% | 101% |
| Class F | -0.49% |  | -0.28% |  | -0.27% | 0.92 | 0.92 |  |  | -0.36% |  | 0.84 | 0.84 | -0.46% | 1.03 | 1.02 | -0.26% | 1.09 | 1.10 | 0.29% | 100% | 91% |
| Class TGM | -1.00% |  | -0.57% |  | -0.47% | 0.85 | 0.85 |  |  | -0.54% |  | 0.79 | 0.78 | -0.90% | 0.91 | 0.91 | -0.50% | 0.99 | 0.99 | 0.08% | 100% | 85% |

Table 14: results of CE7-1.3.b with reduced number of maximum context coded bins at CTC QP( Anchor: VTM-6.0 with 2 bins per coefficient; test: CE7-1.3b with 1.75 bins per coefficient) ( runtime is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.12% | 0.10% | 0.15% | 101% | 106% |
| Class SCC | 0.28% | 0.26% | 0.26% | 101% | 105% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.07% | 0.11% | 0.20% | 100% | 114% |
| Class SCC | 0.12% | 0.13% | 0.13% | 100% | 115% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.01% | -0.02% | 0.46% | 101% | 120% |
| Class SCC | 0.07% | 0.11% | 0.10% | 101% | 119% |

Table 15: results of CE7-1.3.b-alt with reduced number of maximum context coded bins at CTC QP ( Anchor: VTM-6.0 with 2 bins per coefficient; test: CE7-1.3b-alt with 1.75 bins per coefficient) ( runtime is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.12% | 0.04% | 0.09% | 101% | 106% |
| Class SCC | 0.33% | 0.33% | 0.31% | 100% | 106% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.11% | 0.10% | 0.18% | 100% | 114% |
| Class SCC |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.14% | 0.44% | 0.64% | 101% | 119% |
| Class SCC | -0.02% | 0.16% | 0.17% | 102% | 119% |

Table 16: results of CE7-1.3.b with reduced number of maximum context coded bins under low QP( Anchor: VTM-6.0 with 2 bins per coefficient; test: CE7-1.3b with 1.75 bins per coefficient) ( runtime is not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.48% | -0.23% | -0.22% | 99% | 99% |
| Class SCC | -0.99% | -0.65% | -0.65% | 98% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.27% | -0.20% | -0.14% | 100% | 110% |
| Class SCC | -1.14% | -0.55% | -0.55% | 100% | 114% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.29% | -0.21% | -0.16% | 101% | 100% |
| Class SCC | -0.53% | -0.31% | -0.27% | 100% | 98% |

Table 17: results of CE7-1.3.b-alt with reduced number of maximum context coded bins at low QP ( Anchor: VTM-6.0 with 2 bins per coefficient; test: CE7-1.3b-alt with 1.75 bins per coefficient)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.08% | -0.38% | -0.34% | 99% | 99% |
| Class SCC | -1.16% | -0.73% | -0.72% | 98% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.23% | -0.47% | -0.45% | 100% | 110% |
| Class SCC | -1.67% | -0.59% | -0.56% | 100% | 114% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.44% | -0.27% | -0.19% | 100% | 101% |
| Class SCC | -0.74% | -0.38% | -0.31% | 100% | 98% |

Table 18: results of CE7-1.3.b.alt on top of VTM-6.0 lossless software ( Both anchor and test has bug fix version and BDPCM on)

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM-6.0-Lossless + Bug-fix + BDPCM ON | VTM-6.0-Lossless + Bug-fix + BDPCM ON + CE7.1.3b-alt |  | VTM-6.0-Lossless + Bug-fix + BDPCM ON | VTM-6.0-Lossless + Bug-fix + BDPCM ON + CE7.1.3b-alt |  | VTM-6.0-Lossless + Bug-fix + BDPCM ON | VTM-6.0-Lossless + Bug-fix + BDPCM ON + CE7.1.3b-alt |  |
| Class A1 | 2.2 | 2.2 | -0.18% |  |  |  |  |  |  |
| Class A2 | 1.8 | 1.8 | -0.66% |  |  |  |  |  |  |
| Class B | 2.1 | 2.1 | -0.44% |  |  |  |  |  |  |
| Class C | 2.1 | 2.1 | -0.45% |  |  |  |  |  |  |
| Class D | 2.0 | 2.0 | -0.70% |  |  |  |  |  |  |
| Class E | 3.0 | 3.0 | -1.01% |  |  |  |  |  |  |
| Class F | 5.7 | 5.7 | -0.68% |  |  |  |  |  |  |
| TGM | 12.5 | 12.5 | -0.58% |  |  |  |  |  |  |
| Overall | 2.2 | 2.2 | -0.53% |  |  |  |  |  |  |
| Enc Time[%] | 97% |  |  |  |  |  |  |  |  |
| Dec Time[%] | 98% |  |  |  |  |  |  |  |  |