| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0079-v1 |
| --- | --- |

| Title: | CE7: Modifications to transform-skip residual coding (CE7-1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Hongtao Wang, Yung-Hsuan Chao, Muhammed Coban (Qualcomm) | Tel: Email: | +1 858 658 5673 martak@qti.qualcomm.com hongtaow@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE7-1.2. In which, several modifications to the coefficient coding of transform-skip (TS) mode was proposed. There are 4 sub tests: In CE7-1.2a, the coefficient coding of TS mode was performed in a 2-pass manner. In CE7-1.2b, the parity flag was moved to the end of the 2nd pass of TS coefficient coding, i.e. after all the gtx flags. Test CE7-1.2c modifies binarization and encoding of the coefficients after the context bin limit is hit on top of CE7-1.2a, so that the coefficients were coded without been split into 8 bypass bins + remainder as previously done. And in CE7-1.2d, the modification to the binarization and encoding of coefficients after reaching the context bin limit is applied to the 3-pass TS coefficient coding in the current VVC design.

The results of the tests are summarized as follows.

CTC QP set

- CE7-1.2a: ClassF: 0.02% AI, -0.01% RA, -0.03% LB, ClassTGM 0.01% AI, 0.02% RA, -0.02%LB

- CE7-1.2b: ClassF: -0.05% AI, -0.02% RA, 0.03% LB, ClassTGM -0.05% AI, -0.09% RA, -0.04% LB

- CE7-1.2c: ClassF: -0.03% AI, -0.07% RA, 0.08% LB, ClassTGM -0.04% AI, -0.07% RA, -0.06% LB

- CE7-1.2c Alternative method: ClassF: -0.01% AI, 0.00% RA, -0.08% LB, ClassTGM 0.06% AI, 0.00% RA, -0.02% LB

- CE7-1.2d: ClassF: -0.05% AI, -0.03% RA, -0.15%LB, ClassTGM -0.15% AI, -0.10% RA, -0.10% LB

- CE7-1.2d: Alternative method: ClassF: -0.06% AI, -0.03% RA, 0.05% LB, ClassTGM -0.12% AI, -0.09% RA, -0.06% LB

Low QP

- CE7-1.2a: ClassF: -0.10% AI, -0.11% RA, -0.16% LB, ClassTGM -0.31% AI, -0.20% RA, -0.30% LB

- CE7-1.2b: ClassF: -0.06% AI, -0.13% RA, -0.13% LB, ClassTGM -0.20% AI, -0.27% RA, -0.14% LB

- CE7-1.2c: ClassF: -1.02% AI, -0.99% RA, -0.48% LB, ClassTGM -1.82% AI, -2.26% RA, -1.33% LB

- CE7-1.2c Alternative method: ClassF: -1.47% AI, -1.83% RA, -0.67% LB, ClassTGM -1.53% AI, -2.15% RA, -1.05% LB

- CE7-1.2d: ClassF: -1.33% AI, -1.28% RA, -0.50% LB, ClassTGM -2.03% AI, -2.56% RA, -1.33% LB

- CE7-1.2d: Alternative method: ClassF: -1.56% AI, -2.00% RA, -0.68% LB, ClassTGM -1.97% AI, -2.63% RA, -1.31%LB

# CE Description

## CE7-1.2a 2-pass TS coefficient coding

In this sub CE test, the coefficients of TS mode were encoded in a 2-pass manner: In the first pass, sig_coeff_flag, coeff_sign_flag, abs_level_gt1_flag, par_level_flag, abs_level_gtX_flags are coded, in the second pass, the remainder is coded. If the context bin limit is reached in the 1st pass, all the syntax elements will be bypass coded.

## CE7-1.2b repositioning of the parity flag

In this sub CE test, the par_level_flag was moved to the 2nd pass of the coefficient coding, the par_level_flag is located after all the abs_level_gtX_flags.

## CE7-1.2c Rice-Golomb coding after bypass switch for 2-pass scan

In this sub CE, before hitting the context bin limit, a coefficient is encoded in the same mays as that in CE7-1.2a. More specifically, If the abs_level_gt1_flag of the coefficient should be encoded as a bypass bin according to the method in CE7-1.2a, then in this sub CE, the abs(level) – 1 of the coefficient will no longer be split into abs_level_gt1_flag, par_level_flag, abs_level_gtX_flags and abs_remainder, instead of that, the abs(level) – 1 will be coded as a remainder via Rice-Golomb coding. Otherwise, the coefficient is coded in the same was as that in CE7-1.2a.

The rice parameter table was designed as follows:

A table of 32 entries are used for rice parameter derivation:

auiGoRicePars[32] = {0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 3, 3, 3, 3, 3, 3, 3, 3, 3, 3}

For the remainder after abs_level_gtX_flags, the parameter auiGoRicePars[ std::min(std::max(sum -8, 0), 31) ]; was used.

For the remainder that with the value abs(level) – 1. The value auiGoRicePars[std::min(sum + 4, 31)] was used.

### An alternative method of Rice-Golomb coding after bypass switch(2-pass)

An alternative method of Rice-Golomb coding was tested. In the first pass, when there’s no context bin left for a coefficient, the value of abs(level) was encoded directly by Rice-Golomb coding and followed by a bypass coded coeff_sign_flag.

The same rice parameter table as described in 1.3 was used in this case except that the rice parameter is derived as follows:

auiGoRicePars[std::min(sum, 31)]

In the case of the significance of the coefficient is inferred (the last coefficient is the only non-zero coefficient in the coefficient group), the value of abs(level) – 1 instead of abs(level) is coded. The rice parameter is derived based on the same table:

auiGoRicePars[std::min(sum+4, 31)]

## CE7-1.2d Rice-Golomb coding after bypass switch for 3-pass scan

In this sub CE, the Rice-Golomb method described in 1.3 was applied to the 3-pass scan method of the current VVC design.

The sig_coeff_flag and the coeff_sign_flag are coded in the 1st pass. And abs_level_gt1_flag, par_level_flag, abs_level_gtX_flags in the 2nd pass. For the bins encoded in the 1st pass. Bypass are used if context bin limit is hit. For the bins encoded in the 2nd pass. If non of the bins can be encoded via a context bin, then these components were not coded in the 2nd pass and the value of abs(level) – 1 is coded via Rice-Golomb coding in the 3rd pass.

The same method of deriving rice parameters as that was described in 1.3 was used.

### An alternative method of Rice-Golomb coding after bypass switch (3-pass)

An alternative method of Rice-Golomb coding was tested for the 3-pass case. Similar to the method that are described in 1.3.1, if there’s no context bin that can be used for any of the component of a coefficient, the abs(level) is coded using Rice-Golomb coding and followed by a bypass coded sign flag. Otherwise, for a coefficient, if one of more bins in the 1st pass is context coded but there’s no context bin for any component in the 2nd pass. abs(level) – 1 is coded similar to.

The way to derive the rice parameters are derived by the same ways that were described in 1.3 and 1.3.1:

- Remainder after abs_level_gtX_flags

  - auiGoRicePars[ std::min(std::max(sum -8, 0), 31) ]

- abs(level) – 1

  - auiGoRicePars[std::min(sum + 4, 31)]

- abs(level)

  - auiGoRicePars[std::min(sum, 31)]

# Experimental results

## CE7-1.2a: 2-pass TS coefficient coding

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 1 and table 2.

Table 1 Simulation results of 2-pass coefficient coding on VTM-6.0, QP=22,27,32,37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.02% | 100% | 101% |
| Class A2 | 0.00% | -0.02% | 0.03% | 101% | 101% |
| Class B | 0.00% | 0.01% | 0.01% | 101% | 100% |
| Class C | -0.01% | -0.04% | 0.01% | 101% | 99% |
| Class E | 0.01% | 0.03% | -0.05% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.01% | -0.01% | -0.09% | 101% | 98% |
| Class F | 0.02% | 0.00% | -0.13% | 100% | 98% |
| TGM | 0.01% | 0.06% | 0.05% | 101% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.02% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | 0.03% | 101% | 100% |
| Class B | 0.00% | 0.15% | 0.13% | 101% | 100% |
| Class C | 0.01% | 0.07% | -0.01% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.06% | 0.05% | 101% | 100% |
| Class D | -0.04% | -0.24% | -0.20% | 101% | 101% |
| Class F | -0.01% | 0.02% | 0.13% | 102% | 101% |
| TGM | 0.02% | 0.02% | 0.01% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.03% | -0.42% | 99% | 100% |
| Class C | 0.00% | 0.10% | -0.09% | 101% | 98% |
| Class E | -0.10% | 0.20% | -0.08% | 102% | 99% |
| Overall | -0.03% | 0.07% | -0.22% | 101% | 99% |
| Class D | -0.03% | 0.02% | -0.24% | 103% | 100% |
| Class F | 0.06% | -0.25% | 0.28% | 102% | 99% |
| TGM | -0.02% | -0.02% | 0.06% | 99% | 101% |

Table 2 Simulation results of 2-pass coefficient coding on VTM-6.0, QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.01% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | -0.01% | 0.02% | 0.03% | 100% | 101% |
| Class E | -0.01% | -0.05% | -0.02% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.01% | 100% | 100% |
| Class D | -0.01% | -0.01% | 0.03% | 100% | 97% |
| Class F | -0.10% | -0.07% | -0.06% | 100% | 99% |
| TGM | -0.31% | -0.19% | -0.19% | 99% | 102% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class C | 0.00% | -0.02% | 0.00% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -0.05% | -0.10% | 100% | 100% |
| Class F | -0.11% | -0.16% | -0.13% | 99% | 100% |
| TGM | -0.20% | -0.21% | -0.26% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class C | 0.00% | -0.01% | -0.03% | 99% | 95% |
| Class E | -0.01% | -0.01% | 0.02% | 99% | 101% |
| Overall | -0.01% | -0.01% | -0.01% | 100% | 99% |
| Class D | -0.01% | 0.04% | -0.04% | 100% | 97% |
| Class F | -0.16% | -0.19% | -0.17% | 100% | 97% |
| TGM | -0.30% | -0.20% | -0.16% | 100% | 98% |

## CE7-1.2b: re-positioning of the parity flag

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 3 and Table 4.

Table 3 Simulation results of parity repositioning on VTM-6.0, QP=22,27,32,37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class C | 0.00% | -0.05% | 0.04% | 101% | 101% |
| Class E | -0.01% | 0.04% | -0.04% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | -0.02% | -0.08% | 0.08% | 101% | 102% |
| Class F | -0.05% | -0.05% | 0.07% | 101% | 101% |
| TGM | -0.05% | -0.05% | -0.05% | 101% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.02% | 99% | 99% |
| Class A2 | 0.02% | 0.09% | 0.01% | 99% | 100% |
| Class B | 0.00% | 0.02% | 0.06% | 102% | 100% |
| Class C | 0.01% | 0.03% | -0.02% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.02% | 101% | 100% |
| Class D | -0.03% | -0.24% | 0.02% | 102% | 100% |
| Class F | -0.02% | 0.01% | 0.02% | 102% | 100% |
| TGM | -0.09% | -0.03% | -0.09% | 102% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | 0.40% | -0.06% | 98% | 100% |
| Class C | -0.02% | -0.04% | -0.22% | 101% | 98% |
| Class E | -0.02% | -0.65% | -0.53% | 102% | 100% |
| Overall | -0.02% | -0.01% | -0.23% | 100% | 99% |
| Class D | 0.03% | -0.41% | 0.16% | 101% | 99% |
| Class F | 0.03% | 0.11% | 0.90% | 101% | 98% |
| TGM | -0.04% | -0.02% | 0.04% | 98% | 102% |

Table 4 Simulation results of parity flag re-positioning on VTM-6.0, QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | 0.00% | 0.00% | -0.01% | 99% | 97% |
| Class C | 0.00% | -0.01% | 0.03% | 99% | 99% |
| Class E | 0.01% | -0.01% | 0.01% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | -0.02% | -0.01% | 99% | 97% |
| Class F | -0.06% | -0.08% | -0.07% | 99% | 96% |
| TGM | -0.20% | -0.18% | -0.18% | 99% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.01% | 0.01% | 99% | 99% |
| Class C | -0.01% | -0.03% | -0.01% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | -0.01% | 0.07% | 99% | 98% |
| Class F | -0.13% | -0.07% | -0.01% | 98% | 99% |
| TGM | -0.27% | -0.16% | -0.16% | 98% | 96% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.01% | 99% | 100% |
| Class C | -0.01% | 0.02% | -0.02% | 98% | 96% |
| Class E | 0.00% | 0.02% | 0.00% | 98% | 97% |
| Overall | 0.00% | 0.01% | -0.01% | 99% | 98% |
| Class D | -0.02% | -0.01% | -0.01% | 99% | 98% |
| Class F | -0.13% | -0.04% | -0.04% | 99% | 97% |
| TGM | -0.14% | -0.18% | -0.08% | 99% | 100% |

## CE7-1.2c: 2-pass scan with modified rice-golomb coding

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 5 and Table 6.

Table 5 Simulation results of 2-pass scan with rice-golomb change on VTM-6.0, QP=22,27,32,37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | -0.02% | 0.02% | 100% | 103% |
| Class B | 0.00% | 0.05% | 0.00% | 101% | 101% |
| Class C | 0.01% | -0.11% | 0.01% | 101% | 102% |
| Class E | 0.00% | 0.04% | -0.07% | 101% | 100% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class D | -0.01% | 0.06% | 0.01% | 101% | 99% |
| Class F | -0.03% | 0.04% | -0.05% | 99% | 98% |
| TGM | -0.04% | 0.02% | 0.00% | 99% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.03% | 101% | 100% |
| Class A2 | 0.02% | -0.04% | 0.06% | 100% | 99% |
| Class B | 0.00% | 0.09% | 0.01% | 101% | 100% |
| Class C | 0.00% | 0.00% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.00% | 101% | 100% |
| Class D | -0.05% | -0.30% | -0.08% | 101% | 99% |
| Class F | -0.07% | -0.10% | -0.06% | 101% | 100% |
| TGM | -0.07% | -0.05% | -0.05% | 102% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.03% | 100% | 100% |
| Class C | -0.07% | -0.04% | -0.17% | 101% | 99% |
| Class E | -0.14% | 0.25% | 0.31% | 101% | 102% |
| Overall | -0.06% | 0.07% | 0.01% | 100% | 100% |
| Class D | 0.00% | 0.05% | 0.07% | 101% | 98% |
| Class F | 0.08% | 0.07% | 0.08% | 100% | 100% |
| TGM | -0.06% | -0.02% | -0.05% | 99% | 98% |

Table 6 Simulation results of 2-pass scan with rice-golomb change on VTM-6.0 QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | -0.01% | 99% | 96% |
| Class B | -0.02% | 0.00% | 0.00% | 99% | 97% |
| Class C | -0.03% | -0.03% | 0.00% | 99% | 99% |
| Class E | -0.01% | 0.00% | 0.02% | 99% | 99% |
| Overall | -0.01% | -0.01% | 0.00% | 99% | 98% |
| Class D | -0.03% | -0.02% | -0.02% | 99% | 97% |
| Class F | -1.02% | -0.54% | -0.49% | 98% | 95% |
| TGM | -1.82% | -1.47% | -1.47% | 97% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | -0.02% | 100% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.01% | 0.01% | 98% | 99% |
| Class C | -0.02% | -0.03% | -0.02% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | -0.03% | 0.02% | 99% | 99% |
| Class F | -0.99% | -0.63% | -0.50% | 98% | 98% |
| TGM | -2.26% | -1.29% | -1.28% | 98% | 96% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | 0.00% | 99% | 100% |
| Class C | -0.02% | 0.00% | -0.02% | 99% | 96% |
| Class E | -0.01% | 0.00% | 0.01% | 98% | 97% |
| Overall | -0.01% | 0.00% | -0.01% | 99% | 98% |
| Class D | -0.03% | 0.03% | -0.05% | 99% | 97% |
| Class F | -0.48% | -0.41% | -0.38% | 98% | 96% |
| TGM | -1.33% | -1.06% | -1.00% | 97% | 99% |

### The alternative method tested in CE7-1.2c

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 7 and Table 8.

Table 7 Simulation results of 2-pass scan with alternative rice-golomb change on VTM-6.0, QP=22, 27, 32, 37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.04% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.03% | 101% | 100% |
| Class C | -0.01% | -0.05% | 0.04% | 101% | 102% |
| Class E | 0.00% | 0.03% | -0.08% | 101% | 102% |
| Overall | 0.00% | 0.00% | -0.02% | 101% | 101% |
| Class D | 0.00% | -0.02% | -0.09% | 101% | 100% |
| Class F | -0.01% | 0.05% | -0.07% | 101% | 99% |
| TGM | 0.06% | 0.12% | 0.09% | 101% | 97% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 100% | 99% |
| Class A2 | 0.01% | -0.01% | 0.02% | 100% | 100% |
| Class B | 0.00% | 0.10% | 0.08% | 101% | 100% |
| Class C | -0.02% | 0.08% | -0.06% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.02% | 101% | 100% |
| Class D | -0.06% | -0.34% | -0.05% | 101% | 99% |
| Class F | 0.00% | -0.11% | 0.10% | 101% | 100% |
| TGM | 0.00% | 0.02% | 0.05% | 102% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.20% | 0.02% | 100% | 98% |
| Class C | -0.02% | -0.09% | -0.18% | 100% | 99% |
| Class E | -0.17% | 0.30% | -0.21% | 101% | 101% |
| Overall | -0.04% | 0.13% | -0.11% | 100% | 99% |
| Class D | 0.05% | -0.05% | 0.64% | 102% | 101% |
| Class F | -0.08% | 0.33% | 0.25% | 100% | 98% |
| TGM | -0.02% | 0.01% | 0.03% | 99% | 100% |

Table 8 Simulation results of 2-pass scan with alternative rice-golomb change on VTM-6.0 QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class B | -0.03% | -0.01% | 0.00% | 100% | 98% |
| Class C | -0.07% | -0.05% | -0.02% | 99% | 100% |
| Class E | -0.06% | -0.09% | -0.05% | 99% | 98% |
| Overall | -0.03% | -0.03% | -0.01% | 99% | 98% |
| Class D | -0.07% | -0.07% | -0.11% | 99% | 97% |
| Class F | -1.47% | -0.65% | -0.64% | 99% | 94% |
| TGM | -1.53% | -1.15% | -1.14% | 99% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | -0.02% | 100% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02% | 0.02% | 0.01% | 99% | 98% |
| Class C | -0.04% | -0.01% | -0.01% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.07% | -0.16% | -0.07% | 99% | 99% |
| Class F | -1.83% | -0.60% | -0.54% | 98% | 97% |
| TGM | -2.15% | -0.97% | -0.98% | 98% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.02% | 100% | 100% |
| Class C | -0.03% | -0.01% | -0.04% | 99% | 97% |
| Class E | -0.07% | 0.03% | 0.06% | 98% | 99% |
| Overall | -0.03% | 0.00% | -0.01% | 99% | 99% |
| Class D | -0.07% | -0.02% | -0.06% | 99% | 97% |
| Class F | -0.67% | -0.47% | -0.51% | 99% | 98% |
| TGM | -1.05% | -0.68% | -0.62% | 98% | 99% |

## CE7-1.2d: 3-pass scan with modified rice-golomb coding

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 9 and Table 10.

Table 9 Simulation results of 3-pass scan with rice-golomb change on VTM-6.0, QP=22,27,32,37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | 0.03% | 101% | 101% |
| Class B | 0.00% | -0.02% | -0.04% | 101% | 101% |
| Class C | 0.00% | -0.07% | 0.07% | 101% | 101% |
| Class E | -0.01% | 0.06% | -0.06% | 100% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class D | -0.02% | 0.02% | 0.05% | 101% | 99% |
| Class F | -0.05% | -0.05% | -0.15% | 99% | 99% |
| TGM | -0.15% | -0.11% | -0.12% | 99% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.02% | 101% | 98% |
| Class A2 | 0.02% | 0.04% | 0.04% | 100% | 99% |
| Class B | -0.01% | 0.07% | -0.02% | 101% | 100% |
| Class C | 0.00% | -0.02% | 0.02% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.01% | 101% | 99% |
| Class D | -0.03% | -0.24% | 0.05% | 101% | 99% |
| Class F | -0.03% | -0.11% | -0.04% | 101% | 100% |
| TGM | -0.10% | -0.07% | -0.12% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.08% | -0.23% | 100% | 100% |
| Class C | -0.02% | 0.11% | -0.05% | 100% | 98% |
| Class E | -0.01% | 0.09% | -0.52% | 101% | 103% |
| Overall | -0.01% | 0.09% | -0.24% | 100% | 100% |
| Class D | 0.05% | 0.06% | 1.00% | 101% | 100% |
| Class F | -0.15% | -0.03% | 0.15% | 100% | 98% |
| TGM | -0.10% | -0.06% | -0.04% | 99% | 100% |

Table 10 Simulation results of 3-pass scan with rice-golomb change on VTM-6.0 QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.00% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | -0.01% | 99% | 97% |
| Class B | -0.03% | -0.01% | 0.00% | 99% | 97% |
| Class C | -0.08% | -0.02% | -0.02% | 99% | 99% |
| Class E | 0.03% | 0.00% | 0.04% | 99% | 99% |
| Overall | -0.02% | -0.01% | 0.00% | 99% | 98% |
| Class D | -0.12% | -0.12% | -0.08% | 99% | 96% |
| Class F | -1.33% | -0.71% | -0.71% | 97% | 94% |
| TGM | -2.03% | -1.67% | -1.66% | 97% | 98% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 100% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.00% | -0.02% | 99% | 98% |
| Class C | -0.02% | -0.01% | -0.01% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.04% | -0.07% | -0.05% | 99% | 100% |
| Class F | -1.28% | -0.78% | -0.80% | 98% | 97% |
| TGM | -2.56% | -1.32% | -1.30% | 98% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.01% | 0.00% | 99% | 98% |
| Class C | -0.02% | 0.01% | 0.00% | 99% | 95% |
| Class E | 0.01% | -0.04% | -0.03% | 99% | 99% |
| Overall | -0.01% | 0.00% | -0.01% | 99% | 98% |
| Class D | -0.02% | -0.01% | -0.08% | 99% | 99% |
| Class F | -0.50% | -0.41% | -0.54% | 98% | 98% |
| TGM | -1.33% | -1.17% | -1.15% | 97% | 99% |

### The alternative method tested in CE7-1.2d

The proposed method is implemented on top of VTM6.0 and evaluated for the intra-only (AI), random-access (RA) and low delay B slice (LB) configurations. IBC, HashME, and RDPCM are turned on only for class F and class TGM. Simulation results for standard QPs (22, 27, 32, 37) and low QPs (2, 7, 12, 17) are shown in Table 11 and Table 12.

Table 11 Simulation results of 3-pass scan with the alternative rice-golomb change on VTM-6.0, QP=22,27,32,37

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | 0.02% | 101% | 102% |
| Class B | 0.00% | -0.01% | -0.03% | 102% | 100% |
| Class C | -0.01% | -0.07% | 0.05% | 102% | 100% |
| Class E | -0.01% | 0.07% | -0.05% | 101% | 102% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 100% |
| Class D | -0.01% | 0.02% | 0.05% | 102% | 99% |
| Class F | -0.06% | -0.04% | -0.14% | 101% | 98% |
| TGM | -0.12% | -0.11% | -0.11% | 101% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | 0.01% | 100% | 100% |
| Class A2 | 0.02% | 0.03% | 0.05% | 100% | 100% |
| Class B | 0.00% | 0.04% | -0.05% | 101% | 101% |
| Class C | -0.01% | 0.06% | -0.11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | -0.04% | 101% | 100% |
| Class D | -0.03% | -0.38% | 0.05% | 101% | 101% |
| Class F | -0.10% | -0.08% | -0.15% | 101% | 101% |
| TGM | -0.09% | -0.11% | -0.09% | 102% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.18% | -0.14% | 100% | 100% |
| Class C | 0.01% | 0.13% | -0.13% | 101% | 98% |
| Class E | -0.01% | 0.21% | -0.47% | 101% | 101% |
| Overall | 0.00% | 0.17% | -0.22% | 100% | 99% |
| Class D | -0.02% | 0.09% | 0.25% | 101% | 99% |
| Class F | 0.05% | -0.12% | 0.29% | 100% | 98% |
| TGM | -0.06% | -0.01% | -0.04% | 98% | 102% |

Table 12 Simulation results of 3-pass scan with the alternative rice-golomb change on VTM-6.0 QP=2,7,12,17

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | -0.01% | 100% | 99% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 97% |
| Class B | -0.03% | -0.01% | -0.01% | 100% | 98% |
| Class C | -0.09% | -0.04% | -0.04% | 99% | 100% |
| Class E | 0.00% | -0.06% | -0.03% | 100% | 99% |
| Overall | -0.03% | -0.02% | -0.02% | 100% | 98% |
| Class D | -0.14% | -0.09% | -0.09% | 100% | 97% |
| Class F | -1.56% | -0.78% | -0.80% | 99% | 95% |
| TGM | -1.97% | -1.61% | -1.61% | 98% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.00% | 0.00% | 99% | 99% |
| Class C | -0.04% | -0.03% | -0.04% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | -0.07% | -0.09% | 99% | 99% |
| Class F | -2.00% | -0.78% | -0.67% | 98% | 97% |
| TGM | -2.63% | -1.31% | -1.35% | 98% | 97% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.00% | -0.01% | 99% | 99% |
| Class C | -0.02% | -0.03% | -0.03% | 98% | 97% |
| Class E | 0.00% | 0.00% | -0.06% | 99% | 99% |
| Overall | -0.01% | -0.01% | -0.03% | 99% | 98% |
| Class D | -0.07% | -0.02% | -0.08% | 99% | 97% |
| Class F | -0.68% | -0.62% | -0.57% | 99% | 97% |
| TGM | -1.31% | -1.01% | -0.94% | 98% | 98% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).