| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0083 |
| --- | --- |

| Title: | CE1-3: Reference picture resampling filters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jiancong Luo Mohammed Golam Sarwer Yan Ye | Tel: Email: | +1 862 596 2086 jiancong.luo@alibaba-inc.com |
| Source: | Alibaba Group Inc. |  |  |

_____________________________

# Abstract

This document reports the results of core experiment (CE) 1 test 3 on using SHM down-sampling filters for motion compensation (MC) interpolation when the reference picture is of higher resolution than the current picture. The test results reportedly show that compared to the CE1 anchors which use existing motion compensation interpolation filter for reference down-sampling, average BD rate savings of PSNR1 are {-6.47%, -4.55%, -4.62%} for ratio 2:1 and {-3.13%, -2.50%, -2.28%} for ratio 1.5:1, and average BD rate savings of PSNR2 are {-3.98%, -2.67%, -3.00%} for ratio 2:1 and {-2.23%, -2.21%, -1.87%} for ratio 1.5:1.

# Introduction

The contribution JVET-O0641 proposed to use SHM down-sampling filters for reference picture down-sampling. In CE1 test 3, motion compensation interpolation filtering is performed as described below:

- If the reference picture is of higher resolution than the current picture, the motion compensation interpolation uses 12-tap SHM down-sampling filters. The selection of filters is based on the down-sampling ratio as in SHM-12.4. The same filter is used for both luma and chroma components. Table 1 and 2 show the filter coefficients for down-sample ratio 2:1 and 1.5:1.

- Otherwise (the resolution of reference picture is equal or smaller than that of the current picture), the motion compensation interpolation uses existing motion compensation interpolation filters which is the same as in VVC draft 6.

Table 1: interpolation filter coefficients for 2:1 reference down-sampling

| Fractional sample position p | interpolation filter coefficients for 2:1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 2 | -3 | -9 | 6 | 39 | 58 | 39 | 6 | -9 | -3 | 2 | 0 |
| 1 | 2 | -3 | -9 | 4 | 38 | 58 | 43 | 7 | -9 | -4 | 1 | 0 |
| 2 | 2 | -2 | -9 | 2 | 35 | 58 | 44 | 9 | -8 | -4 | 1 | 0 |
| 3 | 1 | -2 | -9 | 1 | 34 | 58 | 46 | 11 | -8 | -5 | 1 | 0 |
| 4 | 1 | -1 | -8 | -1 | 31 | 57 | 47 | 13 | -7 | -5 | 1 | 0 |
| 5 | 1 | -1 | -8 | -2 | 29 | 56 | 49 | 15 | -7 | -6 | 1 | 1 |
| 6 | 1 | 0 | -8 | -3 | 26 | 55 | 51 | 17 | -7 | -6 | 1 | 1 |
| 7 | 1 | 0 | -7 | -4 | 24 | 54 | 52 | 19 | -6 | -7 | 1 | 1 |
| 8 | 1 | 0 | -7 | -5 | 22 | 53 | 53 | 22 | -5 | -7 | 0 | 1 |
| 9 | 1 | 1 | -7 | -6 | 19 | 52 | 54 | 24 | -4 | -7 | 0 | 1 |
| 10 | 1 | 1 | -6 | -7 | 17 | 51 | 55 | 26 | -3 | -8 | 0 | 1 |
| 11 | 1 | 1 | -6 | -7 | 15 | 49 | 56 | 29 | -2 | -8 | -1 | 1 |
| 12 | 0 | 1 | -5 | -7 | 13 | 47 | 57 | 31 | -1 | -8 | -1 | 1 |
| 13 | 0 | 1 | -5 | -8 | 11 | 46 | 58 | 34 | 1 | -9 | -2 | 1 |
| 14 | 0 | 1 | -4 | -8 | 9 | 44 | 58 | 35 | 2 | -9 | -2 | 2 |
| 15 | 0 | 1 | -4 | -9 | 7 | 43 | 58 | 38 | 4 | -9 | -3 | 2 |

Table 2: interpolation filter coefficients for 1.5:1 down-sampling

| Fractional sample position p | interpolation filter coefficients for 1.5 : 1 downsampling |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | p0 | p1 | p2 | p3 | p4 | p5 | p6 | p7 | p8 | p9 | p10 | p11 |
| 0 | 0 | 5 | -6 | -10 | 37 | 76 | 37 | -10 | -6 | 5 | 0 | 0 |
| 1 | 0 | 5 | -4 | -11 | 33 | 76 | 40 | -9 | -7 | 5 | 0 | 0 |
| 2 | -1 | 5 | -3 | -12 | 29 | 75 | 45 | -7 | -8 | 5 | 0 | 0 |
| 3 | -1 | 4 | -2 | -13 | 25 | 75 | 48 | -5 | -9 | 5 | 1 | 0 |
| 4 | -1 | 4 | -1 | -13 | 22 | 73 | 52 | -3 | -10 | 4 | 1 | 0 |
| 5 | -1 | 4 | 0 | -13 | 18 | 72 | 55 | -1 | -11 | 4 | 2 | -1 |
| 6 | -1 | 4 | 1 | -13 | 14 | 70 | 59 | 2 | -12 | 3 | 2 | -1 |
| 7 | -1 | 3 | 1 | -13 | 11 | 68 | 62 | 5 | -12 | 3 | 2 | -1 |
| 8 | -1 | 3 | 2 | -13 | 8 | 65 | 65 | 8 | -13 | 2 | 3 | -1 |
| 9 | -1 | 2 | 3 | -12 | 5 | 62 | 68 | 11 | -13 | 1 | 3 | -1 |
| 10 | -1 | 2 | 3 | -12 | 2 | 59 | 70 | 14 | -13 | 1 | 4 | -1 |
| 11 | -1 | 2 | 4 | -11 | -1 | 55 | 72 | 18 | -13 | 0 | 4 | -1 |
| 12 | 0 | 1 | 4 | -10 | -3 | 52 | 73 | 22 | -13 | -1 | 4 | -1 |
| 13 | 0 | 1 | 5 | -9 | -5 | 48 | 75 | 25 | -13 | -2 | 4 | -1 |
| 14 | 0 | 0 | 5 | -8 | -7 | 45 | 75 | 29 | -12 | -3 | 5 | -1 |
| 15 | 0 | 0 | 5 | -7 | -9 | 40 | 76 | 33 | -11 | -4 | 5 | 0 |

# Simulation results

The proposed filters are implemented on top of VTM-6.1. Test condition described in JVET-O2021 is used in the simulation. The anchor of the comparison is JVET-O1164, which uses the existing motion compensation filters for reference picture down-sampling. Both results of PSNR1 (codec PSNR measured between the input of encoder and output of decoder at coding resolution by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE) and PSNR2 (end-to-end PSNR measured between the original video source and the up-sampled (if needed) decoded pictures using  the existing VTM interpolation filters) are reported. The simulation results are shown in Tables 3 - 6.

Table 3: Simulation results of PSNR1 for down-sample ratio 2:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.44% | -2.48% | -2.90% | 100% | 101% |
| Class C | -7.13% | -5.72% | -5.56% | 99% | 102% |
| Class E | -10.64% | -6.44% | -6.22% | 98% | 101% |
| Overall | -6.47% | -4.55% | -4.62% | 99% | 102% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 4: Simulation results of PSNR1 for down-sample ratio 1.5:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.40% | -1.21% | -1.39% | 100% | 101% |
| Class C | -4.11% | -3.70% | -3.61% | 98% | 101% |
| Class E | -4.70% | -3.06% | -1.98% | 99% | 101% |
| Overall | -3.13% | -2.50% | -2.28% | 99% | 101% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 5: Simulation results of PSNR2 for down-sample ratio 2:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.29% | -1.57% | -1.89% | 100% | 101% |
| Class C | -4.25% | -2.89% | -2.82% | 99% | 102% |
| Class E | -6.45% | -4.20% | -5.10% | 98% | 101% |
| Overall | -3.98% | -2.67% | -3.00% | 99% | 102% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

Table 6: Simulation results of PSNR2 for down-sample ratio 1.5:1

|  |  |  | Low delay B Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE1 anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.96% | -1.02% | -1.14% | 100% | 101% |
| Class C | -2.93% | -2.96% | -2.76% | 98% | 101% |
| Class E | -3.42% | -3.18% | -1.92% | 99% | 101% |
| Overall | -2.23% | -2.21% | -1.87% | 99% | 101% |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Alibaba Group Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).