| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0085-v1 |
| --- | --- |

| Title: | Bit Stream Analyzer for Coding Tool Statistics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Christian Herglotz Matthias Kränzler André Kaup | Tel: Email: | +49 9131 8527101 Christian.herglotz@fau.de |
| Source: | Multimedia Communications and Signal Processing, Friedrich-Alexander University Erlangen-Nürnberg (FAU) |  |  |

_____________________________

# Abstract

This contribution presents an analyzer add-on for the VTM decoder. It counts the use of certain coding tools used for reconstruction. The analyzer counts the occurrence of tools like intra prediction, transformations, residual coefficients, and in-loop filtering operations on slice level, block level, or pel level. The authors report that the tool is lightweight in terms of complexity and that the results of the analyzer can be used for further interpretation and assessment of coding tools.

# Introduction

With the development of the new video coding standard VVC, a large number of new coding modes and advanced coding tools was developed and potentially integrated into the standard. For evaluation, a common method is to assess the rate savings at a constant objective quality and the complexity increase for encoding and decoding of a certain set of sequences [1]. Making use of a visualizing software, it is possible to examine the use of coding modes on block level [2].

This contribution proposes to exploit another method for analyzing the use and efficiency of new coding modes. A software was developed that counts the occurrence of coding modes, tools, or other coding methods, which, in the following, are called features. The extension derives overall statistics on the occurrence of these features for further interpretation and assessment. To give an example, a feature can be the use of the affine coding mode for a certain CU block size. Another feature can be the number of non-zero residual coefficients.

The proposed tool is reported to be computationally lightweight in comparison with standard tracing and provides the following information:

- Occurrence of coding modes depending on block sizes

- Overall sequence-wise statistics

With the help of this information, the following benefits could be drawn:

- Information on the use of newly introduced coding modes.

- Information on the use of established coding modes, which might not or barely be used anymore.

- Investigation of the influence of external parameters (e.g., QP, encoder configuration) on the use of coding tools.

- Exploitation for energy modeling [3].

- Exploitation for decoding-energy optimal encoding [4].

# Bit Stream Analysis in Decoding

To implement the analyzer, the decoder of VTM-6.0 was extended with additional functionality. The main extension is a structure called featureCounter, which includes integer variables that count the occurrence of features. During decoding, after parsing and interpretation of the bit stream information, the variables are incremented if a certain feature occurs. After finishing the overall decoding process, the counted numbers are written to a Matlab-readable text file.

The current version of the analyzer provides information for all features listed in Table 1. The features comprise parsing, prediction, transformation, block size, and loop filter information

For further clarification, we distinguish features by a reference unit. Namely, features are counted on slice level (slice level features), CTB level (CTB features), block level (block features), on pel level (pel features), or on boundary level (boundary features). Block level features are counted depending on the block (mostly CU) dimension as explained in Section 2.1.

To perform feature counting during decoding, a command line flag must be set as

--visualizer=3,

which causes the decoder to write a Matlab-readable text file in the source folder of the bit stream.

## Block-Wise Feature Counting

Most of the features are counted block-wise, which reflects the multi-type tree coding structure implemented in VTM. As such, a large variety of block dimensions is allowed which potentially ranges from the smallest block including a single pel (size 1x1) up to CTU size of 128x128 pels. To simplify calculations and interpretations, the proposed analyzer considers all possible square and rectangular block sizes by using a matrix structure, where each entry corresponds to a single block size. The matrix has dimensions 8x8, which corresponds to block dimensions in powers of two. The resulting matrix in terms of block dimensions is defined as

,

where the first dimension indicates the width and the second dimension the height of the corresponding block. As in practice, not all block dimensions occur, some of the resulting matrices may be sparse. However, readability is ensured.

## Currently Implemented Features

The implemented features can be categorized according to the main components of a hybrid decoder. In this work, the categories GENERAL, PREDICTION, TRANSFORM, PARSING, and IN-LOOP filtering are considered. All features, depending on their category, are listed in Table 1.

Table 1 Features implemented in the VTM analyzer software.

| Category | Index | Feature name | Level |
| --- | --- | --- | --- |
| GENERAL | 1 | Islice | Slice |
|  | 2 | Pslice | Slice |
|  | 3 | Bslice | Slice |
|  | 4 | QP | Slice |
| PREDICTION | 5 | intraBlock | Block |
|  | 6 | intraPlanarLuma | Block |
|  | 7 | intraDCLuma | Block |
|  | 8 | intraHVDLuma | Block |
|  | 9 | intraAngLuma | Block |
|  | 10 | intraPlanarChroma | Block |
|  | 11 | intraDCChroma | Block |
|  | 12 | intraHVDChroma | Block |
|  | 13 | intraAngChroma | Block |
|  | 14 | intraCrossComp | Block |
|  | 15 | intraMIP | Block |
|  | 16 | intraPDPC | Block |
|  | 17 | intraSubPartitionsHor | Block |
|  | 18 | intraSubPartitionsVer | Block |
|  | 19 | interBlocks | Block |
|  | 20 | interInter | Block |
|  | 21 | interMerge | Block |
|  | 22 | interSkip | Block |
|  | 23 | interAffine | Block |
|  | 24 | interAffineInter | Block |
|  | 25 | interAffineMerge | Block |
|  | 26 | interSubblockMerge | Block |
|  | 27 | interTriangleSplit | Block |
|  | 28 | uniPredPel | Pel |
|  | 29 | biPredPel | Pel |
|  | 30 | fracPelHor | Pel |
|  | 31 | fracPelVer | Pel |
|  | 32 | fracPelBoth | Pel |
|  | 33 | copyPel | Pel |
| TRANSFORM | 34 | Transform | Block |
|  | 35 | TransformDCT2 Hor | Block |
|  | 36 | TransformDCT8 Hor | Block |
|  | 37 | TransformDST7 Hor | Block |
|  | 38 | TransformDCT2 Ver | Block |
|  | 39 | TransformDCT8 Ver | Block |
|  | 40 | TransformDST7 Ver | Block |
|  | 41 | TransformSkip | Block |
|  | 42 | TransformNoCBF | Block |
|  | 43 | TransformLFNST | Block |
| PARSING | 44 | nCoeff | Pel |
|  | 45 | coeffG1 | Pel |
|  | 46 | valueOfCoeff | Log-domain |
| IN-LOOP | 47 | Bs0 | Boundary |
|  | 48 | Bs1 | Boundary |
|  | 49 | SAOLumaEO | CTB |
|  | 50 | SAOLumaBO | CTB |
|  | 51 | SAOChromaEO | CTB |
|  | 52 | SAOChromaBO | CTB |
|  | 53 | ALFLuma | CTB |
|  | 54 | ALFChroma | CTB |

Features of the category GENERAL count slice level features like the number of I-, P-, and B-slices in the analyzed bit stream. The feature QP collects all slice-QPs in a vector of size 64, which reflects the allowed number of QPs.

Prediction features count features related to intraframe and interframe prediction. In the current implementation, all prediction modes are counted on CB-level. For example, intraBlock counts the number of intra-coded CBs, where luma and chroma CBs are considered separately. The terms “Planar”, “DC”, “HVD”, and “Ang” refer to the planar, DC, and angular intra modes, where “HVD” comprises the horizontal, the vertical, and the diagonal intra prediction modes. “Ang” includes the remaining angular modes.

It is worth mentioning that feature 5 (intraBlock) sums up all prediction modes covered with features 6-15. The other features related to intra (intraPDPC, intraSubPartitionsHor, and intraSubPartitionsVer) consider coding tools that are executed on top of the regular coding modes.

For inter prediction, the overall feature interBlocks counts all inter predicted blocks. Adding the features interBlocks with intraBlocks and multiplying with the number of pels per block (cf. the matrix from Section 2.1) returns all luma and chroma pels of the complete bit stream. interBlocks is further split into interInter, interMerge, interSkip, interAffine, interSubblockMerge, and interTriangleSplit. InterAffine is split into interAffineInter and interAffineMerge.

Feature indices 28 to 33 correspond to motion compensation on pel level, where unipredpel counts pels predicted from a single reference picture, bipredpel counts pels predicted from two reference pictures (bi-prediction), fracPelHor counts pels predicted from a horizontally fractional position, fracPelHor from a vertically fractional position, and fracPelBoth from a position that is fractional in both horizontal and vertical direction. Finally, copyPel corresponds to the number of pels that do not have to be filtered by  fractional pel filtering. The sum of unipredpel and bipredpel as well as the sum of fracPelHor, fracPelVer, fracPelBoth, and copyPel yield the same value as the overall number of inter predicted pels.

The TRANSFORM features comprise features related to transformation and are counted on TU level. The feature Transform counts all 2D-transforms that are used for reconstruction of the whole sequence. Similar to the prediction features, it is counted on block level. The features with the indices 35 to 40 indicate the use of one of the additional transforms in a certain direction. As such, adding up all horizontal transforms as well as adding up all vertical transforms leads to the feature value of the feature Transform.

The feature TransformSkip counts all blocks which are not transformed and the feature TransformNoCBF counts all blocks that have no significant coefficient. As a consequence, the sum of Transform, TransformSkip, and TransformNoCBF covers all pels of the considered bit stream. The feature TransformLFNST counts all additional low-frequency transforms.

As PARSING features, a few features are defined representing the residual coefficient decoding process. Therefore, the number of nonzero coefficients (nCoeff), the number of coefficients greater one (coeffG1), and the value of the coefficients is considered. To represent the number of bits needed to transmit the value of the coefficients, a simple approximation is used by adding all absolute values in the log2-domain (cf. [4]).

Finally, the IN-LOOP features consider tools related to in-loop filtering. The featuers Bs0 and Bs1 count block boundaries that are subject to deblocking filtering (Bs1 comprises the boundary strengths 1 and 2). The SAO features count the CTBs that are subject to SAO filtering, where band offset (BO) and edge offset (EO) are considered separately. The ALF features consider CTBs that filtered with the adaptive loop filter.

Finally, it shall be noted that the implemented list of features is not exhaustive in describing the overall decoding process. Further features considering coding tools like decoder-side motion vector derivation, bi-direction optical flow, intra block copy, or others could also be implemented. The authors claim that extending the code with new features is straightforward and simple as will be explained in the next subsection.

## Extension with New Features

To include new features for counting, the following steps must be undertaken:

- Add a new feature to the structure featureStruct in TypeDef.h.

- Set the initial value to zero.

- Increment counter after parsing (mostly in the function decompressCTU() in DecCu.cpp).

- Make a new output fprintf() in the if clause if (usePrunedFeatureSet == 0) in DecApp.cpp.

Afterwards, the newly introduced feature is printed to the Matlab-file output and can be further used for analysis.

## Complexity

To assess the complexity of the analysis, we encoded 26 sequences from the common test conditions for standard dynamic range (SDR) video with the four QPs 22, 27, 32, and 37 and the three encoder configurations intra, lowdelay, and randomaccess. The set is the same as used in [5]. Afterwards, we decoded the resulting 312 bit streams with both the conventional VTM-6.0 decoder and the decoder including the analyzer add-on. Processing was performed on an Intel i7-8700 CPU. On average, the decoder complexity increased by 2.79% where in general, the complexity increase is larger for intra-only coded sequences (4.61% on average).

# Applications

This section presents the use of the feature numbers for practical cases.

## Example

In this example, the PartyScene sequence is considered with 32 frames coded. The QP was set to 32, the encoder configuration was set to randomaccess. An excerpt of the resulting feature number output file is displayed in Figure 1.

For example, the number of I-slices and the number of B-slices adds up to the overall number of frames. The feature intraBlockSizes indicates that throughout the whole bit stream, 4692 CBs of size 8x2 were intra predicted (the entry at matrix position (2,4)). The feature interBlocks indicates that all in all, 81 CBs of size 128x128 were inter predicted (the index bottom right).

The feature uniPredPel indicates that 10.8 million pels were predicted from a single reference picture, which corresponds to 10.8 Mio / 32 / 832 / 480 / 1.5 = 56% of all pels. In comparison, bipredicted pels only account for 4.36% of all pels.

As a final example, the feature saoLumaBO yields 30, which means that 30 luma CTBs were filtered with the SAO band offset filter. All other features, including the features not displayed in this example, can be interpreted accordingly.

Figure 1 Excerpt from the result of bit stream analysis.

## Analysis of New Coding Tools

The feature numbers allow to calculate the overall share of pels that are affected by the features. An example shall visualize this property.

In the example, the use of the matrix intra prediction (MIP) mode is assessed. Two sequences are coded using the intra configuration, the first sequence with MIP switched on, the second sequence with MIP switched off. MIP is enabled and disabled using the encoder switch in the configuration file, VTM version 6.0 was used. Three frames of the PartyScene sequence were coded at a QP of 32. The first sequence (MIP on) has a Y-PSNR of 33.4695 dB at a bitrate of 2321.8833 kbps. The second sequence (MIP off) has a PSNR of 33.4559 dB and a bitrate of 2332.0667 kbps.

The use of the analyzer now allows to assess the frequency of occurrence of the intra coding modes. Therefore, Figure 2 shows pie-charts on the relative share of intra prediction modes for all luma pels of the bit stream.

Figure 2 Share of intra prediction modes  for the sequence PartyScene at a QP of 32.

The plot shows that MIP intra prediction mainly replaces the planar mode, while the shares of DC prediction and angular prediction remain approximately constant.

## Exploitation for Energy Modeling

As was already proposed in [3], the feature analysis can be used for decoder energy modeling. Using a large number of test sequences and corresponding energy measurements [5], linear energy coefficients can be trained and used for assessing the energy efficiency of certain coding tools. If training is adequately performed, the energy efficiency of different coding tools can be compared.

Furthermore, as was shown in [4], such an energy model can be used for decoding energy optimal encoding. A corresponding algorithm for decoding-energy-rate-distortion optimization (DERDO) in intra coding was presented in [6].

# Concluding Remarks

This contribution introduced an analyzer add-on for the VTM-decoder software. The software is reported to be lightweight in complexity. It enables to obtain detailed statistics on the use of compression tools such as prediction modes, transformation, coefficient coding, and in-loop filtering. For further assessment and in-depths investigations of coding tools, it is proposed to include the analyzer into the VTM-reference software.

# Patent rights declaration(s)

The Friedrich-Alexander University Erlangen-Nürnberg (FAU) does not have any current or pending patent rights relating to the technology described in this contribution.

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, March 2019.

[2] J. Sauer, J. Schneider, M. Bläser, M. Wien, “Reference software extension for coding block statistics”, JVET-K0149, July 2018.

[3] C. Herglotz, D. Springer, M. Reichenbach, B. Stabernack, and A. Kaup, „Modeling the energy consumption of the HEVC decoding process”, IEEE Transactions on Circuits and Systems for Video Technology, 28:217–229, Jan 2018.

[4] C. Herglotz, A. Heindel, and A. Kaup. Decoding energy distortion optimization for video coding. IEEE Transactions on Circuits and Systems for Video Technology, 29:171-182, Jan. 2019.

[5] M. Kränzler, C. Herglotz, A. Kaup, “Decoding Energy Assessment of VTM-6.0”, JVET-P0084, October 2019.

[6] C. Herglotz, M. Kränzler, A. Kaup, “Decoding-Energy-Rate-Distortion Optimization”, JVET-K0108, July 2018.