| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0090 |
| --- | --- |

| Title: | Non-CE4: 32 bits coding for abs_mvd_minus2 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Muhammed Coban, Hongtao Wang, Vadim Seregin, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 hanhuang, mcoban, hongtaow, vseregin, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes two solutions to limit the worst-case code length to 32 bits for coding the syntax element abs_mvd_minus2 in VVC. In one solution, the MVD resolution of 1/16 in affine AMVR is replace with 1/8, the Y, U, V bd-rate performance is -0.01%, 0.05%, -0.02% in random access configurations. In the other solution, a new binarization scheme is applied for abs_mvd_minus2, the bd-rate performance under common test condition is not changed.

# Introduction

In the JVET O meeting, extension of MVD range [1] to [-217, 217-1] was adopted. MVD value is coded with 2 context coded bins representing greater than 0 (abs_mvd_greater0_flag) and greater than 1 (abs_mvd_greater1_flag) followed by bypass coded absolute remaining value abs(MVD) – 2 (abs_mvd_minus2) if abs_mvd_greater1_flag is 1. Finally, the sign of the MVD is coded as single bypass coded bit. With 18-bit MVD values, the largest value of abs_mvd_minus2 can be 217-2 = 131070. Currently in VTM6.0, the remaining value abs_mvd_minus2 is coded with EG1 (exponential golomb code with order 1). The length of this max value coded using EG1 code is 34 bits (0x3fffc0000). However, limiting the worst-case code length to 32 bits is desirable for implementation in hardware as well as in software [2][3].

# Proposed

This contribution proposes two solutions to limit the worst-case code length to 32 bits for coding the syntax element abs_mvd_minus2.

In one solution, the MVD resolution of 1/16 in affine AMVR is replaced with 1/8, then the MVD range is changed to [-216, 216-1]. Therefore, EG1 coding of abs_mvd_minus2 is within 32 bits.

In the other solution, a new binarization scheme is applied for abs_mvd_minus2. For the value range from 0 to 131069, the EG1 is applied as in current VTM. For the value of 131070, it’s code by truncated unary prefix with 16 bit suffix, i.e. 1111 1111 1111 1111 0000 0000 0000 0000. The binarization of abs_mvd_minus2 is shown in Table 1. The bd-rate performance under common test condition is not changed.

Table 1 Binarization of abs_mvd_minus2

| Input Value | Codeword prefix | Codeword suffix | Prefix code length | Suffix code length | Total codeword length |
| --- | --- | --- | --- | --- | --- |
| [0,1] | 0 | x | 1 | 1 | 2 |
| [2,5] | 10 | xx | 2 | 2 | 4 |
| [6,13] | 110 | xxx | 3 | 3 | 6 |
| [14,29] | 1110 | xxxx | 4 | 4 | 8 |
| [30, 61] | 1 1110 | x xxxx | 5 | 5 | 10 |
| [62, 125] | 11 1110 | xx xxxx | 6 | 6 | 12 |
| [126, 253] | 111 1110 | xxx xxxx | 7 | 7 | 14 |
| [254, 509] | 1111 1110 | xxxx xxxx | 8 | 8 | 16 |
| [510, 1021] | 1 1111 1110 | x xxxx xxxx | 9 | 9 | 18 |
| … | … | … | … | … | … |
| [65534, 131069] | 1111 1111 1111 1110 | xxxx xxxx xxxx xxxx | 16 | 16 | 32 |
| [131070] | 1111 1111 1111 1111 | 0000 0000 0000 0000 | 16 | 16 | 32 |

# Test results

The test results for solution 1 under random access test condition is shown in Table 2. The overall Y, U, V, BD-rate changes are -0.01%, 0.05%, -0.02%. The affine AMVR is not enabled in low delay B common test condition. Therefore, no bd-rate performance change in low delay B. The test results of solution 2 are shown in Table 3. No bd-rate performance difference is observed.

Table 2 Test results of solution 1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.07% | -0.06% | 100% | 100% |
| Class A2 | -0.01% | 0.00% | 0.03% | 102% | 101% |
| Class B | -0.01% | 0.08% | -0.06% | 100% | 100% |
| Class C | -0.01% | 0.03% | 0.02% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.05% | -0.02% | 101% | 100% |
| Class D | -0.03% | 0.02% | 0.02% | 103% | 110% |
| Class F | -0.03% | -0.07% | 0.01% | 100% | 100% |

Table 3 Test results of solution 2

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 103% |
| Class B | 0.00% | 0.00% | 0.00% | 104% | 105% |
| Class C | 0.00% | 0.00% | 0.00% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 103% |
| Class F | 0.00% | 0.00% | 0.00% | 104% | 98% |
| TGM | 0.00% | 0.00% | 0.00% | 105% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 104% |
| Class E | 0.00% | 0.00% | 0.00% | 104% | 107% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 104% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 105% | 98% |
| TGM | 0.00% | 0.00% | 0.00% | 100% | 98% |

# Conclusion

The proposed ensure that all the syntax elements in VVC are coded within 32 bits. It’s suggested to adopt one of the proposed solutions.

# References

- Z. Deng, H. Liu, L. Zhang, K. Zhang, Y. Wang, “Non-CE4: Cleanups on MVD ranges”, JVET-O0567, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Gothenburg, SE, July 2019.

- M. Budagavi, V. Sze, “coeff_abs_level_remaining maximum codeword length reduction”, JCTVC-J0142, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Stockholm, SE, 11–20 July 2012.

- “CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements” JVET-M0470, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Marrakech, MA, Jan. 2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).