| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0091 |
| --- | --- |

| Title: | CE4-related: Simplification of PROF and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Wei-Jung Chien, and Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 | Tel: Email: | +1-858-845-4257 {hanhuang, wchien, martak}@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution proposes to move the right shift operation in refinement offset calculation of PROF and BDOF to the delta MV calculation that is outside the loop. Compared to VTM-6.0 anchors, it is reported that average BD-rate impacts of {0.00%, 0.04%, 0.05%} for RA configurations when the proposed method applies to BDOF, and {0.01%, 0.03%, -0.01%} for RA and {0.02%, 0.01%, 0.04%} for LDB when the proposed method applies to both BDOF and PROF.

# Introduction

In the current VTM, two optical flow based sample refinement methods, bidirectional optical flow (BDOF) and prediction refinement with optical flow (PROF), are applied to improve conventional motion compensation. The BDOF is applied to enable the motion refinement for bi-predicted coding units (CUs) that has both forward and backward predictions. In the BDOF, gradients and motion refinements are derived from the initial L0 and L1 prediction samples. The gradients are calculated based on one three-tap filter [-1, 0, 1] and the motion refinements are derived in the precision of 1/32-pel and clipped in the range of [-32, 32].

The horizontal and vertical commonent of motion refinement are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres,	
	−( sGxdI << 3 ) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres, ( ( sGydI << 3 ) − 	
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

The prediction refinement is performed as follows:

bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1	
	+ ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1

pbSamples[ x ][ y ] = Clip3( 0, ( 2bitDepth ) − 1, ( predSamplesL0[ x + 1 ][ y + 1 ]  + offset4 +	
	predSamplesL1[ x + 1 ][ y + 1 ] + bdofOffset )  >>  shift4 )

The PROF is applied to refine the prediction samples of affine mode. The PROF refinement process is similar to BDOF, but the motion refinements used by the PROF are derived from the affine control-point motion vectors (CPMVs). In the current design, the PROF motion refinement is derived in the precision of 1/64-pel and clipped to the range [-64, 63]. The prediction refinement is derived as follows:

dI = gradientH[ x ][ y ] * diffMv[ x ][ y ][ 0 ] + gradientV[ x ][ y ] * diffMv[ x ][ y ][ 1 ]

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 )

# Proposed

It’s proposed to remove the right shift operation in refinement offset calculation of PROF and BDOF. Instead, the right shift is performed on the derivation of motion refinements.

The prediction refinement offset calculation in BDOF is modified as follows:

bdofOffset = ( vx * ( gradientHL0[ x + 1 ][ y + 1 ] − gradientHL1[ x + 1 ][ y + 1 ] ) ) >> 1	
	+ ( vy * (gradientVL0[ x + 1 ][ y + 1 ] − gradientVL1[ x + 1 ][ y + 1 ] ) ) >> 1

The horizontal and vertical commonent of motion refinement in BDOF are derived as:

vx = sGx2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1,	
	−( sGxdI << 3 2) >> Floor( Log2( sGx2 ) ) )  :  0

vy = sGy2 > 0  ?  Clip3( −mvRefineThres, mvRefineThres-1, ( ( sGydI << 3 2 ) − 	
	( ( vx * sGxGym ) << 12 + vx * sGxGys ) >> 1 ) >> Floor( Log2( sGy2 ) ) )  :  0

The prediction refinement in PROF is modified as follows:

pbSamples[ x ][ y ] = predSamples[ x + 1 ][ y + 1 ] + ( ( dI + 1 ) >> 1 )

The derivation process for motion vector arrays from affine control point motion vectors is modified as follows:

- The variables sbWidth and sbHeight, dmvLimit, posOffsetX and posOffsetY are derived as follows:

sbWidth  =  cbWidth / numSbX		(8-729)

sbHeight  =  cbHeight / numSbY		(8-730)

dmvLimit  =  1 << Max( 65, BitDepthY − 67 )		(8-731)

posOffsetX  =  6 * dHorX + 6 * dVerX		(8-732)

posOffsetY  =  6 * dHorY + 6 * dVerY		(8-733)

- For x = 0..sbWidth − 1 and y = 0..sbHeight − 1, the following applies:

diffMv[ x ][ y ][ 0 ] = x * ( dHorX << 2 ) + y * ( dVerX << 2 ) – posOffsetX	(8-734)

diffMv[ x ][ y ][ 1 ] = x * ( dHorY << 2 ) + y * ( dVerY << 2 ) – posOffsetY	(8-735)

  - For i = 0..1, the following applieis:

    - The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to diffMv[ x ][ y ][ i ], rightShift set equal to 7 8, and leftShift set equal to 0 as inputs and the rounded diffMv[ x ][ y ][ i ] as output.

    - The value of diffMv[ x ][ y ][ i ] is clipped as follows:

diffMv[ x ][ y ][ i ] = Clip3( −dmvLimit, dmvLimit − 1, diffMv[ x ][ y ][ i ]) 	(8-736)

# Simulation results

Two tests were performed in this contribution. In test 1, the proposed method is only applied in BDOF. In test 2, the proposed method is applied in both BDOF and PROF.  The test results are shown in Table 1 and Table 2.  The overall average bd-rate performance is {0.00%, 0.04%, 0.05%} for RA configurations in Test 1. The overall average bd-rate performance is {0.01%, 0.03%, -0.01%} for RA and {0.02%, 0.01%, 0.04%} for LDB in Test 2.

Table 1 Test 1 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.04% | 0.06% | 100% | 101% |
| Class A2 | 0.01% | 0.03% | 0.05% | 102% | 101% |
| Class B | -0.01% | 0.03% | 0.03% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.05% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.04% | 0.05% | 101% | 101% |
| Class D | 0.01% | -0.02% | 0.08% | 104% | 110% |
| Class F | 0.00% | -0.03% | -0.03% | 101% | 101% |

Table 2 Test 2 result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 101% | 101% |
| Class A2 | 0.05% | -0.10% | 0.01% | 103% | 102% |
| Class B | 0.01% | 0.08% | -0.07% | 99% | 99% |
| Class C | 0.00% | 0.06% | 0.07% | 102% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | -0.01% | 101% | 102% |
| Class D | -0.03% | 0.00% | 0.11% | 106% | 113% |
| Class F | 0.01% | -0.03% | 0.00% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.05% | -0.37% | 100% | 100% |
| Class C | 0.01% | -0.09% | 0.24% | 104% | 107% |
| Class E | 0.04% | 0.26% | 0.46% | 104% | 104% |
| Overall | 0.02% | 0.01% | 0.04% | 102% | 103% |
| Class D | -0.05% | 0.22% | -0.13% | 105% | 118% |
| Class F | 0.10% | -0.20% | 0.41% | 101% | 102% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).