| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0095 |
| --- | --- |

| Title: | AHG12/AHG17: On signalling of picture-specific syntax elements in access unit delimiter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The following proposal items are included in this contribution:

- It is proposed to make access unit delimiter layer-specific and rename it to picture header (PH) or layer access unit delimiter (LAUD).

- The PH/LAUD is proposed to include an au_id syntax element to determine AU boundaries. When either the temporal_id_plus1 value or the au_id value in a PH/LAUD differs from those of the previous PH/LAUD in decoding order, the PH/LAUD is the first NAL unit of an access unit.

- It is proposed to make the signalling of tile, brick, and rectangular slice partitioning optional in PPS. When not present in the PPS, the signalling of tile, brick, and rectangular slice partitioning shall be present in the PH/LAUD.

- For signalling of slice ID and/or subpicture ID mapping to slice and/or subpicture locations, it is proposed to include a loop of slice ID and/or subpicture ID values in the decoding order of slices and/or subpictures in the PH/LAUD (and not include such a loop in the PPS).

- It is proposed to include mixed_nalu_types_in_pic_flag in the PH/LAUD for indicating the presence of mixed VCL NAL unit types within a coded pictur. The semantics and constraints related to the flag are identical to those proposed in JVET-O0140-v2.

- It is proposed to move no_ouptut_of_prior_pics_flag from the slice header to the PH/LAUD.

# Introduction

## Signalling of tile structure without changing the PPS

In JVET#15, The access unit delimiter (AUD) was agreed to be mandatorily present in each access unit (AU) to detect the first VCL NAL unit of a coded picture.

In JVET#15, the following agreement was achieved (an excerpt from JVET-O_Notes):

It was agreed in principle that we should be able to change tile structure of a picture in some way without changing the PPS. For example, syntax in the AUD or SH or an APS could be used. Further study is needed to define how to do that. It was suggested for the existing syntax to remain as it is, with this being just some optional different mode.

Signalling the tile structure in a slice header could be costly in bit count.

Signalling the tile structure in APS could be vulnerable to accidental losses of APSs, since a loss of an APS cannot be detected if an APS with the same ID (but different content) was decoded earlier.

It is preferred to use AUD for tile structure signalling, when the tile structure is not present in the PPS, since:

- The tile structure is present only once per coded picture and is hence asserted to be efficient in terms of bit count.

- A wrong tile structure does not get accidentally used in decoding, since a loss of an AUD can be detected and decoding of a slice can be discontinued when the PPS referenced by a slice does not contain the tile structure and AUD has been lost.

However, the tile structure is picture-specific, whereas an AUD is AU-specific and could hence "address" several coded pictures of different layers. It is therefore proposed in this contribution to change the access unit delimiter to be specific to a single coded picture. Consequently, it is also proposed to change the name of the NAL unit to picture header (PH) or layer access unit delimiter (LAUD).

## Signalling of Slice ID and/or subpicture ID mapping to slice and/or subpicture locations

This issue was discussed in more details in JVET-O0394.

In VVC Draft 6, the PPS contains a loop that maps slice ID values to locations of rectangular slices (in raster scan order of rectangular slices). In JVET#15, there was some discussion whether slice_address or slice ID values should be relative to subpictures and whether subpicture IDs should be signalled in the slice header, and JVET-P0126 proposes such a design. It is expected that the topic will be further discussed in JVET#16. However, regardless of the design choices it is believed that signalling a mapping of slice IDs and/or subpicture IDs to slice and/or subpicture locations is needed for extraction and merging use cases.

It is proposed to move the mapping of slice IDs and/or subpicture IDs to slice and subpicture positions, respectively, from the syntax of parameter sets to PH/LAUD. The same number and IDs of parameter sets can be kept in the merged bitstream as in the originally encoded resolution and bitrate versions. Thus, it is asserted that the proposal has the following advantages:

- No rewriting of slice headers is needed since that same PPS ID values are referenced.

- Only a single set of SPS(s) and PPS(s) for the merged bitstream are needed, matching the SPS(s) and PPS(s) of the source bitstreams one-to-one, regardless of client's selection of which slices/subpictures are included in the merged bitstream.

- A content author could provide SPS(s) and PPS(s) of potential merged bitstream(s) through systems means; thus, the client would neither need to create nor rewrite of SPS(s) and PPS(s).

## Mixed VCL NAL unit types within a coded picture

In JVET#15, the following was noted regarding JVET-O0140 that proposed indicating the mixed presence of IRAP and non-IRAP NAL unit types within a coded picture (an excerpt from JVET-O_Notes):

A motivation is region-wise random access using subpictures. An example use case is viewport dependent streaming of 360° video. Basically it would be from a BEAMer scenario. It was commented that using layers would be an alternative way to provide this functionality while constraining to have only one NAL unit type per picture per layer.

There would be a level of indirection involved in identifying the random access properties of the picture from the NAL unit header. It was commented that this indication could be put into the AUD.

Mandating AUD could potentially simplify a lot of basic needs.

It was also commented that something like an SEI message instead of NAL unit type can be used to indicate the predictive properties of the bitstream.

The encoding of the different regions would need to be very carefully coordinated.

Further study of this was needed.

## no_output_of_prior_pics_flag

no_output_of_prior_pics_flag remains unchanged for an entire coded picture. It could be used by a bitstream splicer and hence changing the value of the flag should be made straightforward. It is believed that the PH/LAUD is a more suitable location for the flag, since it there is only a single flag per picture that needs to be written by the bitstream splicer rather than rewriting all the slice headers of a coded picture and since the flag in the PH/LAUD can be made to be accessible at a fixed bit position, whereas VVC Draft 6 requires parsing of the active SPS and PPS and the syntax elements in the slice header preceding no_output_of_prior_pics_flag to change its value.

# Overview of the proposal

The proposal can be summarized as follows:

- It is proposed to make access unit delimiter layer-specific and rename it to picture header (PH) or layer access unit delimiter (LAUD).

- The PH/LAUD is proposed to include an au_id syntax element to determine AU boundaries. When either the temporal_id_plus1 value or the au_id value in a PH/LAUD differs from those of the previous PH/LAUD in decoding order, the PH/LAUD is the first NAL unit of an access unit.

- It is proposed to make the signalling of tile, brick, and rectangular slice partitioning optional in PPS. When not present in the PPS, the signalling of tile, brick, and rectangular slice partitioning shall be present in the PH/LAUD.

- For signalling of slice ID and/or subpicture ID mapping to slice and/or subpicture locations, it is proposed to include a loop of slice ID and/or subpicture ID values in the decoding order of slices and/or subpictures in the PH/LAUD (and not include such a loop in the PPS).

- It is proposed to include mixed_nalu_types_in_pic_flag in the PH/LAUD for indicating the presence of mixed VCL NAL unit types within a coded pictur. The semantics and constraints related to the flag are identical to those proposed in JVET-O0140-v2.

- It is proposed to move no_ouptut_of_prior_pics_flag from the slice header to the PH/LAUD.

# Proposed specification text

## Syntax

The proposed changes are highlighted according to their "category" as follows:

- signalling of tile structure without changing the PPS

- signalling of slice ID and/or subpicture ID mapping to slice and/or subpicture locations

- other (au_id, indicating the presence of mixed VCL NAL unit types within a coded picture, no_output_of_prior_pics_flag, reserved bits)

| layer_au_delimiter_rbsp( ) { | Descriptor |
| --- | --- |
| au_id | u(5) |
| pic_type | u(3) |
| mixed_nalu_types_in_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| slice_tile_brick_flag | u(1) |
| signalled_slice_id_flag | u(1) |
| reserved_zero_4bits | u(4) |
| if( slice_tile_brick_flag ) |  |
| slice_tile_brick( ) |  |
| if( signalled_slice_id_flag ) { |  |
| if( !laud_slice_tile_brick_flag ) { |  |
| laud_pic_parameter_set_id | ue(v) |
| num_slices_in_pic_minus1 | ue(v) |
| } |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_tile_brick( ) { | Descriptor |
| --- | --- |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| … // as in PPS in VVC Draft 6 |  |
| } |  |
| } |  |

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | ue(v) |
| output_flag_present_flag | u(1) |
| pps_slice_tile_brick_flag | u(1) |
| if( pps_slice_tile_brick_flag ) |  |
| slice_tile_brick( ) |  |
| single_tile_in_pic_flag | u(1) |
| if( !single_tile_in_pic_flag ) { |  |
| … |  |
| } |  |
| if( rect_slice_flag ) { |  |
| signalled_slice_id_flag | u(1) |
| if( signalled_slice_id_flag ) { |  |
| signalled_slice_id_length_minus1 | ue(v) |
| for( i = 0; i <= num_slices_in_pic_minus1; i++ ) |  |
| slice_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( nal_unit_type = = IDR_W_RADL ¦ ¦ nal_unit_type = = IDR_N_LP ¦ ¦ nal_unit_type = = CRA_NUT ¦ ¦ nal_unit_type = = GDR_NUT ) |  |
| no_output_of_prior_pics_flag | u(1) |
| … |  |

## Semantics

au_id is used for determining the first NAL unit of an access unit as specified in clause 7.4.2.4.3. The value of au_id shall be the same within an access unit. When the TemporalId derived for the VCL NAL units of the current access unit is equal to the TemporalId derived from the VCL NAL units of the previous access unit in decoding order, the value of au_id shall not be equal to the value of au_id of the previous access unit.

no_output_of_prior_pics_flag affects the output of previously-decoded pictures in the decoded picture buffer after the decoding of a CLVSS picture that is not the first picture in the bitstream as specified in Annex C. When the coded picture following this NAL unit is not an IDR, CRA or GDR picture, no_output_of_prior_pics_flag shall be equal to 0.

## Access unit boundary determination

##### 7.4.2.4.3	Order of NAL units and coded pictures and their association to layer access units and access units

This clause specifies the order of NAL units and coded pictures and their association to layer access units and access units for CVSs that conform to one or more of the profiles specified in Annex A and that are decoded using the decoding process specified in clauses 2 through 10.

A layer access unit consists of one coded picture, which comprises of one or more VCL NAL units, and zero or more non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.4.

An access unit consists of an access unit delimiter NAL unit and one or more layer access units in increasing order of nuh_layer_id.

The first access unit in the bitstream starts with the first NAL unit of the bitstream. Each subsequent access unit starts with a layer access unit delimiter NAL unit in which the value of au_id is not equal to the value of au_id of the previous layer access unit delimiter NAL unit in decoding order or the value of temporal_id_plus1 is not equal to the value of temporal_id_plus1 of the previous layer access unit delimiter NAL unit in decoding order.

Each layer access unit starts with a layer access unit delimiter NAL unit. There shall be at most one and only one layer access unit delimiter NAL unit in anyeach layer access unit.

The order of the coded pictures and non-VCL NAL units within a layer access unit or an access unit shall obey the following constraints:

–	When any DPS NAL units, VPS NAL units, SPS NAL units, PPS NAL units, APS NAL units, prefix SEI NAL units, NAL units with nal_unit_type equal to RSV_NVCL_26, or NAL units with nal_unit_type in the range of UNSPEC_28..UNSPEC_29 are present in a layer access unit, they shall not follow the last VCL NAL unit of the layer access unit.

–	NAL units having nal_unit_type equal to SUFFIX_SEI_NUT, FD_NUT, or RSV_NVCL_27, or in the range of UNSPEC_30..UNSPEC_31 in a layer access unit shall not precede the first VCL NAL unit of the layer access unit.

–	When an end of sequence NAL unit is present in an access unit, it shall be the last NAL unit among all NAL units with in the access unit other than an end of bitstream NAL unit (when present).

–	When an end of bitstream NAL unit is present in an access unit, it shall be the last NAL unit in the access unit.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).