| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0099 |
| --- | --- |

| Title: | Level limits for number of luma samples, tiles, and subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

Assuming that Levels 1 to 6.2 would be specified for VVC similarly to HEVC, the following level limits are proposed in this contribution:

- The maximum number of luma samples per picture specified identically to the limits in HEVC

- The maximum number of tile rows and tile columns specified identically to the respective limits in HEVC

- The maximum number of subpictures specified to be the maximum number of tile rows multiplied by the maximum number of tile columns.

# Proposed specification text

### A.x.y	General tier and level limits

…

When the specified level is not level 8.5, bitstreams conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex ‎C:

- PicSizeInSamplesY shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.XX.

- The value of num_tile_columns_minus1 shall be less than MaxTileCols and num_tile_rows_minus1 shall be less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.XX.

- The value of max_subpics_minus1 shall be less than MaxTileCols * MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.XX.

Table A.XX – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slices per picture MaxSlicesPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 |  |  |  | 1 | 1 |
| 2 | 122 880 |  |  |  | 1 | 1 |
| 2.1 | 245 760 |  |  |  | 1 | 1 |
| 3 | 552 960 |  |  |  | 2 | 2 |
| 3.1 | 983 040 |  |  |  | 3 | 3 |
| 4 | 2 228 224 |  |  |  | 5 | 5 |
| 4.1 | 2 228 224 |  |  |  | 5 | 5 |
| 5 | 8 912 896 |  |  |  | 11 | 10 |
| 5.1 | 8 912 896 |  |  |  | 11 | 10 |
| 5.2 | 8 912 896 |  |  |  | 11 | 10 |
| 6 | 35 651 584 |  |  |  | 22 | 20 |
| 6.1 | 35 651 584 |  |  |  | 22 | 20 |
| 6.2 | 35 651 584 |  |  |  | 22 | 20 |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).