| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0100 |
| --- | --- |

| Title: | AHG8: Unaligned IRAP pictures across layers and layer-wise start-up |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes that IRAP pictures need not be aligned across layers, which is handled in the proposed decoding process using layer-wise start-up similarly to the respective HEVC processes.

# Introduction

IRAP pictures that are not aligned across layers have been found useful e.g. in the following usage scenarios:

- A base layer with frequent IRAP pictures enables clients to tune in and seek at fine time granularity, whereas the IRAP picture interval in the enhancement layer(s) is greater than that in the base layer. Since the base layer has smaller bitrate than the enhancement layer(s), the rate-distortion efficiency is improved compared to having as frequent IRAP pictures in the enhancement layer(s) as in the base layer.

- A base layer has the entire 360° coverage and contains IRAP pictures at the interval that is desired for random access (e.g. every 10 seconds). Enhancement layers provide "tiles" of the content and are selectively received in viewport-dependent manner. The enhancement layers contain frequent IRAP pictures to enable immediate reaction to viewing orientation changes.

- To enable fast start-up in a streaming player, layers can be step-wise added to the transmitted stream. A sufficient level of receiver buffer occupancy in terms of media duration is obtained quickly since the base layer is likely to have considerably smaller bitrate than the experienced throughput.

IRAP pictures that are not aligned across layers is enabled in HEVC.

This contribution proposes specification text changes that enable the support for the usage scenarios above similarly to HEVC.

# Proposal

The proposed specification text changes are summarized as follows:

- A variable CrossLayerStartFlag is derived and used in the decoding process. CrossLayerStartFlag equal to 1 indicates a decoding refresh across all layers.

  - CrossLayerStartFlag is derived or set by external means as follows: CrossLayerStartFlag is derived to be equal to 1 when the referenced VPS changes. CrossLayerStartFlag can be set by external means to indicate cross-layer splice points at CVSS access units (i.e., CVS start access units).

  - As a consequence of CrossLayerStartFlag being equal to 1 all reference pictures are marked as "unused for reference".

CrossLayerStartFlag is asserted to be similar to NoClrasOutputFlag in HEVC.

- The end of sequence (EOS) NAL unit is proposed to apply to the layer indicated by its nuh_layer_id value and all its direct and indirect dependent layers. The reference pictures at the layer containing the EOS NAL unit and at all its direct and indirect dependent layers are marked as "unused for reference".

The proposed handling of EOS NAL units is asserted to be similar to how they are handled in HEVC.

A receiver that selectively receives and/or decodes layers can insert an EOS NAL unit into the bitstream to be decoded whenever it selects to discontinue the reception of a particular layer. Consequently, when the reception and/or decoding of the same layer is re-started later on, the decoding process handles the layer as if it were not decoded before.

- The decoding process is controlled by two arrays, FirstPicInLayerIdxDecodedFlag[ layerIdx ] and LayerIdxInitializedFlag[ layerIdx ], as follows:

  - FirstPicInLayerIdxDecodedFlag[ layerIdx ] equal to 1 indicates that the first picture in the layer with layer index layerIdx has been decoded, whereas FirstPicInLayerIdxDecodedFlag[ layerIdx ] equal to 0 indicates that the first picture in the layer with layer index layerIdx has not been decoded.

  - LayerIdxInitializedFlag[ layerIdx ] equal to 1 indicates that the layer with layer index layerIdx is correctly decoded, whereas LayerIdxInitializedFlag[ layerIdx ] equal to 0 indicates that the decoded pictures (if any) of the respective layer might not be correct in content, e.g. due to missing reference pictures.

  - LayerIdxInitializedFlag[ layerIdx ] is set equal to 1 for an IRAP picture when all the reference layers also have LayerIdxInitializedFlag[ refLayerIdx ] equal to 1 or at the first picture at or subsequent to the recovery point for a GDR picture.

  - CrossLayerStartFlag equal to 1 resets FirstPicInLayerIdxDecodedFlag[ layerIdx ] and LayerIdxInitializedFlag[ layerIdx ] to 0 for all layers.

  - An EOS NAL unit resets FirstPicInLayerIdxDecodedFlag[ layerIdx ] and LayerIdxInitializedFlag[ layerIdx ] to 0 in the layer containing the EOS NAL unit and all its dependent layers.

  - PictureOutputFlag is set equal to 0 for all pictures in a layer layerIdx as long as LayerIdxInitializedFlag[ layerIdx ] remains equal to 0.

FirstPicInLayerIdxDecodedFlag[ ] and LayerIdxInitializedFlag[ ] are asserted to be similar to FirstPicInLayerDecodedFlag[ ] and LayerInitializedFlag[ ] of HEVC, respectively. Likewise, the impact of the proposed variables is similar in the proposed VVC text when compared to the impact of the respective variables in HEVC.

- The decoding process for generating unavailable reference pictures is invoked when the first picture in a layer (with FirstPicInLayerIdxDecodedFlag[ layerIdx ] equal to 0) is not an IDR picture.

This step is asserted to be similar to the HEVC decoding process for generating unavailable reference pictures for pictures first in decoding order within a layer.

The details of the proposed specification text changes are indicated with change marks in the accompanying specification text document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).