| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0101 |
| --- | --- |

| Title: | AHG8: POC derivation for a multi-layer bitstream |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes changes related to picture order count derivation. The proposed changes are summarized as follows:

- prevTid0Pic for a dependent layer is selected to be the previous picture (in decoding order) in the current layer or in any direct or indirect reference layer that has TemporalId equal to 0 and is not RASL or RADL picture.

- poc_msb_reset_flag is added in the slice header. When poc_msb_reset_flag is equal to 1, PicOrderCntMsb is reset to 0. The VVC Draft 6 text where a CLVSS picture resets PicOrderCntMsb to 0 is removed.

- JVET-P0100 proposes the derivation of CrossLayerStartFlag, which when equal to 1 indicates an independent cross-layer decoding refresh point. If JVET-P0100 is adopted, CrossLayerStartFlag is proposed to affect PicOrderCntMsb derivation the same way as poc_msb_reset_flag, i.e., when CrossLayerStartFlag is equal to 1, PicOrderCntMsb is reset to 0.

# Problem statement

Figure 1 presents an example of a two-layer bitstream where the dependent layer has an IRAP picture that is not aligned with an IRAP picture in its reference layer. Any temporal inter prediction hierarchy can be used within each layer.

Figure 1. Example of a two-layer bitstream with an access unit containing an IRAP picture in the dependent layer but not in the reference layer.

VVC Draft 6 specifies that if the current picture is a CLVSS picture, PicOrderCntMsb is set equal to 0. Since the IRAP picture in the dependent layer highlighted with yellow in Figure 1 is a CLVSS picture, its PicOrderCntMsb is set equal to 0 according to VVC Draft 6. However, the PicOrderCntMsb of the reference layer picture in the same access unit is derived from earlier pictures and is likely not to be equal to 0. Consequently, the PicOrderCntVal values derived for the pictures in the same access unit would differ. However, VVC Draft 6 requires all pictures in any particular access unit to have the same value of PicOrderCntVal.

Figure 2 presents an example of a two-layer bitstream where the independent layer has an IRAP picture that is not aligned with an IRAP picture in a dependent layer predicted from the independent layer. The dependent layer may contain prediction references "beyond" the IRAP picture in the reference layer. For proper motion vector scaling in temporal motion vector prediction, the POC values in the dependent layer should be continuous and PicOrderCntMsb should not be reset to 0 at the IRAP picture in the independent layer. However, since the IRAP picture is a CLVSS picture, its PicOrderCntMsb is set equal to 0 according to VVC Draft 6. This leads to a contradiction of deriving unequal PicOrderCntVal values for the pictures of the same access unit, similarly to Figure 1.

Figure 2. Example of a two-layer bitstream with an access unit containing an IRAP picture in the reference layer but not in the dependent layer.

# Summary of the proposal

The following ingredients are proposed:

- prevTid0Pic for a dependent layer is selected to be the previous picture (in decoding order) in the current layer or in any direct or indirect reference layer that has TemporalId equal to 0 and is not RASL or RADL picture.

This aspect is asserted to be similar to how the prevTid0Pic is selected in HEVC.

Thus, according to this proposal, in the example of Figure 1 the POC for the IRAP picture highlighted in yellow is derived from the reference layer picture in the same access unit. This design is asserted to be robust for layer down-switching and subsequent layer up-switching as illustrated in Figure 3. In such as case deriving the POC from the previous picture in the same layer might cause the derivation of the PicOrderCntMsb to fail, since the pictures that are not received might have caused the encoder to update PicOrderCntMsb, whereas the decoder is unable to update PicOrderCntMsb the same way.

Figure 3. Proposed selection of prefTid0Pic for a picture in a dependent layer.

The proposed selection of prevTid0Pic also handles the case where the picture rates are unequal between layers. In the example presented in Figure 4, there is no picture in the reference layer in the access unit that contains the current picture in the dependent layer. Thus, according to the proposal, the previous picture in the dependent layer gets selected as prevTid0Pic.

Figure 4. Proposed selection of prefTid0Pic for a picture in a dependent layer.

- poc_msb_reset_flag is added in the slice header. When poc_msb_reset_flag is equal to 1, PicOrderCntMsb is reset to 0. The VVC Draft 6 text where a CLVSS picture resets PicOrderCntMsb to 0 is proposed to be removed.

When poc_msb_reset_flag is set equal to 1 in any independent layer, it should be set equal to 1 for the next picture in each layer. poc_msb_reset_flag equal to 1 should be used at a cross-layer splice point.

- JVET-P0100 proposes the derivation of CrossLayerStartFlag, which when equal to 1 indicates an independent cross-layer decoding refresh point. If JVET-P0100 is adopted, CrossLayerStartFlag is proposed to affect PicOrderCntMsb derivation the same way as poc_msb_reset_flag, i.e., when CrossLayerStartFlag is equal to 1, PicOrderCntMsb is reset to 0.

The proposed specification text changes are indicated with change marks in the accompanying document.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).