| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0107-v3 |
| --- | --- |

| Title: | CE4-Related: Geometric Merge Mode (GEO) Simplifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

Geometric merge mode (GEO) is proposed in 15th Gothenburg JVET meeting as extension of the existing triangle prediction mode (TPM), and is defined in several CE tests including CE4-1.1, CE4-1.2 and CE4-1.14. It’s desirable to reduce the complexity of GEO. In this proposal, threes complexity reduction method are tested, including further reducing number of split modes, removing partIdx assignment and slope based GEO

With further reducing number of split modes to 64, the coding performance on top of VTM-6.0 -0.34%/-0.37%/-0.53% RA with EncT 102% and DecT 100%; -0.67%/-0.67%/-0.62% LDB with EncT 103%	 and DecT 100%.

With removing partIdx assignment on top of CE4-1.1, the coding performance on top of VTM-6.0 achieves -0.35%/-0.43%/-0.52% RA with EncT 104% and DecT 100%; -0.68%/-0.70%/-0.59% LDB with EncT 105% and DecT 99%.

With the slope based GEO, on top of CE4-1.14, the coding performance of version 1 on top of VTM-6.0 achieves -0.xx%/-0.xx%/-0.xx% RA with EncT xxx% and DecT xxx%; -0.xx%/-0.xx%/-0.xx% LDB with EncT xxx% and DecT xxx%. The coding performance of version 2 on top of VTM-6.0 achieves -0.xx%/-0.xx%/-0.xx% RA with EncT xxx% and DecT xxx%; -0.xx%/-0.xx%/-0.xx% LDB with EncT xxx% and DecT xxx%.

# Introduction

Geometric merge mode is proposed in [1], which using the same predictors blending concept as TPM and extended the blending masks up to 140 different modes with 32 angles and 5 offsets. In CE4-1.2, the number of split modes are reduced to 108 and 80 modes

In the first simplification, split modes of GEO are further reduced to 64, 50, 36 and 32 modes, the 64 modes reserves the coding performance without loss compared with CE4-1.2.

In the second simplification, the partIdx assignment during luma blending operation is removed.

In the third simplification, the angle is defined based on slopes, which significant reduced the multiplication computation complexity (from 6bits to 2 bits). Two versions of implementation are provided.

# Proposal

## Further reducing number of split modes

As descripted in CE-4-1.1 proposal [2], the geo split modes are descripted with angle index φi and distance index ρi.

In the proposed 64 modes, angle index is quantized with fix step 11.25 degrees around 360 degrees, in addition,

- the angle index near to vertical angle direction (horizontal spilt directions) with all distances are removed, since most nature objects or motions are vertically

- Index 4, 12, 20, 28 which indicates 45 or 135 degrees with all distances are also removed, since it’s likely overlapped with the existed TPM modes.

Distance index is quantized into 4 steps, in addition, the fourth distance index for horizontal and vertical angles (0, 8, 16, 24) are removed, since these 4 split modes are likely overplayed with ternary tree split. Figure 1 shows the angles quantization of 64 modes GEO.

In total, 4 angles with 2 distances, 8 angles with 3 distances and 8 angles with 4 distances, which achieves 64 split modes.

Figure 1 Angle quantization of 64 split modes GEO

On top of GEO with 64 split modes, further removing angle index (2, 14, 18, 30), a 50 split modes GEO is achieved, the angle quantization is shown in Figure 2

Figure 2 Angle quantization of 50 split modes GEO

On top of GEO with 50 split modes, further removing angle index (1, 15, 17, 31) a 36 split modes GEO is achieved, the angle quantization is shown in Figure 3

Figure 3 Angle quantization of 36 and 32 split modes GEO

On top of GEO with 36 split modes, further removing the distance index 1 split for horizontal and vertical angles, since they are near to the binary tree, 32 split modes GEO is achieved.

The number of angles, distances and modes compared with CE are summarized in Table 1

Table 1 number of angles distances and modes

|  | CE4-1.1 | CE4-1.2a | CE4-1.2b | 64 modes | 50 modes | 36 modes | 32 modes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Number of angles | 32 | 32 | 24 | 20 | 16 | 12 | 12 |
| Number of distance | 5 | 5 | 4 | 4/2* | 4/2* | 4/2* | 4/1# |
| Number of partitions | 140 | 108 | 80 | 64 | 50 | 36 | 32 |

Note: *Horizontal and vertical angles has 2 distances

#Horizontal and vertical angles has 1 distance

## Removing partIdx assignment

In CE4-1.1, the final predictor PB with the 3 bits blending mask W0 and W0 as in Eq. (1)

PB=(W0P0+W1P1+4)≫3             (1)

The weighting array is pre-calculated based on the distance between the sample position and the split boundary. In order to reduce the code word length signaling, the bottom left samples of current block is guaranteed predicted from P0. Therefore the partIdx is assigned as following:

- when the distFromLine of the bottom-left sample is negative, W0 is equal to the calculated weighting array and W1is equal to 8-W0.

- Otherwise (the distFromLine of the bottom-left sample is positive), W1 is equal to the calculated weighting array and W0is equal to 8-W1.

In the proposed method, the partIdx assignment operation is removed, the W0 is fixed as equal to the calculated weighting array as shown if Figure 4

(a) CE4-1.1 partIdx assignment 								(b) CE4-1.1 partIdx assignment

Figure 4 Removing partIdx assignment

## Slope based GEO

### version 1

In the CE4-1 tests, the GEO split mode is represent angle index φi and distance index ρi, the angle index φi is equally quantized with step 11.25 degrees between 0  and 360 degrees. In order to derive the blending mask, the distance from the split line to the sample position is calculated by equation:

distFromLine = ((x<<1) + 1)*Dis[displacementX] + ((y<<1) + 1))*Dis[displacementY] –rho         (3)

where Dis[] is a 6 bits precision look up table with 32 entries.

In the slope based GEO, the angle is not quantized with fixed steps, but with fixed slops (0, 1/4, 2/4, 3/4, 4/4, 4/3, 4/2, 4/1). In this case, the tan⁡(φi) is equal to the slopes, which means the ratio of sin(φi) and cos⁡(φi) is fixed and equal to the slope. In this case, only 2 bits Dis[.] are necessary for equation (3), which reduced the computation complexity of blending mask derivation and motion storage process of GEO.

With the slope based GEO, the 6 bits Dis[.] table in CE4-1 test will be replaced with Table 2

Table 2 2 bits Dis[.] look up table for slope based GEO

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 4 | 4 | 4 | 4 | 4 | 3 | 2 | 1 | 0 | -1 | -2 | -3 | -4 | -4 | -4 | -4 |
| idx | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| Dis[idx] | -4 | -4 | -4 | -4 | -4 | -3 | -2 | -1 | 0 | 1 | 2 | 3 | 4 | 4 | 4 | 4 |

And the equation (3) will be replaced by following equation:

distFromLine = ((x<<1) + 1)*(Dis[displacementX] <<4) + ((y<<1) + 1))*(Dis[displacementY]<<4) –rho

Comparing with the TPM blending mask derivation in following, multiplication (up to 4 bits shift) and addition are necessary as highlighted. The highlighted sections in TPM are equivalent to distFromLine  calculation in GEO. With the slope based GEO implication, the computation complexity blending mask and motion storage derivation for GEO and TPM are in a same level.

TPM:

… The variable wIdx is derived as follows: - If cIdx is equal to 0 and triangleDir is equal to 0, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) ) (8-842) : ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) ) - Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) (8-843) ( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) ) - Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) ) (8-844) : ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) ) - Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies: wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) (8-845) ( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) ) …

GEO:

Accoding to the table 2, the achievable distance calculation equations examples are as below:

Angle idx is 0: ((x<<1) + 1)*(4 <<4) - ((y<<1) + 1))*(1<<4) –rho

Angle idx is 3: ((x<<1) + 1)*(4 <<4) - ((y<<1) + 1))*(3<<4) –rho

Angle idx is 6: ((x<<1) + 1)*(2 <<4) - ((y<<1) + 1))*(4<<4) –rho

Note that compared to TPM, the only partition in slope based GEO that is different is the partition with slope 3/4 or 4/3.

The slope based GEO can be easily combined with reduced angles or distance proposal.

### Version 2

Based on the slope based GEO version 1, the slope 4/3 and 3/4 angle indices (3, 5, 11, 13, 19, 21, 27, 29) are removed. In slope based GEO version 2, The Dis[.] look up table is illustrated in Table 3

Table 3 2 bits Dis[.] look up table for slope based GEO

| idx | 0 | 1 | 2 | 4 | 6 | 7 | 8 | 9 | 10 | 12 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 4 | 4 | 4 | 4 | 2 | 1 | 0 | -1 | -2 | -4 | -4 | -4 |
| idx | 16 | 17 | 18 | 20 | 22 | 23 | 24 | 25 | 26 | 28 | 30 | 31 |
| Dis[idx] | -4 | -4 | -4 | -4 | -2 | -1 | 0 | 1 | 2 | 4 | 4 | 4 |

With the slope based GEO version 2, the computation complexity of geo blending mask derivation is considered as multiplication (up to 2 bits shift) and addition. There is no different partitions compared to TPM. Furthermore, the rounding operation of distFromLine is removed in order to easier store the blending mask. This bugfix guarantees that sample weights are repeated in each row or column in a shifted fashion.

For example:

Angle idx is 0: ((x<<1) + 1)*(4 <<1) – ((y<<1) + 1))*(1<<1) –rho

Angle idx is 4: ((x<<1) + 1)*(4 <<1) – ((y<<1) + 1))*(4<<1) –rho

Angle idx is 6: ((x<<1) + 1)*(2 <<1) – ((y<<1) + 1))*(4<<1) –rho

# Simulation Results

## Further reducing number of split modes

Table 4 summarize the simulation results of further reducing number of split modes, compared with CE4-1.2 80 modes, the 64 modes GEO has negligible coding loss, which provided the best trade-off between number of split modes and coding efficiently

Table 4 Further reducing number of split modes

| #CE | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-1.2_80 | -0.34% | -0.39% | -0.52% | 103% | 100% | -0.68% | -0.46% | -0.60% | 103% | 99% |
| 64 modes | -0.34% | -0.37% | -0.53% | 102% | 100% | -0.67% | -0.67% | -0.62% | 103% | 100% |
| 50 modes | -0.31% | -0.35% | -0.44% | 102% | 100% | -0.65% | -0.60% | -0.55% | 102% | 100% |
| 36 modes | -0.28% | -0.32% | -0.41% | 102% | 100% | -0.58% | -0.50% | -0.65% | 102% | 99% |
| 32 modes | -0.27% | -0.31% | -0.41% | 102% | 100% | -0.54% | -0.47% | -0.58% | 102% | 99% |

## Removing partIdx assignment

Table 5 shows the simulation results of remiving partIdx assignment on top of CE4-1 tests. The detail results partIdx assignment removing on top of other Ces attached in the excel tables.

Table 5 Removing partIdx assignment on top of CE4-1 tests

| On top of CE | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE4-1.1 | -0.35% | -0.43% | -0.52% | 104% | 100% | -0.68% | -0.70% | -0.59% | 105% | 99% |
| CE4-1.2_108 | -0.34% | -0.45% | -0.53% | 104% | 100% | -0.65% | -0.65% | -0.75% | 104% | 99% |
| CE4-1.2_80 | -0.32% | -0.42% | -0.51% | 103% | 100% | -0.62% | -0.58% | -0.64% | 103% | 99% |
| CE4-1.2_64 | -0.32% | -0.41% | -0.49% | 102% | 100% | -0.64% | -0.59% | -0.56% | 103% | 99% |
| CE4-1.14 | -0.35% | -0.41% | -0.48% | 104% | 100% | -0.69% | -0.81% | -0.78% | 105% | 99% |
| CE4-1.14_108 | -0.33% | -0.41% | -0.49% | 104% | 100% | -0.70% | -0.63% | -0.74% | 104% | 99% |
| CE4-1.14_80 | -0.32% | -0.38% | -0.50% | 103% | 100% | -0.61% | -0.52% | -0.39% | 103% | 99% |
| CE4-1.2_64 | -0.33% | -0.40% | -0.48% | 102% | 100% | -0.63% | -0.58% | -0.55% | 103% | 99% |

## Slope based GEO

### Version 1

The version 1 is simulated on top of CE4-1.14(140 modes with simplified motion storage), Table 6 shows the slope based GEO version 1 vs VTM-6.0

Table 6 Slope based version 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.17% | -0.10% | -0.26% | 104% | 100% |
| Class A2 | -0.24% | -0.30% | -0.26% | 104% | 100% |
| Class B | -0.25% | -0.19% | -0.28% | 105% | 100% |
| Class C | -0.76% | -1.01% | -1.16% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.37% | -0.41% | -0.51% | 104% | 100% |
| Class D | -0.56% | -0.82% | -0.72% | 105% | 100% |
| Class F | -0.36% | -0.52% | -0.40% | 102% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.41% | -0.41% | -0.82% | 105% | 100% |
| Class C | -0.93% | -1.15% | -1.12% | 106% | 99% |
| Class E | -1.23% | -0.45% | -0.17% | 104% | 99% |
| Overall | -0.78% | -0.66% | -0.76% | 105% | 100% |
| Class D | -0.84% | -1.06% | -0.88% | 106% | 100% |
| Class F | -0.48% | -0.42% | -0.55% | 104% | 99% |

Table 7 shows supplementary results of further reducing the geo mode to 64.on top of slope based version 1.

Table 7 Slope based version 1 64 modes

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.23% | -0.24% | 102% | 100% |
| Class A2 | -0.22% | -0.23% | -0.14% | 103% | 100% |
| Class B | -0.21% | -0.11% | -0.25% | 103% | 100% |
| Class C | -0.73% | -0.98% | -1.05% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | -0.39% | -0.44% | 103% | 100% |
| Class D | -0.51% | -0.78% | -0.66% | 103% | 100% |
| Class F | -0.31% | -0.45% | -0.40% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.41% | -0.48% | 103% | 99% |
| Class C | -0.82% | -0.88% | -1.22% | 103% | 99% |
| Class E | -0.99% | -0.41% | -0.45% | 102% | 100% |
| Overall | -0.67% | -0.57% | -0.72% | 103% | 100% |
| Class D | -0.76% | -0.85% | -1.16% | 104% | 100% |
| Class F | -0.41% | -0.69% | 0.20% | 102% | 100% |

### Version 2

The version 2 is simulated on top of CE4-1.14(104 modes with simplified motion storage), Table 8 shows the slope based GEO version 2 vs VTM-6.0

Table 8 Slope based version 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.15% | -0.25% | 103% | 100% |
| Class A2 | -0.23% | -0.25% | -0.24% | 103% | 100% |
| Class B | -0.23% | -0.19% | -0.35% | 103% | 100% |
| Class C | -0.77% | -1.14% | -1.16% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | -0.45% | -0.52% | 103% | 100% |
| Class D | -0.55% | -0.90% | -0.69% | 103% | 100% |
| Class F | -0.37% | -0.53% | -0.44% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.87% | -1.18% | -1.31% | 104% | 99% |
| Class E | -1.08% | -0.53% | -0.48% | 103% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.86% | -1.00% | -0.57% | 105% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 8 shows supplementary results of further reducing the geo mode to 64.on top of slope based version 2 (removing near vertical angles, 7,9,23,25, remove last distance of vertical angle, 4 distances).

Table 9 Slope based version 2 64 modes

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.18% | -0.16% | -0.22% | 102% | 100% |
| Class A2 | -0.21% | -0.17% | -0.20% | 102% | 100% |
| Class B | -0.23% | -0.20% | -0.32% | 102% | 100% |
| Class C | -0.75% | -1.10% | -1.08% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.35% | -0.43% | -0.48% | 102% | 100% |
| Class D | -0.58% | -0.80% | -0.76% | 103% | 100% |
| Class F | -0.33% | -0.42% | -0.40% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | -0.88% | -1.03% | -1.13% | 103% | 100% |
| Class E | -0.98% | -0.51% | 0.28% | 102% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.82% | -1.09% | -0.81% | 104% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# References

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, J. Sauer  “Non-CE4: Geometrical partitioning for inter blocks”, JVET-O0489, Gothenburg, SE, 3–12 July 2019

- S. Esenlik, H. Gao, E. Alshina, A. M. Kotra, B. Wang, M. Bläser, J. Sauer  “CE4: CE4-1.1, CE4-1.2 and CE4-1.14: Geometric Merge Mode (GEO)”, JVET-P0068, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.