| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0108 |
| --- | --- |

| Title: | AHG17: On parsing dependency between slice data and APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Anand Meher Kotra, Biao Wang, Han Gao, Elena Alshina | Tel: Email: | semih.esenlik@huawei.com |
| Source: | Huawei Technologies Dusseldorf GmbH |  |  |

_____________________________

# Abstract

In VVC draft 6, parsing of the slice data depends on the syntax elements signalled in the Adaptation Parameter Set. It is asserted that unlike VPS, SPS or PPS, the APS would most likely be transmitted in low priority data channels and hence would be subject to packet losses or errors in the network. It is proposed to eliminate the parsing dependency between APS and the slice data, such that a slice can be parsed even in the case of a loss of the APS that is referred.

# Problem Statement

The problems are identified in the current VVC draft 6. First of all the presence of the alf_ctb_filter_alt_idx is conditioned on the aps_alf_chroma_num_alt_filters_minus1, which describes the number of ALF chroma filters in the APS. If the number of chroma filters in the APS are greater than or equal to 2, the alf_ctb_filter_alt_idx is present in the bitstream, otherwise it is not present.

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |

Table 1

Similarly the binarization of the alf_ctb_filter_alt_idx depends on the value of aps_alf_chroma_num_alt_filters_minus1, this is presented in the table below.

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_tree_unit( ) | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |

Table 2

It is asserted that APS would have different properties to PPS and SPS, as APS is expected to be transmitted much more frequently due to the fact that different slices typically use different ALF filter sets. It is asserted that the above listed parsing dependency is undesirable for applications prone to transmission errors.

Furthermore it is noted that in the VVC draft 6 the syntax element aps_alf_chroma_num_alt_filters_minus1 is not defined, however it seems that alf_chroma_num_alt_filters_minus1 and aps_alf_chroma_num_alt_filters_minus1 are identical and the absence of “aps_” prefix is a typo.

# Proposed Solution

## Alternative 1

According to the first alternative solution the alf_chroma_num_alt_filters_minus1 is duplicated in the slice header as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| if( sps_alf_enabled_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ){ |  |
| slice_alf_aps_id_chroma | u(3) |
| slice_alf_chroma_num_alt_filters_minus1 | u(3) |
| } |  |
| } |  |
| } |  |

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && slice_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && slice_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_tree_unit( ) | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = slice_alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |

Table 2

alf_chroma_num_alt_filters_minus1 plus 1 specifies the number of alternative filters for chroma components in the APS.

slice_alf_chroma_num_alt_filters_minus1 plus 1 specifies the number of alternative filters for chroma components that can be referred in the slice data. It is requirement of the bitstream conformance that value of slice_alf_chroma_num_alt_filters_minus1 must be equal to or smaller than the value of alf_chroma_num_alt_filters_minus1 of the APS that is referred by the slice for obtaining ALF chroma filters.

## Alternative 2

The presence condition and the binarization of the alf_ctb_filter_alt_idx is modified as follows:

| coding_tree_unit( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_alf_chroma_idc = = 1 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 0 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |
| if( slice_alf_chroma_idc = = 2 ¦ ¦ slice_alf_chroma_idc = = 3 ) { |  |
| alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| if( alf_ctb_flag[ 2 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] && aps_alf_chroma_num_alt_filters_minus1 > 0 ) |  |
| alf_ctb_filter_alt_idx[ 1 ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] | ae(v) |
| } |  |

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| coding_tree_unit( ) | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = 7 alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |

alf_ctb_filter_alt_idx[ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] specifies the index of the alternative chroma filter applied to the coding tree block of the chroma component, with chromaIdx equal to 0 for Cb and chromaIdx equal 1 for Cr, of the coding tree unit at luma location ( xCtb, yCtb ).When alf_ctb_filter_alt_idx[ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] is not present, it is infered to be equal to zero. It is requirement of the bitstream conformance that value of alf_ctb_filter_alt_idx[ chromaIdx ][ xCtb >> CtbLog2SizeY ][ yCtb >> CtbLog2SizeY ] must be equal to or smaller than the value of alf_chroma_num_alt_filters_minus1 + 1 of the APS that is referred by the slice for obtaining ALF chroma filters.

# Simulation Results

## Alternative 1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class E | 0.01% | 0.01% | 0.01% | 103% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 103% | 102% |
| Class D | 0.01% | 0.01% | 0.01% | 103% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 103% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class C | 0.01% | 0.01% | 0.01% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 102% | 101% |
| Class D | 0.02% | 0.02% | 0.02% | 102% | 101% |
| Class F | 0.01% | 0.01% | 0.01% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | 0.01% | 0.01% | 102% | 102% |
| Class E | 0.01% | 0.01% | 0.01% | 102% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | 0.02% | 0.02% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

It is noted that the reason for encoding and decoding time increase are likely due to different load conditions on the simulation cluster, as no actual difference is expected.

## Alternative 2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.03% | -0.04% | 102% | 102% |
| Class A2 | 0.00% | -0.03% | -0.01% | 103% | 102% |
| Class B | 0.01% | -0.04% | -0.05% | 103% | 102% |
| Class C | 0.02% | 0.00% | 0.00% | 103% | 102% |
| Class E | 0.03% | 0.02% | -0.01% | 103% | 102% |
| Overall | 0.01% | -0.01% | -0.03% | 103% | 102% |
| Class D | 0.01% | 0.04% | 0.08% | 103% | 102% |
| Class F | 0.02% | 0.00% | -0.01% | 103% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.00% | 102% | 100% |
| Class A2 | 0.03% | -0.02% | 0.01% | 102% | 101% |
| Class B | -0.02% | 0.14% | 0.14% | 102% | 101% |
| Class C | -0.01% | 0.19% | 0.06% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.09% | 0.06% | 102% | 101% |
| Class D | 0.01% | 0.06% | 0.04% | 102% | 101% |
| Class F | 0.00% | 0.05% | 0.07% | 102% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | 0.04% | -0.10% | 102% | 101% |
| Class E | 0.01% | -0.01% | 0.39% | 101% | 101% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.02% | -0.13% | -0.26% | 102% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

It is noted that the reason for encoding and decoding time increase are likely due to different load conditions on the simulation cluster, as no actual difference is expected.

# Conclusion

It is asserted that in VVC draft 6 parsing of the slice data depends on one syntax element that is signalled in the APS. The proposed solution alternatives eliminate this problem wherein no change in the coding performance is expected in both solutions. It is proposed to include one of the solutions in the next version of the VVC draft specification.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).