| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0110 |
| --- | --- |

| Title: | AHG15: Quantization matrices with single identifier and enhanced prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Leleannec, Edouard François, Karam Naser | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution is based on JVET-O0223, which proposed to change quantization matrix (QM) signaling and prediction, and allow prediction across all QMs regardless of block size, by identifying a QM with a single index, transmitting QMs in decreasing block size order, and specifying prediction as either a copy or decimation process. This contribution adds a scale factor to QM prediction, and refinement with a variable-length residual than can be used for full specification, adjustment of low-frequency coefficients, or QM offset.

The amount of the text needed to support QMs is reported to be reduced by half compared to VVC draft 6, and the proposed technique is said to save more than half of the bits needed to signal example QMs from a provided test package.

The proposed features: scaled prediction across all QMs and variable-size refinement, are said to enable efficient adaptation of QMs at picture level.

# Introduction

Quantization matrices (QMs) were introduced in VVC during the 14th JVET meeting, based on contribution JVET-N0847, which proposed a direct adaptation of the support of QMs available in HEVC. Compared to HEVC, VVC needs more quantization matrices due to a higher number of block sizes.

In VVC draft 6, custom QMs can be signaled in an APS, which can be referred to in slice header. To reduce QM coding cost, prediction can be used as in HEVC, i.e. a QM can be copied from another one instead of being explicitly signalled, provided that both QMs have the same block size identifier (sizeId).

It is worth noting that, like in HEVC, the semantics describe a specific QM decoding process for each block size. This is related to the specificities of some size ids: in VVC, block size 2x2 is used for chroma only, block size 64x64 is for luma only, and some QM coefficients for size 64x64 are not coded because they are never used. Additional text describes the process of upsampling QM data for blocks greater than 8x8 (like in HEVC) and downsampling for rectangular blocks (new in VVC, but similar to what was discussed in JCT-VC for HEVC).

JVET-O0223 proposed to identify QMs with a single id, encompassing both block size and type, so that prediction can be used across all QMs regardless of block size.

In addition, this contribution proposes to add a scale factor to prediction, and to adjust prediction with a residual. Details are presented step by step in the next section, followed by draft text impact analysis and test results.

# Proposal description

The following subsections describe the features of the proposed technique with a step-by-step approach:

- Allow prediction across all QMs by using a single QM identifier (as in JVET-O0223)

- Allow a residual on top of prediction

- Allow prediction with a scale factor instead of just copy

- Remove the prediction flag

Note: for the following, we refer to QMs for a given family of block sizes (square or rectangular) as size-N, in relation to sizeId and the square block size it is used for: for example, for TU sizes 16x16 or 16x4, the QMs are identified as size-16. The size-N notation is used to differentiate from exact block shape, and from the number of signaled QM coefficients (limited to 8x8, as shown in tables 1 and 2)

## Starting point: single QM identifier and prediction across all QMs

Whereas QM coding scheme in VVC draft 6 allows copy from QMs associated to the same transform size only, as illustrated in Figure 1, JVET-O0223 allowed prediction (i.e. copy, or down-sample) of a QM from another one associated to a different block size or type, as illustrated in Figure 2.

This is motivated by a better match expected between QMs of the same type (intra/inter, luma/chroma) but for different block size, than QMs for the same transform size but of different type: for example, there is probably a better chance that 16x16 and 8x8 INTRA luma QMs are similar, compared to 8x8 INTRA luma and 8x8 INTER Cb.

Instead of separate sizeId/matrixId (see Table 1), JVET-O0223 then defined a single QM identifier (matrixId, see Table 2) that, while simplifying specification, allows prediction across all QMs even for different block type or size. To make this possible, QMs are ordered by decreasing block size, with prediction either a copy or decimation as needed.

Tests (see §4) indicate that such inter-size prediction provides significant bit savings (-43% with the provided test set).

Figure 1: QM ids and prediction paths in VVC draft 6

Figure 2: proposed QM ids and prediction paths

Table 1: (matrixId, sizeId) combinations (VVC draft 6)

| INTRA | Y |  | 0, 2 | 0, 3 | 0, 4 | 0, 5 | 0, 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Cb | 1, 1 | 1, 2 | 1, 3 | 1, 4 | 1, 5 |  |
|  | Cr | 2, 1 | 2, 2 | 2, 3 | 2, 4 | 2, 5 |  |
| INTER | Y |  | 3, 2 | 3, 3 | 3, 4 | 3, 5 | 3, 6 |
|  | Cb | 4, 1 | 4, 2 | 4, 3 | 4, 4 | 4, 5 |  |
|  | Cr | 5, 1 | 5, 2 | 5, 3 | 5, 4 | 5, 5 |  |
| Block size: max(width,height) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| Signalled QM size |  | 2x2 | 4x4 | 8x8 | 8x8 + DC |  |  |

Table 2: unified matrixId (proposed)

| Y | INTRA | 0 |  | 6 |  | 12 |  | 18 |  | 24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | INTER | 1 |  | 7 |  | 13 |  | 19 |  | 25 |  |
| Cb | INTRA |  | 2 |  | 8 |  | 14 |  | 20 |  | 26 |
|  | INTER |  | 3 |  | 9 |  | 15 |  | 21 |  | 27 |
| Cr | INTRA |  | 4 |  | 10 |  | 16 |  | 22 |  | 28 |
|  | INTER |  | 5 |  | 11 |  | 17 |  | 23 |  | 29 |
| TU size: luma max(width,height) |  | 64 |  | 32 |  | 16 |  | 8 |  | 4 |  |
| Block size: max(width,height) |  | 64 | 32 |  | 16 |  | 8 |  | 4 |  | 2 |
| Signalled QM size |  | 8x8 + DC |  |  |  |  | 8x8 |  | 4x4 |  | 2x2 |

The syntax changes from VVC draft 6 are the following:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( sizeId = 1; sizeId < 7; sizeId++ ) |  |
| for( matrixId = 0; matrixId < 30; matrixId ++ ) { |  |
| if( ! ( ( ( sizeId = = 1 ) && ( matrixId % 3 = = 0 ) ) ¦ ¦ ( ( sizeId = = 6 ) && ( matrixId % 3 != 0 ) ) ) ) { |  |
| scaling_list_pred_mode_flag[ sizeId ][ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ sizeId ][ matrixId ] ) |  |
| scaling_list_pred_matrix_id_delta[ sizeId ][ matrixId ] | ue(v) |
| else { |  |
| nextCoef = 8 |  |
| coefNum = (matrixId < 20) ? 64 : (matrixId < 26) ? 16 : 4 |  |
| if( matrixId < 14 ) { |  |
| scaling_list_dc_coef_minus8[ sizeId − 4 ][ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ sizeId − 4 ][ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < coefNum; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if ( !( sizeId = = 6 && x >= 4 && y >= 4) ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ sizeId ][ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

All coefficients of size-64 luma QMs are transmitted, so that they can be used for prediction; while simplifying spec, tests indicate a clear benefit in doing so, and the worst case overhead can be limited at encoder side to 2x16 bits by forcing delta_coefficients to zero in the bottom-right quadrant.

In addition, the process of selecting the QM (matrixId) for a given block type and size, and resizing it to match block size, is moved from semantics to a specific section called “Scaling matrix derivation process”, within “Scaling, transformation and array construction process” section. This makes it clearly separate from QM decoding process and storage.

- The selection of QM is based on the maximal dimension of TU, as shown in the following equation:

matrixId = 6 * sizeId + matrixTypeId	(1)
with log2TuWidth = log2( blkWidth ) + ( cIdx > 0 ) ? log2( SubWidthC ) : 0,
log2TuHeight = log2( blkHeight ) + ( cIdx > 0 ) ? log2( SubHeightC ) : 0,
sizeId = 6 – max( log2TuWidth, log2TuHeight ), and
matrixTypeId = ( 2 * cIdx + ( predMode = =  MODE_INTRA ? 0 : 1 ) )

- The adaptation to block size is then performed by applying appropriate bit shifts to x/y coordinates when scanning the selected QM, as illustrated in the following equation:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]	(2)
with i = ( x << log2MatrixSize ) >> log2( blkWidth ), and
j = ( y << log2MatrixSize ) >> log2( blkHeight )

## Step1: variable-size residual on top of prediction

In addition to the single QM identifier and prediction across all QMs taken from JVET-O0223, this contribution proposes to allow refining prediction with a residual. That residual reuses the syntax currently used to transmit the list of QM coefficients, but with variable length. This way, as coefficients are DPCM-coded in up-right diagonal scanning order, transmitting a single coefficient generates a global offset; this allows simple changes at low cost, and follows the idea that the most impacting QM coefficients are the low-frequency ones, which are transmitted first.

Transmitted coefficients are always considered relative to a reference, but when no prediction is signalled, reference matrix and DC coefficient are inferred to all-8 and all coefficients are transmitted.

Tests indicate additional bit savings on top of the previous step (-10 to -20%).

The syntax changes for this step are illustrated below:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( matrixId = 0; matrixId < 30; matrixId ++ ) { |  |
| matrixSize = (matrixId < 20) ? 8 : (matrixId < 26) ? 4 : 2 |  |
| scaling_list_pred_mode_flag[ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ matrixId ] ) { |  |
| scaling_list_pred_matrix_id_delta[ matrixId ] | ue(v) |
| coefNum | ue(v) |
| } else { |  |
| coefNum = matrixSize * matrixSize |  |
| nextCoef = 0 |  |
| if( matrixId < 14 ) { |  |
| scaling_list_dc_coef_minus8[ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef_minus8[ matrixId ] + 8 |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| if( i < coefNum ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| } |  |
| ScalingList[ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

## Step2: prediction with a scale factor

In addition to prediction across all QMs and refinement with a residual, it is proposed to allow prediction to use a scale factor instead of just copy.

Allowing to tune the slope of QMs is expected to meet the need to adjust QM dynamic range across different block types or sizes, which seems to be commonplace. For example, it closely approaches the adaptation between INTRA and INTER observed on default QMs of AVC or HEVC, as show in Figure 3 and Figure 4.

Figure 3. Comparison of intra and inter default AVC matrices.
Left: inter (solid) and intra (grid); Right: inter (solid), intra scaled (grid). Scale = 11/16.

Figure 4. Comparison of intra and inter default HEVC matrices.
Left: inter (solid) and intra (grid); Right: inter (solid), intra scaled (grid). Scale = 12/16.

Scale is expressed in 1/16ths, limited to 0 – 32 range, and coded as an offset from 16, to minimize 16/16 = 1.0 scale coding cost (coded as 0). This is believed to fit needs while keeping implementation simple.

The reference matrix is scaled around 16, which is the neutral value for QMs, as shown in equation below, and the result is clipped to 1 – 255 range.

scaledQM = ( ( refQM – 16 ) * scale + 8 ) >> 4 ) + 16	(3)

Tests indicate significant additional bit savings (-13%), and even more (-26%) when a small error is allowed, which reflects the improved quality of prediction (or approximation capability).

The syntax changes for this step are illustrated below:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( matrixId = 0; matrixId < 30; matrixId ++ ) { |  |
| matrixSize = (matrixId < 20) ? 8 : (matrixId < 26) ? 4 : 2 |  |
| scaling_list_pred_mode_flag[ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ matrixId ] ) { |  |
| scaling_list_pred_matrix_id_delta[ matrixId ] | ue(v) |
| scaling_list_pred_scale_minus16[ matrixId ] | se(v) |
| coefNum | ue(v) |
| } else |  |
| coefNum = matrixSize * matrixSize |  |
| nextCoef = 0 |  |
| if( matrixId < 14 ) { |  |
| scaling_list_dc_coef[ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef[ matrixId ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| if( i < coefNum ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| } |  |
| ScalingList[ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |

## Step3: removal of prediction flag

The prediction flag can be removed because it is possible to emulate the non-prediction case by using zero-scale, which makes the prediction flat (but all-16 instead of all-8).

This simplifies the specification and saves 30 bits of prediction flags, at the expense of always transmitting the number of coefficients. Tests indicate a very small gain.

The final syntax is then:

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( matrixId = 0; matrixId < 30; matrixId ++ ) { |  |
| matrixSize = (matrixId < 20) ? 8 : (matrixId < 26) ? 4 : 2 |  |
| scaling_list_pred_mode_flag[ matrixId ] | u(1) |
| if( !scaling_list_pred_mode_flag[ matrixId ] ) { |  |
| scaling_list_pred_matrix_id_delta[ matrixId ] | ue(v) |
| scaling_list_pred_scale_minus16[ matrixId ] | se(v) |
| coefNum | ue(v) |
| } else |  |
| coefNum = matrixSize * matrixSize |  |
| nextCoef = 0 |  |
| if( matrixId < 14 ) { |  |
| scaling_list_dc_coef[ matrixId ] | se(v) |
| nextCoef = scaling_list_dc_coef[ matrixId ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| if( i < coefNum ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| } |  |
| ScalingList[ matrixId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |

# Draft text size

The following table illustrates the amount of text needed to specify the support of QMs in VVC (statistics from Microsoft Word with all changes accepted):

|  | JVET-O2001-vE | without QMs | Proposed |
| --- | --- | --- | --- |
| Pages | 455 | -5 | +3 |
| Paragraphs | 20781 | -475 | +79 |
| Lines | 29477 | -579 | +216 |
| Words | 233399 | -2691 | +1057 |
| Characters | 1076018 | -10402 | +5332 |

A trend similar to JVET-O0223 is observed: the amount of text is approximately cut by half in the current proposal compared to VVC draft 6. This indicates that additional features (prediction scale and residual) do not add a significant burden.

# Performance tests

To evaluate performance of the proposed coding techniques, a test software has been implemented in the form of GNU Octave scripts, and test matrices (hereafter called “test set”) have been collected and reworked, in a way allowing direct comparison between HEVC, VVC draft 6, and the different steps described in this contribution.

The test bench containing the test matrices is provided in “testbench.zip” attached file, and results are available in “JVET-O0110.xlsx”. Impact on BD-rate can be evaluated by entering bit count values in “JVET-O0110-bdrate.xlsx”.

## Test set description

### HEVC test set

Test matrices used in related JCT-VC quantization core experiments, JCTVC-G1204 and JCTVC-H1104, have been reused, and augmented with other QM sets derived from default or recommended QMs of popular standards (JPEG, MPEG2, AVC, HEVC), and a capture of a DTT AVC broadcast.

Matrices from JCT-VC that are larger than 8x8 have been downscaled to make HEVC-compliant tests (maximum 8x8 coefficients are coded) and copied from smaller size when very little difference was found (i.e. interpolation was used originally to generate the larger size).

For standards specifying only an 8x8 intra matrix (JPEG), 8x8 inter matrix was derived by applying a scale (0.67014) and offset (5.95569) computed from the relation between 8x8 intra and 8x8 inter AVC default matrices.

For missing 4x4 matrices, these were downsampled from 8x8 matrices.

For missing chroma matrices, these were copied from luma, except for DTT-like test.

For DTT-like intra 8x8 chroma: luma matrix up-right diagonals were first averaged to make constant diagonals, then an affine rule was applied (scale 2, same DC, clipping to 80). For DTT-like inter 8x8 chroma: from averaged up-right diagonals, stretched interpolation (pchip) from 4x4 size was applied along the down-right diagonal.

To extend the test matrices to HEVC needs, the missing size-16 and size-32 matrices have been copied from size-8.

Compared to JVET-O0223, a few errors have been fixed: wrong 4x4 matrices for JPEG-like and MPEG2-like QMs, and omitted chroma matrices for JPEG-like QMs. This leads to slightly different test results.

### VVC test set

To extend the test matrices to VVC needs, size-2 matrices have been downsampled from size-4 ones, size-32 matrices have been moved to size-64, and size-32 matrices have been copied from size-16.

## Test bench

A testbench consisting of GNU Octave scripts and the test sets described above is attached to this contribution. Instructions for use are provided in a readme.txt file.

Different coding techniques are compared: HEVC, VVC draft 6, JVET-O0223, and the 3 additional steps of the current proposal described in §2.2, §2.3 and §2.4. An error tolerance feature has been implemented for all coding techniques so that the approximation capabilities of each technique can be evaluated. An encoding/decoding match feature is provided for verification purpose.

Using default QMs is avoided, so that the results are independent from hypothetical default QMs, that may introduce a bias. The feature is available, though, for anyone wishing to test.

For JVET-O0223 and current proposal techniques, bottom-right quadrant of size-64 QMs are optimized when not used for prediction, as explained in §2.1. Compared to the testbench provided in JVET-O0223, a bug in this feature has been fixed (the results of JVET-O0223 were too optimistic because it was erroneously optimizing the 3 bottom and right quadrants).

## Results

The following figure and tables, extracted from JVET-P0110.xlsx attached, illustrate the number of bits needed to encode scaling_list_data using different methods: HEVC, VVC draft 6, JVET-O0223, and proposed; HEVC uses the HEVC test set (24 matrices per test), and other methods use the VVC test set (30 matrices per test). JVET-P0110-1 is for step 1 described in §2.2, and so on. JVET-P0110 alone or “proposed” means the final step (step 3).

Figure 5: QM bit counts with different coding methods

Table 3: Average QM bit count comparison with different coding methods (lossless)

| ref. | HEVC | VVC draft 6 | JVET-O0223 | JVET-P0110-1 | JVET-P0110-2 |
| --- | --- | --- | --- | --- | --- |
| VVC draft 6 | +27% |  |  |  |  |
| JVET-O0223 | -28% | -43% |  |  |  |
| JVET-P0110-1 | -34% | -48% | -9% |  |  |
| JVET-P0110-2 | -43% | -55% | -21% | -13% |  |
| JVET-P0110-3 | -43% | -55% | -21% | -13% | +0.4% |

Table 4: Average QM bit count comparison with different coding methods (lossy:  MAD <= 1.5)

| ref. | HEVC | VVC draft 6 | JVET-O0223 | JVET-P0110-1 | JVET-P0110-2 |
| --- | --- | --- | --- | --- | --- |
| VVC draft 6 | +26% |  |  |  |  |
| JVET-O0223 | -29% | -44% |  |  |  |
| JVET-P0110-1 | -43% | -55% | -19% |  |  |
| JVET-P0110-2 | -58% | -67% | -40% | -26% |  |
| JVET-P0110-3 | -58% | -67% | -41% | -27% | -0.5% |

For this test set, the proposed technique saves a significant amount of bits (-55%), even when compared to HEVC (-43%) whereas it encodes more matrices. The lossy tests show bit savings of -67% with a maximal per-QM MSE of 1.5 which is thought to be a sweet point. This additional gain indicate that the proposed technique can approach a target at low cost using improved prediction and variable refinement.

The effect of error tolerance and the contribution of each step described in the current proposal are presented in Figure 6. It is observed that the prediction scaling feature takes advantage of a small error. This also illustrates the possible tradeoffs between accuracy and bit cost provided by the proposed technique.

Figure 6: Impact of error tolerance on coding cost (step by step)

Considering the average bit count of example QMs with error tolerance equal to 1.5, if QMs are sent every picture, the current proposal reduces the BDrate impact of QMs in RA from 7.27% to 2.45%, which is a 4.5% gain. If QMs are sent every I-period, the BDrate impact in RA is reduced from 0.14% to 0.05% (0.09% gain).

# Conclusion

In addition to single QM identifier and prediction across all QMs proposed in JVET-O0223, this contribution proposes to add a scale factor to QM prediction and a variable-length residual.

Related draft text size is similar to JVET-O0223 and about half of VVC draft 6, thanks to indexing uniformity and unified derivation for all QMs.

Compared to VVC draft 6, QM bit count is reduced by more than half, and 2/3 if allowing a small error.

The ability to define one QM and derive others with scale, offset, and small adjustments at very low cost is considered a desirable feature, that allows fine-tuned picture-level QM adaptation without hindering the expected benefits (probably not above 5%) by QM coding overhead (an example 7% with VVC draft 6 gets reduced to 2.5% by this proposal, with smaller figures possible).

# Annex: chroma format adaptation

As a side effect of equations (1) and (2) illustrated in following example, the proposed adaptation to 4:2:2 and 4:4:4 formats is different from what is currently adopted. Instead of looking for a matrix that matches chroma block size, the size matching is based on the corresponding (luma) TU size, and coefficients are repeated if needed. This makes the QM design independent from the chroma format.

### Example: chroma block for 8x4 TU (4:4:4 format)

For an 8x4 TU, the selected QM is a size-8 one, where luma QMs have 8x8 coefficients and chroma QMs have 4x4 coefficients. However, the 4:4:4 format needs twice as many rows and columns as 4:2:0 (where chroma TB would then be 4x2). Here, as illustrated on Figure 7, chroma QM columns must be repeated, thus x is right-shifted by 1, but the decimation of rows (because of rectangular shape) can be skipped, thus y not shifted.

Indeed, blkWidth equals 8, blkHeight equals 4, and log2MatrixSize equals 2, hence equation (2) takes the following form:

m[ x ][ y ] = ScalingMatrix[ matrixId ][ i ][ j ]
with i = ( x << 2 ) >> 3, and
j = ( y << 2 ) >> 2

where x = 0..7 and y = 0..3

Figure 7: example rectangular QM for chroma 4:4:4

# References

[JVET-O2001]	B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 6),” JVET-O2001, Gothenburg, SE, July 2019

[JVET-O0223]	P. de Lagrange, F. Le Léannec, E. François, K. Naser, “Non-CE7: Quantization matrices with single identifier and prediction from larger ones,” JVET-NO0223, Gothenburg, SE, July 2019

[JCTVC-G1204]	K. Sato et al., “Description of Core Experiment 4 (CE4): Quantization,” JCTVC-G1204, Geneva, CH, November 2011

[JCTVC-H1104]	K. Sato, M. Zhou, R. Joshi, S. Liu, “Description of Core Experiment 4: Quantization Matrices,” JCTVC-H1104, San Jose, CA, USA, February 2012

# Patent rights declaration

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).