| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0116-v1 |
| --- | --- |

| Title: | AHG8: Scalability - random access |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

It is asserted that the scalability design in VVC version 1 should be as simple as possible as long as main scalability use cases are addressed, while those fancy features that are not necessary for main scalability use cases, whenever they complicates the design and the specification, should not be included in VVC version 1.

This contribution focuses on enabling sufficient random access and layer switching capabilities.

The following aspects are proposed:

- Firstly, it is allowed to have unsynchronized IRAP pictures across layers. This reportedly requires POC handling to make sure POC values of all pictures within an AU are the same. For this purpose, it is allowed to signal POC MSB values for random access point (RAP) pictures, which may be IDR, CRA, or GDR pictures. This reportedly allows a significantly simpler design for POC derivation compared to the mixed use of POC resetting and POC MSB signalling mechanism in SHVC and MV-HEVC. The corresponding POC signalling and derivation are also proposed.

- Secondly, incomplete AUs (i.e., an AU that does not contain a picture for each of the layers in the CVS) are allowed, e.g., to enable different layers have different picture rates. However, for simplicity, it is required that each CVSS AU (i.e., an AU that starts a new CVS) is a complete AU and it is further required that each picture in a CVSS AU is a CLVSS picture.

Note that this still allows unsynchronized IRAP pictures across layers, just that for simplicity, the decoding process starting from an AU containing an IRAP picture in the lowest layer but non-IRAP picture in other layers is not normatively specified. However, it is asserted that as long as the design allows unsynchronized IRAP pictures across layers, the technical benefits of such allowance can still be achieved in practice by implementations.

The proposed specification text changes are included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0116 RA" in that document are for this contribution.

# Introduction

In SHVC and MV-HEVC, mixed RAP and non-RAP pictures within an access unit. However, while the design is flexible, it is also rather complicated. For example, even the general decoding process is specified in SHVC and MV-HEVC using 9 pages of text, in clause F.8.1 of the latest HEVC spec. The text involves the use of the many global variables/lists, including NoClrasOutputFlag, LayerInitializedFlag, LayerResetFlag, FirstPicInLayerDecodedFlag, NumPredictedLayers, IdPredictedLayer, IdDirectRefLayer, NumDirectRefLayers, NumActiveRefLayerPics, NumActiveRefLayerPics0, NumActiveRefLayerPics1, etc. Many other places in SHVC and MV-HEVC for support of mixed RAP and non-RAP pictures within an access unit are also complicated. Another example is the POC derivation process.

# Proposal

Due to the need of having a rather simple scalability design in VVC version 1, a design that is summarized as follows is proposed:

- Firstly, it is allowed to have unsynchronized IRAP pictures across layers. This reportedly requires POC handling to make sure POC values of all pictures within an AU are the same. For this purpose, it is allowed to signal POC MSB values for random access point (RAP) pictures, which may be IDR, CRA, or GDR pictures. This reportedly allows a significantly simpler design for POC derivation compared to the mixed use of POC resetting and POC MSB signalling mechanism in SHVC and MV-HEVC. The corresponding POC signalling and derivation are also proposed.

- Secondly, incomplete AUs (i.e., an AU that does not contain a picture for each of the layers in the CVS) are allowed, e.g., to enable different layers have different picture rates. However, for simplicity, it is required that each CVSS AU (i.e., an AU that starts a new CVS) is a complete AU and it is further required that each picture in a CVSS AU is a CLVSS picture.

Note that this still allows unsynchronized IRAP pictures across layers, just that for simplicity, the decoding process starting from an AU containing an IRAP picture in the lowest layer but non-IRAP picture in other layers is not normatively specified. However, it is asserted that as long as the design allows unsynchronized IRAP pictures across layers, the technical benefits of such allowance can still be achieved in practice by implementations.

# Proposed text changes

The proposed changes text changes are provided below, and included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0116 RA" in that document are for this contribution.

## Definitions

coded video sequence start (CVSS) access unit: An access unit in which there is a layer access unit for each layer in the CVS and the coded picture in each layer access unit is a CLVSS picture.

## SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| poc_msb_in_rap_pics_flag | u(1) |
| if( poc_msb_in_rap_pics_flag > 0 ) |  |
| poc_msb_len_minus1 | ue(v) |
| ... |  |
| } |  |

## Slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| slice_pic_order_cnt_lsb | u(v) |
| if( poc_msb_in_rap_pics_flag && nal_unit_type >= IDR_W_RADL && nal_unit_type <= GDR_NUT ) |  |
| poc_msb_val | u(v) |
| ... |  |
| } |  |

## NAL unit header semantics

...

nal_unit_type specifies the NAL unit type, i.e., the type of RBSP data structure contained in the NAL unit...

...

The value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a layer access unit is referred to as having the same NAL unit type as the coded slice NAL units of the picture or layer access unit.

The value of nal_unit_type shall be the same for all pictures of a CVSS access unit.

....

## Order of access units and association to CVSs

A bitstream conforming to this Specification consists of one or more CVSs.

A CVS consists of one or more access units.

The first access unit of a CVS is a CVSS access unit, wherein each present layer access unit is a CLVSS layer access unit, which is either an IRAP layer access unit with NoIncorrectPicOutputFlag equal to 1 or a GDR layer access unit with NoIncorrectPicOutputFlag equal to 1.

Each CVSS access unit shall have a picture in each of the layers present in the CVS.

It is a requirement of bitstream conformance that, when present, each layer access unit in the next access unit after an access unit that contains an end of sequence NAL unit or an end of bitstream NAL unit shall be an IRAP layer access unit, which may be an IDR layer access unit or a CRA layer access unit, or a GDR layer access unit.

## SPS semantics

...

log2_max_pic_order_cnt_lsb_minus4 specifies the value of the variable MaxPicOrderCntLsb that is used in the decoding process for picture order count as follows:

MaxPicOrderCntLsb = 2( log2_max_pic_order_cnt_lsb_minus4 + 4 )

The value of log2_max_pic_order_cnt_lsb_minus4 shall be in the range of 0 to 12, inclusive.

poc_msb_in_rap_pics_flag equal to 1 specifies that the poc_msb_val syntax element is signalled for pictures referring to the SPS and for which the VCL NAL units have nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive. poc_msb_in_rap_pics_flag equal to 0 specifies that the poc_msb_val syntax element is not signalled for pictures referring to the SPS.

poc_msb_len_minus1 plus 1 specifies the length, in bits, of the poc_msb_val syntax elements, when present. The value of poc_msb_len_minus1 shall be in the range of 0 to 32 − log2_max_pic_order_cnt_lsb_minus4 − 5, inclusive.

...

## Slice header semantics

...

slice_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

poc_msb_val specifies the POC MSB value of the current picture. The length of the syntax element poc_msb_val is poc_msb_len_minus1 + 1 bits.

...

## Decoding process for a coded picture

...

–	PictureOutputFlag is set as follows:

–	If one of the following conditions is true, PictureOutputFlag is set equal to 0:

–	the current picture is a RASL picture and NoIncorrectPicOutputFlag of the associated IRAP picture is equal to 1.

–	gdr_enabled_flag is equal to 1 and the current picture is a GDR picture with NoIncorrectPicOutputFlag equal to 1.

–	gdr_enabled_flag is equal to 1, the current picture is associated with a GDR picture with NoIncorrectPicOutputFlag equal to 1, and PicOrderCntVal of the current picture is less than RpPicOrderCntVal of the associated GDR picture.

–	vps_output_layer_mode is equal to 0 and the current access unit contains a picture that has pic_output_flag equal to 1, has nuh_layer_id nuhLid greater than that of the current picture, and belongs to an output layer (i.e., OutputLayerFlag[ GeneralLayerIdx[ nuhLid ] ] is equal to 1).

–	The current picture does not belong to an output layer. [Ed. Note (YK): This can be expressed by as the following wording based on the OLS signalling in JVET-P0115: OutputLayerFlag[ OlsLayeIdx[ TargetOlsIdx ][ nuh_layer_id ] ] is equal to 0."]

–	vps_output_layer_mode is equal to 2 and OutputLayerFlag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

–	OutputLayerFlag[ OlsLayeIdx[ TargetOlsIdx ][ nuh_layer_id ] ] is equal to 0, i.e., the current picture does not belongs to an output layer of the OLS with OLS index equal to TargetOlsIdx.

–	Otherwise, PictureOutputFlag is set equal to pic_output_flag.

...

## Decoding process for picture order count

Output of this process is PicOrderCntVal, the picture order count of the current picture.

Each coded picture is associated with a picture order count variable, denoted as PicOrderCntVal.

When poc_msb_val is not present for the picture and the current picture is not a CLVSS picture, the variables prevPicOrderCntLsb and prevPicOrderCntMsb are derived as follows:

- Let prevTid0Pic be the previous picture in decoding order that has nuh_layer_id equal to the nuh_layer_id of the current picture and TemporalId equal to 0 and that is not a RASL or RADL picture.

- The variable prevPicOrderCntLsb is set equal to slice_pic_order_cnt_lsb of prevTid0Pic.

- The variable prevPicOrderCntMsb is set equal to PicOrderCntMsb of prevTid0Pic.

The variable PicOrderCntMsb of the current picture is derived as follows:

- If poc_msb_val is present for the picture, PicOrderCntMsb is set equal to poc_msb_val * MaxPicOrderCntLsb.

- Otherwise (poc_msb_val is not present for the picture), if the current picture is a CLVSS picture, PicOrderCntMsb is set equal to 0.

- Otherwise, PicOrderCntMsb is derived as follows:

if( ( slice_pic_order_cnt_lsb < prevPicOrderCntLsb )  &&
		( ( prevPicOrderCntLsb − slice_pic_order_cnt_lsb )  >=  ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb + MaxPicOrderCntLsb	(8-1)
else if( (slice_pic_order_cnt_lsb > prevPicOrderCntLsb )  &&
		( ( slice_pic_order_cnt_lsb − prevPicOrderCntLsb ) > ( MaxPicOrderCntLsb / 2 ) ) )
	PicOrderCntMsb = prevPicOrderCntMsb − MaxPicOrderCntLsb
else
	PicOrderCntMsb = prevPicOrderCntMsb

PicOrderCntVal is derived as follows:

PicOrderCntVal = PicOrderCntMsb + slice_pic_order_cnt_lsb	(8-2)

NOTE 1 – All CLVSS pictures for which poc_msb_val is not present will have PicOrderCntVal equal to slice_pic_order_cnt_lsb since for those pictures PicOrderCntMsb is set equal to 0.

The value of PicOrderCntVal shall be in the range of −231 to 231 − 1, inclusive.

In one CVS, the PicOrderCntVal values for any two coded pictures with the same value of nuh_layer_id shall not be the same.

All pictures in any particular access unit shall have the same value of PicOrderCntVal.

The function PicOrderCnt( picX ) is specified as follows:

PicOrderCnt( picX ) = PicOrderCntVal of the picture picX	(8-3)

The function DiffPicOrderCnt( picA, picB ) is specified as follows:

DiffPicOrderCnt( picA, picB ) = PicOrderCnt( picA ) − PicOrderCnt( picB )	(8-4)

The bitstream shall not contain data that result in values of DiffPicOrderCnt( picA, picB ) used in the decoding process that are not in the range of −215 to 215 − 1, inclusive.

NOTE 2 – Let X be the current picture and Y and Z be two other pictures in the same CVS, Y and Z are considered to be in the same output order direction from X when both DiffPicOrderCnt( X, Y ) and DiffPicOrderCnt( X, Z ) are positive or both are negative.

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).