| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0117-v1 |
| --- | --- |

| Title: | AHG8: Scalability - PTL and decoder capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

It is asserted that the scalability design in VVC version 1 should be as simple as possible as long as main scalability use cases are addressed, while those fancy features that are not necessary for main scalability use cases, whenever they complicates the design and the specification, should not be included in VVC version 1.

This contribution proposes the following aspects regarding profile, tier, level (PTL), and decoder capability for VVC scalability:

- Like in HEVC layered extensions, PTL is specified for each layer individually.

- Unlike in HEVC layered extensions, where PTL is signalled for each layer in the context of an OLS, PTL is signalled for each layer that applies to all OLSs.

  - For each layer, one profile and one tier is signalled or inferred.

  - For each layer, up to two levels are signalled or inferred, one for the case when the layer is an output layer, and the other for the case when the layer is a non-output layer.

- Decoder capability is expressed similarly as in HEVC layered extensions.

The proposed specification text changes are included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0117 PTL" in that document are for this contribution.

# Discussion

In HEVC layered extensions, the profile to which a layer conforms depends on the tools used by the layer and its (direct or indirect) reference layers. Therefore, the profile of a layer would not be different for a layer in different OLSs.

The main factor that determines which tier a layer conforms to is the bit rate of the layer itself, which does not really change across the OLSs containing the layer. Therefore, the profile of a layer would not be different for a layer in different OLSs.

A particular layer may be an output layer in some OLSs and at the same time may be a non-output layer in some other OLSs. The level to which a layer conforms depends on the required decoding capability, required CPB and DPB sizes etc. Given a layer, the main difference between it being an output layer and being a non-output layer is the DPB size, which can affect the level value. Therefore, it would be sufficient to signal at most two level values for any particular layer.

# Proposal

The proposed text changes are provided below, and are also included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0117 PTL" in that document are for this contribution.

## SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| same_nonoutput_level_and_dpb_size_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| if( !same_nonoutput_level_and_dpb_size_flag ) |  |
| profile_tier_level( 0, sps_max_sub_layers_minus1 ) |  |
| ... |  |
| } |  |

## SPS semantics

...

same_nonoutput_level_and_dpb_size_flag equal to 1 specifies that the same set of level and DPB size parameters applies to each layer referring to the SPS regardless of whether the layer is an output layer. same_nonoutput_level_and_dpb_size_flag equal to 0 specifies that two separate sets of level and DPB size parameters apply to each layer referring to the SPS depending on whether the layer is an output layer.

Alternatively, two separate flags, are used, one specifies whether the same set of level parameters applies to each layer referring to the SPS regardless of whether the layer is an output layer, and the other specifies whether the same set of DPB size parameters applies to each layer referring to the SPS regardless of whether the layer is an output layer.

...

## Profile, tier, and level syntax:

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| num_sub_profiles | u(8) |
| for( i = 0; i <= num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

Profile, tier, and level semantics:

A profile_tier_level( ) syntax structure provides level information and, optionally, profile, tier, sub-profile, and general constraints information (denoted as PT information).

When the profile_tier_level( ) syntax structure is included in a DPS, the BitstreamInScope is the entire bitstream that refers to the DPS. When the profile_tier_level( ) syntax structure is included in an SPS, the BitstreamInScope is a CLVS that refers to the SPS.

Each SPS includes one or two profile_tier_level( ) syntax structures. The first profile_tier_level( ) syntax structure in an SPS contains both PT inforrmation and level information. When present, the second profile_tier_level( ) syntax structure in an SPS contains level information only. The PT information in the first profile_tier_level( ) syntax structure in an SPS applies to a layer referring to the SPS regardless of whether the layer is an output layer in an OLS. The level information in the first profile_tier_level( ) syntax structure in an SPS applies to a layer referring to the SPS when the layer is an output layer of an OLS. The level information included in the second profile_tier_level( ) syntax structure, when present, in an SPS applies to a layer referring to the SPS when the layer is a non-output layer of an OLS. When an SPS includes only one profile_tier_level( ) syntax structure, the level information for the layer as a non-output layer is inferred to be the same as that for the layer as an output layer.

general_profile_idc indicates a profile to which BitstreamInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

...

- Annex A

Profiles, tiers and levels

(This annex forms an integral part of this Recommendation | International Standard.)

  - Overview of profiles, tiers and levels

Profiles, tiers and levels specify restrictions on bitstreams and hence limits on the capabilities needed to decode the bitstreams. Profiles, tiers and levels may also be used to indicate interoperability points between individual decoder implementations.

Each profile specifies a subset of algorithmic features and limits that shall be supported by all decoders conforming to that profile.

Each level of a tier specifies a set of limits on the values that may be taken by the syntax elements of this Specification. The same set of tier and level definitions is used with all profiles, but individual implementations may support a different tier and within a tier a different level for each supported profile. For any given profile, a level of a tier generally corresponds to a particular decoder processing load and memory capability.

  - Requirements on video decoder capability

Capabilities of video decoders conforming to this Specification are specified in terms of the ability to decode video streams conforming to the constraints of profiles, tiers and levels specified in this annex and other annexes. When expressing the capabilities of a decoder for a specified profile, the tier and level supported for that profile should also be expressed.

A decoder that conforms to a list ptlList consisting of profile, tier, and level triplets ( Pi, Ti, Li ) for i in the range of 0 to d − 1, inclusive, where d is a positive integer, shall be capable of decoding any OLS containing b layers, where b is less than or equal to d, when the following condition applies:

–	There exists a reordered list, orderedPtlList, of the profile, tier, and level capability triplets ( orderedPj, orderedTj, orderedLj ) of the list ptliList such that all the following conditions apply for each value of j in the range of 0 to b − 1, inclusive, where the variable bLayer[ j ] represents the j-th layer in the OLS:

–	The profile to which bLayer[ j ] is indicated to conform is a compatible profile of the profile orderedPj. [NOTE: For example, the AVC Constrained Baseline profile is a compatible profile of the AVC Baseline profile.]

–	The tier to which bLayer[ j ] is indicated to conform is lower than or equal to the tier orderedTj.

–	The level to which bLayer[ j ] is indicated to conform is lower than or equal to the level orderedLj.

  - Profiles

    - Profile A

For a layer in an OLS in a bitstream, the layer being conforming to Profile A, the subBitstream consisting of the layer and all its (direct and indirect) reference layers shall obey the following constraints:

- All referenced VPSs shall have ....

- All referenced SPSs for layers in subBitstream shall have ....

- ...

...

  - Tiers and levels

...

Each layer with nuh_layer_id equal to currLayerId conforming to a profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in clause C, where "access unit" is used to denote the layer access unit:

- The value of ... shall be ...

...

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).