| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0117-v2 |
| --- | --- |

| Title: | AHG8: Scalability - PTL and decoder capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Email: | ye-kui.wang@futurewei.com |
| Source: | Futurewei Technologies, Inc. |  |  |

_____________________________

# Abstract

It is asserted that the scalability design in VVC version 1 should be as simple as possible as long as main scalability use cases are addressed, while those fancy features that are not necessary for main scalability use cases, whenever they complicates the design and the specification, should not be included in VVC version 1.

This contribution proposes the following aspects regarding profile, tier, level (PTL), and decoder capability for VVC scalability:

- Like in HEVC layered extensions, PTL is specified for each layer individually.

- Unlike in HEVC layered extensions, where PTL is signalled for each layer in the context of an OLS, PTL is signalled for each layer that applies to all OLSs.

  - For each layer, one profile and one tier is signalled or inferred.

  - For each layer, up to two levels are signalled or inferred, one for the case when the layer is an output layer, and the other for the case when the layer is a non-output layer.

- Decoder capability is expressed similarly as in HEVC layered extensions.

The proposed specification text changes are included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0117 PTL" in that document are for this contribution.

In v2 of this contribution, the proposal has been updated according to earlier JVET decisions on scalability, particularly to have PTL to be OLS specific instead of layer specific, and to allow single-layer bitstreams without the need of VPSs.

# Discussion

In HEVC layered extensions, the profile to which a layer conforms depends on the tools used by the layer and its (direct or indirect) reference layers. Therefore, the profile of a layer would not be different for a layer in different OLSs.

The main factor that determines which tier a layer conforms to is the bit rate of the layer itself, which does not really change across the OLSs containing the layer. Therefore, the profile of a layer would not be different for a layer in different OLSs.

A particular layer may be an output layer in some OLSs and at the same time may be a non-output layer in some other OLSs. The level to which a layer conforms depends on the required decoding capability, required CPB and DPB sizes etc. Given a layer, the main difference between it being an output layer and being a non-output layer is the DPB size, which can affect the level value. Therefore, it would be sufficient to signal at most two level values for any particular layer.

# Proposal

The proposed text changes are provided below, and are also included in the accompanying specification text document with change marks. Only those changes tagged with "JVET-P0117 PTL" in that document are for this contribution.

## VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| // OLS syntax |  |
| vps_num_ptls | u(8) |
| for( i = 0; i < vps_num_ptls; i++ ) { |  |
| if( i > 0 ) |  |
| pt_present_flag[ i ] | u(1) |
| if( vps_max_sub_layers_minus1 > 0 && !vps_all_layers_same_num_sub_layers_flag ) |  |
| ptl_max_temporal_id[ i ] | u(3) |
| } |  |
| while( !byte_aligned( ) ) |  |
| vps_ptl_byte_alignment_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i < vps_num_ptls; i++ ) |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) |  |
| ols_ptl_idx[ i ] | u(8) |
| ... |  |
| } |  |

## VPS semantics

...

vps_num_ptls specifies the number of profile_tier_level( ) syntax structures in the VPS.

pt_present_flag[ i ] equal to 1 specifies that profile, tier, and general constraints information are present in the i-th profile_tier_level( ) syntax structure in the VPS. pt_present_flag[ i ] equal to 0 specifies that profile, tier, and general constraints information are not present in the i-th profile_tier_level( ) syntax structure in the VPS. The value of pt_present_flag[ 0 ] is inferred to be equal to 0. When pt_present_flag[ i ] is equal to 0, the profile, tier, and general constraints information for the i-th profile_tier_level( ) syntax structure in the VPS are inferred to be the same as that for the ( i − 1 )-th profile_tier_level( ) syntax structure in the VPS.

ptl_max_temporal_id[ i ] specifies the TemporalId of the highest sub-layer representation for which the level information is present in the i-th profile_tier_level( ) syntax structure in the VPS. The value of ptl_max_temporal_id[ i ] shall be in the range of 0 to vps_max_sub_layers_minus1, inclusive. When vps_max_sub_layers_minus1 is equal to 0, the value of ptl_max_temporal_id[ i ] is inferred to be equal to 0. When vps_max_sub_layers_minus1 is greater than 0 and vps_all_layers_same_num_sub_layers_flag is equal to 1, the value of ptl_max_temporal_id[ i ] is inferred to be equal to vps_max_sub_layers_minus1.

vps_ptl_byte_alignment_zero_bit shall be equal to 0.

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls − 1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

...

## SPS syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_present_flag | u(1) |
| if( sps_ptl_dpb_present_flag ) |  |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| ... |  |
| } |  |

## SPS semantics

...

sps_video_parameter_set_id, when greater than 0, specifies the value of vps_video_parameter_set_id for the VPS referred to by the SPS. When sps_video_parameter_set_id is equal to 0, the SPS does not refer to a VPS and no VPS is referred to when decoding each CLVS referring to the SPS, and the value of GeneralLayerIdx[ nuh_layer_id ] is inferred to be equal to 0, and the value of vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is inferred to be equal to 1.

When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the SPS referred to by a CLVS with a particluar nuh_layer_id value nuhLayerId shall have nuh_layer_id equal to nuhLayerId.

...

sps_reserved_zero_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for sps_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC.

sps_ptl_dpb_present_flag equal to 1 specifies that a profile_tier_level( ) syntax structure and a dpb_parameters( ) syntax structure are present in the SPS. sps_ptl_dpb_present_flag equal to 0 specifies that no profile_tier_level( ) syntax structure and no dpb_parameters( ) syntax structure are present in the SPS. The value of sps_ptl_dpb_present_flag shall be equal to vps_independent_layer_flag[ nuh_layer_id ].

...

## Profile, tier, and level syntax

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| num_sub_profiles | u(8) |
| for( i = 0; i <= num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sub_layer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sub_layer_level_present_flag[ i ] ) |  |
| sub_layer_level_idc[ i ] | u(8) |
| } |  |

## Profile, tier, and level semantics

A profile_tier_level( ) syntax structure provides level information and, optionally, profile, tier, sub-profile, and general constraints information.

When the profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is the OLS that includes all layers in the entire bitstream that refers to the DPS. When the profile_tier_level( ) syntax structure is included in a VPS, the OlsInScope is one or more OLSs specified by the VPS. When the profile_tier_level( ) syntax structure is included in an SPS, the OlsInScope is the OLS that includes only the layer that is the lowest layer among the layers that refers to the SPS, which shall be an independent layer.

general_profile_idc indicates a profile to which OlsInScope conforms as specified in Annex A. Bitstreams shall not contain values of general_profile_idc other than those specified in Annex A. Other values of general_profile_idc are reserved for future use by ITU-T | ISO/IEC.

...

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).