| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0119-v1 |
| --- | --- |

| Title: | AHG8: On reference picture resampling (RPR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Hendry San Diego, CA, USA | Email: | Jianle.chen@futurewei.com fhendry@futurewei.com |
| Source: | Futurewei Technologies Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to use the cropping window offsets in the reference sample location derivation of reference picture resampling process to avoid the location misalignment of the reference sample block and the current block.

# Proposal

In order to support adaptive picture resolution and spatial scalability, reference picture resampling (RPR) is introduced in VVC Draft 6. In the resampling process, the resolution of output picture is used to calculate scaling factor between the current picture and reference pictures containing the reference block / samples. With the scaling factor, the following process is used to locate the corresponding samples during the interpolation process.

refxSbL = ( ( xSb  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp	(8-755)

refxL = ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 )  >>  8 )
		+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-756)

refySbL = ( ( ySb  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp	(8-757)

refyL = ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *
		( ( vert_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6	(8-758)

Although cropping window offsets are considered in calculation of scaling factor, they are not taken into account in the reference sample location derivation process as mentioned above, which may result in location misalignment of the current block and the reference block. We propose to use the cropping window offsets in the reference sample location derivation process, as follows:

The variable fRefLeftOffset is set equal to the conf_win_left_offset of the reference picture.

The variable fRefTopOffset is set equal to the conf_win_top_offset of the reference picture.

refxSbL = ( ( xSb  <<  4 ) + refMvLX[ 0 ] ) * hori_scale_fp	(8-755)

refxL = ( SubWidthC * fRefLeftOffset << 4 ) + ( ( ( Sign( refxSb ) * ( ( Abs( refxSb ) + 128 ) 
 >>  8 )+ xL * ( ( hori_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6 )	(8-756)

refySbL = ( ( ySb  <<  4 ) + refMvLX[ 1 ] ) * vert_scale_fp	(8-757)

refyL = ( SubHeightC * fRefTopOffset << 4 ) + (  ( ( Sign( refySb ) * ( ( Abs( refySb ) + 128 )  >>  8 ) + yL *( ( vert_scale_fp + 8 )  >>  4 ) ) + 32 )  >>  6 )	(8-758)

# Patent rights declaration(s)

Futurewei Technologies, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).